3GPP TSG-RAN WG4 Meeting #112bis									                     R4- 2415993

Hefei, China, Oct. 14 – 18, 2024

Agenda item:			6.6.3

Source:	Ericsson

Title:	Discussion on the impact on UE RF requirements of fragmented carriers

Document for:	Approval

# Introduction

Fragmented carriers in the same band requiring multiple Rx chains may limit the UE CA capabilities if part of complex band combinations with many DL CCs combined with DL MIMO in fragmented operator spectrum holdings. A new RAN4 study item was approved in the RAN #104 meeting with the aim of investigating solutions to address this.

The objectives of the SI are as follows [1]:

-	Identify methods for reducing the number of UE Rx chains (e.g. from separate RF chains per CC to shared RF chains) needed for single DL band with frequency span ≤ 100 MHz, containing two non-contiguous CCs within a CA combination for the inter-operator co-located scenario, considering:

-	Which RF requirements could be adjusted for the inter-operator co-located BS scenario, e.g. existing UE RF requirements such as ΔRIBNC, ACS and in-band blocking [RAN4];

- Other requirements are not precluded, if identified

-	The ability to semi-statically switch hardware resources (i.e. Rx chains) [RAN4, RAN2 – See note 2];

-	Up to 6 dB DL received power spectral density imbalance between the two non-contiguous CCs [RAN4];

-	Determine a reasonable level for the power spectral density difference between carriers of co-located adjacent channel operators for study [RAN4]

-	Impacts on DL performance [RAN4] ;

-	Means for a UE to inform the network of appropriate CA configuration it can support with adjusted RF requirements [RAN4, RAN2 – See note 2].

NOTE 1:	No RAN1 impact is foreseen

NOTE 2:	RAN2 work, if necessary, will be triggered by RAN4 LS

NOTE 3:	This study starts from single DL band. Sharing RF chain is not considered among inter-band DL carriers.

- Whether and how to apply the conclusions to intra-band component of inter-band CA will be decided after the study on 2CC in a single DL band is completed.

In the last meeting, several agreements were captured in the following WF [2].

#### Issue 2-1-1: Applicability and clarification on the scope

Online agreements:

Agreement:

- The goal of the DL fragmented carrier study is to study the method to enable support of DL non-contiguous 2CC by using a single Rx RF chain

    - The two non-contiguous CCs is still under the current DL non-contiguous CA framework

      - Each fragment is treated as an individual CC

    - The study should be future-proof for the higher order inter/intra-band combinations or additional CC(s) within the same band with fragments within 100MHz.

    - [The scope should be those bands, where the fragments are fully confined within 100MHz, which includes all FDD/SDL bands and TDD bands

      - Consider n2/n25, n3, n7, n66, n41, n39 as the example bands.

        - Provide the details related to gap between fragmented DL blocks]

Topic #3: Impacts on UE RF requirements and DL performance

#### Issue 3-1-4: Means for a UE to inform the network of appropriate CA configuration it can support with adjusted RF requirements

Recommended WF

- Proposal 1: put off the signalling related discussions until the UE performance aspects are clear.

The following sections will discuss the impact on UE RF requirements of fragmented carriers.

# Discussions

## ACS and In-band blocking

To study the requirements for fragmented carriers, we will begin by reviewing the existing ACS and IBB requirements for non-contiguous CA. The current ACS and IBB have been defined based on the worst-case scenario for each carrier and therefore it should hold for various deployment situations.

The current requirements explicitly specify the condition and location where the existing ACS and in-band blocking requirements are applicable to the intra-band non-contiguous CA of two or more sub-blocks with channel bandwidth larger or equal to 5 MHz. The following text was referenced for information:

For the additional requirements for intra-band non-contiguous carrier aggregation of two or more sub-blocks with channel bandwidth larger than or equal to 5 MHz, the existing adjacent channel selectivity requirements, in-band blocking requirements (for each case), and narrow band blocking requirements apply for in-gap tests only if the corresponding interferer frequency offsets with respect to the two measured carriers satisfy the following condition in relation to the sub-block gap size Wgap for at least one of these carriers j = 1,2, so that the interferer frequency position does not change the nature of the core requirement tested:

Wgap ≥ 2∙|FInterferer (offset),j|  – BWChannel(j)

where FInterferer (offset),j for a sub-block with a single component carrier is the interferer frequency offset with respect to carrier j as specified in clause 7.5, clause 7.6.2 and clause 7.6.4 for the respective requirement and BWChannel(j) the channel bandwidth of carrier j. FInterferer (offset),j for a sub-block with two or more contiguous component carriers is the interference frequency offset with respect to the carrier adjacent to the gap is specified in clause 7.5A, 7.6A.2 and 7.6A.3. The interferer frequency offsets for adjacent channel selectivity, each in-band blocking case and narrow- band blocking shall be tested separately with a single in-gap interferer at a time [2].

Figure 1 provides a visual representation of the previously described equation. ACS and In-band blocking (IBB) are located within the gap between two non-contiguous CCs, in which -56 dBm and -44 dBm are the interference powers of IBB in test cases 1 and 2, respectively.

Figure 1. Visual representation of the equation.

Consider n25 in Toronto as an example of an asymmetric fragmented spectrum [5]. For single operator or two operators coordinated via RAN sharing or spectrum pooling. CC1: 1945-1970 MHz (25 MHz bandwidth), Gap: 1970-1980 MHz (10 MHz gap, potentially used by another operator or legacy system), CC2: 1980-1995 MHz (15 MHz bandwidth).

Figure 2. An example band combination of n25 in Toronto.

For the case in Figure 2, the ACS requirements apply but not the in-band blocking requirements, as according to the equations above, the ACS and in-gap IBB requirements are only defined and applicable if the gap width is large enough. A UE vendor can implement this configuration with a single Rx chain if the existing requirement is met: 30 dB for the 15 MHz carrier and 26 dB for the 25 MHz carrier. Should relaxed ACS conditions be specified for the gap to facilitate a single Rx configuration further, then this would require additional BC indication to distinguish this from a “standard” BC for which the existing requirements are met. Relaxed selectivity might not be accepted for all deployments. Configuration of the BC is then up to the network (i.e. the operator).

We note that the existing non-CA ACS requirements can be met with a good margin.

Relaxing the ACS and in/band blocking requirements could lead to:

- Increasing interference between adjacent fragments potentially degrades overall network performance and user experience.

- Furthermore, existing UEs are designed to meet current requirements. Relaxing standards could create compatibility issues with the existing device ecosystem.

Rather than relaxing the RF minimum requirements for deploying the fragmented carriers, the focus should be on innovative receiver designs based on the existing requirements, leaving the implementation up to the UE vendor.

- Existing ACS and in-band blocking requirements are based on worst-case scenarios for each carrier and apply to various deployment scenarios.

- Relaxing ACS and in-band blocking requirements could create compatibility issues with the existing device ecosystem.

- Maintaining the current ACS and IBB requirements is important to ensure UE selectivity. Implementing non-contiguous intra-band carriers with a single Rx chain is up to UE implementation.

# Conclusion

In the previous sections, we made the following observations:

Observation 1	Existing ACS and in-band blocking requirements are based on worst-case scenarios for each carrier and apply to various deployment scenarios.

Observation 2	Relaxing ACS and in-band blocking requirements could create compatibility issues with the existing device ecosystem.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Maintaining the current ACS and IBB requirements is important to ensure UE selectivity. Implementing non-contiguous intra-band carriers with a single Rx chain is up to UE implementation.

# References

- RP-241669, Study on NR FR1 DL Fragmented Carriers

- R4-2414457, WF on fragmented DL carriers study

- RP-180557, LS response on work to support IMT-2020/5G in the Transport Network

- TS. 38.133 v18.5.0

- TS. 38.101-1 v18.5.0

- RP-240503, Fragmented carriers in the DL