3GPP TSG-RAN WG4 Meeting #112bis	R4-2415997

Hefei, China, 14th – 18th October, 2024

Agenda item:	6.1.1.3.2

Source:            	Google

Title:                	Discussion on MIMO layer for 6Rx UE

Document for:	Approval

# Introduction

In RAN#103, a new WID on RF enhancements for NR FR1/FR2 and EN-DC was approved as one of Release 19 work items [1]. One of the objectives in the WID is to discuss 6Rx requirements for handheld and FWA UEs, which is shown as follows. In this contribution, we will focus on the 6 MIMO layers support for 6Rx handheld and FWA UEs.

6Rx for handheld and FWA UE - Specify the core requirements to enable 6Rx for higher frequency bands (>2.5GHz) targeting at support of handheld UE for NR FR1 single carrier scenario - Example bands: n41, n77/n78, n79, n104 - Support 4 MIMO layers at least, and study the gain and feasibility and if feasible, support 6 MIMO layers - Specify the Rx requirements including reference sensitivity requirements for support 6Rx - Note: the specified requirements can be applicable to both handheld UE and FWA devices - Specify the requirements to support SRS antenna switching including t1r6, t2r6, t3r6, t4r6 depending on UE capability - Study the issue of insertion loss imbalance across SRS ports, and if justified, specify the corresponding solution.

In RAN4#112, the 6 MIMO layers support for 6Rx handheld and FWA UEs has been discussed and some open issues are captured in the approved WF [2]. In WF, the main issue is to discuss whether 6 MIMO layer is supported for 6Rx handheld and FWA UEs. In this paper, we would like to share our view in the following.

# Topic 3: MIMO layer evaluation for 6Rx UE ## Sub-topic 3-1: General considerations for MIMO layer evaluation for 6Rx UE Issue 3-1-1: Tightening BS EVM requirement Agreement: - Do not tighten the minimum requirements of BS Tx EVM - Further discuss the assumption of Tx EVM for 6-layer performance evaluation. Issue 3-1-2: 6-Layer Performance Evaluation Assumptions Way Forward: RAN4 to further discuss and align the 6-Layer MIMO performance evaluation assumptions considering realistic antenna correlation assumptions and deployment scenarios. Issue 3-1-4: Performance requirements for 6Rx Agreement: RAN4 to defer decision after evaluation of 6 MIMO layer performance for handheld and FWA devices. ## Sub-topic 3-2: 6-layer Support Issue 3-2-1: 6-layer Support Way Forward: RAN4 to further discuss the following options. - Option 1: 6-layer support is feasible for handheld and FWA based on some company evaluations considering realistic antenna correlation assumptions and deployment scenarios. - Option 2: 6-layer support is feasible for FWA devices. - Option 3: Companies to align on the feasibility criteria and simulation assumptions to evaluate 6-Layer feasibility using the following as a starting point. - Feasibility criteria - Consider 4-layer vs 6-layer throughput performance. - Simulation assumptions - Only consider link-level simulation for performance evaluations. - Simulation assumptions provided in Annex A as a starting point. Companies to clarify the settings used in the simulation assumptions for BS correlation matrix and Tx EVM. - Option 4: 6-layer support is not feasible for handheld UEs. Issue 3-2-2: 6-layer Support as optional feature Way Forward: RAN4 to further discuss if 6-Layer support should be considered an optional feature.

# Discussion

In 3GPP technology, multiple antennas technique is a key feature to enhance system data DL throughput. In modern commercial implementation, the handheld UE is often designed to be equipped with four Rx antennas when the frequency ranges are around mid/high band (MHB), e.g., n7 or n41 and so on, and ultra-high band (UHB), e.g., n77 or n79, and so on. However, due to physical form factor constraints, the number of multiple antennas cannot be increased without limitation. Hence, in current specification, 4Rx has been introduced for handheld UE and FWA UE from Rel-15 and 8Rx was introduced only for FWA UE in Rel-18.

In RAN4#112, MIMO layer evaluation for 6Rx handheld and FWA UE has been discussed, and some open issues for 6Rx UE with 6 MIMO layer are captured in the WF. One of the open issues is to discuss whether 6 MIMO layer should be introduced for handheld UE and FWA UE. Several options about whether it is feasible to support six MIMO layer for handheld UE and/or FWA UE are listed in the WF. Option 1 is that 6 MIMO layer is feasible for handheld UE and FWA UE, and Option 2 is that 6 MIMO layer is only feasible for FWA UE. Option 3 is that simulation assumption for 6 MIMO layer should be aligned, and Option 4 is that 6 MIMO layer is not feasible for handheld UE.

In our understanding, from commercial handheld UE design perspective, it is beneficial for the high-end handheld UE to be equipped with physical antennas as many as possible. The reason is in order to accommodate as many RF frequency bands as possible to meet as many operator’s requests from different region as possible. Hence, if the handheld UE would like to support 6Rx, it would be challenging that there is very limited space which can be allocated for two additional new antennas without impact to overall legacy antennas performance.

In other way, if the handheld UE would like to support 6Rx by performing antenna sharing between legacy antennas and two new additional antennas, it would also be challenging to guarantee that the legacy antenna performance would not be degraded too much because these legacy antennas need to cover very large frequency range. In addition, in the same form factor, the distance among each 6Rx antenna may also be decreased, and the antenna correlation may become higher which may cause unacceptable antenna performance.

One may question that the foldable phone may have large physical size which can highly reduce antenna correlation for 6Rx handheld UE. However, we think that it may not be always true for foldable phone in all scenarios. For example, if the foldable UE is in the unfolded/open mode, the larger distance among 6Rx antennas would make sense to reduce antenna correlation. However, if the foldable UE is in the folded/close mode, the antenna correlation may not be reduced and it may cause even higher antenna correlation than non-foldable handheld UE depending on different antenna design for the handheld foldable UE. The one possible design is that 6Rx is allocated as far as each other in the open mode, and some Rx antennas may be very close and/or overlapped which cause even higher antenna correlation than non-foldable handheld UE. The other possibility is that one of the four side metal frames in the close mode are often used to support mechanical operation from open mode to close mode operation for the foldable phones. For this one side metal frame in the close mode, it is rarely designed to accommodate any antenna and only other three side metal frames can be used to cover the overall antenna design. In this case, the strategy is different from the handheld non-foldable phone antenna design that all four metal frames can be designed to allocate antennas. Hence, we think it is straightforward that the antenna correlation for 6Rx UE may not be reduced for the foldable phone in the close mode because less antenna space can be allocated to avoid antenna overlapping.

Observation 1: If the foldable handheld UE is in the folded/close mode, the antenna correlation may not be reduced because some Rx antennas may be very close and/or overlapped or less antenna space can be allocated to avoid antenna overlapping, which may cause even higher antenna correlation than non-foldable handheld UE.

In our view, as the above discussion, it is straightforward that the antenna implementation complexity is very high for the handheld UE to be equipped with 6Rx. In addition, the antenna correlation would be much higher for 6Rx handheld UE than 4Rx handheld UE. Considering that 6 MIMO layer may suffer more performance degradation from higher antenna correlation than 4 MIMO layers, it is proposed not to introduce 6 MIMO layer for 6Rx handheld UE in Rel-19. In addition, if RAN4 consensus is to support 6 MIMO layer for 6Rx UE, it is proposed that that 6 MIMO layer for 6Rx UE should be optional feature from Rel-19.

Proposal 1: For the handheld UE equipped with 6Rx, considering that 6 MIMO layer may suffer more performance degradation from higher antenna correlation than 4 MIMO layers and that the antenna implementation complexity is very high, it is proposed not to introduce 6 MIMO layer for 6Rx handheld UE in Rel-19.

Proposal 2: If RAN4 consensus is to support 6 MIMO layer for 6Rx UE, it is proposed that 6 MIMO layer with 6Rx UE should be optional feature from Rel-19.

# Conclusion

The proposals in this contribution are summarized in the following.

Observation 1: If the foldable handheld UE is in the folded/close mode, the antenna correlation may not be reduced because some Rx antennas may be very close and/or overlapped or less antenna space can be allocated to avoid antenna overlapping, which may cause even higher antenna correlation than non-foldable handheld UE.

Proposal 1: For the handheld UE equipped with 6Rx, considering that 6 MIMO layer may suffer more performance degradation from higher antenna correlation than 4 MIMO layers and that the antenna implementation complexity is very high, it is proposed not to introduce 6 MIMO layer for 6Rx handheld UE in Rel-19.

Proposal 2: If RAN4 consensus is to support 6 MIMO layer for 6Rx UE, it is proposed that 6 MIMO layer with 6Rx UE should be optional feature from Rel-19.

# Reference

- RP-240828, “New WID: UE RF enhancements for FR1/FR2 and EN-DC, Phase 4”, RAN4 chair (Huawei), RAN#103

- R4-2414285, “WF on 6Rx UE requirements”, AT&T, RAN4#112

- R4-2412928, “Discussion on MIMO layer for 6Rx UE”, Google, RAN4#112