3GPP TSG-RAN WG4 Meeting # 112-bis														R4-2415998

Hefei, China, 14th – 18th October, 2024

Agenda item:			6.19.2

Source:	Samsung

Title:	Views on UE RF requirements of Rel-19 NR MIMO

Document for:	Discussion

1	Introduction

In RAN#105, the WID on Rel-19 NR MIMO Phase 5 was revised to update additional scope aimed at the minimal impact and TU budget as highlighted below [1].

- Specify enhancement to facilitate UE-initiated/event-driven beam management for reducing overhead and/or latency, assuming the unified TCI while leveraging (as much as possible) legacy CSI measurement and reporting configuration frameworks, targeting FR2 and sTRP with intra- and inter-cell beam management - UL signaling content(s) (and procedure(s) as required) for UE-initiated/event-driven beam reporting facilitating fast beam switching - UL signaling medium/container considering the UE-initiated/event-driven nature of the UL transmission, designed primarily for the purpose of beam reporting - Specify CSI support for up to 128 CSI-RS ports, targeting FR1 - Type-I codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks - Type-II codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks, without modifying any codebook parameter other than introducing additional values for the number of ports codebook parameter(s) - Extension of CRI(s)-based CSI reporting (CQI/PMI/RI calculated per CRI for ≥1 CRIs) for hybrid beamforming supporting up to a total of 128 CSI-RS ports across all resources, with up to 32 CSI-RS ports per resource, without new codebook design - SRS port grouping and its association to the two codewords for the 6/8Rx low complexity receiver supporting more than 4 layers, with legacy codebook - No enhancement on codeword-to-layer mapping, DL resource allocation, CSI feedback, and DCI format - Note: Whether to support 6Rx with more than 4 layers is to be decided in RAN4 Rel-19 RF enhancements WI - Specify UE reporting enhancement for CJT deployments under non-ideal synchronization and backhaul, targeting FR1, both FDD and TDD - Inter-TRP time misalignment and frequency/phase offset measurement and reporting, assuming legacy CSI-RS design, with stand-alone aperiodic reporting on PUSCH - Specify non-coherent UL codebook to facilitate 3-antenna-port codebook-based transmissions, enhancement(s) to enable 3T6R SRS antenna switching, as well as UE capability signaling for 3T3R antenna switching and 3-antenna-port non-codebook-based transmissions, without enhancement on UL full power transmission and without enhancement on SRS resource Note: UL full power transmission mode 1 and 2 are not supported. Note: Other than UE capability signaling, no other enhancement is specified for 3T3R SRS antenna switching. - Specify enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios, assuming intra-band intra-DU non-co-located mTRP scenarios, without changing existing cell definition or defining a new cell (e.g. UL-only cell), assuming the Rel-17/18 unified TCI framework and fully reusing the legacy QCL/UL spatial relation rules, targeting FR1 and FR2 - Two closed-loop PC adjustment states for SRS, both separate from PUSCH; and pathloss offset configurations for pathloss calculation to UL TRP(s), when the pathloss RS is from DL sTRP. - Two TAs through reusing Rel-18 specification of two TAs for multi-DCI-based multi-TRP and removing the restriction that coresetPoolIndex needs to be configured, assuming legacy PRACH resources

Since RAN4 also had their first WI discussion at the last meeting, and found that the 3-antenna-port transmissions should be focused to specify its UE RF discussion in Rel-19 MIMO WI, in this contribution, we would first like to check any additional impact from the revised WID on 3Tx, and continue providing our view based on the outcome of the last meeting [2].

2	Discussion

Looking at the updated WID on 3Tx first, it can be drawn two additional goals from the fourth objective of the WI which has the clear guideline as below [1].

- Specify non-coherent UL codebook to facilitate 3-antenna-port codebook-based transmissions, enhancement(s) to enable 3T6R SRS antenna switching, as well as UE capability signaling for 3T3R antenna switching and 3-antenna-port non-codebook-based transmissions, without enhancement on UL full power transmission and without enhancement on SRS resource Note: UL full power transmission mode 1 and 2 are not supported. Note: Other than UE capability signaling, no other enhancement is specified for 3T3R SRS antenna switching.

As marked in red, the first addition to the 3Tx study would be SRS antenna switching including 3T6R and 3T3R. However, as considered in RAN4 Rel-19 UE RF enhancements WI, the requirements to support SRS antenna switching for 3T6R can be specified in the 6Rx discussion under the UE RF WI [3]. Then, for our Rel-19 MIMO WI, no discussion is required to support the SRS antenna switching on 3T6R.

Proposal 1:		No discussion is required to support the SRS antenna switching on 3T6R.

The other scope is to consider 3-antenna-port non-codebook-based transmissions on top of codebook-based transmissions, which enable UE to determine its transmission strategy without relying on a predefined codebook. Generally, it might provide better accuracy in terms of the precoder since it can be determined directly from measurements by the UE rather than from an indicated codebook/TPMI index by the network. However, in our understanding, such precoding determination procedure would not be considered in RAN4 for UE RF requirements. Then, no discussion is required to support the non-codebook-based transmissions for 3Tx.

Proposal 2:		No discussion is required to support the non-codebook-based transmissions for 3Tx.

As mentioned earlier, in RAN4#112, RAN4 had its first meeting of Rel-19 NR MIMO Phase 5, and it was identified that RAN4 first needs to study the impact of 3Tx on UE RF requirements. Some agreements were also made for 3Tx on the scope of Rel-19 discussions as captured in [2]. In this contribution, we would like to focusing on the open issues above.

Issue 2-1: Applicable UE types - Proposals - Option 1: Handheld UE included - Option 2: Non-handheld UE only - Option 3: Depend on condition (e.g., ULFPTx) - Agreement - Both handheld and non-handheld UEs will be considered for UE RF requirements - FFS on whether a single set or two sets of UE RF requirements, e.g., MPR, will be specified. Issue 2-2: Applicable Frequency Ranges - Proposals - Option 1: FR1 only - Option 2: Others - Agreement - Both FR1 and FR2 will be considered for potential UL mTRP requirements - Only FR1 is considered for 3Tx objective Issue 2-3: Power classes and PA configuration for 3Tx UL MIMO - Proposals - Option 1: PC3 and PC2 - Option 2: PC1.5 only - Option 3: Others - Way forward - Companies are encouraged to provide clear view on 3Tx power class and its PA assumptions Issue 2-5: Non ULFPTx mode - Proposals - Option 1: 3-layer with precoding matrix of W=13100010001. - Option 2: Others - Agreement - 3-layer with precoding matrix of W=13100010001. Issue 2-6: ULFPTx mode 0 - Proposals - Option 1: Add both 1-layer and 2-layer for 3Tx with - Option 2: Add 1-layer for 3Tx with W=13100,13010,13001 - Tentative agreement - Take 1-layer as baseline - Further discuss whether 2-layer is needed

Regarding the applicable power class for 3Tx, legacy power classes such as PC3/2/1.5 were discussed for 3Tx which are still on the table. As discussed in the last meeting, the supportive power class for 3Tx UL MIMO should also consider the uplink full power transmission mode, which is assumed for mode 0 only in Rel-19. Then, in consideration of mode 0, PC1.5 (29dBm) cannot be supported at current level of maximum single PA output, e.g., 26dBm. This is because mode 0 also means to support a target power class with a single full rated port, e.g., one of three ports (A/B/C) in Figure 1.

Figure 1: Possible PA configurations for 3Tx

Observation 1:	PC1.5 (29dBm) cannot be supported in case of mode 0.

On the other hand, given that it is optional feature for UE to support the uplink full power transmission (ULFPTx) for UL MIMO and the motivation of 3Tx described in [1], PC1.5 can also be considered for the maximum output power for 3Tx. However, since PC1.5 is not able to be defined as a single power class for 3Tx for mode 0 as mentioned earlier, in our understanding, RAN4 can consider both PC3 and PC2 first. PC1.5 can be added for the next step as the power class discussion can be independent with uplink full power mode 0.

Proposal 3:		RAN4 can consider both PC3 and PC2 first. PC1.5 can be added for the next step as the power class discussion can be independent with uplink full power mode 0.

For supported MIMO layer for mode 0, it is our understanding that it would be dependent on the supportive power class of 3Tx for mode 0 since one- and/or two-port should have the full rated PA with the maximum output power in the ULFPTx mode 0. In addition, in our understanding, 2-layer can be practically supported as long as a UE supports 3-layer UL MIMO by default, and the rank adaptation in real networks would be varying between layer 1, 2 and 3 rather than 1 and 3 only.

Observation 2:	2-layer can be practically supported as long as a UE supports 3-layer UL MIMO by default.

Therefore, we support the tentative agreement in [2], which is to take 1-layer as a baseline, and have further discussion if 2-layer is needed for mode 0.

Proposal 4:		RAN4 to take 1-layer as a baseline, and have further discussion if 2-layer is needed for mode 0.

Regarding the objective of asymmetric DL sTRP/UL mTRP in [2], RAN4 also had discussed whether it has impact on the UE RF requirement. At the last meeting, one contribution proposed to adopt ‘scenario 2’ for Rel-19 discussion, which assumes all TRPs transmit location information for beam management. However, based on the WID [1] and RAN1 discussion, the main objective on this agenda item is to specify necessary enhancements for supporting UL mTRP deployment scenarios without any DL signal, which is depicted as in Figure 2. TRP1 (i.e., both DL/UL capable TRP) in this scenario can operate both DL transmission and UL reception, while TRP2 (i.e., UL TRP) can only operate UL reception.

Figure 2: Asymmetric DL sTRP/UL mTRP scenario

Basic assumptions of UL TRP are as follows:

-	From UL TRP, there is neither DL control information nor DL RS including pathloss RS (PL RS), hence scheduling/control information for UL TRP could be transmitted from TRP1 (i.e., both DL/UL capable TRP), and pathloss between UL only and a UE can be calculated by PL RS from TRP1, and pathloss offset (PL offset) configuration is additionally considered which is defined as difference between two pathloss values where the 1st pathloss is between TRP1 and UE, and the 2nd pathloss is between TRP2 and UE, respectively.

-	Considering heterogeneous network deployment, i.e., the coverage of TRP1 (i.e., both DL/UL capable TRP) and TRP2 (i.e., UL TRP) could be different (e.g., TRP2 can provide a smaller coverage than TRP1).

Within the assumptions as above, the potential FR2 enhancement topic on asymmetric DL sTRP/UL mTRP scenarios proposed in [4] can be deferred until RAN1 discusses additional scenarios or RAN4 gets concrete proposals on the FR2 enhancements with the existing scenario.

Observation 3:	For asymmetric DL sTRP/UL mTRP scenario, TRP1 (i.e., both DL/UL capable TRP) can operate both DL transmission and UL reception, while TRP2 (i.e., UL TRP) can only operate UL reception.

Proposal 5:		RAN4 to defer the potential FR2 enhancement topic on asymmetric DL sTRP/UL mTRP scenarios.

3	Conclusion

This contribution focuses on updated WID and remaining open issues from the outcome of the last meeting [1, 2]. Following observations and proposals were derived for consideration in this meeting.

Proposal 1:		No discussion is required to support the SRS antenna switching on 3T6R.

Proposal 2:		No discussion is required to support the non-codebook-based transmissions for 3Tx.

Observation 1:	PC1.5 (29dBm) cannot be supported in case of mode 0.

Proposal 3:		RAN4 can consider both PC3 and PC2 first. PC1.5 can be added for the next step as the power class discussion can be independent with uplink full power mode 0.

Observation 2:	2-layer can be practically supported as long as a UE supports 3-layer UL MIMO by default.

Proposal 4:		RAN4 to take 1-layer as a baseline, and have further discussion if 2-layer is needed for mode 0.

Observation 3:	For asymmetric DL sTRP/UL mTRP scenario, TRP1 (i.e., both DL/UL capable TRP) can operate both DL transmission and UL reception, while TRP2 (i.e., UL TRP) can only operate UL reception.

Proposal 5:		RAN4 to defer the potential FR2 enhancement topic on asymmetric DL sTRP/UL mTRP scenarios.

Reference

[1] 	RP-242394,	WID revision: NR MIMO Phase 5, RAN#105

[2] 	R4-2414310,	WF on UE RF requirements for Rel-19 MIMO enhancement, RAN4#112

[3] R4-2410751, 	WF on requirements for 6Rx, RAN4#111

[4] R4-2413224,	On asymmetric connection scenarios for NR MIMO Phase 5, RAN4#112