3GPP TSG-RAN WG4 Meeting #112-bis														R4-2416001

Hefei, China, 14th – 18th October, 2024

Agenda item:			6.2.4.2

Source:	Samsung

Title:	Views on BS and UE parameters for 14800 to 15350 MHz frequency range

Document for:	Discussion

1 Introduction

Based on the LS from ITU-R WP 5D [1], a Study Item was approved in RAN#103 to task RAN4 to study the IMT parameters relevant for sharing and compatibility for the frequency ranges, and to send LS reply to ITU-R WP 5D directly (cc to RAN) [2].

In spite of the challenging schedule of the SI to complete a set of IMT parameters for each of three different frequency ranges as below, RAN4 successfully sent two reply LSs on time based on legacy parameters and companies’ understanding of the importance [3, 4]:

- 4 400-4 800 MHz	Estimated date for completion: May 2024 (RAN WG4#111)

- 7 125-8 400 MHz	Estimated date for completion: August 2024 (RAN WG4#112)

- 14.8-15.35 GHz	Estimated date for completion: November 2024 (RAN WG4#113)

In RAN4#112-bis, thanks to the completion of other ranges [3, 4], RAN4 is supposed to continue the discussion focusing on 15 GHz range based on the options and agreements until the last meeting. In this contribution, we would like to provide our views on overall IMT parameters for both BS and UE based on our initial study and the options from the last meeting, focusing on of the new frequency range of 14800 to 15350 MHz.

2 Discussion

As a background, at its earlier stage of the 15 GHz discussion, it was agreed to conduct coexistence studies in the new frequency range to derive appropriate IMT parameters based on the study outcome.

In addition, on top of the parameters for simulation purpose, RAN4 has discussed them for the LS purpose since multiple options are still captured for some simulation assumptions as agreed in two WFs for 15 GHz BS and UE parameters, respectively [5, 6]. This contribution would start with BS and system part, followed by UE part from practical feasibility perspectives.

2.1 BS and system parameters

For BS parameters, following topics should be highlighted in this meeting, which are trickier with multiple options than others, i.e., antenna array size and system parameters in [5]:

Issue 2-1: BS antenna array and sub-array size - Proposals - Option 1: BS array size as 1536 and the sub-array size as 4 (CATT) - Option 2: At least 1k elements (Qualcomm) - Option 3: 1536=16x24 (Sub Array size 4) per polarization. (Nokia) - Option 4: Two options based on the simulation assumptions (dual polarization): (Ericsson) - 2048 AEs: 8x32 array and 4x1 sub-array - 1536 AEs: 8x16 and 6x1 sub-array - Recommended for further discussions - 3072 AEs dual polarization : - 16x24 array and 4x1 sub-array - 4096 AEs dual polarization: - 8x32 array and 8x1 sub-array - 16x32 array and 4x1 sub-array - 2048 AEs dual polarization: - 8x32 array and 4x1 sub-array - 1536 AEs dual polarization: - 8x16 and 6x1 sub-array - Other options are not precluded. Issue 1-1: Duplex Mode Agreement: - TDD as a baseline. - Suggestion for TR text found in R4-2411521 which can be further discussed Issue 1-2: Channel Bandwidth Agreement: - For LS about 15GHz, choose the single value out of 100MHz, 200MHz or 400MHz as baseline with understanding that the other channel bandwidths are not precluded.

For the potential BS antenna array and sub-array size, in general, such implementation driven parameters could have a consensus in RAN4 based on both legacy products and future trends. However, given the various implementation factors and plans, legacy parameters in the previous two LSs could also be good references to determine antenna related parameters for the new frequency range of 15 GHz. Table 1 summarizes those parameters of we had sent for Urban Macro scenario in [3, 4].

Observation 1:	Given the various implementation factors and plans, legacy parameters in the previous two LSs could also be good references to determine antenna related parameters for 15 GHz.

Table 1: Antenna element and sub-array size for parameters Urban Macro scenario [3, 4]

|  | Antenna element | Sub-array size | Total (dual pol) |
| --- | --- | --- | --- |
| 4400-4800 MHz | 4 x 8 | 3 x 1 | 192 |
| 7125-8400 MHz | 8 x 16 | 3 x 1 | 768 |

Taking into account Table 1 and remaining options on the table in [5], for the both simulation work and LS, we would propose RAN4 to take 1536 or 3072 AEs (dual polarization) with 6x1 sub-array and dual polarization, which are still double and quadruple AEs of 8 GHz, respectively. From our prior studies, we also found that the larger antenna array size does not necessarily guarantee the better performance in terms of the efficiency and coverage at 15 GHz range, which is higher than the previous frequency ranges.

- 1546 AEs: 8 x 16 array and 6x1 sub-array

- 3072 AEs: 8 x 32 array and 6x1 sub-array

Observation 2:	From our prior studies, A larger antenna array size does not necessarily guarantee the better performance in terms of the efficiency and coverage at 15 GHz range, which is higher than the previous frequency ranges.

Proposal 1:		RAN4 to take 1536 or 3072 AEs (dual polarization) with 6x1 sub-array and dual polarization, which are still double and quadruple AEs of 8 GHz, respectively.

Regarding channel bandwidth, as agreed in [5], RAN4 is supposed to choose the single value out of three options for the LS about 15GHz, e.g., 100/200/400 MHz. In our view, given the legacy maximum channel bandwidth of FR1 and FR2, 200 MHz would be an easier option for the frequency range in-between FR1 and FR2. On top of that, considering the potential contiguous spectrum with other legacy services around the range, e.g., fixed and passive satellite services, military uses, etc., available channel bandwidth for single carrier would be limited for larger channel bandwidth than 200 MHz. In this context, we prefer 200 MHz as a nominal channel bandwidth for the frequency range around 15 GHz.

Observation 3:	Potential contiguous spectrum would be limited given other legacy services around the range, e.g., fixed and passive satellite services, military uses, etc.

Proposal 2:		200 MHz could be considered as a nominal channel bandwidth for the frequency range around 15 GHz.

For duplex mode, TDD would be a baseline parameter for duplexing scheme in 15 GHz based on potential spectrum plans as well as existing methods for adjacent frequency ranges. At the same time, SBFD should also be considered as a promising scheme for this frequency range based on the study from Rel-18. In our view, TDD can be sent in the future LS while TR 38.922 captures SBFD as a promising duplex scheme, as mentioned in other frequency ranges during this SI.

Proposal 3:		TDD can be sent in the future LS while TR 38.922 captures SBFD as a promising duplex scheme with other frequency ranges.

2.2 UE antenna parameters

For UE parameters, its antenna and UE type assumption should precede others because it is critical to determine UE operations for both transmission and reception. RAN4 has continued the discussion and reached following options for further discussion and final decision in this meeting [6]:

Issue 3-1: UE Type assumption for comparison of FR1 Vs FR2 like UE performance - FFS the below options for UE types: - Revisit RAN4#111 UE type assumption with the following options - Companies are encouraged to clarify their assumption when they provide simulation results. - FR1-like UE: - Option 1: 1TX assumption with omnidirectional antenna - Option 2: Two discrete elements following Table 1-1 below with maximum-gain across elements selection criterion. - Option 3: For [N] Tx/Rx, [G] dBi gain for link of interest regardless the AoA, and 0dB for signals from all non-serving co-channel and adjacent-channel links regardless the AoA. - Companies to clearly state what value of N and G assumed. - FR2-like UE: - FR2-likUE: Two panels with 1x2 array at each panel following Table 1-2 below with maximum gain across panels critertion. Issue 3-3: UE noise factor for both simulation and LS reply - FFS the below options: - Option 1: 11 dB i.e. follow simulation assumption - Option 2: 8 dB - Option 3: More than 11 dB

As shown in the WF above, there are four options including one option for FR2-like, i.e., three for FR1-like and one for FR2-like. So, it would be beneficial to decide whether to take FR1-like or FR2-like firstly based on potential performance and feasibility.

For FR2-like, as provided in [7], the first question or consideration can be raised whether such analogue beamforming is beneficial even for smaller antenna arrays, e.g., 2x1. For example, given that 1x4 phase array antennas have been assumed for the FR2 requirements from Rel-15, and is widely adopted in FR2 handheld UEs up to now, it could be foreseen that smaller antenna arrays could be used taking into account the limited form factor of the handheld UE. In our view, considering the complexity of beam management, the benefit from using a beam steering antenna system with such a small number of elements would be cancelled, at least for the handheld UE form factor at 15 GHz frequency range.

Observation 4:	For FR2 like UE type, taking the complexity of beam management into account, the benefit from using a beam steering antenna system with such a small number of elements need to be reconsidered.

Also, for the expected number of antenna modules/panels at 15 GHz, it should be noted that it is not even easy to mount single patch antennas on four edges or the backside of the handheld device due to the larger array size than FR2 as shown in [7].

Therefore, in our understanding, it would not make sense for handheld UE to expect the similar uplink and downlink performances when compared with FR2, considering the estimated antenna array size and limited number of antenna modules/panels. In terms of both performance and feasibility, the option for FR2-like UE type with 2x2 or 2x1 can be excluded from the simulation work and LS.

Observation 5:	It would not make sense for handheld UE to expect the similar uplink and downlink performances when compared with FR2, considering the estimated antenna array size and limited number of modules/panels.

Proposal 4:		In terms of both performance and feasibility, the option for FR2-like UE type with 2x2 or 2x1 can be excluded from the simulation work and LS.

Regarding the other options for FR1-like UE type, it has been discussed that, for omnidirectional antennas, how many antennas should be considered for the transmission and reception to achieve better performances in terms of antenna gains, e.g., 5dBi, at the higher frequency range than FR1. Theoretically, at the 15 GHz band, the higher number of antennas than currently used for FR1 could be implemented taking into account the smaller antenna size. However, it should be noted that increasing the number of antennas does not necessarily guarantee the better performance given the higher frequency ranges and the limited form factor. For example, additional antennas will result in the increased coupling among antennas, leading to a degradation in MIMO performance. Also, due to the higher insertion loss per unit length at 15 GHz than FR1, it would be not easy to optimize all the antenna feeding lines to ensure the better performance. Therefore, although we could see some directivity gain due to the higher frequency, 5dBi with more than 4-port transmission cannot be considered for UE antenna parameters at this stage.

Observation 6:	For FR1-like UE type, it should be noted that increasing the number of antennas does not necessarily guarantee the better performance given the higher frequency ranges and the limited form factor.

Proposal 5:		Although we could see some directivity gain due to the higher frequency, 5dBi with more than 4-port transmission cannot be considered for UE antenna parameters at this stage.

Based on observations and proposals above, for UE antenna parameters, our preference would be to take FR1-like UE type with omnidirectional antenna, e.g., 0dBi, as a baseline for the both simulation work and LS. The targeted total antenna gain can be further discussed or refined together with the default Tx number for this frequency range, if necessary.

At the same time, considering 6G or future technologies for 15 GHz, it could be one possible solution for RAN4 to have both options according to the UE type. For example, FWA UE with larger form factors can adopt FR2 like beamforming antennas while the handheld UE with limited form factors can implement FR1 like omnidirectional antenna. Details such as achievable total antenna gain and power level for handheld UE can be further discussed based in Option 1.

Proposal 6:		RAN4 to take FR1-like UE type with omnidirectional antenna, e.g., 0dBi, as a baseline for the both simulation work and LS. The targeted total antenna gain can be further discussed or refined together with the default Tx number for this frequency range, if necessary.

Proposal 7:		It can be considered for RAN4 to have both FR1-like and FR2-like options depending on the UE form factor/size.

2.3 Proposed parameters

As presented above, Table 2 and 3 summarize our preference for IMT parameters at 15 GHz frequency range for both BS and UE based on our study of technical trends and implementation feasibility in practice. Some parameters are marked FFS which might need coexistence results.

Table 2: IMT technology related parameters in 14800 – 15350 MHz

|  |  | IMT |  |  |
| --- | --- | --- | --- | --- |
| No. | Parameter | Base station (non-AAS) (Note 1) | Base station (AAS) | Mobile station |
| 1 | Duplex Method | TDD |  |  |
| 2 | Channel bandwidth (MHz) | 200/[100] MHz typical |  |  |
| 3 | Signal bandwidth (MHz) | Signal bandwidth = NRB × SCS × 12 |  |  |
| 4 | Transmitter characteristics |  |  |  |
| 4.1 | Power dynamic range (dB) | 0 dB |  | 56 dB typical |
| 4.2 | Spectral mask (dB) | FFS | FFS | FFS |
| 4.3 | ACLR | FFS |  | FFS |
| 4.4 | Spurious emissions | FFS |  | FFS |
| 4.5 | Maximum output power | See [1], § 6.2. | See Item No. 1.13 in Table 4 for typical values | 23 dBm |
| 5 | Receiver characteristics |  |  |  |
| 5.1 | Noise figure (dB) | 8 dB (Wide Area BS) 13 dB (Medium Range BS) 16 dB (Local Area BS) For BS class definitions, see [1], § 4.4 |  | 11 dB |
| 5.2 | Sensitivity (dBm) | To be specified | To be specified | To be specified |
| 5.3 | Blocking response | FFS | FFS | FFS |
| 5.4 | ACS | FFS |  | FFS |
| 5.5 | SINR operating range (dB) | See below “SINR operating range and mapping function” |  |  |

Table 3: Beamforming antenna characteristics for IMT in 14800 to 15350 MHz

|  |  | Macro suburban | Macro urban | Small cell outdoor/ Micro urban | Small cell indoor/ Indoor urban |
| --- | --- | --- | --- | --- | --- |
| 1 | Base station Antenna Characteristics |  |  |  |  |
| 1.1 | Antenna pattern | Table 3 |  | Refer to Recommendation ITU-R M.2101 | N/A |
| 1.2 | Element gain (dBi) (Note 2) | 6.4 | 6.4 | 6.4 | N/A |
| 1.3 | Horizontal/vertical 3 dB beam width of single element (degree) | 90º for H 65º for V | 90º for H 65º for V | 90º for H 65º for V | N/A |
| 1.4 | Horizontal/vertical front-to-back ratio (dB) | 30 for both H/V | 30 for both H/V | 30 for both H/V | N/A |
| 1.5 | Antenna polarization | Linear ±45º polarized sub-array | Linear ±45º polarized sub-array | Linear ±45º polarized sub-array | N/A |
| 1.6 | Antenna array configuration (Row × Column) (Note 4) | 8 × 16 or 8 x 32 | 8 × 16 or 8 x 32 | 8 × 8 | N/A |
| 1.7 | Horizontal/Vertical radiating sub-array or element spacing (Note 5) | 0.5 of wavelength for H, 4.2 or 4.8 of wavelength for V | 0.5 of wavelength for H, 4.2 or 4.8 of wavelength for V | 0.5 of wavelength for H, 0.7 of wavelength for V | N/A |
| 1.7a | Number of element rows in sub-array | 6 | 6 | N/A | N/A |
| 1.7b | Vertical element separation in sub-array (dv,sub) | 0.7 or 0.8 of wavelength for V | 0.7 or 0.8 of wavelength for V | N/A | N/A |
| 1.7c | Pre-set sub-array down-tilt (degrees) (Note 6) | 3 | 3 | N/A | N/A |
| 1.8 | Array Ohmic loss (dB) (Note 2) | 2 | 2 | ## 2 | N/A |
| 1.9 | Conducted power (before Ohmic loss) per sub-array or element (dBm) (Note 3) | 19 or 22 | 19 or 22 | 16 | N/A |
| 1.10 | Base station horizontal coverage range (degrees) | ±60 | ±60 | ±60 | N/A |
| 1.11 | Base station vertical coverage range (degrees) (Note 1) | 90-100 | 90-100 | 90-120 | N/A |
| 1.12 | Mechanical down-tilt (degrees) | 6 | 6 | N/A | N/A |
| 1.13 | Base station output power/sector (e.i.r.p.) (dBm) (Note 7) | 81.3 or 84.3 | 81.3 or 84.3 | 61.5 | N/A |

3	Conclusion

This contribution provides following observations and proposals to be considered by this meeting. Table 2 and 3 shall also be taken for further discussion and considerations.

<BS and system parameters>

Observation 1:	Given the various implementation factors and plans, legacy parameters in the previous two LSs could also be good references to determine antenna related parameters for 15 GHz.

Observation 2:	From our prior studies, A larger antenna array size does not necessarily guarantee the better performance in terms of the efficiency and coverage at 15 GHz range, which is higher than the previous frequency ranges.

Proposal 1:		RAN4 to take 1536 or 3072 AEs (dual polarization) with 6x1 sub-array and dual polarization, which are still double and quadruple AEs of 8 GHz, respectively.

Observation 3:	Potential contiguous spectrum would be limited given other legacy services around the range, e.g., fixed and passive satellite services, military uses, etc.

Proposal 2:		200 MHz could be considered as a nominal channel bandwidth for the frequency range around 15 GHz.

Proposal 3:		TDD can be sent in the future LS while TR 38.922 captures SBFD as a promising duplex scheme with other frequency ranges.

<UE antenna parameters>

Observation 4:	For FR2 like UE type, taking the complexity of beam management into account, the benefit from using a beam steering antenna system with such a small number of elements need to be reconsidered.

Observation 5:	It would not make sense for handheld UE to expect the similar uplink and downlink performances when compared with FR2, considering the estimated antenna array size and limited number of modules/panels.

Proposal 4:		In terms of both performance and feasibility, the option for FR2-like UE type with 2x2 or 2x1 can be excluded from the simulation work and LS.

Observation 6:	For FR1-like UE type, it should be noted that increasing the number of antennas does not necessarily guarantee the better performance given the higher frequency ranges and the limited form factor.

Proposal 5:		Although we could see some directivity gain due to the higher frequency, 5dBi with more than 4-port transmission cannot be considered for UE antenna parameters at this stage.

Proposal 6:		RAN4 to take FR1-like UE type with omnidirectional antenna, e.g., 0dBi, as a baseline for the both simulation work and LS. The targeted total antenna gain can be further discussed or refined together with the default Tx number for this frequency range, if necessary.

Proposal 7:		It can be considered for RAN4 to have both FR1-like and FR2-like options depending on the UE form factor/size.

Table 2: IMT technology related parameters in 14800 – 15350 MHz

|  |  | IMT |  |  |
| --- | --- | --- | --- | --- |
| No. | Parameter | Base station (non-AAS) (Note 1) | Base station (AAS) | Mobile station |
| 1 | Duplex Method | TDD |  |  |
| 2 | Channel bandwidth (MHz) | 200/[100] MHz typical |  |  |
| 3 | Signal bandwidth (MHz) | Signal bandwidth = NRB × SCS × 12 |  |  |
| 4 | Transmitter characteristics |  |  |  |
| 4.1 | Power dynamic range (dB) | 0 dB |  | 56 dB typical |
| 4.2 | Spectral mask (dB) | FFS | FFS | FFS |
| 4.3 | ACLR | FFS |  | FFS |
| 4.4 | Spurious emissions | FFS |  | FFS |
| 4.5 | Maximum output power | See [1], § 6.2. | See Item No. 1.13 in Table 4 for typical values | 23 dBm |
| 5 | Receiver characteristics |  |  |  |
| 5.1 | Noise figure (dB) | 8 dB (Wide Area BS) 13 dB (Medium Range BS) 16 dB (Local Area BS) For BS class definitions, see [1], § 4.4 |  | 11 dB |
| 5.2 | Sensitivity (dBm) | To be specified | To be specified | To be specified |
| 5.3 | Blocking response | FFS | FFS | FFS |
| 5.4 | ACS | FFS |  | FFS |
| 5.5 | SINR operating range (dB) | See below “SINR operating range and mapping function” |  |  |

Table 3: Beamforming antenna characteristics for IMT in 14800 to 15350 MHz

|  |  | Macro suburban | Macro urban | Small cell outdoor/ Micro urban | Small cell indoor/ Indoor urban |
| --- | --- | --- | --- | --- | --- |
| 1 | Base station Antenna Characteristics |  |  |  |  |
| 1.1 | Antenna pattern | Table 3 |  | Refer to Recommendation ITU-R M.2101 | N/A |
| 1.2 | Element gain (dBi) (Note 2) | 6.4 | 6.4 | 6.4 | N/A |
| 1.3 | Horizontal/vertical 3 dB beam width of single element (degree) | 90º for H 65º for V | 90º for H 65º for V | 90º for H 65º for V | N/A |
| 1.4 | Horizontal/vertical front-to-back ratio (dB) | 30 for both H/V | 30 for both H/V | 30 for both H/V | N/A |
| 1.5 | Antenna polarization | Linear ±45º polarized sub-array | Linear ±45º polarized sub-array | Linear ±45º polarized sub-array | N/A |
| 1.6 | Antenna array configuration (Row × Column) (Note 4) | 8 × 16 or 8 x 32 | 8 × 16 or 8 x 32 | 8 × 8 | N/A |
| 1.7 | Horizontal/Vertical radiating sub-array or element spacing (Note 5) | 0.5 of wavelength for H, 4.2 or 4.8 of wavelength for V | 0.5 of wavelength for H, 4.2 or 4.8 of wavelength for V | 0.5 of wavelength for H, 0.7 of wavelength for V | N/A |
| 1.7a | Number of element rows in sub-array | 6 | 6 | N/A | N/A |
| 1.7b | Vertical element separation in sub-array (dv,sub) | 0.7 or 0.8 of wavelength for V | 0.7 or 0.8 of wavelength for V | N/A | N/A |
| 1.7c | Pre-set sub-array down-tilt (degrees) (Note 6) | 3 | 3 | N/A | N/A |
| 1.8 | Array Ohmic loss (dB) (Note 2) | 2 | 2 | ## 2 | N/A |
| 1.9 | Conducted power (before Ohmic loss) per sub-array or element (dBm) (Note 3) | 19 or 22 | 19 or 22 | 16 | N/A |
| 1.10 | Base station horizontal coverage range (degrees) | ±60 | ±60 | ±60 | N/A |
| 1.11 | Base station vertical coverage range (degrees) (Note 1) | 90-100 | 90-100 | 90-120 | N/A |
| 1.12 | Mechanical down-tilt (degrees) | 6 | 6 | N/A | N/A |
| 1.13 | Base station output power/sector (e.i.r.p.) (dBm) (Note 7) | 81.3 or 84.3 | 81.3 or 84.3 | 61.5 | N/A |

Reference

[1] R4-2400333	Parameters of terrestrial component of IMT for sharing and compatibility studies in the frequency
 				bands 4 400-4 800 MHz, 7 125-8 400 MHz and 14.8-15.35 GHz

[2]	RP-241326	Study on IMT parameters for 4400 to 4800 MHz, 7125 to 8400 MHz and 14800 to 15350 MHz

[3]	R4-2410576	LS on Parameters for 4400 to 4800 MHz of terrestrial component of IMT for sharing and 
 				compatibility studies in preparation for WRC-27

[4]	R4-2414449	LS Reply on Parameters for 7125 to 8400 MHz of terrestrial component of IMT for sharing and
 				compatibility studies in preparation for WRC-27

[5]	R4-2414283	WF on 15GHz BS parameters

[6]	R4-2414284	WF on 15GHz UE parameters

[7]	R4-2415315	TP on general considerations for the UE antenna parameters for 14800-15350MHz