3GPP TSG-RAN WG4 Meeting # 112-bis														R4-2416003

Hefei, China, 14th – 18th October, 2024

Agenda item:			6.1.1.3.4

Source:	Samsung

Title:	Views on SRS IL imbalance

Document for:	Discussion

1	Introduction

As one of objectives, ”6Rx for handheld and FWA UE” under Rel-19 WI on UE RF enhancements for NR FR1/FR2 and EN-DC (Phase 4) [1], the issue of insertion loss imbalance across SRS ports has been discussed to study whether the corresponding solution is required or not.

However, even though RAN4 is still struggling for the justification, more than 10 solutions for the SRS IL imbalance issue have been proposed, which mostly are the same with theirs from the last release when RAN4 had similar arguments. In our view, RAN4 would repeat the Rel-18 situation until proven the real network performance issue as found in the past meetings. Therefore, in this contribution, we would like to provide our additional view to conclude a unproductive dreary discussion based on our analysis [2, 3] and the last WF [4].

Issue 4-1-1: Whether to solve SRS IL imbalance issue in Rel-19 Way Forward: RAN4 to further discuss the following options. - Option 1: RAN4 should not continue the discussion on how to solve the SRS IL imbalance issue. - Option 2: Continue to pursue a solution to the SRS IL imbalance issue dependent on the outcome of Issue 4-1-2 in the approved WF in R4-2410751. Issue 4-1-2: Initial Considerations for SRS IL imbalance issue Way forward: Given the different views amongst companies, RAN4 to further discuss the set of initial considerations which will allow companies to have a common understanding for the study including existing UE behavior for SRS transmissions in case of SRS IL. - Companies are encouraged to bring analysis on the existing UE behavior and achievable power imbalance for SRS transmissions based on current specification and UE implementations in case of SRS IL. - Companies are encouraged to analyse the impact of SRS IL imbalance on NW performance degradation. Issue 4-2-1: Candidate solutions for the SRS IL imbalance issue Way forward: Companies encouraged to provide compromised solutions for consideration with minimal impact to the specification and to indicate the specific impacts to RAN1, RAN2, and RAN4 specifications and performance gain.

2 Discussion

As mentioned earlier, also in the WID [1], the most important thing is that RAN4 has to identify the issue of the IL imbalance across SRS ports clearly from the real network operations. Otherwise, the meaningless arguments would repeat to find out a solution for the undefined problem as previous discussions in Rel-18. Therefore, RAN4 should discuss and conclude whether the IL imbalance issue needs to be handled in our specifications as early as possible.

Observation 1:	The most important thing is that RAN4 should identify the issue of the IL imbalance across SRS ports clearly from the real network operations. Otherwise, the meaningless arguments would repeat to find out the solution for the undefined problem as previous discussions in Rel-18.

Proposal 1:		RAN4 should discuss and conclude whether the IL imbalance issue needs to be handled in the specification before moving forward to the solutions.

Theoretically, such power imbalance between SRS antenna ports can be an issue itself. It is also possible that the SRS IL imbalance could affect overall uplink and downlink performance due to the low SRS power of Rx antenna ports if single PA based antenna switching is conducted. This would be one motivation from Rel-18 to strive for the solution to compensate the imbalance between the ports. The simulation results might show the issue as they ideally assumed impractical scenarios.

For example, in their simulation without compensation for the SRS IL imbalance across SRS ports, the network may strictly assume to configure lower MCS levels leading to throughput losses due to the artificial levels of IL. On the other hand, for generous compensation with or without UE reporting scheme, such UE might be able to calculate and report the exact IL level dynamically for every port, or the network can perfectly estimate the original channel state based on the self-compensation for the gap between received SRS and the reported values.

In our understanding, the simulation scenario described above has led to the conclusion like “non-negligible performance degradation” in other WG, which is some companies are still referring to. However, according to our prior study, we found that this kind of assumption or conclusion does not apply to real world as we have provided in [2, 3]. As a preliminary check for the potential issue, we have conducted some measurements under the various networks in the field to find out the best implement solution to compensate the imbalance. In summary, we found following facts from the efforts in addition to the measurement results in our previous contribution [2, 3].

[Measurement scenarios] - Network speed and MCS test at 4 different sites/networks - Control Tx power of SRS port 1/2/3 while keeping SRS port 0 as 26 dBm - Check DL/UL throughput per SRS port 1/2/3 power (0, 10, 15, 20 dBm) Figure 1: DL performance by SRS power imbalance [Observations] - DL throughput impact can vary with the network - Some networks show DL throughput degradation due to lower MCS level and MIMO layer at SRS power of 0dBm and 10dBm (more than 16 dB imbalance) - Some networks do not show DL throughput degradation even at 0dBm (26 dB imbalance)

Observation 2:	No performance degradation can be shown from SRS IL imbalance up to 16 dB in real networks.

Observation 3:	Throughput loss is negligible even if 26 dB imbalance gap between SRS port (up to 0.5 dB) in real networks.

Furthermore, in addition to the IL imbalance from the UE, networks are not simply able to acknowledge the SRS power gap if it is from IL imbalance or from others. This is because we also found that the gap is more dependent on the channel fading, hand grip loss, or other issues than IL. The fundamental issue cannot be removed since many other underlying factors like path differences between antennas and LPAF (LNA/PA/Filter/switch) are still there affecting the imbalance even though some IL imbalance related reporting schemes or capabilities are introduced expecting the better network behavior. In this regard, introducing new capability for IL imbalance reporting would be a meaningless solution for improving accuracy of the downlink channel estimation.

Observation 4:	Introducing new capability for IL imbalance reporting would be a meaningless solution for improving accuracy of the downlink channel estimation given the measurement.

Therefore, in our view, the SRS IL imbalance does not affect the practical system performance, nor any enhancement to resolve the issue would work effectively. We do believe that 3GPP should not only add a reporting scheme for an issue, but focus on defining a solution promising any performance enhancement. However, given that RAN4 is now in a deadlock to move forward, and agreed to consider minimal impact to the RAN1, RAN2, and RAN4 specifications and performance gain, it is time for RAN4 to discuss how to make a compromise way based on the agreement. In our view, RAN4 can have further discussion as long as RAN4 strives for the least impact to the legacy specification and UE behaviour, e.g., signaling support only, if required.

Proposal 2:		At the current stage, the SRS IL imbalance does not affect the practical system performance, nor any enhancement to resolve the issue would work effectively.

Proposal 3:		As a compromise, RAN4 can have further discussion as long as RAN4 strives for the least impact to the legacy specification and UE behaviour, e.g., signaling support only, if required.

3	Conclusion

In this contribution, following the direction of the approved WF in the last meeting [4], we summarize the conclusion from the measurements in [2, 3], and provide a possible way for RAN4 at this stage. Following observations and proposals are provided to be considered by this meeting.

Observation 1:	The most important thing is that RAN4 should identify the issue of the IL imbalance across SRS ports clearly from the real network operations. Otherwise, the meaningless arguments would repeat to find out the solution for the undefined problem as previous discussions in Rel-18.

Proposal 1:		RAN4 should discuss and conclude whether the IL imbalance issue needs to be handled in the specification before moving forward to the solutions.

Observation 2:	No performance degradation can be shown from SRS IL imbalance up to 16 dB in real networks.

Observation 3:	Throughput loss is negligible even if 26 dB imbalance gap between SRS port (up to 0.5 dB) in real networks.

Observation 4:	Introducing new capability for IL imbalance reporting would be a meaningless solution for improving accuracy of the downlink channel estimation given the measurement.

Proposal 2:		At the current stage, the SRS IL imbalance does not affect the practical system performance, nor any enhancement to resolve the issue would work effectively.

Proposal 3:		As a compromise, RAN4 can have further discussion as long as RAN4 strives for the least impact to the legacy specification and UE behaviour, e.g., signaling support only, if required.

Reference

[1] RP-241656	New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4

[2] R4-2409460	Views on SRS IL imbalance

[3]	R4-2412136	Views on SRS IL imbalance issue based on performance measurements

[4] R4-2414285	WF on 6Rx UE requirements