3GPP TSG RAN WG4 Meeting #112-bis	R4-2416009

Hefei, China 14 Oct. 2024 – 18 Oct. 2024

Agenda Item:	6.10.4

Source: 	Ericsson

Title:	Further Discussion on Rel-19 ATG CA RRM requirements

Document for:	Discussion

# Introduction

In Rel-18, RAN4 has finalized the air-to-ground (ATG) network work item (WI), emphasizing the FR1 single carrier scenario. During the RAN plenary meeting #103, a new WI [1] was endorsed to enhance the ATG network for NR by supporting carrier aggregation (CA). RAN4#112 meeting was the first meeting where we looked at some of the RRM requirements that may be impacted by introduction of ATG DL CA. The Agreements are listed below from WF [2].

Issue 1-1-1: Scenario Agreement: - Following the WID, only consider FR1 co-located DL intra-band contiguous CA and FR1 co-located DL inter-band CA. - For other ATG scenario characteristics like UE speed, ISD and so on, the R18 scenario captured in TR 38.876 will be reused as baseline. Issue 1-1-4: Whether to support multiple downlink SCells Agreement - Only consider single SCell for R19 ATG CA Issue 1-1-5: UE antenna type Agreement: - For intra-band contiguous CA, same antenna type should be applied on each carrier. - For inter-band CA, either different or same antenna type can be applied on different carriers. - For inter-band CA, consider the following cases: - 1. ATG UE with omnidirectional antennas on both PCell and SCell. - 2. ATG UE with an omnidirectional antenna on PCell and an antenna array on SCell. - 3. ATG UE with an antenna array on PCC and an omnidirectional antenna on SCell. - [4. ATG UE with an antenna array on both PCell and SCell with only one antenna panel.] - 5. ATG UE with an antenna array on both PCell and SCell with separate antenna panels. - Note: If some of the cases will be precluded in RF discussion, they will be removed for RRM requirement discussion. Issue 1-3-2: MRTD Agreement: - For ATG intra-band contiguous CA, not define MRTD requirement. - For ATG inter-band CA, MRTD requirement is: - Option 1: A little larger than 3us, i.e., 3.545 µs - Option 2: 33us - Option 3: 3us - Option 4: further discuss.

This paper discusses our views on the other open issues listed in WF [1] for further discussion.

# Discussion

In this section, we discuss the important open issues for this meeting.

## General Issues

## Co-Location Signaling

In the first meeting of this WI, RAN4#112, it is proposed to study to check whether co-location information in inter-band CA configuration should be provided to UE

Issue 1-1-3: Co-location signaling - Proposal 1: FFS whether co-location information in inter-band CA configuration should be provided to UE.

For intra-band contiguous CA and inter-band CA, we believe we do not need to specifically signal the co-location for ATG, as only the co-located scenario is considered in ATG DL CA case. When an ATG UE configures inter-band CA, it automatically considers co-location by default unless signaled otherwise, which is out of the scope of this WI.

RAN 4 does not need to study and define any co-location information in inter-band CA configuration.

## Signalling for UE Antenna Type

In the WF [2] of RAN4#112, 5 different UE types are agreed. For ATG CA, the UE must conduct a measurement and report the results to the network. This is a new measurement scenario where the UE can perform measurements on different serving cells using different types of antennas. However, the network has no prior knowledge about what type of measurement behaviour the UE will apply, how much time it will take to complete a measurement, and when to expect measurement reporting. Therefore, the network needs to know which type of antenna panels combinations is used to perform measurement when network configures the measurements.

The Network needs to understand which type of antenna panel combination is used at the UE side.

## Specification Structure of ATG CA

In the WF [2], it is proposed that CA-related requirements be added to the sections defined for ATG UE.

Issue 1-1-6: Specification structure - Option 1: The CA related requirements can be directly added in the sections defined for R18 ATG UE - Other Options are not precluded.

we support option 1 as, to the best of our knowledge, this is the structure followed for other features.

The CA related requirements can be directly added in the sections defined for R18 ATG UE

## RRC_IDLE/ RRC_INACTIVE state mobility

## Cell Re-selection

The cell re-selection procedure allows the UE to choose a more suitable cell and camp on it. The NR intra-frequency and inter-frequency carriers specification defines the UE's measurement capability for ATG cell re-selection in clause 4.2D of TS 38.133. However, the measurement capability requirements for ATG in IDLE mode do not consider CA. Therefore, we need to study and define clause 4.2D.2 based on the legacy requirements of clause 4.2.2 to support ATG CA.

RAN4 to reuse NR cell re-selection as a baseline for ATG cell re-selection for legacy UE.

## Measurement Report for Fast CA Setup

ATG UEs in co-located scenarios must perform measurements and report to the network. Early measurement reporting (EMR) saves delays and allows for fast SCell activation in such scenarios. Hence, If RAN4 decides to support EMR while the T331 timer is running, the general requirements for ATG DL CA need to be specified. Due to advancements in cell re-selection measurement techniques, leveraging the Rel-18 capabilities would be more appropriate. Also, Rel-18 resolves issues observed in Rel-16. Therefore, it is recommended that RAN4 consider supporting early measurement reporting for ATG by adopting the Rel-18 NR CA EMR as a baseline.

RAN4 to define EMR for ATG DL CA by using the Rel-18 NR CA EMR as a baseline.

## Timing Requirements

## Maximum Receive Time Difference (MRTD)

According to the WID [1], The scope of the WI is co-located cases. In co-located scenarios, the antennas are generally expected to be co-located within a single site. According to [3], co-location means placing antennas at the same physical location or in proximity, such as on different parts of a rooftop where a gNB represents a logical grouping of functionalities typically implemented within a base station at a single site. This definition is valid in the context of 5G networks when using technologies like MIMO, Tx diversity, and intra-band contiguous carrier aggregation. In this section, we take this definition into consideration for further calculation.

Section 7.6.4 of TS 38.133 defines the minimum requirements for NR CA across different configurations, including Intra-band contiguous CA, Intra-band non-contiguous CA, and Inter-band NR CA. In the context of ATG, the specific deployment conditions may have implications for MRTD requirements.

MRTD can be calculated based on the following formula.

MRTD = TAE + ΔRF_prop + Dispersion

Where,

TAE = Time alignment error defined in 38.104 clause 6.5.3.3 for CA.

ΔRF_prop= is the difference in the time taken for signals to travel from the base station to the receiver across different paths or frequencies.

Dispersion = the spread of signal arrival times at the receiver due to multipath propagation.

Now for the inter-band CA in ATG deployments, where base station sites are co-located and the UE is within 100 meters on an aircraft, the MRTD is calculated as case,

TAE = 3 µs.

ΔRF_prop= 300ns, calculated for the ATG cell edge case, as depicted in Fig 1, where the airplane-to-BS distance is 20km, the cell coverage range is 300km, the flight speed is 1200km/hr, and the distance of the collocated antenna is 100m.

Dispersion = 245ns. In the Rel-15 study report [4], section 6.7.2 the Delay Spread (DS) worst case LOS scenario for 20 degrees or higher elevation has a Delay spread (DS) lgDS = log10(DS/1s) with mlgDS = -7.28 and slgDS = 0.67. This means that mlgDS +slgDS = -7.28 + 0.67 = -6.61 and DS < 0.245 µs with 68 % probability, if we assume Gaussian statistics [4, Table 6.7.2-1a].

So, for inter-band co-located CA

MRTD= 3 µs+ .3 µs+.245 µs= 3.545 µs

The MRTD specification of 33 µs for non-colocated deployments is significantly larger than the co-located ATG MTRD requirement. Hence, RAN4 should further study and define MRTD requirements to ensure they are aligned with co-located ATG deployments.

Fig 1: ATG in the cell edge case

IN WF [2] it is agreed to discuss the following 4 options in this meeting.

Issue 1-3-2: MRTD Agreement: - For ATG inter-band CA, MRTD requirement is: - Option 1: A little larger than 3us, i.e., 3.545 µs - Option 2: 33us - Option 3: 3us - Option 4: further discuss.

Based on the above calculation, we support option 1.

RAN4 should discuss and define 3.545 µs as MRTD requirements for co-located ATG inter-band CA deployments.

## Maximum transmit timing difference (MTTD)

In general, the maximum transmit timing difference (MTTD) in uplink depends on the maximum receive timing difference (MRTD) allowed in the DL. As from the previous section, it is evident that we may need to revise the MRTD value for ATG inter-band Co-located CA; we need to check the MTTD of ATG inter-band Co-located CA as well.

RAN4 should discuss and decide to study MTTD requirements for ATG inter-band Co-located CA.

As we agreed in the last meeting, the MRTD requirement for ATG intra-band contiguous CA is not defined, and we do not expect any changes on the propagation path due to the co-location case; RAN4 can agree that ATG intra-band contiguous co-located CA, MTTD requirement is not defined.

RAN4 should discuss and decide MTTD baseline requirements for NR can be reused for ATG intra-band contiguous co-located CA.

## Signaling Characteristics

## SCell Activation/Deactivation

The TN Scell activation requirements are defined for standalone NR carrier aggregation in FR1. This includes known and unknown Scells, intra-band, and inter-band serving cells. The procedure and requirements provide a complete framework covering possible ATC DL CA scenarios. The complete procedures are captured in releases 15 to 18. Therefore, the Scell activation requirements for NR CA can be reused as a baseline for ATG DL CA. RAN4 should consider all requirements from releases 15 to 18.

RAN4 to define ATG Scell activation requirements based on current SCell activation requirements from rel-15 to rel-18. Such as:

- MAC-CE based Scell Activation

- RRC based Scell Activation

- Fast Scell Activation

- PUCCH Scell Activation

- MAC-CE with L3-reporting during SCell activation

Table 1: Proposed Scenarios in which there is always a co-located active serving cell

| # Case | # To-be-activated target SCell | # Rules |
| --- | --- | --- |
| # #1 | # FR1 known Scell with meas. period <= 2400ms | # Known, measurement period |
| # #2 | # FR1 known Scell with meas. period > 2400ms |  |
| # #3 | # FR1 unknown Scell with active serving cell(s) on the same band | # Unknown, with ssb-PositionInBurst, TCI indication |
| # #4 | # FR1 with active serving cell(s) on the same band | # SSB-less |
| # #5 | # FR2 with active serving cell(s) on the same band with configured SMTC | # Active serving cell in the same band(SSB-less) |
| # #6 | # FR2 with active serving cell(s) on the same band but without configured SMTC, and the RS(s) of Scell being activated is QCL-TypeD with RS of one active serving cell on that FR2 band |  |

Based on the WID[1] we always have a co-located PCell with SCell. Hence, we propose to consider ATG SCell activation based on the scenarios in which there is always a co-located active serving cell. The following Scenarios listed in Table 1 should be considered in this WI.

RAN4 to define ATG SCell activation down select to the scenarios in which there is always a co-located active serving cell.

#### SCell activation delay

From RAN4#111, Fukuoka meeting agreements, we can see that the RAN4 RF agreed there are no limitations on antenna types for ATG CA. The text from the meeting minutes is copied below:

Sub-topic 3-1 antenna type for inter-band CA Issue 3-1: clarify the antenna type for each band for inter-band CA Agreement: - No limitation on antenna types for ATG CA - FFS on whether to assume omni-antenna type can be assumed for both band n3 and n39 in DL CA_n3-n39 - FFS on whether new capability is needed for ATG CA

In accordance with the agreement, we agreed for inter-band CA to consider the following cases:

1. ATG UE with omnidirectional antennas on both PCell and SCell.

2. ATG UE with an omnidirectional antenna on PCell and an antenna array on SCell.

3. ATG UE with an antenna array on PCC and an omnidirectional antenna on SCell.

[4. ATG UE with an antenna array on both PCell and SCell with only one antenna panel.]

5. ATG UE with an antenna array on both PCell and SCell with separate antenna panels.

To better understand the impact of different antenna types on the SCell activation requirement, let's examine the SCell activation time requirements.

Upon receiving SCell activation command in slot n, the UE shall be capable to transmit valid CSI report and apply actions related to the activation command for the SCell being activated no later than in slot n+THARQ+Tactivation_time+TCSI_ReportingNR slot length , where: THARQ (in ms) is the timing between DL data transmission and acknowledgement as specified in TS 38.213 [3] Tactivation_time is the SCell activation delay in millisecond. If the SCell is known and belongs to FR1, Tactivation_time is: - TFirstSSB+ 5ms, if the measurement period of the SCell being activated is equal to or smaller than 2400ms. - TFirstSSB_MAX + Trs + 5ms, if the measurement period of the SCell being activated is larger than 2400ms. If the SCell is unknown and belongs to FR1, and if one of the following conditions is met - ‘ssb-PositionInBurst’ indicates only one SSB is being actually transmitted, or - ‘ssb-PositionInBurst’ indicates multiple SSBs and TCI indication is provided in same MAC PDU with SCell activation, provided that the side condition Ês/Iot ≥ -2dB is fulfilled, Tactivation_time is: - If UE supports shortMeasInterval-r18, then - TFirstSSB_MAX, enhanced + TSMTC_MAX, enhanced + Trs, enhanced + 5ms, if the following conditions are met, - the SCell is contiguous to an active serving cell in the same band, and - its ssb-PositionInBurst is same as the one of contiguous FR1 active serving cell, and - its SMTC offset is same as the one of contiguous FR1 active serving cell, and - its RTD with contiguous FR1 active serving cell is smaller than or equal to 260ns with respect to the to-be-activated SCell’s SSB numerology, and its reception power difference with contiguous FR1 active serving cell is smaller than or equal to 6dB; - TFirstSSB_MAX, enhanced + TSMTC_MAX, enhanced + 2*Trs, enhanced + 5ms, otherwise. - Otherwise - TFirstSSB_MAX + TSMTC_MAX + Trs + 5ms, if the following conditions are met, - the SCell is contiguous to an active serving cell in the same band, and - its ssb-PositionInBurst is same as the one of contiguous FR1 active serving cell, and - its SMTC offset is same as the one of contiguous FR1 active serving cell, and - its RTD with contiguous FR1 active serving cell is smaller than or equal to 260ns, and its reception power difference with contiguous FR1 active serving cell is smaller than or equal to 6dB; - TFirstSSB_MAX + TSMTC_MAX + 2*Trs + 5ms, otherwise. otherwise, provided that the side condition Ês/Iot ≥ -2dB is fulfilled, Tactivation_time is: - If UE supports shortMeasInterval-r18, then - 6ms + TFirstSSB_MAX, enhanced + TSMTC_MAX, enhanced + Trs, enhanced + TL1-RSRP, enhanced_measure + TL1-RSRP, report + THARQ + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP), if semi-persistent CSI-RS is used for CSI reporting, - 3ms + TFirstSSB_MAX, enhanced + TSMTC_MAX, enhanced + Trs, enhanced + TL1-RSRP, enhanced_measure + TL1-RSRP ,report + max(THARQ + Tuncertainty_MAC + 5ms + TFineTiming, Tuncertainty_RRC + TRRC_delay), if periodic CSI-RS is used for CSI reporting. - Otherwise - 6ms + TFirstSSB_MAX + TSMTC_MAX + Trs + TL1-RSRP, measure + TL1-RSRP,report + THARQ + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP), if semi-persistent CSI-RS is used for CSI reporting, - 3ms + TFirstSSB_MAX + TSMTC_MAX + Trs + TL1-RSRP, measure + TL1-RSRP,report + max(THARQ + Tuncertainty_MAC + 5ms + TFineTiming, Tuncertainty_RRC + TRRC_delay), if periodic CSI-RS is used for CSI reporting. However, when the following conditions are fulfilled, no activation requirement will be applied for this unknown SCell: - the SCell is contiguous to an active serving cell in the same band, and - A single SSB is used in the unknown SCell; or multiple SSBs are used in the SCell and TCI state indication for PDCCH is provided by the same MAC PDU used for SCell activation; and - its ssb-PositionInBurst is same as the one of contiguous FR1 active serving cell, and - its SMTC offset is same as the one of contiguous FR1 active serving cell - its RTD with contiguous FR1 active serving cell is larger than 260ns, or its reception power difference with contiguous FR1 active serving cell is larger than 6dB; If the SCell being activated belongs to FR1 and if there is at least one active serving cell contiguous to the SCell on that FR1 band, if the UE is not provided with SSB configuration (absoluteFrequencySSB) nor SMTC configuration for the target SCell, Tactivation_time is 3 ms for UE supporting scellWithoutSSB, provided - The RTD between the target SCell and the contiguous active serving cell is within within ±260ns, and - The difference of the reception power with the contiguous active serving cell is <= 6dB, and - The RS(s) of SCell being activated is (are) QCL-TypeA with TRS(s) of the SCell being activated, and the TRS(s) of the SCell being activated is (are) further QCL-TypeC with SSB(s) of any active serving cell that is contiguous to the SCell being activated on that FR1 band. For a UE supporting scellWithoutSSB-InterBandCA-r18if the SCell being activated belongs to FR1 and if the UE is not provided with SSB configuration (absoluteFrequencySSB) in the target SCell (FrequencyInfoDL) nor SMTC configuration for the target SCell, and if there is one collocated active reference serving cell on different FR1 band, when the following conditions are fulfilled, - The RTD between the target SCell and the collocated reference serving cell is within CP where CP is corresponding to the SCS of SSB-less SCell, and - The [EPRE] difference at the UE is smaller than or equal to [12] dB, where, [EPRE] difference is the power difference between TRS/A-TRS symbol on the SSB-less SCell and SSB symbol on the reference serving cell [after the compensation for AGC], and - The RS(s) of the SSB-less SCell being activated is (are) QCL-TypeA with TRS(s) of the SSB-less SCell being activated, and the TRS(s) of the SSB-less SCell being activated is (are) further QCL-TypeC with SSB(s) of an inter-band active serving cell, and the inter-band active serving cell shall be same as the reference serving cell. where the reference serving cell can be indicated by higherlayer parameter referenceCell-r18. If UE is not indicated with referenceCell-r18, the reference serving cell is assumed to be the QCL-typeC source cell if there is only one active QCL-typeC source cell configured. Editor notes: FFS whether and how to capture the wording “after the compensation for AGC”. Tactivation_time is - Tfirst_TRS + TTRS +5 ms, if aperiodic CSI-RS resources are not configured for SCell activation or UE do not support aperiodicCSI-RS-FastScellActivation-r17 - Tfirst_ATRS + Tgap + TATRS + 5 ms if aperiodic CSI-RS resources are configured for Scell activation for UE supporting aperiodicCSI-RS-FastScellActivation-r17

As marked in the boxed text, the activation delay of SCell may vary depending on the combination of different antenna types in PCell and SCell/s. Hence, SCell activation should be further investigated for ATG CA in Rel-19.

SCell activation requirements shall be further investigated for potential impact, considering the band combinations stated in the WID and the UE antenna types.

## Interruption Requirements

As we agreed to to consider the following cases for inter-band CA:

1. ATG UE with omnidirectional antennas on both PCell and SCell.

2. ATG UE with an omnidirectional antenna on PCell and an antenna array on SCell.

3. ATG UE with an antenna array on PCC and an omnidirectional antenna on SCell.

[4. ATG UE with an antenna array on both PCell and SCell with only one antenna panel.]

5. ATG UE with an antenna array on both PCell and SCell with separate antenna panels.

We need to study the impact of this UE antenna pattern on the interruption requirements. We may need to update the following two tables of clause 8.2.2.2.3.

Table 8.2.2.2.2-1: Interruption length X2 for SCell activation/deactivation for inter-band CA NR Slot length (ms) of victim cell; Interruption length X2 (slots). 0; 1; 1. 1; 0.5; 1. 2; 0.25; Both aggressor cell and victim cell are on FR2; 2. Either aggressor cell or victim cell is on FR1; 3. 3; 0.125; Aggressor cell is on FR2; 4. Aggressor cell is on FR1; 5. 5; 0.03125; Aggressor cell is on FR1; 17. 6; 0.015625; Aggressor cell is on FR1; 33. Table 8.2.2.2.2-2: Interruption duration for SCell activation/deactivation for intra-band CA NR Slot length (ms); Interruption length (slots). 0; 1; 1 + TSMTC_duration * Nslotsubframe,μ. 1; 0.5; 1 + TSMTC_duration * Nslotsubframe,μ. 2; 0.25; 2 + TSMTC_duration * Nslotsubframe,μ. 3; 0.125; 4 + TSMTC_duration * Nslotsubframe,μ. 5; 0.03125; 16+ TSMTC_duration * Nslotsubframe,μ. 6; 0.015625; 32+ TSMTC_duration * Nslotsubframe,μ. NOTE 1: TSMTC_duration measured in subframes is - the longest SMTC duration among all above active serving cells and the SCell being activated when one SCell is activated. If SSB configuration (absoluteFrequencySSB) but no SMTC configuration is provided for the SCell being activated, the SSB transmission periodicity is assumed to be 5ms and TSMTC duration for the SCell being activated is x ms, where x = the number of consecutive subframes containing all SSBs in one SSB burst transmitted by the SCell being activated. If no SSB configuration (absoluteFrequencySSB) nor SMTC configuration is provided for the SCell being activated, TSMTC duration for the SCell being activated is 0ms; - the longest SMTC duration among all active servingNOTE 2: Nslotsubframe,μ is as defined in TS 38.211 [6]..

Interruption requirements shall be further investigated for potential impact, considering the band combinations stated in the WID and the UE antenna types.

## Active BWP switch delay on multiple CCs

The legacy Active BWP switch delay requirements are specified in clause 8.6 of TS 38.133. As per our understanding, legacy NR requirements can be reused for ATG DL CA. Hence, RAN4 should reuse the legacy requirements relevant to support ATG DL CA.

RAN4 should reuse the legacy Active BWP switch delay requirements relevant to support ATG DL CA.

## Beam Failure recovery in SCell

The delay requirement for beam failure recovery in SCell should be considered. The delay requirements of beam failure recovery in SCell can reuse the legacy requirements defined in Rel-18 clause 8.5.9 in 38.133.

RAN4 to define delay requirements for beam failure recovery in SCell for ATG CA, and the NR legacy procedure and requirements from Rel-18 can be reused.

## Measurement Procedure

    - Measurement Gap

The measurement gap defined in Rel-18 for NR carrier aggregation should be reused as a baseline for ATG DL CA.

RAN4 to reuse NR CA measurement gap as the baseline for ATG DL CA.

Furthermore, in the Rel-18 ATG single-carrier scenario, the pre-configured measurement gap is supported. RAN4 should also consider supporting the SCell activation/deactivation as an event for triggering pre-configured measurement gap activation/deactivation. The NR legacy requirement can be reused.

RAN4 to Introduce Pre-configured measurement gap activation/deactivation delay requirement upon SCell activation/deactivation, reuse Rel-18 NR legacy requirements as the baseline.

    - Carrier specific scaling factor

In TS 38.133, carrier-specific scaling factor (CSSF) is specified in clause 9.1D.5. RAN4 needs to study this section and accommodate CSSF for ATG DL CA for FR1.

RAN4 to study and accommodate CSSF requirements for ATG DL CA, consider legacy requirements as a baseline.

    - Scheduling Availability/Restrictions

Rel-18 specifies different scheduling availabilities for ATG omnidirectional antennas and antenna arrays. However, as no limitations on antenna types for ATG CA were agreed upon in the RAN4#111 meeting, we need to study the possible impact of different antenna combinations on scheduling availability for ATG CA.

RAN4 to further study the impact of UE antenna patterns on scheduling availability ATG CA.

## Deactivated SCell measurement

In legacy NR specification, we use a deactivated Scell measurement cycle in SCell to achieve the UE power-saving gain, one of NR's primary KPIs. However, from our understanding, power saving is not the primary issue of ATG UE. Hence, Ran4 should study the requirements of deactivated cell measurement.

RAN4 should study the requirements of deactivated cell measurement for ATG DL UE , consider legacy requirements as a baseline..

# Summary and Conclusion

In this contribution, we have analyzed RAN4 aspects for ATG CA and we propose the following:

Proposal 1	RAN 4 does not need to study and define any co-location information in inter-band CA configuration.

Proposal 2	The Network needs to understand which type of antenna panel combination is used at the UE side.

Proposal 3	The CA related requirements can be directly added in the sections defined for R18 ATG UE

Proposal 4	RAN4 to reuse NR cell re-selection as a baseline for ATG cell re-selection for legacy UE.

Proposal 5	RAN4 to define EMR for ATG DL CA by using the Rel-18 NR CA EMR as a baseline.

Proposal 6	RAN4 should discuss and define 3.545 µs as MRTD requirements for co-located ATG inter-band CA deployments.

Proposal 7	RAN4 should discuss and decide to study MTTD requirements for ATG inter-band Co-located CA.

Proposal 8	RAN4 should discuss and decide MTTD baseline requirements for NR can be reused for ATG intra-band contiguous co-located CA.

Proposal 9	RAN4 to define ATG Scell activation requirements based on current SCell activation requirements from rel-15 to rel-18. Such as:

 MAC-CE based Scell Activation

 RRC based Scell Activation

 Fast Scell Activation

 PUCCH Scell Activation

 MAC-CE with L3-reporting during SCell activation

Proposal 10	RAN4 to define ATG SCell activation down select to the scenarios in which there is always a co-located active serving cell.

Proposal 11	SCell activation requirements shall be further investigated for potential impact, considering the band combinations stated in the WID and the UE antenna types.

Proposal 12	Interruption requirements shall be further investigated for potential impact, considering the band combinations stated in the WID and the UE antenna types.

Proposal 13	RAN4 should reuse the legacy Active BWP switch delay requirements relevant to support ATG DL CA.

Proposal 14	RAN4 to define delay requirements for beam failure recovery in SCell for ATG CA, and the NR legacy procedure and requirements from Rel-18 can be reused.

Proposal 15	RAN4 to reuse NR CA measurement gap as the baseline for ATG DL CA.

Proposal 16	RAN4 to Introduce Pre-configured measurement gap activation/deactivation delay requirement upon SCell activation/deactivation, reuse Rel-18 NR legacy requirements as the baseline.

Proposal 17	RAN4 to study and accommodate CSSF requirements for ATG DL CA, consider legacy requirements as a baseline.

Proposal 18	RAN4 to further study the impact of UE antenna patterns on scheduling availability ATG CA.

Proposal 19	RAN4 should study the requirements of deactivated cell measurement for ATG DL UE, consider legacy requirements as a baseline.

# References

- RP- 240839 “New WID on Enhancements for Air-to-ground network for NR,” Apple

- R4-2413959, “WF on RRM requirements for NR_ATG_enh”, CMCC

- 3GPP TS 38.133: "NR; Requirements for support of radio resource management"

- RP-180037, LS response on work to support IMT-2020/5G in the Transport Network

- TR 38.811, Study on New Radio (NR) to support non-terrestrial networks (Release 15)