# 3GPP TSG-RAN WG4 Meeting # 112-bis	R4-2416012

# Hefei, China, Oct.14th – 18th, 2024

# Source: 	ZTE Corporation, Sanechips

# Title: 	On MSD requirements with intra-band contiguous UL CA

# Agenda item:	5.3.3

# Document for:	Approval

# 1 Introduction

In WF[1], the issue on the MSD requirements with intra-band contiguous UL CA was raised, see the proposals below:

Proposal 1: Companies are encouraged to consider the following options for handling the MSD requirements resulting from intra-band contiguous UL CA configured with non-contiguous allocations: Option 1: No change from TR 38.862 guidelines Option 2: Do not consider all MSD requirements resulting from intra-band contiguous UL CA configured with non-contiguous allocations. Option 3: Do not consider only the MSD requirements resulting from intra-band contiguous UL CA configured with 1RB+1RB allocations. Proposal 2: If Option 2 or Option 3 in Proposal 1 would be considered, which release to start taking effect? Option 1: From earliest release where such MSD requirements have been specified. Option 2: Rel-18 Option 3: Rel-19 Proposal 3: Is there a need to introduce cross-band MSD requirements resulting from intra-band contiguous UL CA configured with fully allocated maximum aggregated BW? Option 1: Yes Option 2: No

In last meeting, companies bring contributions [2~7] on the above issue but no consensus were reached. In addition, the MSD for the cross-band isolation MSD caused by the intra-band UL CA are given in [5][6] for CA_n40A-n41C and CA_n41C-n79A, respectively.

In this contribution, we give some further views on the MSD requirements with intra-band contiguous UL CA.

# 2	Discussion

- 2.1 General

- As we stated in [4], the main reason for not considering the intra-band non-contiguous allocation such as 1RB+1RB MSD test point is that from the network scheduling point of view a network will not schedule another carrier and configure UL intra-band CA if one carrier is not fully utilized since it would impact the UL throughput. In the other words, the non-contiguous allocation MSD test point would not be not a popular scenario in the filed, even it does not

- occur in operators’ networks.

- The 1RB+1RB allocation MSD test point is only used in the TDD band as intra-band UL CA assuming MPR requirement is applied to meet the emissions, and several RB+RB allocation MSD test point is only used in the FDD band as intra-band UL CA assuming that the UE is at PCmax and shared with equal PSD among the two carriers.

- In terms of the discussion in last meeting, the views among the companies are still diverse. Some companies still think it is necessary to keep the current 1RB+1RB MSD test point unchanged, but some other companies would like to consider more practical scenarios for the MSD definition.

- Actually, this issue was somehow triggered by CA_n41C-n79A due to >55dB MSD is foreseen without MPR, and ~43dB MSD with MPR assumption, where such high MSD value cause deployment concerns although it is not the first combination whose MSD value is more than 45dB.

- For the more practical scenarios, to guarantee the UL performance, usually the fully allocated maximum aggregated BW would be configured for the intra-band contiguous UL CA band. The 1RB allocation may possibly happen when the UE is moved/located in the cell edge. However, in this case we think it does not make sense for NW configuring 1RB+1RB in each carrier for the inter-band UL CA considering the serious MSD value since it would cause the REFSEN degraded too much which would shrink the DL coverage. Instead the NW should drop the Scell to avoid large MSD.Therefore, we still prefer to re-consider 1RB+1RB allocations if possible, and it should only be for intra-band TDD UL CA only.

- Proposal 1: Re-considered 1RB+1RB RB allocation for intra-band TDD ULCA within an inter-band DL CA if possible.

Regarding the cross-band MSD requirements resulting from intra-band contiguous UL CA configured with fully allocated maximum aggregated BW, it is a new type of MSD and RAN4 never consider it in the past although there are several inter-band UL CA including intra-band CA configurations in the spec.

- Observation 1: In current spec, there are no cross band isolation MSD defined for intra-band contiguous UL CA configured with fully allocated maximum aggregated BW.

- As we discussed in [3], fully allocated maximum aggregated BW is a meaningful scenario in the field for one band supports intra-band UL CA, we think there might be needed define the MSD since the CA ACLR region caused by intra-band contiguous CA (maximum aggregated BW) is larger than single carrier, the CA ACLR region does not overlap with victim DL for single carrier does not imply the CA ACLR region will not overlap with victim DL for intra-band contiguous CA.

- Proposal 2: If the cross-band MSD requirements resulting from intra-band contiguous UL CA configured with fully allocated maximum aggregated BW is considered to be defined:

- - Only to define new cross band isolation MSD for CA ACLR1/ACLR2 interference source

- - To reuse cross band isolation MSD of single carrier for >ACLR2 interference source

- 2.2 Cross-band isolation MSD for n41C to n40

- In this section, we give some discussions on the cross band isolation MSD caused by the intra-band UL n41C to n40, where fully allocated maximum aggregated BW is considered.

- For CA_n40A-n41C, the aggregated channel bandwidth for UL CA_n41C is 160MHz, both CA ACLR1/2 would falls into the n40 Rx range.

- We use the same RF architecture assumption in [5] for the analysis, and some of the RF assumptions as well. By the calculation, we see the MSD dominate factor is the CA ACLR due to the poor rejections between n40 to n41.

- For the 30dB n41 Tx filter rejection at 2300~2400MHz, the MSD is calculated in the table 1 for CA ACLR1 and CA ACLR2, respectively.

- Table 1. MSD for CA_n41C CA ACLR1 to n40

| total interference noise | Main | Diversity |
| --- | --- | --- |
| Total interference noise referred to antenna port (dBm) | -64.5 | -64.5 |
| Noise floor referred to antenna port (dBm) 5MHz | -96.0 |  |
| The degradation of the REFSEN (dB) | 31.5 | 31.5 |
| MSD after MRC (dB) | 31.5 |  |

- Table 1. MSD for CA_n41C CA ACLR2 to n40

| total interference noise | Main | Diversity |
| --- | --- | --- |
| Total interference noise referred to antenna port (dBm) | -74.3 | -74.3 |
| Noise floor referred to antenna port (dBm) 5MHz | -96.0 |  |
| The degradation of the REFSEN (dB) | 21.7 | 21.7 |
| MSD after MRC (dB) | 21.7 |  |

- Observation 2: The cross band isolation MSD caused by fully allocated maximum aggregated BW of UL CA_n41C falls into n40 is 31.5dB and 21.7dB for CA ACLR1 and CA ACLR2, respectively.

- 2.3 Cross-band isolation MSD for n41C to n79

- Since the frequency separation between the upper n41 frequency and the lower n79 frequency is larger than CA ACLR2 regions, we think there is no need to define additional cross band isolation MSD requirements for CA ACLR2 region since the CA ACLR2 region is already within the single carrier >ACLR2 region, which means the cross band isolation MSD of single carrier for >ACLR2 interference source can be re-used.

- In the existing spec, there are no cross band isolation MSD defined for n41A to n79. Thus there is also no need to define additional MSD requirements for CA ACLR2 region of UL CA_n41C.

- Observation 3: There is no need to define additional cross band isolation MSD requirements caused by fully allocated maximum aggregated BW of UL CA_n41C falls into n79.

# 3 Conclusion

In this contribution, we give some discussions on the MSD requirements with intra-band contiguous UL CA, and also the cross band isolation MSD for CA_n40A-n41C and CA_n41CA-n79A . The proposals and conclusions are:

- Proposal 1: Re-considered 1RB+1RB RB allocation for intra-band TDD ULCA within an inter-band DL CA if possible.

- Observation 1: In current spec, there are no cross band isolation MSD defined for intra-band contiguous UL CA configured with fully allocated maximum aggregated BW.

- Proposal 2: If the cross-band MSD requirements resulting from intra-band contiguous UL CA configured with fully allocated maximum aggregated BW is considered to be defined:

- - Only to define new cross band isolation MSD for CA ACLR1/ACLR2 interference source

- - To reuse cross band isolation MSD of single carrier for >ACLR2 interference source

- Observation 2: The cross band isolation MSD caused by fully allocated maximum aggregated BW of UL CA_n41C falls into n40 is 31.5dB and 21.7dB for CA ACLR1 and CA ACLR2, respectively.

- Observation 3: There is no need to define additional cross band isolation MSD requirements caused by fully allocated maximum aggregated BW of UL CA_n41C falls into n79.

# 4	Reference

[1] R4-2406676, WF on MSD requirements with intra-band contiguous UL CA, Apple

[2] R4-2411551, On intra-band ULCA interference versus single ULCC, Skyworks Solutions Inc.

[3] R4-2411738, Discussion on MSD requirements with intra-band contiguous UL CA, CMCC

[4] R4-2411887, On MSD requirements with intra-band contiguous UL CA,ZTE Corporation, Sanechips

[5] R4-2412934, Discussion on MSD for CA_n40A-n41C with intra-band UL CA_n41C, Huawei, HiSilicon

[6] R4-2412935, Discussion on MSD for CA_n41C-n79A with intra-band UL CA_n41C, Huawei, HiSilicon

[7] R4-2413315, On intra-band ULCA interference versus single ULCC, Skyworks Solutions Inc.