3GPP TSG-RAN WG4 Meeting #112bis	R4-2416016

Hefei, China, Oct 14 – Oct 18, 2024

Source:	Nokia

Title:	On NR MIMO Phase 5 UE-initiated/event-driven beam management

Agenda item:	6.19.3.1

Document for:	Discussion

Introduction

MIMO is one of the key technologies in NR systems, and Rel-19 will bring important enhancements to both FR1 and FR2. Among the enhancements to be studied, the following is listed in the WID [1].

- Specify enhancement to facilitate UE-initiated/event-driven beam management for reducing overhead and/or latency, assuming the unified TCI while leveraging (as much as possible) legacy CSI measurement and reporting configuration frameworks, targeting FR2 and sTRP with intra- and inter-cell beam management - UL signaling content(s) (and procedure(s) as required) for UE-initiated/event-driven beam reporting facilitating fast beam switching - UL signaling medium/container considering the UE-initiated/event-driven nature of the UL transmission, designed primarily for the purpose of beam reporting - Specify CSI support for up to 128 CSI-RS ports, targeting FR1 - Type-I codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks - Type-II codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks, without modifying any codebook parameter other than introducing additional values for the number of ports codebook parameter(s) - Extension of CRI(s)-based CSI reporting (CQI/PMI/RI calculated per CRI for ≥1 CRIs) for hybrid beamforming supporting up to a total of 128 CSI-RS ports across all resources, with up to 32 CSI-RS ports per resource, without new codebook design - SRS port grouping and its association to the two codewords for the 6/8Rx low complexity receiver supporting more than 4 layers, with legacy codebook - No enhancement on codeword-to-layer mapping, DL resource allocation, CSI feedback, and DCI format - Note: Whether to support 6Rx with more than 4 layers is to be decided in RAN4 Rel-19 RF enhancements WI - Specify UE reporting enhancement for CJT deployments under non-ideal synchronization and backhaul, targeting FR1, both FDD and TDD - Inter-TRP time misalignment and frequency/phase offset measurement and reporting, assuming legacy CSI-RS design, with stand-alone aperiodic reporting on PUSCH - Specify non-coherent UL codebook to facilitate 3-antenna-port codebook-based transmissions, enhancement(s) to enable 3T6R SRS antenna switching, as well as UE capability signaling for 3T3R antenna switching and 3-antenna-port non-codebook-based transmissions, without enhancement on UL full power transmission and without enhancement on SRS resource Note: UL full power transmission mode 1 and 2 are not supported. Note: Other than UE capability signaling, no other enhancement is specified for 3T3R SRS antenna switching. - Specify enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios, assuming intra-band intra-DU non-co-located mTRP scenarios, without changing existing cell definition or defining a new cell (e.g. UL-only cell), assuming the Rel-17/18 unified TCI framework and fully reusing the legacy QCL/UL spatial relation rules, targeting FR1 and FR2 - Two closed-loop PC adjustment states for SRS, both separate from PUSCH; and pathloss offset configurations for pathloss calculation to UL TRP(s), when the pathloss RS is from DL sTRP. - Two TAs through reusing Rel-18 specification of two TAs for multi-DCI-based multi-TRP and removing the restriction that coresetPoolIndex needs to be configured, assuming legacy PRACH resources

In this document, we present Nokia’s views regarding the core requirements that RAN4 needs to discuss in relation to the WID, in particular about:

- Objective 1: UE-initiated beam management.

Discussion on reporting delay requirements

Regarding UE-initiated beam management (UEIBM) reporting delay requirements, the following was agreed and discussed in the last RAN4#112 meeting [2]:

Issue 2-1-2: Scope of the scenario of event-triggered L1 beam reporting Agreement: - Intra-cell and inter-cell for FR2. - Note: If specific requirements cannot be applicable, discuss it case by case. Issue 2-1-3: Whether/how to define new delay requirements such as measurement reporting delay/measurement period? Agreement: - RAN4 will define Event triggered measurement reporting delay requirement - Further discuss: - Whether to support both mode A and mode B - Whether to differentiate mode A and mode B - Other related aspects

It is worth recalling that RAN1 is discussing two procedures for actually sending back the UEI report:

- Mode A, where the second UL channel for the UEI report is dynamically scheduled by the gNB; in such procedure the following steps are implemented:

  - The UE sends in a first PUCCH channel an UL indication to request to the gNB resources in a second UL channel to carry the UEI report;

  - The gNB indicates via DCI to the UE a resource in a second UL channel to carry the UEI report;

  - The UE sends the UEI report on the second UL channel.

- Mode B, where the second UL channel for the UEI report is pre-configured by the gNB; in such procedure the following steps are implemented:

  - The UE sends in a first PUCCH channel an UL indication to notify to the gNB that a UEI report will be transmitted in a second UL channel;

  - UE sends the UEI report on the second UL channel.

RAN1 support both modes and has defined Mode A as the baseline, whereas Mode B may be supported only by some UEs. We do not see a need to down select between these modes for the RAN4 requirements, and hence the requirements should support both modes.

Define event-triggered L1 reporting delay requirements for both mode A and mode B.

In both modes A and B, after the UE has realized the reporting condition is met, the UE first sends an UL indication in the first PUCCH channel once an event has been triggered. This indication is used in Mode A to request UL resources for the report, whereas in Mode B it simply notifies the gNB that an upcoming preconfigured UL resource will be used for the UEI report.

In Mode A, the delay to send the UEI report in a second UL channel after the UL indication in a first PUCCH channel in our view simply depends on how quickly the network allocate UL resources to a UE. More specifically, such delay depends on the network-configured time domain allocation K2 value and the PUSCH preparation time N2, as defined in TS 38.214.

Regarding Mode B, RAN1 has agreed on the following in the last RAN1#118 meeting:

Agreement (RAN1 #118 Maastricht) On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding Mode-B, UEI beam report is carried on a first available transmission occasion of the second UL channel X symbols after sending the last symbol of report notification on the first PUCCH channel. Agreement (RAN1 #118 Maastricht) On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding resource mapping/configuration between first and second channel in Mode-B, for a given CSI report configuration, the following is provided for down-selection. - Option-1 (one-to-one): Only one first PUCCH resource and only one pre-configured resource for second UL channel can be associated with the CSI report configuration for UE-initiated/event-driven beam reporting. - Option-1A: Same periodicity between first PUCCH resource and pre-configured resource for second UL channel. - Option-1B: No restriction in terms of periodicity. Option-2 (one-to-M): Only one first PUCCH resource and one or more pre-configured resource(s) for second UL channel can be associated with CSI report configuration for UE-initiated/event-driven beam reporting.

Therefore, in Mode B, RAN1 has already agreed that the UE sends the UEI report on the first available occasion of the second UL channel X symbols after the UL indication in the first PUCCH channel.

In both mode A and mode B the delay to send the UEI beam report in a second UL channel after the UL indication in a first PUCCH channel has already been defined by RAN1 work.

On the other hand, the total event-triggered L1 reporting delay depends also on the delay between the time instant an event is triggered (i.e., at the UE after collecting the last measurement sample that triggers the report) and the UL indication in the first PUCCH channel. Therefore, RAN4 need to also define a maximum delay for the UE transmission of the UL indication in the first PUCCH channel once an event is triggered. This requirement would ensure that the UEI beam report is eventually transmitted by the UE to the network within a certain total delay, and make sure that the information on the reported beams is not outdated.

RAN4 to define event-triggered L1 reporting delay requirements with a maximum delay for the UE transmission of the UL indication in the first PUCCH channel after the event that triggers the beam report is met.

In the context of which event(s) may trigger a UEI beam report, Event-2 has been agreed as the main event that can trigger a UEI beam report. Event-2 is defined as the event where the quality of at least one new beam, such as L1-RSRP, becomes a “threshold value” better than the current beam. For example, the network may configure the UE with a certain threshold, for example 3 dB, and when the UE measures a new beam to have a L1-RSRP which is 3 dB better than the L1-RSRP of the current beam, then a UEI beam report is triggered. Besides Event-2, some other events are currently under discussion, for example Event-1 and Event-7 are expected to be supported, where:

- Event-1 is triggered when the quality of the current beam is worse than a certain threshold. This event may be used to avoid costly procedures like BFR.

- Event-7 is triggered when the quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the M-th best quality. This event may be used to update the active TCI state list.

All UEIBM events are associated to RSs that the UE must monitor/measure. Related to that, there are some issues that need to be taken into account when defining event-triggered L1 reporting delay requirements (see also Figure 1):

- We have at least two beams involved, and therefore at least two RSs that need to be monitored/measured by the UE for triggering the event;

- The periodicities of the two RSs can be different;

- Even if the two RSs have the same periodicity, that can be different from the periodicity of the first PUCCH channel;

- The actual time instant of the RSRP measurement of an SSB or CSI-RS is unique for all the UEs, and just need to meet the RSRP measurement period requirements defined in clause 9.5.4 of TS 38.133.

Therefore, in order to define a maximum delay for the UE transmission of the UL indication in the first PUCCH channel, we should consider as starting point the time instant a RS associated to either the current beam or a candidate new beam that actually triggered the UEIBM event is received by the UE.

The maximum delay requirement for the first PUCCH channel is determined in relation to the reception time instant of the RS that triggered the UEIBM event.

(a)

(b)

Figure 1. UEIBM Event-2 (assuming network configured 3 dB threshold between candidate new beam and current beam) triggered by (a) a candidate new beam RSRP measurement and (b) a current beam RSRP measurement.

Discussion on TCI switching requirements

Regarding UEIBM TCI switching requirements, the following was agreed and discussed in the last RAN4#112 meeting [2]:

Issue 2-1-7: Whether to specify new unified TCI state switching delay? <Way forward> - FFS on whether to define new TCI state switching delay or existing requirements are sufficient Issue 2-1-9: Impact of beam on known/unknown conditions for TCI states? <Way forward> - FFS. Depends on RAN1 further progress.

On the actual structure of the L1-RSRP report format, RAN1 has agreed on the following in the last RAN1#118 meeting:

Agreement (RAN1 #118 Maastricht) On UE-initiated/event-driven beam reporting, regarding L1-RSRP report format Option-3 depending on Event-2, the following differential L1-RSRP report format is supported. CRI or SSBRI #1. CRI or SSBRI #2. …. CRI or SSBRI #N. L1-RSRP #1. Differential L1-RSRP #2. …. Differential L1-RSRP #N. Differential L1-RSRP for current beam, if report mode that current beam is always reported is enabled by RRC. Note: Other contents are not precluded. . - Differential L1-RSRP #2~#N/current beam is determined based on the difference between measured L1-RSRP corresponding to the CRI/SSBRI #2~#N/current beam and the measured L1-RSRP corresponding to CRI/SSBRI #1 - L1-RSRP#1 is the largest measured RSRP among reported ones, and an absolute L1-RSRP. - FFS: range and step size of differential L1-RSRP - FFS: Whether/how to report additional indication of which CRI/SSBRI(s) satisfy the condition of Event-2. - FFS: Additional report content(s) (e.g., reporting configuration ID, indication for synchronization state, event ID, or cell ID).

It is worth recalling that before RAN1#118, it was not clear if the L1-RSRP was reported for every single beam included in the UEI beam report, giving the possibility of having for example a beam meeting the condition of Event-2, measured by the UE but reported to the network only with its beam index and without L1-RSRP, potentially introducing some ambiguity on if such beam should have been considered as known or unknown. For this reason, Issue 2-1-9 was included for discussion in the last RAN4 meeting. In RAN1#118 RAN1 clarified that L1-RSRP is reported for every beam included in the UEI beam report, and therefore we think there is no impact on known/unknown conditions for TCI states and existing known/unknown conditions for TCI switching based on UEI beam reports can be reused.

Reuse known/unknown conditions for Rel-17 unified TCI states when defining requirements for TCI-switching based on UEIBM.

RAN1 has mostly focused up to now on the following aspects, that mainly relate to the new type of UEIBM reporting:

- UL content of the UEI beam report, e.g. N-beams with N RRC configured;

- UL container, e.g., PUCCH vs PUSCH for first and second UL channel;

- Events to trigger UEIBM reporting, e.g., Event-1/2/7;

- RSs to be monitored for triggering the events;

- Procedures for UEIBM reporting, i.e., Mode A and Mode B.

Based on the current RAN1 progress, it is not yet fully clear if new TCI switching requirements need to be defined.

Wait for further RAN1 progress before discussing TCI switching delay requirements.

Discussion on other measurement requirements

Regarding UEIBM, the following was further agreed and discussed in the last RAN4#112 meeting [2]:

Issue 2-1-5: measurement metrics for beam report? Agreement: - Based on L1-RSRP - Note: others can be revisited if RAN1 have further agreement Issue 2-1-6: L1-RSRP for CSI-RS measurement? Enhancement for UE-initiated/event-driven beam management Agreement: Define CSI-RS based L1-RSRP measurement requirements. CSI-RS is periodic CSI-RS. Issue 2-1-8: Whether to filter L1-RSRP? <Way forward> - FFS: RRM requirements for event-triggered L1 reporting without time-window-based filtering or other filtering configured. Issue 2-1-10: Whether to enhance beam sweeping in event triggered L1 beam reporting? <Way forward> - FFS: The assumption of N = 8 fine beam sweeping can be enhanced to realize more efficient BM. For example, the combination of coarse Rx beam and fine Rx beam would facilitate faster Tx and Rx beam refinement. Issue 2-1-11: Whether to define new accuracy requirements by Enhancement for UE-initiated/event-driven beam management? <Way forward> - Option 1: No new accuracy requirements for existing L1 measurements are required. - Option 2: RAN4 need to discuss whether to update legacy accuracy requirements.

Currently, L1-RSRP has been agreed as the baseline quality metric for trigger events, although other quality metrics like L1-SINR are under discussion in RAN1. However, such measurements can be performed by the UE as done in legacy, so there is no need for RAN4 to define new measurement accuracy requirements specific for UEIBM.

Reuse legacy measurement accuracy requirements for measurement reports with UEIBM.

Clause 9.5.4 of TS 38.133 specifies measurement period requirements to compute L1-RSRP to be reported in the baseline periodic/semi-persistent/aperiodic reporting specified up to Rel-18. Such requirements are defined in both FR1 and FR2, and for SSBs in FR2 they read as follows

Table 9.5.4.1-2: Measurement period TL1-RSRP_Measurement_Period_SSB for FR2 Configuration; TL1-RSRP_Measurement_Period_SSB (ms). non-DRX; max(TReport, ceil(M*P*N)*TSSB). DRX cycle ≤ 320ms; max(TReport, ceil(1.5*M*P*N)*max(TDRX,TSSB)). DRX cycle > 320ms; ceil(1.5*M*P*N)*TDRX. Note: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting..

In FR2-1, the baseline value for the beam sweeping factor N is N=8. That means that, for a scenario with M=1, P=1 and without DRX in the previous table, by assuming an SSB periodicity of 20 ms, we have that a UE is required to compute the RSRP for an SSB beam within 160 ms, for example to allow the UE to measure that SSB beam with multiple Rx panels/beams.

In our understanding, while with periodic reporting we have the pros of having many reports sent by the UE at regular periodicity, with UEIBM we would be able to achieve the advantages of many configured reports (like in the periodic reporting) with reduced reporting overhead using just few reports. This is achieved with UEIBM by reporting only relevant and updated measurements. However, this large RSRP measurement delay in FR2 with beam sweeping scaling factor N=8 result in large measurement delays, which are somehow in contradiction with such objective of UEIBM of reducing latency of the reporting.

In Rel-18 a reduced beam sweeping factor has already been supported for example for multi-Rx enhancements, and we think that a reduced beam sweeping factor could be beneficial also for UEIBM.

Specify RAN4 requirements for a UE supporting Rel-18 multi-Rx fast beam sweeping and configured with UEIBM reporting.

In the context of how to determine the triggering event for Event-2 with filtering/windowing of multiple L1 measurements, RAN1 has agreed on the following:

Agreement (RAN1 #117 Fukuoka) Regarding the triggering event determination for Event 2: - If within a time window (which is configurable), the number of Event-2 instance(s) for at least one same new beam is greater than or equal to a configurable number M, UE initiated beam report occurs. - Note: Event-2 instance for a new beam is determined if the L1-RSRP of the new beam becomes a threshold value better than the current beam Above feature is subject to UE capability. - Basic feature: Once the L1-RSRP of the new beam becomes a threshold value better than the current beam, UE initiated beam report occurs FFS: Whether the above is captured in RAN1 or RAN2 specification.

The agreement above introduces the possibility for the network to configure the UE with a time window, such that the UEI beam report (either with Mode A or Mode B) is triggered only when at least M Event-2 instances take place within such configured time window. In other words, that means that the UE needs to measure at least M times within such configured time window that the L1-RSRP of the new beam is a threshold value better than the L1-RSRP of the current beam. Taking Event-2 as an example, this feature makes sure that when a new beam is measured by the UE to be a threshold value better than the current beam, such condition is rather robust, and not caused by a small-scale fading fluctuation of the channel.

Although such feature is subject to a UE capability, and the basic feature just assumes that a UEI beam report is triggered once the L1-RSRP of the new beam becomes a threshold value better than the current beam, RRM requirements should apply both without and with such configured time window for Event-2 determination. However, the details of such feature seem to be still under definition, so we think we should wait for further RAN1 progress before deciding if specific RRM requirements are needed.

Wait for further RAN1 progress before discussing specific RRM requirements with configured time window and configured number of event instances within the time window for event determination.

Conclusion

In this paper, the following observations and proposals were made:

Proposal 1: Define event-triggered L1 reporting delay requirements for both mode A and mode B.

Observation 1: In both mode A and mode B the delay to send the UEI beam report in a second UL channel after the UL indication in a first PUCCH channel has already been defined by RAN1 work.

Proposal 2: RAN4 to define event-triggered L1 reporting delay requirements with a maximum delay for the UE transmission of the UL indication in the first PUCCH channel after the event that triggers the beam report is met.

Proposal 3: The maximum delay requirement for the first PUCCH channel is determined in relation to the reception time instant of the RS that triggered the UEIBM event.

Proposal 4: Reuse known/unknown conditions for Rel-17 unified TCI states when defining requirements for TCI-switching based on UEIBM.

Proposal 5: Wait for further RAN1 progress before discussing TCI switching delay requirements.

Proposal 6: Reuse legacy measurement accuracy requirements for measurement reports with UEIBM.

Proposal 7: Specify RAN4 requirements for a UE supporting Rel-18 multi-Rx fast beam sweeping and configured with UEIBM reporting.

Proposal 8: Wait for further RAN1 progress before discussing specific RRM requirements with configured time window and configured number of event instances within the time window for event determination.

- References

- RP-242394, WID revision: NR MIMO Phase 5, RAN#105, Melbourne, Australia, September 9-12, 2024.

- R4-2414035, WF on RRM requirements for NR_MIMO_Ph5, RAN4#112, Maastricht, Netherlands, 19th ‒ 23rd August, 2024.