# 3GPP TSG-RAN WG4 Meeting #112bis	R4-2416025

# Hefei, Anhui, China, 14th – 18th October, 2024

Agenda Item:	6.22.1

Source: 	Ericsson

Title:	Discussion on RAN1 LS in NES

Document for:	Discussion

# Introduction

In last meeting, RAN1 sent an LS[1] to RAN4 to ask about the timeline of T_min.

RAN1 has made the following agreements for MAC-CE based indication of on-demand SSB (OD-SSB) transmission on SCell. Only the relevant agreements related to RAN1’s question are excerpted. In highlights in the agreements, RAN1 agreed working assumption that the value of T_min for MAC-CE based OD-SSB indication is the same value as UE’s MAC-CE processing time for SCell activation command. RAN1 would like to ask RAN4 if the working assumption of T_min=. can be confirmed to agreement. <RAN1 #116> Agreement Regarding the UE assumption on SSB transmission on a cell supporting on-demand SSB SCell operation, the following cases are identified for further study: - Case #1: No always-on SSB on the cell - Case #2: Always-on SSB is periodically transmitted on the cell - FFS: Whether always-on SSB and on-demand SSB are not cell-defining SSB if transmitted. FFS: Which scenario the above applies for <RAN1 #116 bis> Agreement For the identified scenarios and cases (as per RAN1#116 agreement), on-demand SSB can be triggered by gNB at least for the following scenarios/cases: - Scenario #2 and Case #1 - Scenario #2 and Case #2 - Scenario #2A and Case #1 - Scenario #2A and Case #2 - FFS: Scenario #3A and Case #1 - FFS: Scenario #3A and Case #2 - FFS: Scenario #3B and Case #1 - FFS: Scenario #3B and Case #2 - For Case #1, once on-demand SSB is triggered, its transmission is in a periodic manner. - Note: This does not imply periodic on-demand SSB is transmitted indefinitely after triggered. - Notes: - Scenario #2A refers to - “When UE receives SCell activation command (e.g., as defined in TS 38.321)” - Scenario #3A refers to - “After UE receives SCell activation command (e.g., as defined in TS 38.321) until SCell activation is completed” - Scenario #3B refers to - “When SCell activation is completed and SCell is activated” or - “After SCell activation is completed and SCell is activated” - For discussion purpose under AI 9.5.1, always-on SSB is SSB supported in Rel-18 specifications. - Timing for on-demand SSB transmission (e.g. when the triggered SSB starts and ends) will be separately discussed. <RAN1 #117> Agreement - For a cell supporting on-demand SSB SCell operation, - Support RRC based signaling to indicate on-demand SSB transmission on the cell. - Support MAC CE based signaling to indicate on-demand SSB transmission on the cell. - FFS: Whether to support DCI based signaling to indicate on-demand SSB transmission on the cell. - This DCI signaling does not provide SCell activation/deactivation. - If supported, details on DCI including UE-specific or group-common DCI, DCI contents, etc. - FFS: Scenarios where the above signalings are applicable <RAN1 #118> Agreement For a cell supporting on-demand SSB SCell operation, at least for the following parameter(s), multiple candidate values can be configured by RRC and the applicable value can be indicated by MAC CE for on-demand SSB transmission indication for the cell. - Periodicity of the on-demand SSB - FFS: Any other relevant parameters Agreement The previous RAN1 agreement made in RAN1#117 is revised as follows. - For SSB burst(s) indicated by on-demand SSB SCell operation via MAC CE, UE expects that on-demand SSB burst(s) is transmitted from time instance A which is determined as follows. - Alt 3-1: Time instance A is the beginning of the first slot containing [candidate SSB index 0 or the first actually transmitted SSB index] of on-demand SSB burst [the slot boundary of] the first SSB time domain position [of actually transmitted on-demand SSB burst] which is at least T [slots or symbols] after the [slot or symbol] where UE receives a signalling from gNB to indicate on-demand SSB transmission - The SSB time domain positions of on-demand SSB burst are configured by gNB. - FFS: Details of the value of T (≥ 0) including possibility of T comprising of multiple components - Note: The value of T is not less than existing timeline required for UE’s MAC CE processing for SCell activation - (Working assumption): T is not less than T_min=m+3Nslotsubframe,μ m+3Nslotsubframe,μ+1m+3Nslotsubframe,μ+1 where slot n+m is a slot indicated for PUCCH transmission with HARQ-QCK information when the UE receives MAC CE signaling to indicate on-demand SSB transmission ending in slot n, and Nslotsubframe,μ is as defined in current specification. - RAN4 to confirm that T_min can be equal to m+3Nslotsubframe,μ m+3Nslotsubframe,μ+1 - FFS: Whether the value of T is predefined or indicated/configured by gNB - (Working assumption) T=T_min - FFS: Details of “the [slot or symbol] where UE receives a signalling from gNB” or “the [slot or symbol] where UE transmits HARQ-ACK corresponding to a signalling from gNB to trigger on-demand SSB” - Above applies at least for the case where SCell with on demand SSB transmission and cell with signalling transmission have the same numerology..

In this contribution, we will discuss and reply to the RAN1 LS.

- Timeline T_min

In legacy SCell activation, RAN4 followed the same principle of T_min to handle the SCell activation requirement as RAN1.

### 8.3.2 SCell Activation Delay Requirement for Deactivated SCell … Upon receiving SCell activation command in slot n, the UE shall be capable to transmit valid CSI report and apply actions related to the activation command for the SCell being activated no later than in slot n+THARQ+Tactivation_time+TCSI_ReportingNR slot length , where: THARQ (in ms) is the timing between DL data transmission and acknowledgement as specified in TS 38.213 [3] Tactivation_time is the SCell activation delay in millisecond. … TFirstSSB: is the time to the end of the first complete SSB burst indicated by the SMTC, or within 5ms if SMTC is not configured, after slot n + THARQ+3msNR slot length.

Observation 1: In legacy SCell activation, RAN4 followed the same RAN1 principle to apply SCell activation requirement based on slot n + THARQ+3msNR slot length.

The procedure for UE to activate the OD-SSB based SCell activation is similar as legacy SCell activation. UE will first parse the SCell activation and OD-SSB MAC-CE command and transmit the HARQ feedback. After that, UE will try to receive the first OD-SSB to complete the OD-SSB based SCell activation. Thus, RAN4 can confirm RAN1’s working assumption that T_min equals to m+3Nslotsubframe,μ+1.

Observation 2: The first steps of OD-SSB based SCell activation procedure is similar as legacy SCell activation.

Proposal 1: Proposal: RAN4 to send LS to confirm that T_min equals to m+3Nslotsubframe,μ+1.

- Summary

In this contribution, we have discussed the reply of RAN1 LS. Based on the discussions, we have made following proposals and observations:

Observation 1: In legacy SCell activation, RAN4 follows the same RAN1 principle to apply SCell activation requirement based on slot n + THARQ+3msNR slot length.

Observation 2: The first steps of OD-SSB based SCell activation procedure is similar as legacy SCell activation.

Proposal 1: Proposal: RAN4 to send LS to confirm that T_min equals to m+3Nslotsubframe,μ+1.

- References

- R1-2407565, “LS on timeline for On-demand SSB operation on SCell”, Apple

- Annex: draft LS response on timeline for On-demand SSB operation on SCell

3GPP TSG-RAN WG4 Meeting #112bis R4-22xxxxx Hefei, Anhui, China, 14th – 18th October, 2024 Title: Reply LS on timeline for On-demand SSB operation on SCell Response to: R1-2407565 Release: Release 19 Work Item: Netw_Energy_NR_enh Source: RAN4 To: RAN1 Cc: Contact Person: Name: Zhixun Tang #### E-mail Address: zhixun.tang@ericsson.com Attachments: - 1. Overall Description: RAN4 thanks the LS from RAN1 on timeline of On-demand SSB operation on SCell. During this RAN4 meeting, RAN4 further discussed the questions and achieved the following conclusions: [RAN4 Response]: RAN4 confirms the RAN1’s working assumption of T_min= m+3Nslotsubframe,μ+1. 2. Actions: To RAN1: RAN4 respectfully asks RAN1 to take the above information into account. 3. Date of Next TSG-RAN4 Meetings: TSG-RAN4 #113 18 Nov. – 22 Nov. 2024 Orlando, US TSG-RAN4 #114 17 Feb. – 21 Feb. 2025 Athens, Greece