3GPP TSG-RAN WG4 Meeting # 112bis	R4-2416026

# Hefei, Anhui, China, 14th – 18rd October, 2024

Source:	LG Electronics

Title:	Discussion on RRM requirements of SSB adaptation for Rel-19 NES

Agenda item:	6.22.2.2

Document for:	Discussion

Introduction

In agreed WID for Rel-19 NES operation [1], following objective was captured for RAN4 RRM core requirements:

- Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4] - Adaptation of SSB in time domain, e.g. adapting periodicity - Adaptation of PRACH in time domain - Adaptation of paging occasions including confining the paging occasions in the time domain - Note: there shall be no paging latency increase - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown - Specify the corresponding core requirements, for the above features [RAN4].

In this contribution, we provide our views on RRM core requirements for RRM requirements of adaptation of SSB for Rel-19 NES.

Discussion

In the last RAN4 meeting, RAN4 achieved agreement to disuss the L1/L3 measurement requirement impact as starting point as follows[2]:

Issue 3-1-2: SSB adaptation impact in RRM requirement - Agreement: SSB adaptation impact in RRM requirement - RAN4 to discuss the L1/L3 measurement requirement impact as starting point. - Wayforward: - RAN4 to discuss SSB periodicity adaptation in time-domain as starting point. - Other SSB adaptation mechanism (s) and requirements are not precluded depending on RAN1 progress.

Furthermore, Wayforward considering SSB periodicity adaptation in time-domain as starting point is captured. In general, we are fine with SSB periodicity adapation in time-domain as starting point. However, it is unclear whether the requirement for SSB adaption is overlapped with on-demand SSB operation. In order to compare on-demand SSB and SSB adaptation, we summarized Table1 as follows:

Table 1 On-demand SSB and SSB adaptation

|  | On-demand SSB | SSB adaptation |
| --- | --- | --- |
| Scenario | Case1) OD-SSB Case2) AO-SSB+OD-SSB | Semi-persistant or dynamic signaling changes at least SSB periodicity |
| Signaling | RRC, MAC-CE | TBD |
| Information in indication | At least periodicity | TBD (Periodicity?) |
| Applied cell | SCell | PCell(connected), SCell |

For the target scenario of SSB adaptation, the intention is to send SSB more frequently via semi-persistant or dynamic signaling when it is required. It seems that the mechanism is similar to Case2 of on-demand SSB operation. For example, the periodicity of AO-SSB becomes shorter by indicating OD-SSB from UE perspective. Although signaling and its information has not been determined yet, similar signaling can be introduced by RAN1. For the target cell, on-demand SSB is applied for only SCell. On the other hand SSB adaptation is for PCell(connected) and SCell. From the above analysis, at least at this stage, it seems that the only major difference is applied cell.

Observation 1: On-demand SSB and SSB adaptation has similar operation except for applied cell

Such issue has also been raised by some companies in RAN1, which means that guidance can be provided. In case when two operations are almost same, we think considering the requirements defined in on-demand SSB seems reasonable at least for SCell. We prefer to specify SSB adaptation requirments when it is clearly confirmed. However, if RAN4 needs to specify requiremens for SSB adaptation, we think PCell in connected mode can be prioritized.

Proposal 1: For SSB adaptation requirements, we propose,

- Specify the requirements of SSB adpatiation for PCell

- Consider the requirements of on-demand SSB as baseline at least for SCell

Conclusion

In this contribution, we provide views on RRM requireiments for SSB adaptation, and we propose

Observation 1: On-demand SSB and SSB adaptation has similar operation except for applied cell

Proposal 1: For SSB adaptation requirements, we propose,

- Consider the requirements of on-demand SSB as baseline at least for SCell

- Specify the requirements of SSB adpatiation only for PCell

Reference

[1] RP-242354, “Revised WID: Enhancements of network energy savings for NR,” Ericsson

[2] R4-2413885, “WF on Netw_Energy_NR_enh,” Ericsson