# 3GPP TSG-RAN WG4 Meeting # 112bis	R4-2416051

# Hefei, China, October 14 – 18, 2024

Source: 	Huawei, HiSilicon

Title: 	Discussion on the NTN S band definition

Agenda Item:	5.17.2

Document for:	Approval

Introduction

In the RAN4#112 meeting, RAN4 reached the following way forward [1] to further discuss the UE-to-UE coexistence issue between TN and NTN bands.

- Issue 1-2-5: Applicable TN Bands for Coexistence Agreement: - Focus on coexistence of S-band UL with B2/n2 and B25/n25 DL. - Capture the clarifications on the co-existence issues with B70/n70 and B66/n66 and that there is no 3GPP solution for them in the TR - Issue 1-2-6: TN-NTN UE-to-UE Coexistence Offline Agreement: - Further study the UE-to-UE coexistence between NTN S-band UL and B2/n2, B25/n25 DL, considering: - Realistic usage scenarios - consider UE antenna gain for smartphone of -5.5dBi - UE-to-UE separations distance - probability of the UE-to-UE coexistence scenarios occurring - Companies are encouraged to examine the previous work in TR 38.863 for UE-to-UE coexistence noting that the solution might need to be re-evaluated for this work. - Companies are encouraged to investigate the background scenario for applicability of the -50dBm/MHz requirement to assess its applicability to TN-NTN coexistence (e.g. reference R4-080710) - Companies are also encouraged to investigate other prior work in UE-to-UE coexistence where different values from -50 dBm/MHz have been adopted

In this paper, we’d like to share our views on this WI.

Discussion

As the UL frequency range for this S band is 2000 – 2020 MHz, it’s overlapping with the DL frequency range 1995 ~ 2020 MHz of band n70, which is deployed in North America. If we follow the similar solution for band n256 protecting band n2/n25/70, one not can be clarified for new NTN S-band in the Spurious emissions table for UE co-existence.

Proposal 1: the similar solution for band n256 protecting band n2/n25/70 can be reused for new NTN S-band protecting the NR band n70.

| NR NTN satellite Band | Spurious emission for UE co-existence |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| New NTN S-band | NR Band n70 | FDL_low | - | FDL_high | NA | NA | X |
| NOTE X: The co-existence between New NTN S-band and band n70 is subject to regional/national regulation. |  |  |  |  |  |  |  |

In the TS 36.101, the Spurious emissions table for UE co-existence in band 23 was specified below.

| E-UTRA Band | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| 23 | E-UTRA Band 4, 5, 12, 13, 14, 17, 23, 24, 26, 27, 29, 30, 41, 54, 66, 103, 106 | FDL_low | - | FDL_high | -50 | 1 |  |

Proposal 2: since band 23 can protect the DL frequency range of band 66 with -50dBm/MHz, similarly the new NTN S-band can protect the DL frequency range of band 66 with -50dBm/MHz as well.

In addition, the UL frequency range 2000 – 2020 MHz is very close to the DL frequency range of band n2/n25. There are only 5MHz gap between the DL frequency of band n25 and the UL 2000 – 2020 MHz. The first adjacent channel interference from UL 2000 – 2020 MHz will fall into the DL frequency of band n25/n2. If AMPR solution will be considered to protect the DL frequency of band n25/n2, it’s unclear whether it will lead to the Uplink radio link failure for new NTN S-band due to output power reduction.

Observation 1: If AMPR solution will be considered to protect the DL frequency of band n25/n2, it’s unclear whether it will lead to the Uplink radio link failure for new NTN S-band due to output power reduction.

After checking 47 CFR Part 25 [2], the following regulatory requirements related to S-band (2000 – 2020 MHz) are cited below.

25.265 Acceptance of interference in 2000-2020 MHz. (a) MSS receivers operating in the 2000-2020 MHz band must accept interference from lawful operations in the 1995-2000 MHz band, where such interference is due to: (1) The in-band power of any operations in 1995-2000 MHz (i.e., the portion of transmit power contained in the 1995-2000 MHz band); or (2) The portion of out-of-band emissions contained in 2000-2005 MHz

25.216 Limits on emissions from mobile earth stations for protection of aeronautical radionavigation-satellite service. (e) The e.i.r.p density of emissions from mobile earth stations with assigned uplink frequencies between 1990 MHz and 2025 MHz shall not exceed −70 dBW/MHz, averaged over any 2 millisecond active transmission interval, in frequencies between 1559 MHz and 1610 MHz. The e.i.r.p. of discrete emissions of less than 700 Hz bandwidth from such stations between 1559 MHz and 1605 MHz shall not exceed −80 dBW, averaged over any 2 millisecond active transmission interval. The e.i.r.p. of discrete emissions of less than 700 Hz bandwidth from such stations between 1605 MHz and 1610 MHz manufactured more than six months after FEDERAL REGISTER publication of the rule changes adopted in FCC 03-283 shall not exceed −80 dBW, averaged over any 2 millisecond active transmission interval.

For the emission requirements, it seems that the frequency separation between UL 2000 – 2020 MHz and protected frequency range 1559 ~ 1610 MHz is large enough, so there is no need additional power reduction to comply with this requirements. But it can be reflected in the UE-to-UE coexistence table.

Proposal 3: For new S-band, No additional power reduction is needed in order to comply with the regulatory requirements in the protected frequency range 1559 ~ 1610 MHz. The following Spurious emission for UE co-existence can be considered.

| NR NTN satellite Band | Spurious emission for UE co-existence |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| New NTN S-band | Frequency range | 1559 | - | 1610 | -50 | 0.7 |  |

Summary

Proposal 1: the similar solution for band n256 protecting band n2/n25/70 can be reused for new NTN S-band protecting the NR band n70.

| NR NTN satellite Band | Spurious emission for UE co-existence |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| New NTN S-band | NR Band n70 | FDL_low | - | FDL_high | NA | NA | X |
| NOTE X: The co-existence between New NTN S-band and band n70 is subject to regional/national regulation. |  |  |  |  |  |  |  |

Proposal 2: since band 23 can protect the DL frequency range of band 66 with -50dBm/MHz, similarly the new NTN S-band can protect the DL frequency range of band 66 with -50dBm/MHz as well.

Observation 1: If AMPR solution will be considered to protect the DL frequency of band n25/n2, it’s unclear whether it will lead to the Uplink radio link failure for new NTN S-band due to output power reduction.

Proposal 3: For new S-band, No additional power reduction is needed in order to comply with the regulatory requirements in the protected frequency range 1559 ~ 1610 MHz. The following Spurious emission for UE co-existence can be considered.

| NR NTN satellite Band | Spurious emission for UE co-existence |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| New NTN S-band | Frequency range | 1559 | - | 1610 | -50 | 0.7 |  |

- References

[1] R4-2414287, WF on Rel-19 FR1 NTN bands, Moderator (Inmarsat)

[2] 47 CFR Part 25, https://www.ecfr.gov/current/title-47/chapter-I/subchapter-B/part-25