3GPP TSG-RAN WG4 Meeting # 112bis	R4-2416052

# Hefei, China, October 14 – 18, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.863 | CR | DraftCR | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for TR 38.863 to introduce some regulatory information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_Ku_bands-Core |  |  |  |  |  | Date: |  |  | 2024-09-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To introduce some regulatory information so far for Ku band. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To introduce some regulatory information so far for Ku band. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There is no regulatory information for Ku band. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<Start of Change for TR 38.863>>

2	Reference

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP RP-213691: "Revised WID: Solutions for NR to support non-terrestrial networks (NTN)".

[3]	ITU-R Radio Regulations, 2020 Edition

[4]	ECC Decision 06(09): "Designation of the bands 1980-2010 MHz and 2170-2200 MHz for use by systems in the Mobile-Satellite Service including those supplemented by a Complementary Ground Component (CGC)", Approved 01 December 2006, Amended 05 September 2007,

[5]	3GPP TR 38.811: "Study on New Radio (NR) to support non-terrestrial networks".

[6]	3GPP TR 38.821: "Solutions for NR to support Non-Terrestrial Networks (NTN)".

[7]	3GPP RP-152284: "Revised Work Item: Narrowband IoT ".

[8]	3GPP TR 36.942: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Frequency (RF) system scenarios".

[9]	3GPP TR 45.820: "Cellular system support for ultra-low complexity and low throughput Internet of Things (CIoT)".

[10]	3GPP TR 38.901: "Study on channel model for frequencies from 0.5 to 100 GHz".

[11]	Report ITU-R M.2292: "Characteristics of terrestrial IMT-Advanced systems for frequency sharing/interference analyses".

[12]	3GPP RP-200559: "LS on Parameters of terrestrial component of IMT for sharing and compatibility studies in preparation for WRC-23 (below 5 GHz) ".

[13]	ITU-R Annex 4.4 to Document 5D/716-E.

[14]	3GPP TR 38.921: "Study on International Mobile Telecommunications (IMT) parameters for 6.425 - 7.025 GHz, 7.025 - 7.125 GHz and 10.0 - 10.5 GHz".

[15]	3GPP TR 36.802: "Evolved Universal Terrestrial Radio Access (E-UTRA); NB-IOT; Technical Report for BS and UE radio transmission and reception".

[16]	3GPP TS 38.104: "NR; Base Station (BS) radio transmission and reception".

[17]	3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone".

[18]	3GPP R4-2111460: "On the Rx Parameters and Rx Testing Setup for NTN gNB".

[19]	3GPP R4-2108099: "WF on [312] NTN_Solutions_Part1".

[20]	FCC Online Table of Frequency Allocations, 47 C.F.R § 2.106, February 1, 2021.

[21]	FCC Order 03-15, Report and Order and Notice of Proposed Rule Making, January 29, 2003.

[22]	FCC Order 20-48, Order and Authorization, April 19, 2020.

[23]	FCC Code of Federal Regulations.

[24]	SoftBank, Loon LLC, Nokia, Ericsson, “Proposed deployment and system characteristics of HIBS in the working document towards a preliminary draft new Report ITU-R M.[HIBS-CHARACTERISTICS],” ITU WP-5D contribution, Sep. 28, 2020.

[25]	3GPP TS 38.181: "NR; Satellite Node conformance testing".

[26]	3GPP TS 38.108: "NR; Satellite Node radio transmission and reception".

[27]	3GPP R4-213618: "Moderator’s summary of discussion [94e-41-R17-NRNTN-RAN4Spec] ".

[28]	ERC Recommendation 74-01: "Unwanted emissions in the spurious domain".

[29]	Recommendation ITU-R SM.1541-6: "Unwanted emissions in the out-of-band domain".

[30]	Recommendation ITU-R SM.329: "Unwanted emissions in the spurious domain".

[31]	3GPP TR 38.817-02: "General aspects for Base Station (BS) Radio Frequency (RF) for NR".

[32]	3GPP TR 38.817-01: "	General aspects for User Equipment (UE) Radio Frequency (RF) for NR".

[33]	FCC 16-89, REPORT AND ORDER AND FURTHER NOTICE OF PROPOSED RULEMAKING

[34]	FCC 20-159, REPORT AND ORDER

[35]	FCC 20-166, SECOND REPORT AND ORDER IN IB DOCKET NO. 17-95 AND REPORT AND ORDER IN IB DOCKET NO. 18-315 AND FURTHER NOTICE OF PROPOSED RULEMAKING

[36]	ECC Decision(05)01, The use of the band 27.5-29.5 GHz by the Fixed Service and uncoordinated Earth stations of the Fixed-Satellite Service (Earth-to-space)

[37]	ECC Decision(13)01,  The harmonised use, free circulation and exemption from individual licensing of Earth Stations On Mobile Platforms (ESOMPs) within the frequency bands 17.3-20.2 GHz and 27.5-30.0 GHz

[X]	https://www.miit.gov.cn/jgsj/wgj/wjfb/art/2023/art_8ff006c5a6304a67829e4670e0a4cb96.html

<<Next of Change>>

## 5.2	Regulatory aspects for NTN satellites

### 5.2.1	NTN Satellite band: UL 1980–2010 MHz / DL 2170–2200 MHz

The ITU-R Radio Regulations [3] specify the following service allocation for those frequency ranges, shown in Table 5.2.1-1:

Table 5.2.1-1: Allocation of 1980–2010 MHz and 2170–2200 MHz in the ITU-R Radio Regulations

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 2 170-2 200 FIXED MOBILE MOBILE-SATELLITE (space-to-Earth) 5.351A 5.388 5.389A 5.389F |  |  |
| 1 980-2 010 FIXED MOBILE MOBILE-SATELLITE (Earth-to-space) 5.351A 5.388 5.389A 5.389B 5.389F |  |  |
| 5.351A: For the use of the bands 1 518-1 544 MHz, 1 545-1 559 MHz, 1 610-1 645.5 MHz, 1 646.5-1 660.5 MHz,1 668- 1 675 MHz, 1 980-2 010 MHz, 2 170-2 200 MHz, 2 483.5-2 520 MHz and 2 670-2 690 MHz by the mobile satellite service, see Resolutions 212 (Rev.WRC-07)* and 225 (Rev.WRC-07)**. * This Resolution was revised by WRC-15 and WRC-19. ** This Resolution was revised by WRC-12. 5.388: The frequency bands 1 885-2 025 MHz and 2 110-2 200 MHz are intended for use, on a worldwide basis, by administrations wishing to implement International Mobile Telecommunications (IMT). Such use does not preclude the use of these frequency bands by other services to which they are allocated. The frequency bands should be made available for IMT in accordance with Resolution 212 (Rev.WRC-15)* (see also Resolution 223 (Rev.WRC-15)*). (WRC-15) * This Resolution was revised by WRC-19. 5.389A: The use of the bands 1 980-2 010 MHz and 2 170-2 200 MHz by the mobile-satellite service is subject to coordination under No. 9.11A and to the provisions of Resolution 716 (Rev.WRC-2000)**. ** This Resolution was revised by WRC-12. 5.389B: The use of the frequency band 1 980-1 990 MHz by the mobile-satellite service shall not cause harmful interference to or constrain the development of the fixed and mobile services in Argentina, Brazil, Canada, Chile, Ecuador,the United States, Honduras, Jamaica, Mexico, Paraguay, Peru, Suriname, Trinidad and Tobago, Uruguay and Venezuela. (WRC-19) 5.389F: In Algeria, Cape Verde, Egypt, Iran (Islamic Republic of), Mali, Syrian Arab Republic and Tunisia, the use of the frequency bands 1 980-2 010 MHz and 2 170-2 200 MHz by the mobile-satellite service shall neither cause harmful interference to the fixed and mobile services, nor hamper the development of those services prior to 1 January 2005, nor shall the former service request protection from the latter services. (WRC-19) |  |  |

Following Figure 5.2.1-1 gives an overview of the NR TN bands adjacent to the NTN n256 band.

Figure 5.2.1-1: Adjacent TN bands to NTN band n256

As shown in Figure 5.2.1-1, the NTN satellite band is adjacent to NR bands n1 (FDD) and n34 (TDD). The bands need to be protected and co-existence analysis is required to determine the Adjacent Channel Interference Ratio (ACIR) and in-band power levels.

Also, the NTN band n256 will fully be overlapped by TN NR bands n65 and partly overlapped by TN NR bands n2, n25, n70 and n66, limiting band n256 deployment accordingly to countries where n2, n25 and/or n70 are not deployed or regional regulation applies.

Per ECC Decision 06(09) [4], n256 and Complementary Ground Component (CGC) could operate simultaneously in the upper 30 MHz portion of n65 in CEPT countries.

### 5.2.2	NTN Satellite band: UL 1626.5–1660.5 MHz / DL 1525–1559 MHz

The ITU-R Radio Regulations [3] specify the following service allocation for frequency ranges 1525 – 1559 MHz and 1626.5 – 1660.5 MHz as shown in Table 5.2.2-1.

Table 5.2.2-1: Allocation of 1525 –1559 MHz and 1626.5 – 1660.5 MHz in the ITU-R Radio Regulations

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 1 525-1 530 SPACE OPERATION (space-to-Earth) FIXED MOBILE-SATELLITE (space-to-Earth) 5.208B 5.351A Earth exploration-satellite Mobile except aeronautical mobile 5.349 5.341 5.342 5.350 5.351 5.352A 5.354 | 1 525-1 530 SPACE OPERATION (space-to-Earth) MOBILE-SATELLITE (space-to-Earth) 5.208B 5.351A Earth exploration-satellite Fixed Mobile 5.343 5.341 5.351 5.354 | 1 525-1 530 SPACE OPERATION (space-to-Earth) FIXED MOBILE-SATELLITE (space-to-Earth) 5.208B 5.351A Earth exploration-satellite Mobile 5.349 5.341 5.351 5.352A 5.354 |
| 1 530-1 535 SPACE OPERATION (space-to-Earth) MOBILE-SATELLITE (space-to-Earth) 5.208B 5.351A 5.353A Earth exploration-satellite Fixed Mobile except aeronautical mobile 5.341 5.342 5.351 5.354 | 1 530-1 535 SPACE OPERATION (space-to-Earth) MOBILE-SATELLITE (space-to-Earth) 5.208B 5.351A 5.353A Earth exploration-satellite Fixed Mobile 5.343 5.341 5.351 5.354 |  |
| 1 535-1 559 MOBILE-SATELLITE (space-to-Earth) 5.208B 5.351A 5.341 5.351 5.353A 5.354 5.355 5.356 5.357 5.357A 5.359 5.362A |  |  |
| 1 626.5-1 660 MOBILE-SATELLITE (Earth-to-space) 5.351A 5.341 5.351 5.353A 5.354 5.355 5.357A 5.359 5.362A 5.374 5.375 5.376 |  |  |
| 1 660-1 660.5 MOBILE-SATELLITE (Earth-to-space) 5.351A RADIO ASTRONOMY 5.149 5.341 5.354 5.362A 5.376A |  |  |
| The details of the notes 5.208B, 5.341, 5.342, 5.350, 5.351, 5.351A, 5.352A, 5.353A, 5.354, 5.355, 5.356, 5.357, 5.357A, 5.359, 5.362A, 5.374, 5.375 and 5.376A related to MOBILE-SATELLITE service can be found in [3]. |  |  |

FCC’s specifies the following service allocation for frequency ranges 1525 – 1559 MHz and 1626.5 – 1660.5 MHz in USA[20] as shown in Table 5.2.2-2.

Table 5.2.2-2: Allocation of 1525 –1559 MHz and 1626.5 – 1660.5 MHz in the US

| Allocation to services |  |  |
| --- | --- | --- |
| Federal Table | Non-Federal Table | FCC Rule Part(2) |
| 1525-1535 MOBILE-SATELLITE (space-to-Earth) US315 US380 5.341 5.351 |  | Satellite Communications (25) Maritime (80) |
| 1535-1559 MOBILE-SATELLITE (space-to-Earth) US308 US309 US315 US380 5.341 5.351 5.356 |  | Satellite Communications (25) Maritime (80) Aviation (87) |
| 1626.5-1660 MOBILE-SATELLITE (Earth-to-space) US308 US309 US315 US380 5.341 5.351 5.375 |  | Satellite Communications (25) Maritime (80) Aviation (87) |
| 1660-1660.5 MOBILE-SATELLITE (Earth-to-space) US308 US309 US380 RADIO ASTRONOMY 5.341 5.351 US342 |  | Satellite Communications (25) Aviation (87) |
| The details of the notes 5.341, 5.351, 5.356, 5.375, US308, US309, US315 and US342 related to MOBILE-SATELLITE service can be found in [20]. US380 In the bands 1525-1544 MHz, 1545-1559 MHz, 1610-1645.5 MHz, 1646.5-1660.5 MHz, and 2483.5-2500 MHz, a non-Federal licensee in the mobile-satellite service (MSS) may also operate an ancillary terrestrial component in conjunction with its MSS network, subject to the Commission’s rules for ancillary terrestrial components and subject to all applicable conditions and provisions of its MSS authorization. |  |  |

NOTE:	DL operation in band n24 is restricted to 1526-1536MHz and UL operation is restricted to 1627.5-1637.5MHz and 1646.5-1656.5MHz

Figure 5.2.2-1: Adjacent TN bands to NTN band n255

As shown in Figure 5.2.2-1, the NTN satellite band is not adjacent to any TN NR or LTE bands. The edge-to-edge separation for nearest TN band, n74 DL, is 7 MHz.

Also, the NTN band n255 fully overlaps with TN NR bands n24 and SUL NR band n99. Per note US380 in Table 5.2.2-2 simultaneous operation of n24/n99 and n255 is allowed in USA. Commission’s authorization and technical rules for TN can be found in FCC Orders 03-15 [21] and 20-48 [22] respectively. Commission’s technical rules for its MSS authorization can be found in FCC 47 C.F.R Part 25.

Technical rules for NTN SAN and UEs operating in 1 to 3 GHz from FCC’s 47 C.F.R. § 25.202 [23] are summarized below:

47 C.F.R. § 25.202:

(d) Frequency tolerance, Earth stations. The carrier frequency of each earth station transmitter authorized in these services shall be maintained within 0.001 percent of the reference frequency.

(e) Frequency tolerance, space stations. The carrier frequency of each space station transmitter authorized in these services shall be maintained within 0.002 percent of the reference frequency.

47 C.F.R. § 25.202 (f):

Emission limitations. Except for SDARS terrestrial repeaters and as provided for in paragraph (i), the mean power of emissions shall be attenuated below the mean output power of the transmitter in accordance with the schedule set forth in paragraphs (f)(1) through (f)(4) of this section. The out-of-band emissions of SDARS terrestrial repeaters shall be attenuated in accordance with the schedule set forth in paragraph (h) of this section.

(1)	In any 4 kHz band, the center frequency of which is removed from the assigned frequency by more than 50 percent up to and including 100 percent of the authorized bandwidth: 25 dB;

(2)	In any 4 kHz band, the center frequency of which is removed from the assigned frequency by more than 100 percent up to and including 250 percent of the authorized bandwidth: 35 dB;

(3)	In any 4 kHz band, the center frequency of which is removed from the assigned frequency by more than 250 percent of the authorized bandwidth: An amount equal to 43 dB plus 10 times the logarithm (to the base 10) of the transmitter power in watts;

(4)	In any event, when an emission outside of the authorized bandwidth causes harmful interference, the Commission may, at its discretion, require greater attenuation than specified in paragraphs (f) (1), (2) and (3) of this section.

47 C.F.R. § 25.216:

(a) The e.i.r.p. density of emissions from mobile earth stations placed in service on or before July 21, 2002 with assigned uplink frequencies between 1610 MHz and 1660.5 MHz shall not exceed −70 dBW/MHz, averaged over any 2 millisecond active transmission interval, in the band 1559-1587.42 MHz. The e.i.r.p. of discrete emissions of less than 700 Hz bandwidth generated by such stations shall not exceed −80 dBW, averaged over any 2 millisecond active transmission interval, in that band.

(b) The e.i.r.p. density of emissions from mobile earth stations placed in service on or before July 21, 2002 with assigned uplink frequencies between 1610 MHz and 1626.5 MHz shall not exceed −64 dBW/MHz, averaged over any 2 millisecond active transmission interval, in the band 1587.42-1605 MHz. The e.i.r.p. of discrete emissions of less than 700 Hz bandwidth generated by such stations shall not exceed −74 dBW, averaged over any 2 millisecond active transmission interval, in the 1587.42-1605 MHz band.

(c) The e.i.r.p. density of emissions from mobile earth stations placed in service after July 21, 2002 with assigned uplink frequencies between 1610 MHz and 1660.5 MHz shall not exceed −70 dBW/MHz, averaged over any 2 millisecond active transmission interval, in the band 1559-1605 MHz. The e.i.r.p. of discrete emissions of less than 700 Hz bandwidth from such stations shall not exceed −80 dBW, averaged over any 2 millisecond active transmission interval, in the 1559-1605 MHz band.

(d) As of January 1, 2005, the e.i.r.p. density of emissions from mobile earth stations placed in service on or before July 21, 2002 with assigned uplink frequencies between 1610 MHz and 1660.5 MHz (except Standard A and B Inmarsat terminals used as Global Maritime Distress and Safety System ship earth stations) shall not exceed −70dBW/MHz, averaged over any 2 millisecond active transmission interval, in the 1559-1605 MHz band. The e.i.r.p. of discrete emissions of less than 700 Hz bandwidth from such stations shall not exceed −80 dBW, averaged over any 2 millisecond active transmission interval, in the 1559-1605 MHz band. Standard A Inmarsat terminals used as Global Maritime Distress and Safety System ship earth stations that do not meet the e.i.r.p. density limits specified in this paragraph may continue operation until December 31, 2007. Inmarsat-B terminals manufactured more than six months after Federal Register publication of the rule changes adopted in FCC 03-283 must meet these limits. Inmarsat B terminals manufactured before then are temporarily grandfathered under the condition that no interference is caused by these terminals to aeronautical satellite radio-navigation systems. The full-compliance deadline for grandfathered Inmarsat-B terminals is December 31, 2012.

(e) The e.i.r.p density of emissions from mobile earth stations with assigned uplink frequencies between 1990 MHz and 2025 MHz shall not exceed −70 dBW/MHz, averaged over any 2 millisecond active transmission interval, in frequencies between 1559 MHz and 1610 MHz. The e.i.r.p. of discrete emissions of less than 700 Hz bandwidth from such stations between 1559 MHz and 1605 MHz shall not exceed −80 dBW, averaged over any 2 millisecond active transmission interval. The e.i.r.p. of discrete emissions of less than 700 Hz bandwidth from such stations between 1605 MHz and 1610 MHz manufactured more than six months after Federal Register publication of the rule changes adopted in FCC 03-283 shall not exceed −80 dBW, averaged over any 2 millisecond active transmission interval.

(f) Mobile earth stations placed in service after July 21, 2002 with assigned uplink frequencies in the 1610-1660.5 MHz band shall suppress the power density of emissions in the 1605-1610 MHz band to an extent determined by linear interpolation from −70 dBW/MHz at 1605 MHz to −10 dBW/MHz at 1610 MHz.

(g) Mobile earth stations manufactured more than six months after Federal Register publication of the rule changes adopted in FCC 03-283 with assigned uplink frequencies in the 1610-1626.5 MHz band shall suppress the power density of emissions in the 1605-1610 MHz band-segment to an extent determined by linear interpolation from −70 dBW/MHz at 1605 MHz to −10 dBW/MHz at 1610 MHz averaged over any 2 millisecond active transmission interval. The e.i.r.p of discrete emissions of less than 700 Hz bandwidth from such stations shall not exceed a level determined by linear interpolation from −80 dBW at 1605 MHz to −20 dBW at 1610 MHz, averaged over any 2 millisecond active transmission interval.

(h) Mobile earth stations manufactured more than six months after Federal Register publication of the rule changes adopted in FCC 03-283 with assigned uplink frequencies in the 1626.5-1660.5 MHz band shall suppress the power density of emissions in the 1605-1610 MHz band-segment to an extent determined by linear interpolation from −70 dBW/MHz at 1605 MHz to −46 dBW/MHz at 1610 MHz, averaged over any 2 millisecond active transmission interval. The e.i.r.p of discrete emissions of less than 700 Hz bandwidth from such stations shall not exceed a level determined by linear interpolation from −80 dBW at 1605 MHz to −56 dBW at 1610 MHz, averaged over any 2 millisecond active transmission interval.

(i) The e.i.r.p density of carrier-off state emissions from mobile earth stations manufactured more than six months after Federal Register publication of the rule changes adopted in FCC 03-283 with assigned uplink frequencies between 1 and 3 GHz shall not exceed −80 dBW/MHz in the 1559-1610 MHz band averaged over any two millisecond interval.

(j) A Root-Mean-Square detector shall be used for all power density measurements.

For L-band NTN satellite operation in the US, additional spectrum emission requirements in 47 C.F.R § 25.216 (h) need to be specified for L-band NTN satellite UE. This can be captured as part of additional spurious emission requirements which allows inclusion of frequency ranges that are less than FOOB (MHz) from the edge of the channel bandwidth.

### 5.2.3	NTN Satellite band: UL 27.5-30.0 GHz / DL 17.3-20.2 GHz

#### 5.2.3.1	ITU-R

The ITU-R Radio Regulations [3] specify the following service allocation for those frequency ranges, shown in Table 5.2.3.1-1:

Table 5.2.3.1-1: Allocation of 17.3-20.2 GHz and 27.5-30.0 GHz in the ITU-R Radio Regulations

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 17.3-17.7 FIXED-SATELLITE (Earth-to-space) 5.516 (space-to-Earth) 5.516A 5.516B Radiolocation 5.514 | 17.3-17.7 FIXED-SATELLITE (Earth-to-space) 5.516 BROADCASTING-SATELLITE Radiolocation 5.514 5.515 | 17.3-17.7 FIXED-SATELLITE (Earth-to-space) 5.516 Radiolocation 5.514 |
| 17.7-18.1 FIXED FIXED-SATELLITE (space-to-Earth) 5.484A 5.517A (Earth-to-space) 5.516 MOBILE | 17.7-17.8 FIXED FIXED-SATELLITE (space-to-Earth) 5.517 5.517A (Earth-to-space) 5.516 BROADCASTING-SATELLITE Mobile 5.515 | 17.7-18.1 FIXED FIXED-SATELLITE (space-to-Earth) 5.484A 5.517A (Earth-to-space) 5.516 MOBILE |
|  | 17.8-18.1 FIXED FIXED-SATELLITE (space-to-Earth) 5.484A 5.517A (Earth-to-space) 5.516 MOBILE 5.519 |  |
| 18.1-18.4 FIXED FIXED-SATELLITE (space-to-Earth) 5.484A 5.516B 5.517A (Earth-to-space) 5.520 MOBILE 5.519 5.521 |  |  |
| 18.4-18.6 FIXED FIXED-SATELLITE (space-to-Earth) 5.484A 5.516B 5.517A MOBILE |  |  |
| 18.6-18.8 EARTH EXPLORATION-SATELLITE (passive) FIXED FIXED-SATELLITE (space-to-Earth) 5.517A 5.522B MOBILE except aeronautical mobile Space research (passive) 5.522A 5.522C | 18.6-18.8 EARTH EXPLORATION- SATELLITE (passive) FIXED FIXED-SATELLITE (space-to-Earth) 5.516B 5.517A 5.522B MOBILE except aeronautical mobile SPACE RESEARCH (passive) 5.522A | 18.6-18.8 EARTH EXPLORATION-SATELLITE (passive) FIXED FIXED-SATELLITE (space-to-Earth) 5.517A 5.522B MOBILE except aeronautical mobile Space research (passive) 5.522A |
| 18.8-19.3 FIXED FIXED-SATELLITE (space-to-Earth) 5.516B 5.517A 5.523A MOBILE |  |  |
| 19.3-19.7 FIXED FIXED-SATELLITE (space-to-Earth) (Earth-to-space) 5.517A 5.523B 5.523C 5.523D 5.523E MOBILE |  |  |
| 19.7-20.1 FIXED-SATELLITE (space-to-Earth) 5.484A 5.484B 5.516B 5.527A Mobile-satellite (space-to-Earth) | 19.7-20.1 FIXED-SATELLITE (space-to-Earth) 5.484A 5.484B 5.516B 5.527A MOBILE-SATELLITE (space-to-Earth) | 19.7-20.1 FIXED-SATELLITE (space-to-Earth) 5.484A 5.484B 5.516B 5.527A Mobile-satellite (space-to-Earth) |
| 5.524 | 5.524 5.525 5.526 5.527 5.528 5.529 | 5.524 |
| 20.1-20.2 FIXED-SATELLITE (space-to-Earth) 5.484A 5.484B 5.516B 5.527A MOBILE-SATELLITE (space-to-Earth) 5.524 5.525 5.526 5.527 5.528 |  |  |
| 27.5-28.5 FIXED 5.537A FIXED-SATELLITE (Earth-to-space) 5.484A 5.516B 5.517A 5.539 MOBILE 5.538 5.540 |  |  |
| 28.5-29.1 FIXED FIXED-SATELLITE (Earth-to-space) 5.484A 5.516B 5.517A 5.523A 5.539 MOBILE Earth exploration-satellite (Earth-to-space) 5.541 5.540 |  |  |
| 29.1-29.5 FIXED FIXED-SATELLITE (Earth-to-space) 5.516B 5.517A 5.523C 5.523E 5.535A 5.539 5.541A MOBILE Earth exploration-satellite (Earth-to-space) 5.541 5.540 |  |  |
| 29.5-29.9 FIXED-SATELLITE (Earth-to-space) 5.484A 5.484B 5.516B 5.527A 5.539 Earth exploration-satellite (Earth-to-space) 5.541 Mobile-satellite (Earth-to-space) 5.540 5.542 | 29.5-29.9 FIXED-SATELLITE (Earth-to-space) 5.484A 5.484B 5.516B 5.527A 5.539 MOBILE-SATELLITE (Earth-to-space) Earth exploration-satellite (Earth-to-space) 5.541 5.525 5.526 5.527 5.529 5.540 | 29.5-29.9 FIXED-SATELLITE (Earth-to-space) 5.484A 5.484B 5.516B 5.527A 5.539 Earth exploration-satellite (Earth-to-space) 5.541 Mobile-satellite (Earth-to-space) 5.540 5.542 |
| 29.9-30.0 FIXED-SATELLITE (Earth-to-space) 5.484A 5.484B 5.516B 5.527A 5.539 MOBILE-SATELLITE (Earth-to-space) Earth exploration-satellite (Earth-to-space) 5.541 5.543 5.526 5.527 5.538 5.540 5.542 |  |  |

In addition:

-	WRC-15 authorized ESIM operations in 19.7-20.2 GHz (space-to-Earth) and 29.5-30.0 GHz (Earth-to-space) under certain conditions, and for GSO systems only.

-	WRC-19 authorized ESIM operations in 17.7-19.7 GHz (space-to-Earth) and 27.5-29.5 GHz (Earth-to-space) under certain conditions, and for GSO systems only (Resolution 173).Editor’s note: This TR would be updated with WRC-23 outcomes (Agenda Item 1.16 on NGSO ESIM Ka band).

#### 5.2.3.2	FCC

In 2016, FCC issued Report and Order FCC 16-89 [33] allocating the 27.50-28.35 GHz frequency range for terrestrial licensed mobile operations (IMT 5G). This frequency range corresponds to the 3GPP band n261.

In 2020, FCC issued Report and Order FCC 20-159 [34] which streamlines part 25 rules that govern the licensing of satellite space and earth stations in a satellite system through a unified license. The unified licensing framework specified frequency bands which includes:

-	Space-to-Earth:

-	For GSO: 18.3-18.8 GHz and 19.7-20.2 GHz

-	For NGSO: 17.8-18.6 GHz, 18.8-19.4 GHz and 19.6-20.2 GHz

-	Earth-to-space:

-	For GSO: 28.35-29.1 GHz and 29.25-30.0 GHz.

-	For NGSO: 28.35-29.1 GHz and 29.5-30.0 GHz.

In 2020 as well, FCC issued the Report and Order FCC-20-66 [35] which facilitates the deployment of, and reduce the regulatory burdens on, Earth Stations in Motion (ESIMs). It allows ESIMs to communicate in additional frequency bands with GSO satellites operating in FSS bands and adopts rules for ESIMs to communicate with NGSO satellites in specific FSS bands. Those FSS bands include:

-	Space-to-Earth:

-	For GSO: 17.8-19.4 GHz, 19.6-20.2 GHz.

-	For NGSO: 17.8-18.6 GHz, 18.8-19.4 GHz, 19.6-20.2 GHz

-	Earth-to-space:

-	For GSO: 28.35-29.1 GHz and 29.25-30.0 GHz.

-	For NGSO: 28.4-29.1 GHz and 29.5-30.0 GHz.

#### 5.2.3.3	CEPT

The ECC Decision (05)01 [36] addresses the use of band 27.5-29.5GHz, regulated for Fixed Service and Fixed Satellite Service:

-	The bands 27.5-27.8285 GHz, 28.4445-28.8365 GHz and 29.4525-29.5 GHz are designated for the use of uncoordinated FSS earth stations.

-	The band 28.8365-28.9485 GHz is designated for the use of uncoordinated FSS earth stations, considering, until 1 January 2020, the legacy FS systems licensed in this band in some countries before 18 March 2005.

-	The bands 27.8285-28.4445 GHz and 28.9485-29.4525 GHz are designated for the use of FS systems.

Also, this ECC Decision(05)01 states that the CEPT administrations shall not authorize the deployment of uncoordinated FSS earth stations in the bands designed for the use of FS systems.

Figure 5.2.3.3-1: ECC Decision(05)01 - 27.5-29.5 band segmentation

The ECC Decision(13)01 [37] specifies the harmonized use, free circulation, and exemption from individual licensing of Earth stations on mobile platforms (ESOMPs) within the ranges 17.3-20.2 GHz and 27.5-30.0 GHz,  in the portion identified for their use within the territory of operation, and under the conditions specified in Annexes 1,2 and 3 of this ECC Decision. ESOMPs are terminals with small directional antennas for the provision of broadband communication services. The terminals may be mounted on aircraft, ships or land vehicles or may be transportable devices used in motion or at temporary halts. This ECC Decision also specifies the conditions for those ESOMP devices to operate.

### 5.2.4	NTN Satellite band: UL 14.0-14.5 GHz / DL 10.7-12.5 GHz

#### 5.2.4.1	ITU-R

#### 5.2.4.2	Region 1

#### 5.2.4.3	Region 2

#### 5.2.4.4	Region 3

In China, the MIIT (Ministry of Industry and Information Technology) released an administration document [x] on earth station in motion communicating with Geostationary orbit satellite in 2023. The UL frequency range 14-14.5GHz (Earth-to-Space) and DL frequency range 12.25-12.75GHz (Space-to-Earth) are available for the use of ESIM Geostationary scenario in Ku band. And a lot of regulatory RF requirements for coexistence can be found in the respective document.

<<End of Change>>