# 3GPP TSG-RAN WG4 Meeting # 112bis	R4-2416054

Hefei, China, October 14 – 18, 2024

Source: 	Huawei, HiSilicon

Title: 	Discussion on coexistence study for NR NTN Ku Band

Agenda Item:	6.9.2

Document for:	Approval

Introduction

In the RAN4#112 meeting, RAN4 reached some initial consensus on adjacent coexistence studies for Ku band in the Way forward [1].

- Issue 2-2: Coexistence scenarios Adopt the interference scenarios in [Intelsat] R4-2411507 for the coexistence studies - Issue 2-4: Coexistence frequencies - Use 11GHz for DL coexistence evaluation and 14GHz for UL coexistence evaluation for Ku-band. - Issue 2-8: Coexistence TN channel model For TN channel model, reuse existing version of TR 38.901. - Issue 2-9: Coexistence TN channel model For NTN channel model, reuse parameters from TR 38.811 and collect the following missing information: - Shadow fading and clutter loss tables for the reference frequencies - Tropospheric scintillation. - Issue 2-10: TN Parameters As starting point, reuse the antenna parameters and BS/UE ACLR/ACS from: - The 10.0-10.5 GHz frequency range (TR 38.921) for the reference frequency of 11 GHz - The 14.8-15.35 GHz frequency range (pending on conclusion of SI FS_NR_IMT_4400_7125_14800MHz) for the reference frequency of 14 GHz.

In this paper, we’d like to share our views and studies to continue the discussion of adjacent coexistence study assumption in Ku band.

Discussion

For TN parameters, RAN4 will continue the discussion about the antenna parameters and BS/UE ACLR/ACS by considering the following two references.

•	The 10.0-10.5 GHz frequency range (TR 38.921 [2]) for the reference frequency of 11 GHz

•	The 14.8-15.35 GHz frequency range (pending on conclusion of SI FS_NR_IMT_4400_7125_14800MHz) for the reference frequency of 14 GHz.

For the antenna model of TN BS specified in clause 8.1.1 of TR 38.921 [2], I think we can simply reuse it for this adjacent coexistence study.

- 8.1.1 Array antenna model In Table 8.1.1-1, the parameters used by the parameterized array antenna model are described. Table 8.1.1-1: Parameters of the parameterized array antenna model Parameter; Symbol; Unit. Front to back ratio; Am; dB. Side lobe suppression; SLAv; dB. Horizontal HPBW; 3dB; Degrees. Vertical HPBW; 3dB; Degrees. Array element peak gain; GE,max; dBi. Number of radiating elements rows and columns; (M, N); Integer. Horizontal element separation; dh; m. Vertical element separation; dv; m. Electrical down-tilt angle; etilt; Degrees. Electrical scan angle; escan; Degrees. The parameterized antenna model is built around array antenna model where the element factor, array factor and linear phase progressing is characterized as described by equations in Table 8.1.1-2. Table 8.1.1-2: Array antenna model details Description; Equation; Unit. Peak normalized element radiation pattern; Aθ,φ=-min--min12φφ3dB2,Am-min12θ-90θ3dB2,SLAv ,Am; dB. Peak gain normalized element radiation pattern; AEθ,φ=GE,max+Aθ,φ; dBi. Composite array radiation pattern; AAθ,φ=AEθ,φ+10log10m=1Mn=1Nwm,nvm,n2 , where vm,n=expj2πm-1dvλcosθ+n-1dhλsinθsinφ wm,n=1MNexpj2πm-1dvλsinθetilt-n-1dhλcosθetiltsinφescan; dBi.

However, for the specific array antenna parameters, we are not sure whether we can simply reuse the following Macro Sub-urban BS array antenna parameters for 10GHz.

| Parameter | Macro Sub-urban |
| --- | --- |
| Am (dB) | 30 |
| SLAv (dB) | 30 |
| 3dB (deg.) | 90 |
| 3dB (deg.) | 65 |
| GE,max (dBi) | 6.4 |
| LE (dB) | 2.0 |
| (M, N) | (16, 8) |
| Number of supported polarizations, P | 2 |
| dh (m) | 0.5 |
| dv (m) | 0.7 |
| Horizontal coverage range (deg.) | +/- 60 |
| Vertical coverage range (deg.) | 90 to 100 |
| Conducted power (before ohmic loss) per antenna element, Ptx (dBm) | 22 (Note 4) |
| Mechanical downtilt (deg.) | 6 |
| Note 1: MxN means there are M vertical and N horizontal elements Note 2: LE is included in GE,max Note 3: The vertical coverage range includes the mechanical downtilt. Note 4: The conducted power per element assumes 16x8x2 elements (i.e. power per H/V polarized element). Note 5: The conducted power per element assumes 8x8x2 elements (i.e. power per H/V polarized element). Note 6: The conducted power per element assumes 4x4x2 elements (i.e. power per H/V polarized element). Note 7: Boresight direction is perpendicular to the ceiling. |  |

Although two references i.e. TR 38.921 for 11GHz and Rel-19 SI FS_NR_IMT_4400_7125_14800MHz for 14GHz, were proposed as starting point, the frequency 11GHz and 14GHz are so closed that BS may have similar implementation. In order to simplify the adjacent coexistence studies, it’s better to use the same BS array antenna parameters for both 11GHz and 14GHz.

Proposal 1: To reuse the antenna model of TN BS specified in clause 8.1.1 of TR 38.921 [2] for Ku band adjacent channel coexistence study.

Proposal 2: it’s better to use the same BS array antenna parameters for both 11GHz and 14GHz based on the trade off between TR 38.921 and conclusion of Rel-19 SI FS_NR_IMT_4400_7125_14800MHz.

For the UE type in 10~14GHz, there are some observation in the clause 8.2 of TR 38.921. The UE in 10 - 10.5 GHz range will most likely have a conducted interface with an assumed isotropic radiation pattern antenna and no beamforming.

- 8.2 UE antenna characteristics The outcome of the RAN4 study for collecting technical background information relevant for the frequency range 7 to 24 GHz indicated that the frequency range 7.25-[10-13] GHz would have “FR1 like” requirements, and as such we can assume that in the 10 - 10.5 GHz range this applies. The UE will most likely therefore have a conducted interface with an assumed isotropic radiation pattern antenna and no beamforming.

Proposal 3: For Ku band adjacent channel coexistence study, RAN4 can discuss whether no beamforming “FR1 like” or beamforming “FR2 like” can be assumed for TN UE for both 11GHz and 14GHz.

In last meeting, companies provided some NTN parameters in their contribution [3]. The satellite EIRP density is still FFS as RAN4 haven’t decided the corresponding system channel bandwidth for Ku band.

The FSPL for different scenarios are shown below.

| Scenarios | DL 11GHz FSPL | UL 14GHz FSPL |
| --- | --- | --- |
| GEO (35786km) Nadir Point | 204.34dB | 206.44dB |
| GEO (35786km) 30 deg slant distance 38556km | 205dB | 207.1dB |
| LEO (1200km) Nadir Point | 174.9dB | 177dB |
| LEO (1200km) 30 deg slant distance 1976km | 179.2dB | 181.3dB |
| LEO (600km) Nadir Point | 168.8dB | 170.9dB |
| LEO (600km) 30 deg slant distance 1060km | 173.8dB | 175.9dB |

It can be observed that the FSPL for Ku band is smaller than the Ka band due to lower carrier frequency.

For Ku band VSAT parameters, it seems that the Rx antenna gain (DL 11GHz) is higher than the Tx antenna gain (UL 14GHz) with the same equivalent aperture diameter in the contribution [3]. It seems unreasonable.

Characteristics; VSAT (Note 1). Frequency band; Ku-band (i.e. 14 GHz UL and 12 GHz DL). Antenna type and configuration; Directional Section 6.4.1 of [2] with 60 cm equivalent aperture diameter. Polarisation; Circular. Rx Antenna gain; 36.7 dBi. Antenna temperature; 150 K. Noise figure; 1.2 dB. Tx transmit power; 2 W (33 dBm). Tx antenna gain; 35.1 dBi. NOTE 1: VSAT terminal characteristics could be implemented with phased array antenna.

If we still assume 0.6m equivalent aperture diameter, the Tx antenna gain (14GHz) is 35.1dBi, but the Rx antenna gain should be 33.1dBi. In order to avoid the Noise figure discussion, it’s proposed to consider G/T for VSAT parameters. 4.5dB G/T for Ku VSAT is enough.

In addition, it’s clarified that VSAT terminal characteristics could be implemented with phased array antenna. I’m not sure whether RAN4 need to assume the phased array antenna considering this type of Ku VSAT is the mainstream in the current real market.

Proposal 4: RAN4 can consider the following RF parameters for Ku band VSAT to perform the adjacent coexistence study.

| Characteristics | VSAT (Note 1) |
| --- | --- |
| Frequency band | Ku-band (i.e. 14 GHz UL and 11 GHz DL) |
| Antenna type | Directional FFS antenna type: Option 1: Section 6.4.1 of TR 38.811 Option 2: section 8.1.1 of TR38.921 |
| Antenna equivalent aperture diameter | 60 cm equivalent aperture diameter |
| Polarisation | Circular without polarization gain |
| Rx Antenna gain (DL 11GHz) | 33.1 dBi |
| Antenna temperature | 150 K |
| G/T (dB/K) | 4.5 dB |
| Tx transmit power | 2 W (33 dBm) |
| Tx antenna gain (UL 14GHz) | 35.1 dBi |
| [NOTE 1: VSAT terminal characteristics could be implemented with phased array antenna] |  |

For Ku band Satellite parameters, it seems that satellite Tx part should be 11GHz DL and satellite Rx part should be 14GHz UL. If the same equivalent aperture diameter is assumed for both Tx and Rx, the satellite Rx antenna gain should be higher than the Tx antenna gain. In addition, the same equivalent satellite aperture diameter is assumed as Ka band, but the same antenna gain 58.5/38.5/38.5dBi are assumed for Ku band. Thus, it’s encouraged for companies to double-check the parameters for Ku band Satellite part due to the different carrier frequency from Ka band.

Observation 1: it’s encouraged for companies to double-check the parameters for Ku band Satellite part, e.g. antenna gain, due to the different carrier frequency from Ka band.

Summary

Proposal 1: To reuse the antenna model of TN BS specified in clause 8.1.1 of TR 38.921 [2] for Ku band adjacent channel coexistence study.

Proposal 2: it’s better to use the same BS array antenna parameters for both 11GHz and 14GHz based on the trade off between TR 38.921 and conclusion of Rel-19 SI FS_NR_IMT_4400_7125_14800MHz.

Proposal 3: For Ku band adjacent channel coexistence study, RAN4 can discuss whether no beamforming “FR1 like” or beamforming “FR2 like” can be assumed for TN UE for both 11GHz and 14GHz.

Proposal 4: RAN4 can consider the following RF parameters for Ku band VSAT to perform the adjacent coexistence study.

| Characteristics | VSAT (Note 1) |
| --- | --- |
| Frequency band | Ku-band (i.e. 14 GHz UL and 11 GHz DL) |
| Antenna type | Directional FFS antenna type: Option 1: Section 6.4.1 of TR 38.811 Option 2: section 8.1.1 of TR38.921 |
| Antenna equivalent aperture diameter | 60 cm equivalent aperture diameter |
| Polarisation | Circular without polarization gain |
| Rx Antenna gain (DL 11GHz) | 33.1 dBi |
| Antenna temperature | 150 K |
| G/T (dB/K) | 4.5 dB |
| Tx transmit power | 2 W (33 dBm) |
| Tx antenna gain (UL 14GHz) | 35.1 dBi |
| [NOTE 1: VSAT terminal characteristics could be implemented with phased array antenna] |  |

Observation 1: it’s encouraged for companies to double-check the parameters for Ku band Satellite part, e.g. antenna gain, due to the different carrier frequency from Ka band.

- References

[1] R4-2413519, Way Forward for [112][314] NR_NTN_Ku_Band_General, Moderator (Eutelsat Group)

[2] TR 38.921, Study on International Mobile Telecommunications (IMT) parameters for 6.425-7.025GHz, 7.025-7.125GHz and 10.0-10.5 GHz, V17.1.0

[3] R4-2411507, Ku Band Co-existence, Intelsat