# 3GPP TSG-RAN WG4 Meeting # 112bis	R4-2416055

# Hefei, China, October 14 – 18, 2024

Source: 	Huawei, HiSilicon

Title: 	Discussion on coexistence study for NR NTN HPUE

Agenda Item:	6.8.2.1

Document for:	Approval

Introduction

In the RAN#103 meeting, the WI [1] for Enhanced requirements and test methodology for NR and IoT NTN was established to introduce for HPUE feature. The objectives for NTN HPUE are listed below.

- Specify high power UE (HPUE) for NR-NTN (Non-Terrestrial Networks) and IoT-NTN (NB-IoT and eMTC based NTN) in FR1-NTN bands and the corresponding LTE NTN bands for the single uplink (UL) carrier scenario - Support UE with power class 2 (PC2, 26 dBm) that applies to both handheld and non-handheld based on the co-existence study. - Support other power classes based on the co-existence and feasibility study - Non-hand held UE is prioritized - Specify RF requirements for all considered power classes for NR-NTN and IoT-NTN in FR1-NTN bands and the corresponding LTE NTN bands - Necessary Tx requirements, at least including - Investigate and if necessary, specify the solution to address potential SAR (Specific Absorption Rate restriction) requirements for handheld UE - Specify MPR and A-MPR for the example bands considering the regulation for the corresponding bands, if necessary - Necessary Rx requirements, at least including - Specify the RSD requirements on the example bands, if necessary - Specify the necessary signaling to support the above objectives

One of these objectives is to perform the adjacent channel coexistence studies for PC2 and other power classes for NR NTN.

Based on the approved WF [2] and information paper [3] submitted by rapporteur, the following aspects:

1) Scenarios

2) NTN UE UL power control

3) ACLR mode and mapping

4) How to drop NTN UE

In this paper, we’d like to share our views on these aspects before submitting the results of adjacent channel coexistence studies.

Discussion

Scenarios

Based on the approved WF, RAN4 reached the following consensus on NTN scenarios for co-existence study.

Issue 2-2-1: NTN scenarios for co-ex study Agreement: - For the scenario, consider the scenarios in TR 38.863 as a baseline - Prioritize GEO and LEO1200 for co-existence evaluation - LEO600 is not precluded for the requirements and the conclusion for LEO1200 can be applied to LEO600 Issue 2-2-2: TN scenarios for co-ex study - Agreement: Prioritize Urban Macro for Scenario 4 and Rural Macro for Scenario 5 Issue 2-2-3: Co-ex scenario # to be studied No.; Combination; Aggressor; Victim; Notes; Study Phase. 1; TN with NTN; TN DL; NTN DL; Applicable for satellite operating in e.g. S-band, for e.g. coexistence with n1 FDD.; Phase 1. 2; TN with NTN; TN UL; NTN UL; Applicable for satellite operating in e.g. S-band, for e.g. coexistence with n1 FDD.; Phase 1. 3; TN with NTN; NTN DL; TN DL; Applicable for satellite operating in e.g. S-band, for e.g. coexistence with n1 FDD.; Phase 1. 4; TN with NTN; NTN UL; TN UL; Applicable for satellite operating in e.g. S-band, for e.g. coexistence with n1 FDD.; Phase 1. 5; TN with NTN; NTN UL; TN DL; Applicable for satellite operating in S-band, for e.g. coexistence with n34 TDD.; Phase 1. 6; TN with NTN; TN DL; NTN UL; Applicable for satellite operating in S-band, for e.g. coexistence with n34 TDD.; Phase 1. Agreement: - Consider both Scenario 4 and 5 for co-existence study - Prioritize scenario 4 since it is the worst case - Do not consider Scenario 1, 2, 3 and 6

Although RAN4 agreed to prioritize GEO and LEO1200 for co-existence evaluation, these two scenarios are totally different. Thus, from RAN4 perspective, it’s better to discuss GEO and LEO1200 for NR NTN separately. Generally, due to the large path loss between satellite and NTN UE, only narrow bandwidth communication technology is applicable to GEO scenario. In Rel-17, in order to address the issue that SINR is lower than -10dB, only 2 UL RBs are allocated to the NTN UE. From UL perspective, this scenario is very similar to the NB IoT NTN. Thus, GEO scenario for NR NTN can be combined together with NB IoT NTN scenario, which can help reduce the workloads from co-existence study perspective.

LEO scenarios are applicable to NR NTN, which belongs to wide bandwidth technology. For FSPL mode, UL 20~25RBs can be configured to NTN UE achieve higher throughput.

Proposal 1:

For NR NTN, RAN4 should focus on LEO scenarios to study adjacent coexistence simulation and specify the corresponding RF requirements.

GEO scenario for NR NTN can be combined together with NB-IoT NTN as only a few RBs (one RB ) can be allocated to NTN UE for UL adjacent coexistence evaluation.

| No. | Combination | Aggressor | Victim | Notes | Study Phase |
| --- | --- | --- | --- | --- | --- |
| 4 | NR TN with NR NTN | NR NTN UL | NR TN UL | NR NTN: LEO1200 scenario NR TN: Urban Macro | Phase 1 |
| 4 | NR TN with NB-IoT NTN | NB-IoT NTN UL | NR TN UL | NB-IoT NTN: GEO scenario NR TN: Urban Macro | Phase 1 |

On the other hand, both Scenario 4 and 5 are considered for coexistence study. We simulate scenario 5 assuming 31dBm with 20dB ACLR under Rural victim terrestrial network. The throughput loss is less than 0.1% as shown below.

Figure 1 CDF of DL throughput for 2GHz Rural scenario with LEO1200 interference

Proposal 2: RAN4 should specify ACLR requirements for NTN HPUE based on scenario 4 considering the interference from NTN UE to TN UE in scenario 5 is neglect.

NTN UE UL power control

Referring to the service requirement for the 5G system (TS 22.261 [5]), the expected performance requirements for satellite access can be found in the table 7.4.2-1. For handheld UE, the maximum UL experienced data rate is about 3.5Mbit/s. According to the SINR-throughput calculation, if the targeted SINR is 3dB and UL RB allocation is 5MHz, the targeted throughput is 0.4*log2(1+ 10^(3/10))*5MHz=3.17Mbit/s, which is almost closed to the expected UL data rate for handheld UE. Since large UL RB allocation should be considered, this is only for LEO scenario.

If we go to NB-IoT NTN with one RB UL allocation and 0dB targeted SINR, the targeted throughput is 0.4*log2(1+ 10^(0/10))*180kHz=72kbit/s, which is almost closed to the expected UL data rate for Pedestrian scenario and Narrowband IoT connectivity.

Observation 1: 3dB targeted SINR with UL 5MHz RB allocation can meet the expected UL data rate for handheld UE scenario using NR NTN. 0dB targeted SINR with UL one RB allocation can meet the expected UL data rate for Pedestrian scenario using NB IoT NTN.

In Rel-17, for the coexistence study, the same TPC model of TN for NTN UL scenarios is adopted to align with UE UL power control parameters used in TR 38.821. However, the propagation model between TN BS and TN UE is totally different from the propagation model between NTN SAN and UE. Thus, it’s necessary to reopen the discussion on how to derive the parameters for NTN UE UL power control.

Generally, the TPC model specified in Section 9.1 TR 36.942[4] is applied for TN with following parameters.

where: (For example)

-	Pmax = 26dBm for PC2,

-	Rmin = -56 dB assuming -30dBm minimum output power,

-	CLx-ile and γ are set as following:

-	CLx-ile = 10*log10(Pmax) – (SNRtarget + 10*log10(kTB) + NF ) = 26 – [ 3 + (-174 + 67 + 4.3) ] = 125.7,

-	CLx-ile = 10*log10(Pmax) – (SNRtarget + 10*log10(kTB) + NF ) = 26 – [ 0 + (-174 + 52.6 + 7.4) ] = 140,

Where:

SNRtarget is proposed as 3dB for NR NTN LEO scenario and 0dB for NB-IoT NTN GEO scenario.

NF is the SAN noise figure, i.e. 4.3dB for LEO and 7.4dB for GEO.

B is the UL channel bandwidth, and 5MHz is assumed for LEO and one RB is assumed for GEO.

10log10(kT) = -174dBm/Hz.

-	γ = 1 For uplink scenario.

For different power class, we can summarize the parameters below.

table 1 The parameters for NTN UE UL power control

| Parameters | NR NTN for LEO |  |  | NB-IoT NTN for GEO |  |
| --- | --- | --- | --- | --- | --- |
| Pmax Output power (Power Class) | 26dBm (PC2) | 29dBm (PC1.5) | 31dBm (PC1) | 26dBm (PC2) | 31dBm (PC1) |
| Rmin (dB) | -66 | -69 | -71 | -66 | -71 |
| SNRtarget (dB) | 3 | 3 | 3 | 0 | 0 |
| NFSAN (dB) | 4.3 | 4.3 | 4.3 | 7.4 | 7.4 |
| UL BW (MHz) | 5 | 5 | 5 | 0.18 | 0.18 |
| 10log10(kT) (dBm/Hz) | -174 |  |  |  |  |
| CLx-ile (dB) | 125.7 | 128.7 | 130.7 | 140 | 145 |
| γ | 1 | 1 | 1 | 1 | 1 |

Generally, the free space path loss is 32.45+10log10(fc)+10log10(Distance).

where:

-	fc is the centre frequency in GHz. Here 2GHz is assumed,

-	Distance is the centre frequency in meter. Here it can be 600000m, 1200000m and 36000000m,

The general FSPL can be 154dB, 160dB and 190dB for LEO600, LEO1200 and GEO scenario. Based on assumption in Rel-17, 30dBi antenna gain is assumed for LEO and 51dBi antenna gain is assumed for GEO.

Then the coupling loss (CL) is 124dB, 130dB and 139dB.

For different orbit scenarios, we can summarize the general coupling loss below.

| Orbit Scenarios | LEO600 | LEO1200 | GEO |
| --- | --- | --- | --- |
| Centre frequency (GHz) | 2 | 2 | 2 |
| Distance (meter) | 600000 | 1200000 | 36000000 |
| FSPL (dB) | 154 | 160 | 190 |
| SAN Rx Antenna Gain (dB) | 30 | 30 | 51 |
| The UL coupling loss for NTN UE (dB) | 124 | 130 | 139 |

26dBm/29dBm/31dBm NTN UE Tx power CDF for LEO1200 are shown below.

figure 2 26dBm NTN UE Tx power CDF for LEO1200

figure 3 29dBm NTN UE Tx power CDF for LEO1200

figure 4 31dBm NTN UE Tx power CDF for LEO1200

26dBm/29dBm/31dBm NTN UE UL Throughput CDF for LEO1200 are shown below.

figure 5 26dBm NTN UE UL throughput CDF for LEO1200

figure 6 29dBm NTN UE UL throughput CDF for LEO1200

figure 7 31dBm NTN UE UL throughput CDF for LEO1200

Observation 2:

For 26dBm LEO1200 scenario, almost 15% NTN UEs are under power control and 70% NTN UE can achieve 1Mbps UL throughput (And 10% NTN UE achieve the targeted throughput 3.17Mbps).

For 29dBm LEO1200 scenario, almost 35% NTN UEs are under power control and 85% NTN UE can achieve 1Mbps UL throughput (And 28% NTN UE achieve the targeted throughput 3.17Mbps).

For 31dBm LEO1200 scenario, almost 55% NTN UEs are under power control and 92% NTN UE can achieve 1Mbps UL throughput (And 40% NTN UE achieve the targeted throughput 3.17Mbps).

Thus, it is reasonable to assume 3dB target SINR and 5MHz UL allocation for LEO1200 NR NTN scenario to achieve good UL performance for NR NTN UE.

Proposal 3:

To consider the following parameters for NTN UE UL power control

| Parameters | NR NTN for LEO |  |  | NB-IoT NTN for GEO |  |
| --- | --- | --- | --- | --- | --- |
| Pmax Output power (Power Class) | 26dBm (PC2) | 29dBm (PC1.5) | 31dBm (PC1) | 26dBm (PC2) | 31dBm (PC1) |
| Rmin (dB) | -66 | -69 | -71 | -66 | -71 |
| SNRtarget (dB) | 3 | 3 | 3 | 0 | 0 |
| NFSAN (dB) | 4.3 | 4.3 | 4.3 | 7.4 | 7.4 |
| UL BW (MHz) | 5 | 5 | 5 | 0.18 | 0.18 |
| 10log10(kT) (dBm/Hz) | -174 |  |  |  |  |
| CLx-ile (dB) | 125.7 | 128.7 | 130.7 | 140 | 145 |
| γ | 1 | 1 | 1 | 1 | 1 |

ACLR mode and mapping

Referring to the information paper [3], the ACLR model is assumed below.

For NR-NTN: ACLR modelling for TN and NTN co-existence study referring to clause 5.1.1.4.1 and 5.1.1.4.2 in TR 36.942[8] is used as baseline. The number of RBs refers to Table 2.2.2.1-4 and Table 2.2.2.4-1 respectively.

If we assume four UL NTN aggressor UEs, the general ACLR model is shown below.

figure 8 General ACLR model assuming four UL NTN aggressor UEs

In order to reduce the complexity of simulation, we can change this model slightly. For the NTN UE1 which is closest to the victim TN channel, it’s assumed that flat ACLR1 will have impacts on all the Victim UE(s). For the other NTN UEs except for NTN UE1, it’s assumed that only flat ACLR2 will have impacts on all the Victim UE(s). The illustration is shown below.

figure 9 Simplified ACLR model assuming four UL NTN aggressor UEs

Proposal 4: In order to reduce the complexity of simulation, RAN4 can consider a simplified ACLR model assuming four UL NTN aggressor UEs.

For the NTN UE1 which is closest to the victim TN channel, it’s assumed that flat ACLR1 will have impacts on all the Victim UE(s).

For the other NTN UEs except for NTN UE1, it’s assumed that only flat ACLR2 will have impacts on all the Victim UE(s).

How to drop NTN UE

In last meeting, RAN4 has reached the following agreement for NTN and TN isolation distance.

Issue 2-3-1 NTN and TN network isolation distance Agreement: - Consider isolation distance in scenario 4 and 5 for both NR-NTN HPUE coex and IoT-NTN HPUE coex. - Use the isolation distance 1.5km as the starting point - Other values for isolation distance are not precluded

It seems that companies have concerns on this assumption considering some extreme cases. Generally, the satellite beam diameter is assumed as 250km, 90km and 50km for GEO, LEO1200 and LEO600. However, the cluster radius for terrestrial network is only 2km. Since the number of NTN UE deployed in one cell is limited, generally it is very hard to deploy all the NTN UEs into the area within 2km radius. Given NTN UE has both TN and NTN functionalities, it is reasonable to assume isolation distance. When NTN UE is covered by terrestrial network, NTN UE can access to the terrestrial network. Otherwise, NTN UE can access to satellite network. If RAN4 agree to drop NTN UE into the cluster of terrestrial network, the following red area can be considered with 1.5km above radius as the worst case. However, this scenario is much stricter than the real deployment.

figure 10 One strict method to drop NTN UE into the cluster of terrestrial network

Summary

Proposal 1:

For NR NTN, RAN4 should focus on LEO scenarios to study adjacent coexistence simulation and specify the corresponding RF requirements.

GEO scenario for NR NTN can be combined together with NB-IoT NTN as only a few RBs (one RB ) can be allocated to NTN UE for UL adjacent coexistence evaluation.

| No. | Combination | Aggressor | Victim | Notes | Study Phase |
| --- | --- | --- | --- | --- | --- |
| 4 | NR TN with NR NTN | NR NTN UL | NR TN UL | NR NTN: LEO1200 scenario NR TN: Urban Macro | Phase 1 |
| 4 | NR TN with NB-IoT NTN | NB-IoT NTN UL | NR TN UL | NB-IoT NTN: GEO scenario NR TN: Urban Macro | Phase 1 |

Proposal 2: RAN4 should specify ACLR requirements for NTN HPUE based on scenario 4 considering the interference from NTN UE to TN UE in scenario 5 is neglect.

Observation 1: 3dB targeted SINR with UL 5MHz RB allocation can meet the expected UL data rate for handheld UE scenario using NR NTN. 0dB targeted SINR with UL one RB allocation can meet the expected UL data rate for Pedestrian scenario using NB IoT NTN.

Observation 2:

For 26dBm LEO1200 scenario, almost 15% NTN UEs are under power control and 70% NTN UE can achieve 1Mbps UL throughput (And 10% NTN UE achieve the targeted throughput 3.17Mbps).

For 29dBm LEO1200 scenario, almost 35% NTN UEs are under power control and 85% NTN UE can achieve 1Mbps UL throughput (And 28% NTN UE achieve the targeted throughput 3.17Mbps).

For 31dBm LEO1200 scenario, almost 55% NTN UEs are under power control and 92% NTN UE can achieve 1Mbps UL throughput (And 40% NTN UE achieve the targeted throughput 3.17Mbps).

Proposal 3:

To consider the following parameters for NTN UE UL power control

| Parameters | NR NTN for LEO |  |  | NB-IoT NTN for GEO |  |
| --- | --- | --- | --- | --- | --- |
| Pmax Output power (Power Class) | 26dBm (PC2) | 29dBm (PC1.5) | 31dBm (PC1) | 26dBm (PC2) | 31dBm (PC1) |
| Rmin (dB) | -66 | -69 | -71 | -66 | -71 |
| SNRtarget (dB) | 3 | 3 | 3 | 0 | 0 |
| NFSAN (dB) | 4.3 | 4.3 | 4.3 | 7.4 | 7.4 |
| UL BW (MHz) | 5 | 5 | 5 | 0.18 | 0.18 |
| 10log10(kT) (dBm/Hz) | -174 |  |  |  |  |
| CLx-ile (dB) | 125.7 | 128.7 | 130.7 | 140 | 145 |
| γ | 1 | 1 | 1 | 1 | 1 |

Proposal 4: In order to reduce the complexity of simulation, RAN4 can consider a simplified ACLR model assuming four UL NTN aggressor UEs.

For the NTN UE1 which is closest to the victim TN channel, it’s assumed that flat ACLR1 will have impacts on all the Victim UE(s).

For the other NTN UEs except for NTN UE1, it’s assumed that only flat ACLR2 will have impacts on all the Victim UE(s).

- References

[1] RP-240857, New WID: Enhanced requirements and test methodology for NR and IoT NTN, RAN4 chair (Huawei)

[2] R4-2414452, WF on UE RF requirements for NTN HPUE, Samsung

[3] R4-2414276, Simulation assumption for co-existence study, Samsung

[4] TR 36.942, Radio Frequency (RF) system scenarios, RAN4

[5] TS 22.261, Service requirements for the 5G system, SA1