# 3GPP TSG-RAN WG4 Meeting # 112bis	R4-2416060

# Hefei, China, October 14 – 18, 2024

Source: 	Huawei, HiSilicon

Title: 	Discussion and Draft replied LS on FR2-NTN inclusion to the specifications from RAN1 LS

Agenda Item:	7.1

Document for:	Approval

Introduction

In the RAN1#118 meeting, RAN1 sent an LS [1] to RAN4 and RAN2 and request us to check whether any update to the related specifications is needed.

RAN1 has completed the work on RAN1 specification changes to support NR NTN for the frequency bands defined as part of FR2-NTN, and has endorsed several CRs to include FR2-NTN. During this work, RAN1 identified that some RAN1 specifications are still unclear regarding whether FR2-1 or FR2-2 procedure should apply to the FR2-NTN, as the current RAN1 specifications define different behaviour between FR2-1 and FR2-2. The attached CR R1-2407407 has been agreed. Additionally, RAN1 has noticed that some RRC parameters (e.g., subcarrierSpacing in BWP) and UE capabilities (e.g., channelBWs-DL, configuredUL-GrantType1-v1650), which are expected to be applicable to NTN, may have the similar issues, i.e., only reference the FR1 and FR2-1 bands, or TDD-FR1 bands, TDD-FR2-1 bands and TDD-FR2-2 in the RAN2 specification. Furthermore, RAN1 understands that NOTE 2 in Table 5.1-1 of 38.101-5 states that FR2-NTN are regarded as FR2 bands, likely to minimize the impact when referencing RAN1 and RAN2 specifications. RAN1 would like to ask RAN4 to consider whether NOTE 2 may need to be updated. Table 5.1-1: Definition of NTN frequency ranges Frequency range designation; Corresponding frequency range. FR1-NTN1; 410 MHz – 7125 MHz. FR2-NTN2; 17300 MHz – 30000 MHz. NOTE 1: [NTN bands within this frequency range are regarded as a FR1 band when references from other specifications.] NOTE 2: [NTN bands within this frequency range are regarded as a FR2 band when references from other specifications.].

In this paper, we’d like to go through the discussion for the NOTES specified in table 5.1-1 of TS 38.101-5 and reply LS to RAN2 and RAN1.

Discussion

In the RAN4#106bis-e meeting, companies provided a lot of contributions [2~5] to discuss how to specify the NTN frequency range(s). However, in the end, RAN4 reached the following agreement, which was captured in the WF [6].

Sub-topic 1-1 (Issues 1-1-1, 1-1-2, 1-1-3, 1-1-4), updated: Above 10 GHz NTN band definition and related parameters - Starting with option below and further work on the text proposal of “note 1 and note 2”. LS to RAN1/RAN2 can be considered. - It’s not precluded to further extend the frequency ranges in the future if needed and justified. - Agree the definition of NTN frequency ranges as shown in the Proposed Table 5.1-1 to be introduced to TS 38.101-5 and TS 38.108. Proposed Table 5.1-1: Definition of NTN frequency ranges Frequency range designation; Corresponding frequency range. FR1-NTN1; 410 MHz – 7125 MHz. FR2-NTN2; 17300 MHz – 30000 MHz. NOTE 1: [NTN bands within this frequency range are regarded as a FR1 band when references from other specifications.] NOTE 2: [NTN bands within this frequency range are regarded as a FR2 band when references from other specifications.].

However, in the LS [7] which was sent to RAN1/RAN2, RAN4 didn’t mention anything about the text proposal of NOTE1 and NOTE2 in the table 5.1-1 of TS 38.101-5 and TS 38.108.

Although it’s said that RAN4 will further wok on the text proposal of “note 1 and note 2”, RAN4 never discuss this open issue and left it as what we specified in the spec with square brackets.

Observation 1: based on the previous agreements, RAN4 still need some works to update the text proposal of NOTE1 and NOTE 2 in the table 5.1-1 of TS 38.101-5 and TS 38.108.

In the RAN1’s LS [1], it’s clear stated that RAN1 identified that some RAN1 specifications are still unclear regarding whether FR2-1 or FR2-2 procedure should apply to the FR2-NTN, as the current RAN1 specifications define different behaviour between FR2-1 and FR2-2.

Additionally, RAN1 also identified some other RRC parameters issue that FR1-NTN/FR2-NTN bands can’t simply refer to the TN terms “FR1/FR2” in the RAN2 specification.

Additionally, RAN1 has noticed that some RRC parameters (e.g., subcarrierSpacing in BWP) and UE capabilities (e.g., channelBWs-DL, configuredUL-GrantType1-v1650), which are expected to be applicable to NTN, may have the similar issues, i.e., only reference the FR1 and FR2-1 bands, or TDD-FR1 bands, TDD-FR2-1 bands and TDD-FR2-2 in the RAN2 specification.

Thus, from RF’s perspective, it’s very hard to say FR1-NTN/FR2-NTN bands are regarded as the corresponding FR1/FR2 in other specifications of RAN1/RAN2 or other working groups. As we have agreed to use a different suffix for NTN bands, these two NOTEs are meaningless for RF. There is a risk for other working groups that some other features for terrestrial network may automatically apply to the corresponding FR1-NTN/FR2-NTN bands without carefully checking if RAN4 still keep these two NOTEs in RF specification.

Observation 2: if RAN4 still keep these two NOTEs in RF specification, there is a risk for other working groups that some other features for terrestrial network may automatically apply to the corresponding FR1-NTN/FR2-NTN bands without carefully checking.

In order to avoid any confusion and misunderstanding, RAN4 shall remove these two NOTES in the table 5.1-1 of TS 38.101-5 and TS 38.108. And please other working groups carefully check whether FR1-NTN/FR2-NTN clarification in the corresponding specification are needed or not.

Proposal 1: In order to avoid any confusion and misunderstanding, RAN4 shall remove these two NOTES in the table 5.1-1 of TS 38.101-5 and TS 38.108.

Summary

Observation 1: based on the previous agreements, RAN4 still need some works to update the text proposal of NOTE1 and NOTE 2 in the table 5.1-1 of TS 38.101-5 and TS 38.108.

Observation 2: if RAN4 still keep these two NOTEs in RF specification, there is a risk for other working groups that some other features for terrestrial network may automatically apply to the corresponding FR1-NTN/FR2-NTN bands without carefully checking.

Proposal 1: In order to avoid any confusion and misunderstanding, RAN4 shall remove these two NOTES in the table 5.1-1 of TS 38.101-5 and TS 38.108.

- References

[1] R1-2407406, LS on FR2-NTN inclusion to specifications, RAN1

[2] R4-2305831, Discussion on system parameters for above 10 GHz, THALES

[3] R4-2304567, NTN enhancement: System parameters, Ericsson

[4] R4-2304439, Further discussion on NTN system parameters, CATT

[5] R4-2304938, Discussion on above 10GHz NTN bands, Nokia, Nokia Shanghai Bell

[6] R4-2305925, WF for system parameters of above 10 GHz NTN band, Moderator (THALES)

[7] R4-2305926, LS on the system parameters for NTN above 10 GHz, RAN4

- Annex

Title:	Replied LS on FR2-NTN inclusion to specifications

Response to: 	R1-2407406

Release:	Release 18

Work Item:	NR_NTN_enh-Core

Source:	RAN4

To:	RAN1, RAN2

Cc:

Contact Person:

Name:	   Peng Zhang

Email Address:  zhangpeng169@huawei.com

Send any reply LS to:	 3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

- Overall description

RAN4 thanks RAN1 has completed the work on RAN1 specification changes to support NR NTN for the frequency bands defined as part of FR2-NTN and sent LS to RAN4. RAN4 is aware that it’s very difficult and risky to regard FR1-NTN/FR2-NTN bands as FR1/FR2 bands in the other working group’s specification, even if the purpose is to minimize the impact when referencing other working groups’ specifications.

Thus, RAN4 has agreed to remove the “NOTE1 and NOTE2” in the table 5.1-1 of TS 38.101-5 and TS 38.108 below. In order to avoid any confusion and misunderstanding, RAN4 kindly request RAN1 and RAN2 to carefully check the applicability for FR1-NTN/FR2-NTN and clarify them in the corresponding specifications after RAN4 remove these two NOTEs in the table 5.1-1 of TS 38.101-5 and TS 38.108.

Table 5.1-1: Definition of NTN frequency ranges

| Frequency range designation | Corresponding frequency range |
| --- | --- |
| FR1-NTN1 | 410 MHz – 7125 MHz |
| FR2-NTN2 | 17300 MHz – 30000 MHz |
| NOTE 1: Void. NOTE 2: Void. |  |

- Actions

To RAN WG1 and RAN WG2:

ACTION: RAN4 respectfully request RAN1 and RAN2 to take above information into account for their future work and to carefully check the applicability for FR1-NTN/FR2-NTN in the specifications. If any ambiguities are identified, please kindly clarify the applicability in the specifications.

- Dates of Next TSG-RAN WG1

TSG RAN WG1 Meeting #112bis         14th – 18th October, 2024         Hefei, China

TSG RAN WG1 Meeting #113            18th – 22nd November, 2024      Orlando, USA