3GPP TSG-RAN WG4 Meeting #112bis									R4-2416063

Hefei, China, 14 – 18 October, 2024

Agenda item:	6.12.2.1

Source:	CAICT

Title:	Discussion on XR OTA test method

Document for:	Approval

1	Introduction

In the last meeting, test scenarios and OTA test time reduction for head-worn XR devices have been discussed and relevant conclusions are captured in the WF [1]. In this contribution, we provide our further views on Rel-19 XR OTA test methodology.

2	Discussion

2.1 Device type and performance metric for XR OTA

Issue 2-1-1: Discussions on Reply LS from CTIA on XR OTA test phantom Agreements: - Two types of XR, i.e., glasses and head mounted display are prioritized in this WI. - Need to send this two XR types to CTIA for phantom positioning and MU study. Issue 2-1-4: Whether WI should focus on the XR devices those connected to gNB directly Agreements: - Only test XR devices with 5G link to gNB in networks, not those with side link or wired connections. Issue 2-1-6: Prioritize 1Tx XR devices? Agreements: - Prioritize 1Tx XR

Based on the discussions from the last meeting, priority will be given to 1Tx XR devices. Device types are mainly categorized into glasses and head-mounted displays. Additionally, the WI scope will only include XR devices with a 5G link to gNB in networks, excluding those with sidelink or wired connections. Given this information, it is clear that the performance metrics for XR OTA should reuse those of smartphones, specifically testing 1Tx TRP and TRS.

Additionally, the size of head-mounted displays can vary based on the model and design, and they are generally larger than XR glasses. Many HMDs may have a diameter between 20 and 30 centimeters, but it is uncertain whether some devices might exceed this range. It needs to be further confirmed whether these devices can be fully accommodated within the 30cm quiet zone of the chamber when mounted on a head phantom, otherwise a larger quiet zone size needs to be declared to meet the testing conditions. Feedback and confirmation from vendors will be helpful in this regard.

Proposal 1: Whether all XR devices can be fully accommodate within 30cm QZ when mounted on a head phantom needs further confirmation, otherwise a larger quiet zone size needs to be declared to meet the testing conditions. Feedback from vendors will be helpful.

- 2 Testing time reduction for XR OTA

In OTA testing of XR devices, a major challenge is the limitation of the device's battery capacity, which prevents the device under test from completing comprehensive TRP/TRS tests. In previous meetings, several potential solutions were proposed as below.

Issue 2-2-1: Coarse measurement grids Agreements: The following Options can be further discussed next meeting: Option 1: Stop the measurement in the middle and charge the battery, carefully re-place the XR device into the Chamber to continue the test. Option 2: Test TRP at [X] dB below the maximum transmit power and scale up [X] dB to obtain TRP at maximum transmit power. Options 3: - Measure TRP: Averaging the measurement results from 4 of interpolated 62-points measurement grids by TRPave=10log⁡(10TRP1/10+10TRP2/10+10TRP3/10+10TRP4/104) TRSave=10log4/(110TRS110+110TRS210+110TRS310+110TRS410 to achieve the same accuracy level of using 266-points measurement grids during OTA tests. (Define 4 of 62-points interpolated measurement grids based on 266-points measurement grids at the same time, the locations of each four of 62-points interpolated measurement grids is different from each other on the sphere, they can be considered orthogonal to each other.) - Measure Peak EIPR: Full sphere coarse girds measurement + partial fine grids measurement (For example, using 62-points measurement grids in OTA test to measure peak EIPR first, then based on the direction of peak EIPR, apply corresponding sectors (pre-defined 8 or 16 sectors) of 266-points measurement grids to measure the peak EIPR again to increase the accuracy of measured peak EIRP)

The solution for Option 1 is to stop the measurement midway to charge the battery, then carefully reposition the XR device in the chamber to continue testing. However, since it is not guaranteed that the DUT will be placed in the exact same place and orientation as before, factors such as operator handling and DUT holder positioning may introduce additional uncertainty. Therefore, further assessment of its impact on testing accuracy is necessary, one possible solution is to introduce a separate measurement uncertainty factor to account for this effect.

Proposal 2: Due to the occurrence of repositioning, option 1 will introduce additional uncertainty to the test results, and it is necessary to further evaluate its impact on test accuracy.

For Option 2 (low-power testing approach), it is important to consider the impact of PMAX settings on the measurement results. In traditional full-power TRP tests with SA mode, it does not require PMAX configurations, instead, it send continuously uplink power control "up" commands to allow UE reach PUMAX level. Considering the existence of Test Tolerance, the actual transmission power may exceed the standard power class (without TT).

When using maximum transmit power minus XdB for TRP testing, PMAX must be manually configured. Considering TT and other factors, the TRP at maximum transmission power cannot be simply obtained by scale up XdB with low-power TRP. To address this issue, we propose to first perform 3D scanning at a lower power level to obtain TRPlow_power, while performing SPOT testing to infer TRPmax_power at maximum transmission power based on the offset between single point EIRP testing at low power and full power. This approach accounts for the impacts of Pmax configurations and provides a more accurate estimation of TRP at maximum transmission power.

Proposal 3: Conduct a 3D scan at a lower power level to obtain TRPlow_power. Subsequently, perform SPOT testing to derive TRPmax_power at maximum transmit power by analyzing the offset between single-point EIRP measurements taken at low power and full power.

3	Conclusion

This contribution makes the following observations and proposals.

Proposal 1: Whether all XR devices can be fully accommodate within 30cm QZ when mounted on a head phantom needs further confirmation, otherwise a larger quiet zone size needs to be declared to meet the testing conditions. Feedback from vendors will be helpful.

Proposal 2: Due to the occurrence of repositioning, option 1 will introduce additional uncertainty to the test results, and it is necessary to further evaluate its impact on test accuracy.

Proposal 3: Conduct a 3D scan at a lower power level to obtain TRPlow_power. Subsequently, perform SPOT testing to derive TRPmax_power at maximum transmit power by analyzing the offset between single-point EIRP measurements taken at low power and full power.

References

- R4-2413603, WF for [112][333] TRP_TRS_MIMO_OTA, vivo