3GPP TSG-RAN WG4 Meeting #112bis									R4-2416064

Hefei, China, 14 – 18 October, 2024

Agenda item:	6.12.3.1

Source:	CAICT

Title:	Discussion on TRP TRS requirements

Document for:	Approval

1	Introduction

In the last meeting, framework to specify TRP TRS requirements for 2Tx UEs and Size 2 UEs have been discussed and relevant conclusions are captured in the WF [1]. In this contribution, we provide our views on Rel-19 TRP TRS requirements.

2	Discussion

2.1 2Tx requirements for TRP and TRS

Due to the high workload of Rel-18 and the limited availability of commercial devices, the lab alignment and measurement campaign for 2Tx devices have been postponed in Rel-18. This work is included in the performance part of the Rel-19 OTA WI based on the availability of measurement data. According to the previous WF [1], AC lab alignment for non-coherent UL MIMO at n41/n78 should take place in Rel-19, with a framework similar to Rel-18 [2].

Issue 5-1-2: 2Tx requirements for TRP TRS - Proposals - Proposal 1: AC lab alignment for non-coherent UL MIMO at n41/n78 should be done in Rel-19, the framework can be similar to Rel-18 [5], encourage test labs to join this activity. (vivo) - Recommended WF - TBA

As per the established procedure in Rel-18, the minimum sample number required for each band each device size during the measurement campaign is ~40, and these must be commercially available devices. Given the current market conditions, there is a risk that the number of devices supporting non-coherent UL MIMO may not meet this requirement. It is important to identify the current availability of commercial devices that support non-coherent UL MIMO, which is crucial for planning the measurement campaign. Feedback from vendors is essential for gathering information on available devices.

Observation 1: The existing minimum sample size requirement [~40] may not be achievable for 2Tx devices given the current market situation.

Proposal 1: Confirm the feasibility of obtaining a sufficient number of commercial devices that support non-coherent UL MIMO. Encourage vendors to provide input and support, facilitating the gathering of relevant data on available devices.

If the minimum sample number cannot be achieved, it is essential to discuss potential options for adjusting the requirements. This could include evaluating whether a reduction in the minimum sample size is feasible.

Proposal 2: Discuss the feasibility of relaxing the minimum sample size requirement if the current minimum sample size cannot be met.

2.2 Size 2 TRP TRS requirements (PDA hand)

Issue 5-1-3: Size 2 TRP TRS requirements (PDA hand) Agreements: - RAN4 further discuss how to specify size 2 UE TRP TRS requirements.

According to the WF of the last meeting, RAN4 should further discuss how to specify size 2 UE TRP TRS requirements. Two potential approaches are summarized below:

Option 1: Measurement-based approach (traditional method, widely used for OTA requirements specification)

Option 2: Offset-based approach (efficiency-driven, deriving Size 2 requirements based on Size 1 values for the same band)

It is essential to emphasize that the OTA requirements have traditionally been derived using a measurement-based approach. This methodology has proven effective in establishing accurate requirements across different devices, bands and scenarios.

Observation 2: OTA requirements have traditionally relied on a measurement-based approach, proven effective across various scenarios.

While it is recognized that there are unique cases, such as requirements for different CBW and power classes, where additional measurements may not be necessary, these exceptions rely on the existence of a derivable offset. For example, a fixed power gap exists between different power levels, and TRS performance across varying CBW can be scaled using REFENS RB factors, which is also validated through measurements. However, this does not apply to the relationship between Size 1 and Size 2 handheld UEs, as there is no intuitive or derivable theoretical relationship between their OTA performances.

From a device size perspective, larger devices can offer more space for antenna design. This typically implies that antenna efficiency could be better, or there could be more antennas available for selection, meaning that Size 1 devices may have better OTA performance compared to Size 2 devices. However, this assumption holds true only if both device sizes support the same frequency bands. If the larger device needs to support more frequency bands, the optimization space for each individual band may not necessarily improve.

Furthermore, considering the impact of phantoms, Size 1 devices use the wide hand phantom, whereas Size 2 devices use the PDA hand. These two phantoms may introduce different blocking effects on antenna radiation, which adds more uncertainty to the relationship between the OTA performances of the two device sizes.

Observation 3: There is currently a lack of reliable studies confirming a fixed offset between Size 1 and Size 2 UEs.

Currently, 3GPP lacks reliable studies to confirm a fixed offset between the OTA performance of Size 1 and Size 2 handheld UEs. Moreover, it remains unclear whether the offsets (if have) across different frequency bands are consistent or not. Given this uncertainty, it is critical to conduct measurements to ascertain the OTA performance levels of Size 1 and Size 2 devices. This will help determine whether Size 2 performance can be considered equivalent to Size 1, or if a measurable offset exists.

Observation 4: Without establishing a clear relationship between Size 1 and Size 2 performances, the offset-based approach may not be justified.

Therefore, regardless of the workload implications, these initial measurement steps are indispensable. If a sufficient number of measurement can demonstrate that Size 2 OTA performance aligns with that of Size 1 devices, or if a consistent offset is observed, we can then appropriately discuss the feasibility of an offset-based approach. It is only through rigorous testing and data collection that we can ensure the reliability and validity of our specifications for Size 2 handheld UEs.

Proposal 3: Initiate a preliminary measurement campaign for size 2 UEs, FFS how to down select between measurement-based approach (Option 1) and the offset-based approach (Option 2) based on the measurement results.

3	Conclusion

This contribution makes the following observations and proposals.

Observation 1: The existing minimum sample size requirement [~40] may not be achievable for 2Tx devices given the current market situation.

Proposal 1: Confirm the feasibility of obtaining a sufficient number of commercial devices that support non-coherent UL MIMO. Encourage vendors to provide input and support, facilitating the gathering of relevant data on available devices.

Proposal 2: Discuss the feasibility of relaxing the minimum sample size requirement if the current minimum sample size cannot be met.

Observation 2: OTA requirements have traditionally relied on a measurement-based approach, proven effective across various scenarios.

Observation 3: There is currently a lack of reliable studies confirming a fixed offset between Size 1 and Size 2 UEs.

Observation 4: Without establishing a clear relationship between Size 1 and Size 2 performances, the offset-based approach may not be justified.

Proposal 3: Initiate a preliminary measurement campaign for size 2 UEs, FFS how to down select between measurement-based approach (Option 1) and the offset-based approach (Option 2) based on the measurement results.

References

- R4-2413603, WF for [112][333] TRP_TRS_MIMO_OTA, vivo

- R4-2316995 , Further updated working procedure for Rel-18 lab alignment and harmonization campaign, vivo