3GPP TSG-RAN4 Meeting #112bis	R4-2416066

# Hefei, Anhui, China, 14th – 18th October, 2024

Source:	LG Electronics

Title:	Tx requirements for SL intra-band non-contiguous CA

Agenda item:	6.3.2.2

Document for:	Discussion

# Introduction

In this paper, we provide the updated simulation result on MPR for intra-band non-contiguous SL CA for the Band n47 based on the approved WFs[1, 2] and our view on the MPR requirements.

# Discussion

Based on the approved WFs [1][2], we would like to discuss the UE RF requirements for SL intra-band non-contiguous CA in n47.

General - To capture the intra-band contiguous and non-contiguous SL CA bands under the same sub-clause SCS - All the requirements defined for PC2/PC3 SL intra-band NC CA shall apply under the assumption of the same subcarrier spacing for SL CA Architecture for MPR/A-MPR evaluation for PC2 - Do evaluations based assumptions of 1LO and 2LO - Consider the following architecture for evaluation Arch; description; Remarks. #1-1; 1x26dBm PA+ 1LO; Single–Tx. #1-2; 2x23dBm PA+ 1LO; dual-Tx or txDiversity. #2-2; 2x23dBm PA+ 2LO; dualPA-Architecture. PC3 with 2Tx - use PSSCH/PSCCH with 2x20dBm PA for PC3 sidelink non-contiguous CA evaluation Frequency gap - Consider the worst case for BW gap size between sub-block #1 and #2 for the MPR requirement. - For evaluations, consider all the gap size. Regulation (FCC) - For FCC regulation, work is based on current NS_52 first and wait for FCC update. Tx requirements - Use proposal 1 as the starting point. - Proposal 1: The work in Rel-19 NR SL intra-band NC CA with PC3 and PC2 would include at least - Maximum output power - MPR for PC2 and PC3 - Configured output power - Update output power dynamic, transmit signal quality, out of band emission, spurious emissions Rx requirements - Use proposal 1 as the starting point. - Proposal 1: The description of the following Rx requirements in the specification should be updated from that only for SL intra-band C CA to also accommodate the requirements for SL intra-band NC CA - REFSENS - Maximum input level, - ACS locking - In-band/ out-of-band blocking - Spurious response - Wide band Intermodulation Simulation assumption - Agree on below table for simulation assumption - The carrier leakage can be further discussed Center frequency; 5.9GHz. Bandwidth; per CC: 10/20MHz aggregated CBW: 10+10MHz/ 10+20MHz. Maximum output power for aggregated CBW; 23dBm/26dBm. Numerology; 15 kHz/30kHz/60kHz. Modulation per CC; QPSK/16QAM/64QAM/256QAM. Waveform; CP-OFDM. ACLR; 30dBc(PC3) / 31dBc(PC2). Carrier leakage; 25dBc. IQ image; 34dBc. CIM3; 60dBc. PA calibration; PA calibrated to deliver 30dB ACLR for a fully allocated RBs in 20MHz QPSK DFT- S-OFDM waveform at 1 dB MPR. This is based to share PA between LTE V2X and NR V2X at 5.9GHz as worst case.. MPR - Define MPR to meet -13dBm/MHz and MPR to meet -30dBm/MHz as NR intra-band non-contiguous CA - Consider B = (LCRB1* 12* SCS1 + LCRB2 * 12 * SCS2)/1,000 for PSSCH as NR intra-band non-contiguous CA

## PC2 MPR (PSSCH)

### UE RF architecture

UE RF architecture in Table 2-1 is considered for MPR evaluation.

Table 2-1. UE Architecture for PC2 SL non-contiguous CA

| Arch | description | Remarks |
| --- | --- | --- |
| #1-1 | 1x26dBm PA+ 1LO | Single–Tx |
| #1-2 | 2x23dBm PA+ 1LO | dual-Tx or txDiversity |
| #2-2 | 2x23dBm PA+ 2LO | dualPA-Architecture |

### Simulation assumption

The simulation assumption in Table 2-2 is considered.

Table 2-2. Simulation assumption

| Center frequency | 5.9GHz |
| --- | --- |
| Bandwidth | per CC: 10MHz/20MHz aggregated CBW: 10MHz +10 MHz/ 10MHz+20MHz |
| Aggregated CBW | 10MHz + Gap10MHz + 10MHz 10MHz + Gap50MHz + 10MHz 10MHz + Gap10MHz + 20MHz 10MHz + Gap40MHz + 20MHz |
| Maximum output power for aggregated CBW | 26dBm |
| Numerology | 15 kHz |
| Modulation per CC | QPSK/16QAM/64QAM/256QAM |
| Waveform | CP-OFDM |
| ACLR | 31dBc |
| Carrier leakage | 25dBc |
| IQ image | 34dBc |
| CIM3 | 60dBc |
| PA calibration | PA calibrated to deliver 30dBc ACLR for a fully allocated RBs in 20MHz QPSK DFT- S-OFDM waveform at 1 dB MPR. This is based to share PA between LTE V2X and NR V2X at 5.9GHz as worst case. |

### Evaluation scenario

The MPR evaluation scenario in Table 2-3 is considered.

Table 2-3: PSSCH SL intra-band NC CA MPR evaluation scenarios

| Aggregated CBW | Scenario | CC1 | CC2 | B | Frequency of FIM3,low_block,low & FIM3,high_block,high | SCS1 (= SCS2) |
| --- | --- | --- | --- | --- | --- | --- |
| 10MHz + Gap10MHz+10MHz | 1 | 10RB0 | 10RB42 | 3.6 | SEfreq_-30 | 15 |
|  | 2 | 10RB34 | 10RB8 | 3.6 | SEMfreq_-13 | 15 |
|  | 3 | 12RB0 | 12RB40 | 4.32 | SEfreq_-30 | 15 |
|  | 4 | 12RB33 | 12RB7 | 4.32 | SEMfreq_-13 | 15 |
|  | 5 | 15RB0 | 15RB37 | 5.4 | SEfreq_-30 | 15 |
|  | 6 | 15RB31 | 15RB5 | 5.4 | SEMfreq_-13 | 15 |
|  | 7 | 20RB0 | 20RB32 | 7.2 | SEfreq_-30 | 15 |
|  | 8 | 20RB29 | 20RB3 | 7.2 | SEMfreq_-13 | 15 |
|  | 9 | 25RB0 | 25RB27 | 9.0 | SEfreq_-30 | 15 |
|  | 10 | 25RB26 | 25RB0 | 9.0 | SEMfreq_-25 | 15 |
|  | 11 | 30RB0 | 30RB22 | 10.8 | SEfreq_-30 | 15 |
|  | 12 | 30RB22 | 30RB0 | 10.8 | SEMfreq_-25 | 15 |
|  | 13 | 36RB0 | 36RB16 | 12.96 | SEfreq_-30 | 15 |
|  | 14 | 36RB16 | 36RB0 | 12.96 | SEMfreq_-25 | 15 |
|  | 15 | 40RB0 | 40RB12 | 14.4 | SEfreq_-30 | 15 |
|  | 16 | 40RB12 | 40RB0 | 14.4 | SEfreq_-30 | 15 |
|  | 17 | 45RB0 | 45RB7 | 16.2 | SEfreq_-30 | 15 |
|  | 18 | 45RB7 | 45RB0 | 16.2 | SEfreq_-30 | 15 |
|  | 19 | 50RB0 | 50RB2 | 18.0 | SEfreq_-30 | 15 |
|  | 20 | 50RB2 | 50RB0 | 18.0 | SEfreq_-30 | 15 |
| 10MHz + Gap50MHz+10MHz | 21 | 10RB0 | 10RB42 | 3.6 | SEfreq_-30 | 15 |
|  | 22 | 10RB42 | 10RB8 | 3.6 | SEfreq_-30 | 15 |
|  | 23 | 12RB0 | 12RB40 | 4.32 | SEfreq_-30 | 15 |
|  | 24 | 12RB40 | 12RB0 | 4.32 | SEfreq_-30 | 15 |
|  | 25 | 15RB0 | 15RB37 | 5.4 | SEfreq_-30 | 15 |
|  | 26 | 15RB37 | 15RB0 | 5.4 | SEfreq_-30 | 15 |
|  | 27 | 20RB0 | 20RB32 | 7.2 | SEfreq_-30 | 15 |
|  | 28 | 20RB32 | 20RB0 | 7.2 | SEfreq_-30 | 15 |
|  | 29 | 25RB0 | 25RB27 | 9.0 | SEfreq_-30 | 15 |
|  | 30 | 25RB27 | 25RB0 | 9.0 | SEfreq_-30 | 15 |
|  | 31 | 30RB0 | 30RB22 | 10.8 | SEfreq_-30 | 15 |
|  | 32 | 30RB22 | 30RB0 | 10.8 | SEfreq_-30 | 15 |
|  | 33 | 36RB0 | 36RB16 | 12.96 | SEfreq_-30 | 15 |
|  | 34 | 36RB16 | 36RB0 | 12.96 | SEfreq_-30 | 15 |
|  | 35 | 40RB0 | 40RB12 | 14.4 | SEfreq_-30 | 15 |
|  | 36 | 40RB12 | 40RB0 | 14.4 | SEfreq_-30 | 15 |
|  | 37 | 45RB0 | 45RB7 | 16.2 | SEfreq_-30 | 15 |
|  | 38 | 45RB7 | 45RB0 | 16.2 | SEfreq_-30 | 15 |
|  | 39 | 50RB0 | 50RB2 | 18.0 | SEfreq_-30 | 15 |
|  | 40 | 50RB2 | 50RB0 | 18.0 | SEfreq_-30 | 15 |
| 10MHz + Gap10MHz+20MHz | 41 | 10RB0 | 10RB96 | 3.6 | SEfreq_-30 | 15 |
|  | 42 | 10RB42 | 10RB0 | 3.6 | SEfreq_-13 | 15 |
|  | 43 | 12RB0 | 12RB94 | 4.32 | SEfreq_-30 | 15 |
|  | 44 | 12RB40 | 12RB0 | 4.32 | SEfreq_-13 | 15 |
|  | 45 | 15RB0 | 15RB91 | 5.4 | SEfreq_-30 | 15 |
|  | 46 | 15RB37 | 15RB0 | 5.4 | SEfreq_-13 | 15 |
|  | 47 | 20RB0 | 20RB86 | 7.2 | SEfreq_-30 | 15 |
|  | 48 | 20RB32 | 20RB0 | 7.2 | SEfreq_-13 | 15 |
|  | 49 | 25RB0 | 25RB81 | 9.0 | SEfreq_-30 | 15 |
|  | 50 | 25RB27 | 25RB0 | 9.0 | SEfreq_-25 | 15 |
|  | 51 | 30RB0 | 30RB76 | 10.8 | SEfreq_-30 | 15 |
|  | 52 | 30RB22 | 30RB0 | 10.8 | SEfreq_-25 | 15 |
|  | 53 | 36RB0 | 36RB70 | 12.96 | SEfreq_-30 | 15 |
|  | 54 | 36RB16 | 36RB0 | 12.96 | SEfreq_-25 | 15 |
|  | 55 | 40RB0 | 40RB66 | 14.4 | SEfreq_-30 | 15 |
|  | 56 | 40RB12 | 40RB0 | 14.4 | SEfreq_-30 | 15 |
|  | 57 | 45RB0 | 45RB61 | 16.2 | SEfreq_-30 | 15 |
|  | 58 | 45RB7 | 45RB0 | 16.2 | SEfreq_-30 | 15 |
|  | 59 | 50RB0 | 50RB56 | 18.0 | SEfreq_-30 | 15 |
|  | 60 | 50RB2 | 50RB0 | 18.0 | SEfreq_-30 | 15 |
|  | 61 | 50RB0 | 70RB36 | 21.6 | SEfreq_-30 | 15 |
|  | 62 | 50RB2 | 70RB0 | 21.6 | SEfreq_-30 | 15 |
|  | 63 | 50RB0 | 90RB16 | 25.2 | SEfreq_-30 | 15 |
|  | 64 | 50RB2 | 90RB0 | 25.2 | SEfreq_-30 | 15 |
|  | 65 | 50RB0 | 105RB1 | 27.9 | SEfreq_-30 | 15 |
|  | 66 | 50RB2 | 105RB0 | 27.9 | SEfreq_-30 | 15 |
| 10MHz + Gap40MHz+20MHz | 67 | 10RB0 | 10RB96 | 3.6 | SEfreq_-30 | 15 |
|  | 68 | 10RB42 | 10RB0 | 3.6 | SEfreq_-30 | 15 |
|  | 69 | 12RB0 | 12RB40 | 4.32 | SEfreq_-30 | 15 |
|  | 70 | 12RB40 | 12RB0 | 4.32 | SEfreq_-30 | 15 |
|  | 71 | 15RB0 | 15RB91 | 5.4 | SEfreq_-30 | 15 |
|  | 72 | 15RB37 | 15RB0 | 5.4 | SEfreq_-30 | 15 |
|  | 73 | 20RB0 | 20RB32 | 7.2 | SEfreq_-30 | 15 |
|  | 74 | 20RB32 | 20RB0 | 7.2 | SEfreq_-30 | 15 |
|  | 75 | 25RB0 | 25RB27 | 9.0 | SEfreq_-30 | 15 |
|  | 76 | 25RB27 | 25RB0 | 9.0 | SEfreq_-30 | 15 |
|  | 77 | 30RB0 | 30RB22 | 10.8 | SEfreq_-30 | 15 |
|  | 78 | 30RB22 | 30RB0 | 10.8 | SEfreq_-30 | 15 |
|  | 79 | 36RB0 | 36RB16 | 12.96 | SEfreq_-30 | 15 |
|  | 80 | 36RB16 | 36RB0 | 12.96 | SEfreq_-30 | 15 |
|  | 81 | 40RB0 | 40RB12 | 14.4 | SEfreq_-30 | 15 |
|  | 82 | 40RB12 | 40RB0 | 14.4 | SEfreq_-30 | 15 |
|  | 83 | 45RB0 | 45RB7 | 16.2 | SEfreq_-30 | 15 |
|  | 84 | 45RB7 | 45RB0 | 16.2 | SEfreq_-30 | 15 |
|  | 85 | 50RB0 | 50RB2 | 18.0 | SEfreq_-30 | 15 |
|  | 86 | 50RB2 | 50RB0 | 18.0 | SEfreq_-30 | 15 |
|  | 87 | 50RB0 | 70RB36 | 21.6 | SEfreq_-30 | 15 |
|  | 88 | 50RB2 | 70RB0 | 21.6 | SEfreq_-30 | 15 |
|  | 89 | 50RB0 | 90RB16 | 25.2 | SEfreq_-30 | 15 |
|  | 90 | 50RB2 | 90RB0 | 25.2 | SEfreq_-30 | 15 |
|  | 91 | 50RB0 | 105RB1 | 27.9 | SEfreq_-30 | 15 |
|  | 92 | 50RB2 | 105RB0 | 27.9 | SEfreq_-30 | 15 |

Here, the aggregated CA bandwidth is CA bandwidth class B, which is composed of 2 CCs (CC1 + CC2).

-	LCRB1 is for CC1 which is the component carrier with lower frequency

-	LCRB2 is for CC2 which is the component carrier with higher frequency

-	B =  (LCRB1* 12* SCS1 + LCRB2 * 12 * SCS2)/1,000

-	SCS1 is subcarrier spacing for CC1

-	SCS2 is subcarrier spacing for CC2

-	FIM3,high_block,high = (2 * Fhigh_alloc,high_edge ) – Flow_alloc,low_edge

-	FIM3,low_block,low = (2 * Flow_alloc,low_edge) – Fhigh_alloc,high_edge

-	Flow_alloc,low_edge is the lowermost frequency of the lower transmission bandwidth allocation.

-	Flow_alloc,high_edge is the uppermost frequency of the lower transmission bandwidth allocation.

-	Fhigh_alloc,low_edge is the lowermost frequency of the upper transmission bandwidth allocation.

-	Fhigh_alloc,high_edge is the uppermost frequency of the upper transmission bandwidth allocation.

-	SEM-13,low = Threshold frequency where lower spectral emission mask below the lower channel drops from -13 dBm / MHz to -25 dBm / MHz, as specified in Clause 6.5A.2.2.2 of TS38.101-1.

-	SEM-13,high = Threshold frequency where upper spectral emission mask above the upper channel drops from -13 dBm / MHz to -25 dBm / MHz, as specified in Clause 6.5A.2.2.2 of TS38.101-1.

-	SEMfreq_-13 = (SEM-13,low < FIM3,low_block,low ) and  (FIM3,high_block,high < SEM-13,high )

-	SEfreq_-30  = (FIM3,low_block,low ≤ SEM-25low ) and  (SEM-25,high ≤ FIM3,high_block,high )

Figure 3-1, and Figure 2-2 show the example of Frequency for FIM3,low_block,low & FIM3,high_block,high, such as, SEMfreq_-13, and SEfreq_-30 respectively.

Figure 2-1: Example of SEMfreq_-13 for Frequency of FIM3,low_block,low & FIM3,high_block,high

Figure 2-2: Example of SEMfreq_-30 for Frequency of FIM3,low_block,low & FIM3,high_block,high

### Simulation results

Table 2-4, Table 2-5, and Table 2-6 show the MPR simulation results for the SL non-contiguous CA with architecture #1-1, #1-2, and #2-2 in Table 2-1 respectively.

Table 2-4: PSSCH/PSCCH MPR simulation results for SL Non-contiguous CA with 1x26dBm+1LO

| '10MHz+G10MHz+10MHz' | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 'QPSK' | 16.2 | 7.4 | 15.7 | 6.9 | 14.8 | 6.4 | 13.9 | 6.0 | 13.4 | 5.5 |  |  |  |  |  |  |  |  |
|  | '16QAM' | 16.2 | 7.4 | 15.7 | 6.9 | 14.8 | 6.4 | 13.9 | 6.0 | 13.4 | 5.5 |  |  |  |  |  |  |  |  |
|  | '64QAM' | 16.2 | 7.4 | 15.7 | 6.9 | 14.8 | 6.4 | 14.4 | 6.0 | 13.4 | 5.5 |  |  |  |  |  |  |  |  |
|  | '256QAM' | 16.2 | 7.4 | 15.7 | 6.9 | 15.3 | 6.4 | 13.9 | 6.0 | 13.4 | 5.5 |  |  |  |  |  |  |  |  |
|  | Scenario # | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 |  |  |  |  |  |  |  |  |
|  | 'QPSK' | 13.0 | 7.8 | 12.5 | 8.8 | 12.0 | 10.2 | 11.6 | 11.1 | 11.6 | 11.1 |  |  |  |  |  |  |  |  |
|  | '16QAM' | 13.0 | 7.4 | 12.5 | 8.8 | 12.1 | 10.2 | 11.6 | 11.1 | 11.1 | 11.1 |  |  |  |  |  |  |  |  |
|  | '64QAM' | 13.0 | 7.4 | 12.5 | 8.8 | 12.0 | 10.2 | 11.6 | 11.1 | 11.1 | 11.1 |  |  |  |  |  |  |  |  |
|  | '256QAM' | 13.4 | 7.8 | 13.9 | 8.8 | 13.4 | 13.4 | 13.4 | 13.0 | 13.4 | 13.0 |  |  |  |  |  |  |  |  |
| '10MHz+G50MHz+10MHz' | Scenario # | #21 | #22 | #23 | #24 | #25 | #26 | #27 | #28 | #29 | #30 |  |  |  |  |  |  |  |  |
|  | 'QPSK' | 16.2 | 16.2 | 15.7 | 15.7 | 14.8 | 14.8 | 13.9 | 13.9 | 13.4 | 13.4 |  |  |  |  |  |  |  |  |
|  | '16QAM' | 16.2 | 16.2 | 15.7 | 15.7 | 14.8 | 14.8 | 13.9 | 13.9 | 13.4 | 13.4 |  |  |  |  |  |  |  |  |
|  | '64QAM' | 16.2 | 16.2 | 15.7 | 15.7 | 14.8 | 14.8 | 14.4 | 13.9 | 13.4 | 13.4 |  |  |  |  |  |  |  |  |
|  | '256QAM' | 16.2 | 16.2 | 15.7 | 15.7 | 14.8 | 14.8 | 13.9 | 13.9 | 13.4 | 13.4 |  |  |  |  |  |  |  |  |
|  | Scenario # | #31 | #32 | #33 | #34 | #35 | #36 | #37 | #38 | #39 | #40 |  |  |  |  |  |  |  |  |
|  | 'QPSK' | 13.0 | 13.0 | 12.5 | 12.5 | 12.0 | 12.1 | 11.6 | 11.6 | 11.1 | 11.1 |  |  |  |  |  |  |  |  |
|  | '16QAM' | 13.0 | 13.0 | 12.5 | 12.5 | 12.1 | 12.1 | 11.6 | 11.6 | 11.1 | 11.6 |  |  |  |  |  |  |  |  |
|  | '64QAM' | 13.0 | 13.0 | 12.5 | 12.5 | 12.0 | 12.0 | 11.6 | 11.6 | 11.6 | 11.1 |  |  |  |  |  |  |  |  |
|  | '256QAM' | 13.0 | 13.0 | 13.9 | 13.4 | 13.4 | 13.4 | 13.0 | 13.4 | 13.0 | 13.0 |  |  |  |  |  |  |  |  |
| '10MHz+G10MHz+20MHz' | Scenario # | #41 | #42 | #43 | #44 | #45 | #46 | #47 | #48 | #49 | #50 | #51 | #52 | #53 |  |  |  |  |  |
|  | 'QPSK' | 16.2 | 7.4 | 15.7 | 6.9 | 14.8 | 6.4 | 14.4 | 6.0 | 13.4 | 5.5 | 13.0 | 7.8 | 12.5 |  |  |  |  |  |
|  | '16QAM' | 16.7 | 7.4 | 15.7 | 6.9 | 14.8 | 6.4 | 13.9 | 6.0 | 13.4 | 5.5 | 13.0 | 8.3 | 12.5 |  |  |  |  |  |
|  | '64QAM' | 16.2 | 7.4 | 15.7 | 6.9 | 14.8 | 6.4 | 14.4 | 6.0 | 13.4 | 5.5 | 13.0 | 7.8 | 12.5 |  |  |  |  |  |
|  | '256QAM' | 16.2 | 7.4 | 15.7 | 6.9 | 14.8 | 6.4 | 13.9 | 6.0 | 13.4 | 5.5 | 13.0 | 7.8 | 14.3 |  |  |  |  |  |
|  | Scenario # | #54 | #55 | #56 | #57 | #58 | #59 | #60 | #61 | #62 | #63 | #64 | #65 | #66 |  |  |  |  |  |
|  | 'QPSK' | 9.2 | 12.1 | 10.7 | 11.6 | 11.1 | 11.6 | 11.1 | 10.2 | 10.2 | 8.8 | 8.8 | 7.4 | 7.8 |  |  |  |  |  |
|  | '16QAM' | 9.2 | 12.0 | 10.7 | 11.6 | 11.1 | 11.6 | 11.1 | 10.2 | 10.2 | 8.8 | 8.8 | 7.8 | 7.8 |  |  |  |  |  |
|  | '64QAM' | 9.2 | 12.0 | 10.7 | 11.6 | 11.1 | 11.1 | 11.1 | 10.2 | 10.2 | 8.8 | 8.8 | 7.8 | 7.8 |  |  |  |  |  |
|  | '256QAM' | 9.2 | 13.4 | 13.0 | 13.4 | 13.0 | 13.4 | 13.0 | 13.4 | 13.4 | 12.5 | 13.0 | 13.0 | 13.0 |  |  |  |  |  |
| '10MHz+G40MHz+20MHz' | Scenario # | #67 | #68 | #69 | #70 | #71 | #72 | #73 | #74 | #75 | #76 | #77 | #78 | #70 |  |  |  |  |  |
|  | 'QPSK' | 16.2 | 16.2 | 15.7 | 15.7 | 14.8 | 14.8 | 13.9 | 14.4 | 13.4 | 13.4 | 13.0 | 13.0 | 12.5 |  |  |  |  |  |
|  | '16QAM' | 16.2 | 16.2 | 15.7 | 15.7 | 14.8 | 14.8 | 13.9 | 14.4 | 13.4 | 13.4 | 13.0 | 13.0 | 12.5 |  |  |  |  |  |
|  | '64QAM' | 16.2 | 16.2 | 15.7 | 15.7 | 14.8 | 14.8 | 13.9 | 13.9 | 13.4 | 13.4 | 13.0 | 13.0 | 12.5 |  |  |  |  |  |
|  | '256QAM' | 16.2 | 16.2 | 15.7 | 15.7 | 14.8 | 14.8 | 13.9 | 14.4 | 13.4 | 13.4 | 13.0 | 13.0 | 13.4 |  |  |  |  |  |
|  | Scenario # | #80 | #81 | #82 | #83 | #84 | #85 | #86 | #87 | #88 | #89 | #90 | #91 | #92 |  |  |  |  |  |
|  | 'QPSK' | 12.5 | 12.1 | 12.1 | 11.6 | 11.6 | 11.6 | 11.1 | 11.1 | 11.1 | 10.2 | 10.2 | 9.7 | 9.7 |  |  |  |  |  |
|  | '16QAM' | 12.5 | 12.1 | 12.1 | 11.6 | 11.6 | 11.6 | 11.1 | 11.1 | 11.1 | 10.2 | 10.2 | 9.7 | 9.7 |  |  |  |  |  |
|  | '64QAM' | 12.5 | 12.1 | 12.1 | 11.6 | 11.6 | 11.6 | 11.6 | 11.1 | 11.1 | 10.2 | 10.2 | 9.7 | 9.7 |  |  |  |  |  |
|  | '256QAM' | 13.4 | 13.0 | 13.0 | 13.0 | 13.0 | 13.0 | 13.0 | 13.4 | 13.4 | 12.5 | 12.5 | 12.5 | 12.5 |  |  |  |  |  |

Table 2-5: PSSCH/PSCCH MPR simulation results for SL Non-contiguous CA with 2x23dBm+1LO

| '10MHz+G10MHz+10MHz' | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 'QPSK' | 16.5 | 7.7 | 16.0 | 7.2 | 15.6 | 6.7 | 14.7 | 6.3 | 13.7 | 5.8 |  |  |  |  |  |  |  |  |
|  | '16QAM' | 16.5 | 7.7 | 16.0 | 7.2 | 15.6 | 7.2 | 14.7 | 6.3 | 13.7 | 5.8 |  |  |  |  |  |  |  |  |
|  | '64QAM' | 17.0 | 7.7 | 16.1 | 7.2 | 15.6 | 6.7 | 14.7 | 6.3 | 13.7 | 5.8 |  |  |  |  |  |  |  |  |
|  | '256QAM' | 16.5 | 7.7 | 16.0 | 7.2 | 15.6 | 6.7 | 14.7 | 6.3 | 13.7 | 5.8 |  |  |  |  |  |  |  |  |
|  | Scenario # | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 |  |  |  |  |  |  |  |  |
|  | 'QPSK' | 13.3 | 8.1 | 12.8 | 9.1 | 12.4 | 10.5 | 12.4 | 11.4 | 11.9 | 11.9 |  |  |  |  |  |  |  |  |
|  | '16QAM' | 13.3 | 8.1 | 12.8 | 9.1 | 12.4 | 11.0 | 12.4 | 11.4 | 11.9 | 11.9 |  |  |  |  |  |  |  |  |
|  | '64QAM' | 13.3 | 8.1 | 12.8 | 9.1 | 12.4 | 11.0 | 12.4 | 11.4 | 11.9 | 11.9 |  |  |  |  |  |  |  |  |
|  | '256QAM' | 13.8 | 8.1 | 14.7 | 9.1 | 13.7 | 13.7 | 13.7 | 13.3 | 13.3 | 13.7 |  |  |  |  |  |  |  |  |
| '10MHz+G50MHz+10MHz' | Scenario # | #21 | #22 | #23 | #24 | #25 | #26 | #27 | #28 | #29 | #30 |  |  |  |  |  |  |  |  |
|  | 'QPSK' | 16.5 | 17.0 | 16.5 | 16.0 | 15.6 | 15.6 | 14.7 | 14.7 | 13.8 | 13.7 |  |  |  |  |  |  |  |  |
|  | '16QAM' | 16.5 | 17.0 | 16.1 | 16.1 | 15.6 | 15.6 | 14.7 | 14.7 | 13.7 | 13.8 |  |  |  |  |  |  |  |  |
|  | '64QAM' | 16.5 | 17.0 | 16.5 | 16.0 | 15.6 | 15.6 | 14.7 | 14.7 | 13.8 | 13.8 |  |  |  |  |  |  |  |  |
|  | '256QAM' | 16.5 | 17.0 | 16.5 | 16.1 | 15.6 | 15.6 | 14.7 | 14.7 | 13.8 | 13.7 |  |  |  |  |  |  |  |  |
|  | Scenario # | #31 | #32 | #33 | #34 | #35 | #36 | #37 | #38 | #39 | #40 |  |  |  |  |  |  |  |  |
|  | 'QPSK' | 13.3 | 13.3 | 12.8 | 12.8 | 12.4 | 12.4 | 12.4 | 12.4 | 11.9 | 11.9 |  |  |  |  |  |  |  |  |
|  | '16QAM' | 13.3 | 13.3 | 12.8 | 12.8 | 12.4 | 12.4 | 12.4 | 12.4 | 11.9 | 11.9 |  |  |  |  |  |  |  |  |
|  | '64QAM' | 13.3 | 13.3 | 12.8 | 12.8 | 12.4 | 12.4 | 12.4 | 12.4 | 11.9 | 11.9 |  |  |  |  |  |  |  |  |
|  | '256QAM' | 13.8 | 13.3 | 14.2 | 14.2 | 13.7 | 13.8 | 13.7 | 13.7 | 13.7 | 13.7 |  |  |  |  |  |  |  |  |
| '10MHz+G10MHz+20MHz' | Scenario # | #41 | #42 | #43 | #44 | #45 | #46 | #47 | #48 | #49 | #50 | #51 | #52 | #53 |  |  |  |  |  |
|  | 'QPSK' | 16.5 | 7.7 | 16.0 | 7.2 | 15.6 | 7.2 | 14.7 | 6.3 | 13.7 | 5.8 | 13.3 | 8.6 | 12.8 |  |  |  |  |  |
|  | '16QAM' | 17.0 | 7.7 | 16.0 | 7.2 | 15.6 | 6.7 | 14.7 | 6.3 | 13.7 | 5.8 | 13.3 | 8.6 | 12.8 |  |  |  |  |  |
|  | '64QAM' | 17.0 | 7.7 | 16.0 | 7.2 | 15.6 | 6.7 | 14.7 | 6.3 | 13.7 | 5.8 | 13.3 | 8.6 | 12.8 |  |  |  |  |  |
|  | '256QAM' | 17.0 | 7.7 | 16.0 | 7.2 | 15.6 | 6.7 | 14.7 | 6.3 | 13.7 | 5.8 | 13.3 | 8.6 | 13.7 |  |  |  |  |  |
|  | Scenario # | #54 | #55 | #56 | #57 | #58 | #59 | #60 | #61 | #62 | #63 | #64 | #65 | #66 |  |  |  |  |  |
|  | 'QPSK' | 9.6 | 12.4 | 11.0 | 12.4 | 11.9 | 11.9 | 11.9 | 11.0 | 10.5 | 9.1 | 9.1 | 8.1 | 8.1 |  |  |  |  |  |
|  | '16QAM' | 9.6 | 12.4 | 11.0 | 12.4 | 11.9 | 11.9 | 11.9 | 10.5 | 10.5 | 9.1 | 9.1 | 8.1 | 8.1 |  |  |  |  |  |
|  | '64QAM' | 9.6 | 12.4 | 11.0 | 12.4 | 11.9 | 11.9 | 11.9 | 11.0 | 10.5 | 9.1 | 9.1 | 8.1 | 8.1 |  |  |  |  |  |
|  | '256QAM' | 9.6 | 14.2 | 13.7 | 13.3 | 13.3 | 13.7 | 13.3 | 13.7 | 13.7 | 12.8 | 13.3 | 13.3 | 13.3 |  |  |  |  |  |
| '10MHz+G40MHz+20MHz' | Scenario # | #67 | #68 | #69 | #70 | #71 | #72 | #73 | #74 | #75 | #76 | #77 | #78 | #70 |  |  |  |  |  |
|  | 'QPSK' | 16.5 | 16.5 | 16.0 | 16.5 | 15.6 | 15.6 | 14.7 | 14.7 | 13.7 | 13.8 | 13.3 | 13.3 | 12.8 |  |  |  |  |  |
|  | '16QAM' | 17.0 | 17.0 | 16.1 | 16.5 | 15.6 | 15.6 | 14.7 | 14.7 | 13.7 | 13.8 | 13.3 | 13.3 | 12.8 |  |  |  |  |  |
|  | '64QAM' | 16.5 | 17.0 | 16.0 | 16.1 | 15.6 | 15.6 | 14.7 | 14.7 | 13.8 | 13.7 | 13.3 | 13.3 | 12.8 |  |  |  |  |  |
|  | '256QAM' | 16.5 | 16.5 | 16.0 | 16.0 | 15.6 | 15.6 | 14.7 | 14.7 | 13.7 | 13.7 | 13.3 | 13.3 | 14.2 |  |  |  |  |  |
|  | Scenario # | #80 | #81 | #82 | #83 | #84 | #85 | #86 | #87 | #88 | #89 | #90 | #91 | #92 |  |  |  |  |  |
|  | 'QPSK' | 12.8 | 12.4 | 12.4 | 12.4 | 12.4 | 11.9 | 11.9 | 11.4 | 11.4 | 11.0 | 11.0 | 10.5 | 10.5 |  |  |  |  |  |
|  | '16QAM' | 12.8 | 12.4 | 12.4 | 12.4 | 12.4 | 11.9 | 11.9 | 11.4 | 11.4 | 11.0 | 11.0 | 10.5 | 10.5 |  |  |  |  |  |
|  | '64QAM' | 12.8 | 12.4 | 12.4 | 12.4 | 11.9 | 11.9 | 11.9 | 11.4 | 11.4 | 11.0 | 11.0 | 10.5 | 10.5 |  |  |  |  |  |
|  | '256QAM' | 13.7 | 13.3 | 13.7 | 13.3 | 13.3 | 13.3 | 13.3 | 13.7 | 13.7 | 12.8 | 12.8 | 12.8 | 12.8 |  |  |  |  |  |

Table 2-6: PSSCH/PSCCH MPR simulation results for SL Non-contiguous CA with 2x23dBm+2LO

| '10MHz+G10MHz+10MHz' | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 'QPSK' | 11.6 | 3.9 | 10.7 | 3.5 | 9.3 | 4.7 | 8.3 | 5.1 | 7.8 | 5.1 |  |  |  |  |  |  |  |  |
|  | '16QAM' | 11.6 | 3.9 | 10.7 | 3.5 | 9.7 | 4.7 | 8.8 | 5.1 | 7.8 | 5.1 |  |  |  |  |  |  |  |  |
|  | '64QAM' | 11.6 | 3.9 | 10.7 | 3.5 | 9.3 | 4.7 | 8.8 | 5.1 | 7.8 | 5.1 |  |  |  |  |  |  |  |  |
|  | '256QAM' | 11.6 | 5.1 | 10.7 | 5.1 | 9.3 | 5.1 | 8.8 | 5.5 | 7.8 | 5.5 |  |  |  |  |  |  |  |  |
|  | Scenario # | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 |  |  |  |  |  |  |  |  |
|  | 'QPSK' | 7.4 | 5.5 | 7.4 | 6.0 | 6.9 | 6.4 | 6.9 | 6.4 | 6.9 | 6.9 |  |  |  |  |  |  |  |  |
|  | '16QAM' | 7.4 | 5.5 | 7.4 | 6.0 | 6.9 | 6.4 | 6.9 | 6.9 | 6.9 | 6.9 |  |  |  |  |  |  |  |  |
|  | '64QAM' | 7.4 | 5.5 | 7.3 | 6.0 | 6.9 | 6.4 | 6.9 | 6.4 | 6.9 | 6.9 |  |  |  |  |  |  |  |  |
|  | '256QAM' | 7.4 | 5.5 | 7.4 | 6.0 | 6.9 | 6.4 | 6.9 | 6.9 | 6.9 | 6.9 |  |  |  |  |  |  |  |  |
| '10MHz+G50MHz+10MHz' | Scenario # | #21 | #22 | #23 | #24 | #25 | #26 | #27 | #28 | #29 | #30 |  |  |  |  |  |  |  |  |
|  | 'QPSK' | 11.6 | 11.6 | 10.7 | 10.2 | 9.3 | 9.2 | 8.3 | 8.8 | 7.8 | 7.8 |  |  |  |  |  |  |  |  |
|  | '16QAM' | 11.6 | 11.6 | 10.7 | 10.7 | 9.7 | 9.2 | 8.8 | 8.8 | 7.8 | 7.8 |  |  |  |  |  |  |  |  |
|  | '64QAM' | 11.6 | 11.6 | 10.7 | 10.7 | 9.3 | 9.2 | 8.8 | 8.8 | 7.8 | 7.8 |  |  |  |  |  |  |  |  |
|  | '256QAM' | 11.6 | 11.6 | 10.7 | 10.7 | 9.3 | 9.2 | 8.3 | 8.3 | 7.8 | 7.8 |  |  |  |  |  |  |  |  |
|  | Scenario # | #31 | #32 | #33 | #34 | #35 | #36 | #37 | #38 | #39 | #40 |  |  |  |  |  |  |  |  |
|  | 'QPSK' | 7.8 | 7.3 | 7.4 | 7.3 | 6.9 | 6.9 | 6.9 | 6.9 | 6.9 | 6.9 |  |  |  |  |  |  |  |  |
|  | '16QAM' | 7.4 | 7.8 | 7.3 | 7.3 | 6.9 | 6.9 | 6.9 | 6.9 | 6.9 | 6.9 |  |  |  |  |  |  |  |  |
|  | '64QAM' | 7.4 | 7.8 | 7.4 | 7.3 | 6.9 | 6.9 | 6.9 | 6.9 | 6.9 | 6.9 |  |  |  |  |  |  |  |  |
|  | '256QAM' | 7.4 | 7.3 | 7.4 | 7.3 | 6.9 | 6.9 | 6.9 | 6.9 | 6.9 | 6.9 |  |  |  |  |  |  |  |  |
| '10MHz+G10MHz+20MHz' | Scenario # | #41 | #42 | #43 | #44 | #45 | #46 | #47 | #48 | #49 | #50 | #51 | #52 | #53 |  |  |  |  |  |
|  | 'QPSK' | 11.6 | 3.5 | 10.7 | 3.5 | 9.3 | 3.5 | 8.3 | 3.5 | 7.8 | 5.1 | 7.8 | 5.5 | 7.4 |  |  |  |  |  |
|  | '16QAM' | 11.6 | 3.5 | 10.7 | 3.5 | 9.3 | 3.5 | 8.8 | 3.5 | 7.8 | 5.1 | 7.8 | 5.5 | 7.3 |  |  |  |  |  |
|  | '64QAM' | 11.6 | 3.5 | 10.7 | 3.5 | 9.3 | 3.5 | 8.8 | 3.5 | 7.8 | 5.1 | 7.4 | 5.5 | 7.4 |  |  |  |  |  |
|  | '256QAM' | 11.6 | 5.5 | 10.7 | 5.1 | 9.3 | 5.1 | 8.8 | 5.5 | 7.8 | 5.1 | 7.4 | 5.5 | 7.4 |  |  |  |  |  |
|  | Scenario # | #54 | #55 | #56 | #57 | #58 | #59 | #60 | #61 | #62 | #63 | #64 | #65 | #66 |  |  |  |  |  |
|  | 'QPSK' | 6.0 | 6.9 | 6.4 | 6.9 | 6.9 | 6.9 | 6.9 | 6.4 | 6.4 | 6.4 | 6.4 | 6.4 | 6.4 |  |  |  |  |  |
|  | '16QAM' | 6.0 | 6.9 | 6.4 | 6.9 | 6.9 | 6.9 | 6.9 | 6.4 | 6.4 | 6.4 | 6.4 | 6.0 | 6.4 |  |  |  |  |  |
|  | '64QAM' | 6.0 | 6.9 | 6.4 | 6.9 | 6.9 | 6.9 | 6.9 | 6.4 | 6.4 | 6.4 | 6.4 | 6.0 | 6.0 |  |  |  |  |  |
|  | '256QAM' | 6.0 | 6.9 | 6.4 | 6.9 | 6.9 | 6.9 | 6.9 | 6.4 | 6.4 | 6.4 | 6.4 | 6.0 | 6.4 |  |  |  |  |  |
| '10MHz+G40MHz+20MHz' | Scenario # | #67 | #68 | #69 | #70 | #71 | #72 | #73 | #74 | #75 | #76 | #77 | #78 | #70 |  |  |  |  |  |
|  | 'QPSK' | 11.6 | 11.1 | 10.7 | 10.7 | 9.3 | 9.2 | 8.8 | 8.3 | 7.8 | 7.8 | 7.3 | 7.3 | 7.3 |  |  |  |  |  |
|  | '16QAM' | 11.6 | 11.1 | 10.7 | 10.7 | 9.7 | 9.2 | 8.8 | 8.3 | 7.8 | 7.8 | 7.8 | 7.8 | 7.3 |  |  |  |  |  |
|  | '64QAM' | 11.6 | 11.1 | 10.7 | 10.7 | 9.3 | 9.7 | 8.8 | 8.8 | 7.8 | 7.8 | 7.8 | 7.8 | 7.3 |  |  |  |  |  |
|  | '256QAM' | 11.6 | 11.1 | 10.7 | 10.7 | 9.3 | 9.2 | 8.8 | 8.8 | 7.8 | 7.8 | 7.8 | 7.3 | 7.3 |  |  |  |  |  |
|  | Scenario # | #80 | #81 | #82 | #83 | #84 | #85 | #86 | #87 | #88 | #89 | #90 | #91 | #92 |  |  |  |  |  |
|  | 'QPSK' | 7.3 | 6.9 | 6.9 | 6.9 | 6.9 | 6.9 | 6.9 | 6.4 | 6.4 | 6.0 | 6.0 | 6.0 | 6.0 |  |  |  |  |  |
|  | '16QAM' | 7.3 | 6.9 | 6.9 | 6.9 | 6.9 | 6.9 | 6.9 | 6.4 | 6.4 | 6.0 | 6.0 | 6.0 | 6.0 |  |  |  |  |  |
|  | '64QAM' | 7.3 | 6.9 | 6.9 | 6.9 | 6.9 | 6.9 | 6.9 | 6.4 | 6.4 | 6.0 | 6.4 | 6.0 | 6.0 |  |  |  |  |  |
|  | '256QAM' | 7.3 | 6.9 | 6.9 | 6.9 | 6.9 | 6.9 | 6.9 | 6.4 | 6.4 | 6.0 | 6.0 | 6.0 | 6.0 |  |  |  |  |  |

Same MPR can be defined for all modulation order because big difference between different modulation order is not observed from Table 2-4, Table 2-5 and Table 2-6. .

Table 2-7, Table 2-8, and Table 2-9 show the maximum value of simulation results for SL non-contiguous CA with architecture #1-1, #1-2, and #2-2 in Table 2-1 respectively considering B and frequency of FIM3,low_block,low & FIM3,high_block,high.

Table 2-7 : PSSCH/PSCCH MPR simulation results for SL non-contiguous CA with 1x26dBm PA + 1LO

| B | MPR (dB) for IM3 frequency |  |
| --- | --- | --- |
|  | SEMfreq_-13 | SEfreq_-30 |
| 0 ≤ B < 5.04 | 7.4 | 16.7 |
| 5.04 ≤ B < 10.08 | 6.4 | 15.3 |
| 10.08 ≤ B |  | 14.3 |

Table 2-8 : PSSCH/PSCCH MPR simulation results for SL non-contiguous CA with 2x23dBm PA + 1LO

| B | MPR (dB) for IM3 frequency |  |
| --- | --- | --- |
|  | SEMfreq_-13 | SEfreq_-30 |
| 0 ≤ B < 5.04 | 7.7 | 17.0 |
| 5.04 ≤ B < 10.08 | 7.2 | 15.6 |
| 10.08 ≤ B |  | 14.7 |

Table 2-9 : PSSCH/PSCCH MPR simulation results for SL non-contiguous CA with 2x23dBm PA + 2LO

| B | MPR (dB) for IM3 frequency |  |
| --- | --- | --- |
|  | SEMfreq_-13 | SEfreq_-30 |
| 0 ≤ B < 5.04 | 5.5 | 11.6 |
| 5.04 ≤ B < 10.08 |  | 9.7 |
| 10.08 ≤ B < 20.16 |  | 7.8 |
| 20.16 ≤ B |  | 6.4 |

The MPR can be proposed as Table 2-10, Table 2-11, and Table 2-12 based on the simulation results when considering implementation margin.

Table 2-10 : PSSCH/PSCCH MPR for SL non-contiguous CA with 1x26dBm PA + 1LO

| B | MPR (dB) for IM3 frequency |  |
| --- | --- | --- |
|  | SEMfreq_-13 | SEfreq_-30 |
| 0 ≤ B < 5.04 | ≤ 11.0 | ≤ 18.5 |
| 5.04 ≤ B < 10.08 | ≤ 10.0 | ≤ 17.0 |
| 10.08 ≤ B |  | ≤ 16.0 |

Table 2-11 : PSSCH/PSCCH MPR for SL non-contiguous CA with 2x23dBm PA + 1LO

| B | MPR (dB) for IM3 frequency |  |
| --- | --- | --- |
|  | SEMfreq_-13 | SEfreq_-30 |
| 0 ≤ B < 5.04 | ≤ 11.5 | ≤ 19.0 |
| 5.04 ≤ B < 10.08 | ≤ 11.0 | ≤ 17.5 |
| 10.08 ≤ B |  | ≤ 16.5 |

Table 2-12 : PSSCH/PSCCH MPR for SL non-contiguous CA with 2x23dBm PA + 2LO

| B | MPR (dB) for IM3 frequency |  |
| --- | --- | --- |
|  | SEMfreq_-13 | SEfreq_-30 |
| 0 ≤ B < 5.04 | ≤ 7.0 | ≤ 15.0 |
| 5.04 ≤ B < 10.08 |  | ≤ 13.0 |
| 10.08 ≤ B < 20.16 |  | ≤ 12.0 |
| 20.16 ≤ B |  | ≤ 10.0 |

#### Proposal 1: Consider PSSCH MPR evaluation scenario in Table 2-3.

#### Proposal 2: Consider PC2 PSSCH MPR in Table 2-10 for SL non-contiguous CA with 1x26dBm PA + 1LO.

#### Proposal 3: Consider PC2 PSSCH MPR in Table 2-11 for SL non-contiguous CA with 2x23dBm PA + 1LO.

#### Proposal 4: Consider PC2 PSSCH MPR in Table 2-12 for SL non-contiguous CA with 2x23dBm PA + 2LO.

## PC3 MPR (PSSCH)

### UE RF architecture

UE RF architecture in Table 2-13 is considered for MPR evaluation.

Table 2-13. UE Architecture for PC3 SL non-contiguous CA

| Arch | description | Remarks |
| --- | --- | --- |
| #1-1 | 1x23dBm PA+ 1LO | Single–Tx |
| #1-2 | 2x20dBm PA+ 1LO | dual-Tx or txDiversity |
| #2-2 | 2x20dBm PA+ 2LO | dualPA-Architecture |

### Simulation assumption

The simulation assumption in Table 2-14 is considered.

Table 2-14. Simulation assumption

| Center frequency | 5.9GHz |
| --- | --- |
| Bandwidth | per CC: 10MHz/20MHz aggregated CBW: 10MHz +10 MHz/ 10MHz+20MHz |
| Aggregated CBW | 10MHz + Gap10MHz + 10MHz 10MHz + Gap50MHz + 10MHz 10MHz + Gap10MHz + 20MHz 10MHz + Gap40MHz + 20MHz |
| Maximum output power for aggregated CBW | 23dBm |
| Numerology | 15 kHz |
| Modulation per CC | QPSK/16QAM/64QAM/256QAM |
| Waveform | CP-OFDM |
| ACLR | 31dBc |
| Carrier leakage | 25dBc |
| IQ image | 34dBc |
| CIM3 | 60dBc |
| PA calibration | PA calibrated to deliver 30dBc ACLR for a fully allocated RBs in 20MHz QPSK DFT- S-OFDM waveform at 1 dB MPR. This is based to share PA between LTE V2X and NR V2X at 5.9GHz as worst case. |

### Evaluation scenario

The MPR evaluation scenario in Table 2-3 is considered.

### Simulation results

Table 2-15, Table 2-16, and Table 2-17 show the MPR simulation results for the SL non-contiguous CA with architecture #1-1, #1-2, and #2-2 in Table 2-13 respectively.

Table 2-15: PSSCH/PSCCH MPR simulation results for SL Non-contiguous CA with 1x23dBm+1LO

| '10MHz+G10MHz+10MHz' | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 'QPSK' | 13.7 | 6.7 | 13.3 | 6.4 | 12.8 | 5.9 | 11.9 | 5.6 | 11.4 | 6.7 |  |  |  |  |  |  |  |  |
|  | '16QAM' | 13.7 | 6.7 | 13.7 | 6.4 | 12.8 | 5.9 | 11.9 | 5.6 | 11.4 | 6.7 |  |  |  |  |  |  |  |  |
|  | '64QAM' | 13.7 | 6.7 | 13.3 | 6.4 | 12.8 | 5.9 | 11.9 | 5.6 | 11.4 | 6.7 |  |  |  |  |  |  |  |  |
|  | '256QAM' | 13.7 | 6.7 | 13.3 | 6.4 | 12.8 | 5.9 | 11.9 | 5.6 | 11.4 | 6.7 |  |  |  |  |  |  |  |  |
|  | Scenario # | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 |  |  |  |  |  |  |  |  |
|  | 'QPSK' | 10.5 | 6.7 | 10.0 | 6.7 | 9.5 | 6.7 | 9.1 | 8.1 | 8.6 | 8.6 |  |  |  |  |  |  |  |  |
|  | '16QAM' | 11.0 | 6.7 | 10.0 | 6.7 | 9.5 | 6.7 | 9.1 | 8.1 | 8.6 | 8.6 |  |  |  |  |  |  |  |  |
|  | '64QAM' | 10.5 | 6.7 | 10.0 | 6.7 | 9.5 | 6.7 | 9.5 | 8.1 | 8.6 | 8.6 |  |  |  |  |  |  |  |  |
|  | '256QAM' | 11.0 | 6.7 | 11.2 | 6.7 | 11.3 | 11.3 | 11.3 | 10.8 | 11.3 | 11.3 |  |  |  |  |  |  |  |  |
| '10MHz+G50MHz+10MHz' | Scenario # | #21 | #22 | #23 | #24 | #25 | #26 | #27 | #28 | #29 | #30 |  |  |  |  |  |  |  |  |
|  | 'QPSK' | 13.7 | 13.7 | 13.3 | 13.3 | 12.8 | 12.8 | 11.9 | 11.9 | 11.4 | 11.4 |  |  |  |  |  |  |  |  |
|  | '16QAM' | 13.7 | 13.7 | 13.7 | 13.7 | 12.8 | 12.8 | 11.9 | 11.9 | 11.4 | 11.4 |  |  |  |  |  |  |  |  |
|  | '64QAM' | 13.7 | 13.7 | 13.7 | 13.3 | 12.8 | 12.8 | 11.9 | 11.9 | 11.4 | 11.4 |  |  |  |  |  |  |  |  |
|  | '256QAM' | 13.7 | 13.7 | 13.7 | 13.3 | 12.8 | 12.8 | 11.9 | 11.9 | 11.4 | 11.4 |  |  |  |  |  |  |  |  |
|  | Scenario # | #31 | #32 | #33 | #34 | #35 | #36 | #37 | #38 | #39 | #40 |  |  |  |  |  |  |  |  |
|  | 'QPSK' | 11.0 | 10.5 | 10.0 | 10.0 | 9.5 | 9.5 | 9.1 | 9.1 | 8.6 | 8.6 |  |  |  |  |  |  |  |  |
|  | '16QAM' | 10.5 | 10.5 | 10.0 | 10.0 | 9.5 | 9.5 | 9.1 | 9.1 | 8.6 | 8.6 |  |  |  |  |  |  |  |  |
|  | '64QAM' | 11.0 | 10.5 | 10.0 | 10.0 | 9.5 | 9.5 | 9.1 | 9.1 | 8.6 | 8.6 |  |  |  |  |  |  |  |  |
|  | '256QAM' | 11.0 | 10.5 | 11.2 | 11.3 | 11.3 | 11.3 | 10.8 | 10.8 | 11.3 | 11.3 |  |  |  |  |  |  |  |  |
| '10MHz+G10MHz+20MHz' | Scenario # | #41 | #42 | #43 | #44 | #45 | #46 | #47 | #48 | #49 | #50 | #51 | #52 | #53 |  |  |  |  |  |
|  | 'QPSK' | 13.7 | 6.7 | 13.3 | 6.4 | 12.8 | 5.9 | 11.9 | 5.6 | 11.4 | 6.7 | 10.5 | 6.7 | 10.0 |  |  |  |  |  |
|  | '16QAM' | 13.7 | 6.7 | 13.3 | 6.4 | 12.8 | 5.9 | 11.9 | 5.6 | 11.4 | 6.7 | 11.0 | 6.7 | 10.0 |  |  |  |  |  |
|  | '64QAM' | 13.7 | 6.7 | 13.3 | 6.4 | 12.8 | 5.9 | 11.9 | 5.6 | 11.4 | 6.7 | 11.0 | 6.7 | 10.0 |  |  |  |  |  |
|  | '256QAM' | 13.7 | 6.7 | 13.3 | 6.4 | 12.8 | 5.9 | 11.9 | 5.6 | 12.8 | 6.7 | 11.0 | 6.7 | 10.3 |  |  |  |  |  |
|  | Scenario # | #54 | #55 | #56 | #57 | #58 | #59 | #60 | #61 | #62 | #63 | #64 | #65 | #66 |  |  |  |  |  |
|  | 'QPSK' | 6.7 | 9.5 | 7.2 | 9.1 | 8.6 | 8.6 | 8.6 | 6.7 | 6.7 | 6.7 | 6.7 | 6.7 | 6.7 |  |  |  |  |  |
|  | '16QAM' | 6.7 | 9.5 | 7.2 | 9.1 | 8.6 | 8.6 | 8.6 | 6.7 | 6.7 | 6.7 | 6.7 | 6.7 | 6.7 |  |  |  |  |  |
|  | '64QAM' | 6.7 | 9.5 | 7.2 | 9.1 | 8.6 | 8.6 | 8.6 | 6.7 | 6.7 | 6.7 | 6.7 | 6.7 | 6.7 |  |  |  |  |  |
|  | '256QAM' | 6.7 | 11.3 | 10.8 | 10.8 | 10.8 | 11.3 | 10.8 | 10.8 | 11.3 | 10.3 | 10.8 | 10.8 | 10.8 |  |  |  |  |  |
| '10MHz+G40MHz+20MHz' | Scenario # | #67 | #68 | #69 | #70 | #71 | #72 | #73 | #74 | #75 | #76 | #77 | #78 | #70 |  |  |  |  |  |
|  | 'QPSK' | 13.7 | 13.7 | 13.3 | 13.3 | 12.8 | 12.8 | 11.9 | 11.9 | 11.4 | 11.4 | 10.5 | 10.5 | 10.0 |  |  |  |  |  |
|  | '16QAM' | 13.7 | 13.7 | 13.3 | 13.7 | 12.8 | 12.8 | 11.9 | 11.9 | 11.4 | 11.4 | 10.5 | 10.5 | 10.0 |  |  |  |  |  |
|  | '64QAM' | 13.7 | 13.7 | 13.7 | 13.3 | 12.8 | 12.8 | 11.9 | 11.9 | 11.4 | 11.4 | 10.5 | 10.5 | 10.0 |  |  |  |  |  |
|  | '256QAM' | 13.7 | 13.7 | 13.3 | 13.3 | 12.8 | 12.8 | 11.9 | 11.9 | 11.4 | 11.4 | 10.5 | 10.5 | 10.3 |  |  |  |  |  |
|  | Scenario # | #80 | #81 | #82 | #83 | #84 | #85 | #86 | #87 | #88 | #89 | #90 | #91 | #92 |  |  |  |  |  |
|  | 'QPSK' | 10.0 | 9.5 | 9.5 | 9.1 | 9.1 | 8.6 | 8.6 | 8.1 | 7.7 | 6.7 | 6.7 | 6.7 | 6.7 |  |  |  |  |  |
|  | '16QAM' | 10.0 | 9.5 | 9.5 | 9.1 | 9.1 | 8.6 | 8.6 | 8.1 | 7.7 | 6.7 | 6.7 | 6.7 | 6.7 |  |  |  |  |  |
|  | '64QAM' | 10.0 | 9.5 | 9.5 | 9.1 | 9.1 | 8.6 | 8.6 | 7.7 | 7.7 | 6.7 | 6.7 | 6.7 | 6.7 |  |  |  |  |  |
|  | '256QAM' | 11.7 | 11.3 | 11.3 | 10.8 | 10.8 | 10.8 | 10.8 | 11.3 | 11.3 | 10.3 | 10.3 | 10.3 | 10.3 |  |  |  |  |  |

Table 2-16: PSSCH/PSCCH MPR simulation results for SL Non-contiguous CA with 2x20dBm+1LO

| '10MHz+G10MHz+10MHz' | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 'QPSK' | 14.4 | 7.5 | 13.9 | 7.2 | 13.0 | 6.7 | 12.4 | 6.3 | 11.9 | 7.5 |  |  |  |  |  |  |  |  |
|  | '16QAM' | 14.4 | 7.5 | 13.9 | 7.2 | 13.0 | 6.7 | 12.4 | 6.3 | 11.9 | 7.5 |  |  |  |  |  |  |  |  |
|  | '64QAM' | 14.4 | 7.5 | 13.9 | 7.2 | 13.0 | 6.7 | 12.4 | 6.3 | 11.9 | 7.5 |  |  |  |  |  |  |  |  |
|  | '256QAM' | 14.4 | 7.5 | 13.9 | 7.2 | 13.0 | 6.7 | 12.4 | 6.3 | 11.9 | 7.5 |  |  |  |  |  |  |  |  |
|  | Scenario # | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 |  |  |  |  |  |  |  |  |
|  | 'QPSK' | 11.6 | 7.5 | 11.2 | 7.5 | 10.2 | 7.5 | 9.3 | 9.5 | 9.5 | 9.5 |  |  |  |  |  |  |  |  |
|  | '16QAM' | 11.6 | 7.5 | 11.2 | 7.5 | 10.2 | 7.5 | 8.8 | 9.5 | 9.5 | 9.5 |  |  |  |  |  |  |  |  |
|  | '64QAM' | 11.6 | 7.5 | 11.2 | 7.5 | 10.2 | 7.5 | 9.3 | 9.5 | 9.5 | 9.5 |  |  |  |  |  |  |  |  |
|  | '256QAM' | 11.6 | 7.5 | 11.5 | 7.5 | 11.2 | 11.5 | 11.3 | 11.5 | 11.5 | 11.5 |  |  |  |  |  |  |  |  |
| '10MHz+G50MHz+10MHz' | Scenario # | #21 | #22 | #23 | #24 | #25 | #26 | #27 | #28 | #29 | #30 |  |  |  |  |  |  |  |  |
|  | 'QPSK' | 14.4 | 14.4 | 13.9 | 13.9 | 13.0 | 13.0 | 12.4 | 12.4 | 11.9 | 11.9 |  |  |  |  |  |  |  |  |
|  | '16QAM' | 13.9 | 14.4 | 13.9 | 13.9 | 13.0 | 13.0 | 12.3 | 12.4 | 11.9 | 11.9 |  |  |  |  |  |  |  |  |
|  | '64QAM' | 14.4 | 14.4 | 13.9 | 13.9 | 13.0 | 13.0 | 12.4 | 12.4 | 11.9 | 11.9 |  |  |  |  |  |  |  |  |
|  | '256QAM' | 14.4 | 14.4 | 13.9 | 13.9 | 13.0 | 13.0 | 12.4 | 12.4 | 11.9 | 11.9 |  |  |  |  |  |  |  |  |
|  | Scenario # | #31 | #32 | #33 | #34 | #35 | #36 | #37 | #38 | #39 | #40 |  |  |  |  |  |  |  |  |
|  | 'QPSK' | 11.6 | 11.6 | 11.2 | 11.2 | 10.2 | 10.2 | 9.3 | 8.8 | 9.5 | 9.5 |  |  |  |  |  |  |  |  |
|  | '16QAM' | 11.6 | 11.6 | 11.2 | 11.2 | 10.2 | 10.2 | 8.8 | 8.4 | 9.5 | 9.5 |  |  |  |  |  |  |  |  |
|  | '64QAM' | 11.6 | 11.6 | 11.2 | 11.2 | 10.2 | 10.2 | 9.3 | 8.8 | 9.5 | 9.5 |  |  |  |  |  |  |  |  |
|  | '256QAM' | 11.6 | 11.6 | 11.2 | 11.2 | 11.2 | 11.2 | 11.3 | 10.8 | 11.5 | 11.5 |  |  |  |  |  |  |  |  |
| '10MHz+G10MHz+20MHz' | Scenario # | #41 | #42 | #43 | #44 | #45 | #46 | #47 | #48 | #49 | #50 | #51 | #52 | #53 |  |  |  |  |  |
|  | 'QPSK' | 14.4 | 7.5 | 13.9 | 7.5 | 13.0 | 7.5 | 12.4 | 7.5 | 11.9 | 7.5 | 11.6 | 7.5 | 11.2 |  |  |  |  |  |
|  | '16QAM' | 13.9 | 7.5 | 13.9 | 7.5 | 13.0 | 7.5 | 12.4 | 7.5 | 11.9 | 7.5 | 11.6 | 7.5 | 11.2 |  |  |  |  |  |
|  | '64QAM' | 14.4 | 7.5 | 13.9 | 7.5 | 13.0 | 7.5 | 12.3 | 7.5 | 11.9 | 7.5 | 11.6 | 7.5 | 11.2 |  |  |  |  |  |
|  | '256QAM' | 14.4 | 7.5 | 13.9 | 7.5 | 13.0 | 7.5 | 12.3 | 7.5 | 11.9 | 7.5 | 11.6 | 7.5 | 11.2 |  |  |  |  |  |
|  | Scenario # | #54 | #55 | #56 | #57 | #58 | #59 | #60 | #61 | #62 | #63 | #64 | #65 | #66 |  |  |  |  |  |
|  | 'QPSK' | 7.5 | 10.2 | 7.5 | 8.8 | 9.5 | 9.5 | 9.5 | 7.5 | 7.5 | 7.5 | 7.5 | 7.5 | 7.5 |  |  |  |  |  |
|  | '16QAM' | 7.5 | 10.2 | 7.5 | 9.3 | 9.5 | 9.5 | 9.5 | 7.5 | 7.5 | 7.5 | 7.5 | 7.5 | 7.5 |  |  |  |  |  |
|  | '64QAM' | 7.5 | 10.2 | 7.5 | 9.3 | 9.5 | 9.5 | 9.5 | 7.5 | 7.5 | 7.5 | 7.5 | 7.5 | 7.5 |  |  |  |  |  |
|  | '256QAM' | 7.5 | 11.2 | 11.5 | 11.3 | 11.5 | 11.5 | 11.5 | 11.5 | 11.5 | 11.5 | 11.5 | 11.5 | 11.5 |  |  |  |  |  |
| '10MHz+G40MHz+20MHz' | Scenario # | #67 | #68 | #69 | #70 | #71 | #72 | #73 | #74 | #75 | #76 | #77 | #78 | #70 |  |  |  |  |  |
|  | 'QPSK' | 14.4 | 14.4 | 13.9 | 13.9 | 13.0 | 13.0 | 12.3 | 12.4 | 11.9 | 11.9 | 11.6 | 11.6 | 11.2 |  |  |  |  |  |
|  | '16QAM' | 14.4 | 14.4 | 13.9 | 13.9 | 13.0 | 13.0 | 12.4 | 12.8 | 11.9 | 11.9 | 11.6 | 11.6 | 11.2 |  |  |  |  |  |
|  | '64QAM' | 14.4 | 14.4 | 13.9 | 13.9 | 13.0 | 13.0 | 12.4 | 12.4 | 11.9 | 11.9 | 11.6 | 11.6 | 11.2 |  |  |  |  |  |
|  | '256QAM' | 14.4 | 14.4 | 13.9 | 13.9 | 13.0 | 13.0 | 12.4 | 12.4 | 11.9 | 11.9 | 11.6 | 11.6 | 11.2 |  |  |  |  |  |
|  | Scenario # | #80 | #81 | #82 | #83 | #84 | #85 | #86 | #87 | #88 | #89 | #90 | #91 | #92 |  |  |  |  |  |
|  | 'QPSK' | 10.7 | 10.2 | 10.2 | 8.8 | 8.4 | 9.5 | 9.5 | 8.5 | 8.5 | 7.5 | 7.5 | 7.5 | 7.5 |  |  |  |  |  |
|  | '16QAM' | 10.7 | 10.2 | 10.2 | 8.8 | 8.8 | 9.5 | 9.5 | 8.5 | 8.5 | 7.5 | 7.5 | 7.5 | 7.5 |  |  |  |  |  |
|  | '64QAM' | 11.2 | 10.2 | 10.2 | 9.3 | 8.8 | 9.5 | 9.5 | 8.5 | 8.5 | 7.5 | 7.5 | 7.5 | 7.5 |  |  |  |  |  |
|  | '256QAM' | 11.2 | 11.2 | 11.2 | 11.3 | 10.8 | 11.5 | 11.5 | 11.5 | 11.5 | 11.5 | 11.5 | 11.5 | 11.5 |  |  |  |  |  |

Table 2-17: PSSCH/PSCCH MPR simulation results for SL Non-contiguous CA with 2x20dBm+2LO

| '10MHz+G10MHz+10MHz' | Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 'QPSK' | 9.5 | 3.7 | 9.0 | 3.7 | 8.5 | 3.7 | 8.1 | 4.1 | 8.0 | 5.5 |  |  |  |  |  |  |  |  |
|  | '16QAM' | 9.5 | 3.7 | 9.0 | 3.7 | 8.5 | 3.7 | 8.0 | 4.1 | 8.0 | 5.5 |  |  |  |  |  |  |  |  |
|  | '64QAM' | 9.5 | 3.7 | 9.0 | 3.7 | 8.5 | 4.1 | 8.0 | 4.1 | 8.0 | 5.5 |  |  |  |  |  |  |  |  |
|  | '256QAM' | 9.5 | 4.1 | 9.0 | 4.1 | 8.5 | 4.1 | 8.0 | 4.1 | 8.0 | 5.5 |  |  |  |  |  |  |  |  |
|  | Scenario # | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 |  |  |  |  |  |  |  |  |
|  | 'QPSK' | 7.6 | 5.9 | 7.1 | 5.9 | 7.1 | 6.3 | 7.1 | 6.7 | 7.1 | 7.1 |  |  |  |  |  |  |  |  |
|  | '16QAM' | 7.6 | 5.9 | 7.6 | 5.9 | 7.1 | 6.7 | 7.1 | 6.7 | 7.1 | 7.1 |  |  |  |  |  |  |  |  |
|  | '64QAM' | 7.6 | 5.9 | 7.6 | 5.9 | 7.1 | 6.3 | 7.1 | 6.7 | 7.1 | 7.1 |  |  |  |  |  |  |  |  |
|  | '256QAM' | 7.6 | 5.9 | 7.6 | 5.9 | 7.1 | 6.3 | 7.1 | 7.1 | 7.1 | 7.1 |  |  |  |  |  |  |  |  |
| '10MHz+G50MHz+10MHz' | Scenario # | #21 | #22 | #23 | #24 | #25 | #26 | #27 | #28 | #29 | #30 |  |  |  |  |  |  |  |  |
|  | 'QPSK' | 9.5 | 9.4 | 9.0 | 8.9 | 8.5 | 8.5 | 8.0 | 8.0 | 8.0 | 8.0 |  |  |  |  |  |  |  |  |
|  | '16QAM' | 9.5 | 9.4 | 9.0 | 8.9 | 8.5 | 8.5 | 8.0 | 8.0 | 8.0 | 8.0 |  |  |  |  |  |  |  |  |
|  | '64QAM' | 9.5 | 9.4 | 9.0 | 8.9 | 8.5 | 8.5 | 8.1 | 8.0 | 8.0 | 8.0 |  |  |  |  |  |  |  |  |
|  | '256QAM' | 9.5 | 9.4 | 9.0 | 8.9 | 8.5 | 8.5 | 8.1 | 8.0 | 8.0 | 8.0 |  |  |  |  |  |  |  |  |
|  | Scenario # | #31 | #32 | #33 | #34 | #35 | #36 | #37 | #38 | #39 | #40 |  |  |  |  |  |  |  |  |
|  | 'QPSK' | 7.6 | 7.6 | 7.6 | 7.6 | 7.1 | 7.1 | 7.1 | 7.1 | 7.1 | 7.1 |  |  |  |  |  |  |  |  |
|  | '16QAM' | 7.6 | 7.6 | 7.6 | 7.6 | 7.1 | 7.1 | 7.1 | 7.1 | 7.1 | 7.1 |  |  |  |  |  |  |  |  |
|  | '64QAM' | 7.6 | 7.6 | 7.6 | 7.6 | 7.1 | 7.1 | 7.1 | 7.1 | 7.1 | 7.1 |  |  |  |  |  |  |  |  |
|  | '256QAM' | 7.6 | 7.6 | 7.6 | 7.6 | 7.1 | 7.1 | 7.1 | 7.1 | 7.1 | 7.1 |  |  |  |  |  |  |  |  |
| '10MHz+G10MHz+20MHz' | Scenario # | #41 | #42 | #43 | #44 | #45 | #46 | #47 | #48 | #49 | #50 | #51 | #52 | #53 |  |  |  |  |  |
|  | 'QPSK' | 9.5 | 3.8 | 9.0 | 3.8 | 8.5 | 3.8 | 8.1 | 3.8 | 8.0 | 5.5 | 7.6 | 5.9 | 7.6 |  |  |  |  |  |
|  | '16QAM' | 9.5 | 3.8 | 9.0 | 3.8 | 8.5 | 3.8 | 8.0 | 3.8 | 8.0 | 5.5 | 7.6 | 5.9 | 7.6 |  |  |  |  |  |
|  | '64QAM' | 9.5 | 4.1 | 9.0 | 4.1 | 8.5 | 3.8 | 8.0 | 4.1 | 8.0 | 5.5 | 7.6 | 5.9 | 7.6 |  |  |  |  |  |
|  | '256QAM' | 9.5 | 4.1 | 9.0 | 4.1 | 8.5 | 4.1 | 8.0 | 4.1 | 8.0 | 5.5 | 7.6 | 5.9 | 7.6 |  |  |  |  |  |
|  | Scenario # | #54 | #55 | #56 | #57 | #58 | #59 | #60 | #61 | #62 | #63 | #64 | #65 | #66 |  |  |  |  |  |
|  | 'QPSK' | 6.3 | 7.1 | 6.7 | 7.1 | 7.1 | 7.1 | 7.1 | 6.7 | 6.7 | 6.7 | 6.7 | 6.3 | 6.3 |  |  |  |  |  |
|  | '16QAM' | 6.3 | 7.1 | 6.7 | 7.1 | 7.1 | 7.1 | 7.1 | 6.7 | 6.7 | 6.7 | 6.7 | 6.3 | 6.3 |  |  |  |  |  |
|  | '64QAM' | 6.3 | 7.1 | 6.7 | 7.1 | 7.1 | 7.1 | 7.1 | 6.7 | 6.7 | 6.7 | 6.7 | 6.3 | 6.3 |  |  |  |  |  |
|  | '256QAM' | 6.3 | 7.1 | 7.1 | 7.1 | 7.1 | 7.1 | 7.1 | 6.7 | 6.7 | 6.7 | 6.7 | 6.7 | 6.7 |  |  |  |  |  |
| '10MHz+G40MHz+20MHz' | Scenario # | #67 | #68 | #69 | #70 | #71 | #72 | #73 | #74 | #75 | #76 | #77 | #78 | #70 |  |  |  |  |  |
|  | 'QPSK' | 9.5 | 9.4 | 9.0 | 9.0 | 8.5 | 8.5 | 8.0 | 8.0 | 8.0 | 8.0 | 7.6 | 7.6 | 7.6 |  |  |  |  |  |
|  | '16QAM' | 9.5 | 9.4 | 9.0 | 9.0 | 8.5 | 8.5 | 8.0 | 8.0 | 8.0 | 8.0 | 7.6 | 7.6 | 7.6 |  |  |  |  |  |
|  | '64QAM' | 9.5 | 9.4 | 9.0 | 8.9 | 8.5 | 8.5 | 8.0 | 8.0 | 8.0 | 8.0 | 7.6 | 7.6 | 7.6 |  |  |  |  |  |
|  | '256QAM' | 9.5 | 9.4 | 9.0 | 8.9 | 8.5 | 8.5 | 8.0 | 8.0 | 8.0 | 8.0 | 7.6 | 7.6 | 7.6 |  |  |  |  |  |
|  | Scenario # | #80 | #81 | #82 | #83 | #84 | #85 | #86 | #87 | #88 | #89 | #90 | #91 | #92 |  |  |  |  |  |
|  | 'QPSK' | 7.6 | 7.1 | 7.1 | 7.1 | 7.1 | 7.1 | 7.1 | 6.7 | 6.7 | 6.3 | 6.3 | 6.3 | 6.3 |  |  |  |  |  |
|  | '16QAM' | 7.6 | 7.1 | 7.1 | 7.1 | 7.1 | 7.1 | 7.1 | 6.7 | 6.7 | 6.3 | 6.3 | 6.3 | 6.3 |  |  |  |  |  |
|  | '64QAM' | 7.6 | 7.1 | 7.1 | 7.1 | 7.1 | 6.7 | 7.1 | 6.7 | 6.7 | 6.3 | 6.3 | 6.3 | 6.3 |  |  |  |  |  |
|  | '256QAM' | 7.6 | 7.1 | 7.1 | 7.1 | 7.1 | 6.7 | 7.1 | 6.7 | 6.7 | 6.7 | 6.7 | 6.7 | 6.7 |  |  |  |  |  |

From Table 2-15, Table 2-16, and Table 2-17, same MPR can be defined for all modulation order because big difference is not observed.

Table 2-18, Table 2-19, and Table 2-20 show the maximum value of simulation results for SL non-contiguous CA with architecture #1-1, #1-2, and #2-2 in Table 2-13 respectively considering B and frequency of FIM3,low_block,low & FIM3,high_block,high.

Table 2-18 : PSSCH/PSCCH MPR simulation results for SL non-contiguous CA with 1x23dBm PA + 1LO

| B | MPR (dB) for IM3 frequency |  |
| --- | --- | --- |
|  | SEMfreq_-13 | SEfreq_-30 |
| 0 ≤ B < 5.04 | 6.7 | 13.7 |
| 5.04 ≤ B < 10.08 | 5.9 | 12.8 |
| 10.08 ≤ B |  | 11.3 |

Table 2-19 : PSSCH/PSCCH MPR simulation results for SL non-contiguous CA with 2x20dBm PA + 1LO

| B | MPR (dB) for IM3 frequency |  |
| --- | --- | --- |
|  | SEMfreq_-13 | SEfreq_-30 |
| 0 ≤ B < 5.04 | 7.5 | 14.4 |
| 5.04 ≤ B < 10.08 | 7.5 | 13.0 |
| 10.08 ≤ B |  | 11.6 |

Table 2-20 : PSSCH/PSCCH MPR simulation results for SL non-contiguous CA with 2x20dBm PA + 2LO

| B | MPR (dB) for IM3 frequency |  |
| --- | --- | --- |
|  | SEMfreq_-13 | SEfreq_-30 |
| 0 ≤ B < 5.04 | 4.1 | 9.5 |
| 5.04 ≤ B < 10.08 | 4.1 | 8.5 |
| 10.08 ≤ B < 20.16 |  | 7.6 |
| 20.16 ≤ B |  | 6.7 |

The MPR can be proposed as Table 2-21, Table 2-22, and Table 2-23 based on the simulation results when considering implementation margin.

Table 2-21 : PSSCH/PSCCH MPR for SL non-contiguous CA with 1x23dBm PA + 1LO

| B | MPR (dB) for IM3 frequency |  |
| --- | --- | --- |
|  | SEMfreq_-13 | SEfreq_-30 |
| 0 ≤ B < 5.04 | ≤ 10.0 | ≤ 16.5 |
| 5.04 ≤ B < 10.08 | ≤ 9.0 | ≤ 15,5 |
| 10.08 ≤ B |  | ≤ 15.0 |

Table 2-22 : PSSCH/PSCCH MPR for SL non-contiguous CA with 2x20dBm PA + 1LO

| B | MPR (dB) for IM3 frequency |  |
| --- | --- | --- |
|  | SEMfreq_-13 | SEfreq_-30 |
| 0 ≤ B < 5.04 | ≤ 10.5 | ≤ 17.0 |
| 5.04 ≤ B < 10.08 | ≤ 9.5 | ≤ 16.0 |
| 10.08 ≤ B |  | ≤ 15.5 |

Table 2-23 : PSSCH/PSCCH MPR for SL non-contiguous CA with 2x20dBm PA + 2LO

| B | MPR (dB) for IM3 frequency |  |
| --- | --- | --- |
|  | SEMfreq_-13 | SEfreq_-30 |
| 0 ≤ B < 5.04 | ≤ 6.5 | ≤ 13.5 |
| 5.04 ≤ B < 10.08 | ≤ 6.5 | ≤ 12.5 |
| 10.08 ≤ B < 20.16 |  | ≤ 11.5 |
| 20.16 ≤ B |  | ≤ 10.5 |

#### Proposal 5: Consider PC3 PSSCH MPR in Table 2-21 for SL non-contiguous CA with 1x23dBm PA + 1LO.

#### Proposal 6: Consider PC3 PSSCH MPR in Table 2-22 for SL non-contiguous CA with 2x20dBm PA + 1LO.

#### Proposal 7: Consider PC3 PSSCH MPR in Table 2-23 for SL non-contiguous CA with 2x20dBm PA + 2LO.

## PC2 MPR (PSFCH)

### Evaluation scenario

The MPR evaluation scenario in Table 2-24 is considered.

Table 2-24: PSFCH SL intra-band NC CA MPR evaluation scenarios

| Aggregated CBW | Scenario | CC1 | CC2 | Frequency of FIM3,low_block,low & FIM3,high_block,high | SCS1 (= SCS2) |
| --- | --- | --- | --- | --- | --- |
| 10MHz + Gap10MHz+10MHz | 1 | 1RB51 | 1RB0 | SEMfreq_-13 | 15 |
|  | 2 | 1RB45 | 1RB5 | SEMfreq_-13 | 15 |
|  | 3 | 1RB40 | 1RB10 | SEMfreq_-13 | 15 |
|  | 4 | 1RB35 | 1RB15 | SEMfreq_-13 | 15 |
|  | 5 | 1RB30 | 1RB20 | SEfreq_-30 | 15 |
|  | 6 | 1RB25 | 1RB25 | SEfreq_-30 | 15 |
|  | 7 | 1RB20 | 1RB30 | SEfreq_-30 | 15 |
|  | 8 | 1RB2 | 1RB49 | SEfreq_-30 | 15 |
| 10MHz + Gap50MHz+10MHz | 9 | 1RB51 | 1RB0 | SEfreq_-30 | 15 |
|  | 10 | 1RB45 | 1RB5 | SEfreq_-30 | 15 |
|  | 11 | 1RB40 | 1RB10 | SEfreq_-30 | 15 |
|  | 12 | 1RB35 | 1RB15 | SEfreq_-30 | 15 |
|  | 13 | 1RB30 | 1RB20 | SEfreq_-30 | 15 |
|  | 14 | 1RB25 | 1RB25 | SEfreq_-30 | 15 |
|  | 15 | 1RB20 | 1RB30 | SEfreq_-30 | 15 |
|  | 16 | 1RB2 | 1RB49 | SEfreq_-30 | 15 |
| 10MHz + Gap10MHz+20MHz | 17 | 1RB51 | 1RB0 | SEMfreq_-13 | 15 |
|  | 18 | 1RB45 | 1RB10 | SEMfreq_-13 | 15 |
|  | 19 | 1RB40 | 1RB20 | SEMfreq_-13 | 15 |
|  | 20 | 1RB35 | 1RB30 | SEfreq_-30 | 15 |
|  | 21 | 1RB30 | 1RB40 | SEfreq_-30 | 15 |
|  | 22 | 1RB25 | 1RB50 | SEfreq_-30 | 15 |
|  | 23 | 1RB20 | 1RB60 | SEfreq_-30 | 15 |
|  | 24 | 1RB2 | 1RB103 | SEfreq_-30 | 15 |
| 10MHz + Gap40MHz+20MHz | 25 | 1RB51 | 1RB0 | SEfreq_-30 | 15 |
|  | 26 | 1RB45 | 1RB10 | SEfreq_-30 | 15 |
|  | 27 | 1RB40 | 1RB20 | SEfreq_-30 | 15 |
|  | 28 | 1RB35 | 1RB30 | SEfreq_-30 | 15 |
|  | 29 | 1RB30 | 1RB40 | SEfreq_-30 | 15 |
|  | 30 | 1RB25 | 1RB50 | SEfreq_-30 | 15 |
|  | 31 | 1RB20 | 1RB60 | SEfreq_-30 | 15 |
|  | 32 | 1RB2 | 1RB103 | SEfreq_-30 | 15 |
| 10MHz + Gap10MHz+10MHz | 33 | 1RB23 | 1RB0 | SEMfreq_-13 | 30 |
|  | 34 | 1RB21 | 1RB3 | SEMfreq_-13 | 30 |
|  | 35 | 1RB18 | 1RB6 | SEMfreq_-13 | 30 |
|  | 36 | 1RB16 | 1RB9 | SEMfreq_-13 | 30 |
|  | 37 | 1RB14 | 1RB12 | SEfreq_-30 | 30 |
|  | 38 | 1RB12 | 1RB15 | SEfreq_-30 | 30 |
|  | 39 | 1RB10 | 1RB18 | SEfreq_-30 | 30 |
|  | 40 | 1RB0 | 1RB23 | SEfreq_-30 | 30 |
| 10MHz + Gap50MHz+10MHz | 41 | 1RB23 | 1RB0 | SEfreq_-30 | 30 |
|  | 42 | 1RB21 | 1RB3 | SEfreq_-30 | 30 |
|  | 43 | 1RB18 | 1RB6 | SEfreq_-30 | 30 |
|  | 44 | 1RB16 | 1RB9 | SEfreq_-30 | 30 |
|  | 45 | 1RB14 | 1RB12 | SEfreq_-30 | 30 |
|  | 46 | 1RB12 | 1RB15 | SEfreq_-30 | 30 |
|  | 47 | 1RB10 | 1RB18 | SEfreq_-30 | 30 |
|  | 48 | 1RB0 | 1RB23 | SEfreq_-30 | 30 |
| 10MHz + Gap10MHz+20MHz | 49 | 1RB23 | 1RB0 | SEMfreq_-13 | 30 |
|  | 50 | 1RB21 | 1RB5 | SEMfreq_-13 | 30 |
|  | 51 | 1RB18 | 1RB10 | SEMfreq_-13 | 30 |
|  | 52 | 1RB16 | 1RB15 | SEfreq_-30 | 30 |
|  | 53 | 1RB14 | 1RB20 | SEfreq_-30 | 30 |
|  | 54 | 1RB12 | 1RB25 | SEfreq_-30 | 30 |
|  | 55 | 1RB10 | 1RB30 | SEfreq_-30 | 30 |
|  | 56 | 1RB0 | 1RB50 | SEfreq_-30 | 30 |
| 10MHz + Gap40MHz+20MHz | 57 | 1RB23 | 1RB0 | SEfreq_-30 | 30 |
|  | 58 | 1RB21 | 1RB5 | SEfreq_-30 | 30 |
|  | 59 | 1RB18 | 1RB10 | SEfreq_-30 | 30 |
|  | 60 | 1RB16 | 1RB15 | SEfreq_-30 | 30 |
|  | 61 | 1RB14 | 1RB20 | SEfreq_-30 | 30 |
|  | 62 | 1RB12 | 1RB25 | SEfreq_-30 | 30 |
|  | 63 | 1RB10 | 1RB30 | SEfreq_-30 | 30 |
|  | 64 | 1RB0 | 1RB50 | SEfreq_-30 | 30 |
| 10MHz + Gap10MHz+10MHz | 65 | 1RB10 | 1RB0 | SEMfreq_-13 | 60 |
|  | 66 | 1RB9 | 1RB2 | SEMfreq_-13 | 60 |
|  | 67 | 1RB8 | 1RB4 | SEMfreq_-13 | 60 |
|  | 68 | 1RB7 | 1RB6 | SEMfreq_-13 | 60 |
|  | 69 | 1RB6 | 1RB7 | SEfreq_-30 | 60 |
|  | 70 | 1RB4 | 1RB8 | SEfreq_-30 | 60 |
|  | 71 | 1RB2 | 1RB9 | SEfreq_-30 | 60 |
|  | 72 | 1RB0 | 1RB10 | SEfreq_-30 | 60 |
| 10MHz + Gap50MHz+10MHz | 73 | 1RB10 | 1RB0 | SEfreq_-30 | 60 |
|  | 74 | 1RB9 | 1RB2 | SEfreq_-30 | 60 |
|  | 75 | 1RB8 | 1RB4 | SEfreq_-30 | 60 |
|  | 76 | 1RB7 | 1RB6 | SEfreq_-30 | 60 |
|  | 77 | 1RB6 | 1RB7 | SEfreq_-30 | 60 |
|  | 78 | 1RB4 | 1RB8 | SEfreq_-30 | 60 |
|  | 79 | 1RB2 | 1RB9 | SEfreq_-30 | 60 |
|  | 80 | 1RB0 | 1RB10 | SEfreq_-30 | 60 |
| 10MHz + Gap10MHz+20MHz | 81 | 1RB10 | 1RB0 | SEMfreq_-13 | 60 |
|  | 82 | 1RB9 | 1RB4 | SEMfreq_-13 | 60 |
|  | 83 | 1RB8 | 1RB6 | SEMfreq_-13 | 60 |
|  | 84 | 1RB7 | 1RB8 | SEfreq_-30 | 60 |
|  | 85 | 1RB6 | 1RB10 | SEfreq_-30 | 60 |
|  | 86 | 1RB4 | 1RB12 | SEfreq_-30 | 60 |
|  | 87 | 1RB2 | 1RB15 | SEfreq_-30 | 60 |
|  | 88 | 1RB0 | 1RB23 | SEfreq_-30 | 60 |
| 10MHz + Gap40MHz+20MHz | 89 | 1RB10 | 1RB0 | SEfreq_-30 | 60 |
|  | 90 | 1RB9 | 1RB4 | SEfreq_-30 | 60 |
|  | 91 | 1RB8 | 1RB6 | SEfreq_-30 | 60 |
|  | 92 | 1RB7 | 1RB8 | SEfreq_-30 | 60 |
|  | 93 | 1RB6 | 1RB10 | SEfreq_-30 | 60 |
|  | 94 | 1RB4 | 1RB12 | SEfreq_-30 | 60 |
|  | 95 | 1RB2 | 1RB15 | SEfreq_-30 | 60 |
|  | 96 | 1RB0 | 1RB23 | SEfreq_-30 | 60 |

### Simulation results

Table 2-25, Table 2-26, and Table 2-27 show the MPR simulation results for the SL non-contiguous CA with architecture #1-1, #1-2, and #2-2 respectively.

Table 2-25: PSFCH MPR simulation results for SL Non-contiguous CA with 1x26dBm+1LO

| Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| '10MHz+G10MHz+10MHz' | 10.4 | 10.6 | 10.1 | 10.6 | 11.0 | 11.5 | 14.2 | 14.3 |
| Scenario # | #9 | #10 | #11 | #12 | #13 | #14 | #15 | #16 |
| '10MHz+G50MHz+10MHz' | 14.8 | 14.2 | 14.2 | 14.2 | 14.2 | 14.2 | 14.2 | 14.3 |
| Scenario # | #17 | #18 | #19 | #20 | #21 | #22 | #23 | #24 |
| '10MHz+G10MHz+20MHz' | 9.4 | 9.6 | 10.6 | 11.0 | 13.3 | 13.8 | 14.2 | 14.3 |
| Scenario # | #25 | #26 | #27 | #28 | #29 | #30 | #31 | #32 |
| '10MHz+G40MHz+10MHz' | 14.3 | 14.2 | 14.2 | 14.2 | 13.8 | 14.2 | 14.2 | 14.3 |
| Scenario # | #33 | #34 | #35 | #36 | #37 | #38 | #39 | #40 |
| '10MHz+G10MHz+10MHz' | 10.3 | 10.1 | 10.6 | 9.6 | 11.0 | 13.8 | 13.8 | 13.8 |
| Scenario # | #41 | #42 | #43 | #44 | #45 | #46 | #47 | #48 |
| '10MHz+G50MHz+10MHz' | 13.0 | 14.2 | 13.3 | 14.2 | 13.3 | 14.2 | 13.8 | 13.4 |
| Scenario # | #49 | #50 | #51 | #52 | #53 | #54 | #55 | #56 |
| '10MHz+G10MHz+20MHz' | 10.2 | 9.2 | 9.2 | 11.0 | 13.8 | 13.3 | 14.2 | 13.9 |
| Scenario # | #57 | #58 | #59 | #60 | #61 | #62 | #63 | #64 |
| '10MHz+G40MHz+10MHz' | 14.7 | 13.8 | 13.8 | 14.2 | 14.2 | 13.8 | 13.3 | 14.3 |
| Scenario # | #65 | #66 | #67 | #68 | #69 | #70 | #71 | #72 |
| '10MHz+G10MHz+10MHz' | 9.2 | 8.7 | 9.6 | 10.1 | 13.3 | 13.3 | 13.3 | 13.4 |
| Scenario # | #73 | #74 | #75 | #76 | #77 | #78 | #79 | #80 |
| '10MHz+G50MHz+10MHz' | 13.4 | 13.3 | 13.3 | 12.9 | 13.3 | 12.9 | 13.3 | 12.9 |
| Scenario # | #81 | #82 | #83 | #84 | #85 | #86 | #87 | #88 |
| '10MHz+G10MHz+20MHz' | 9.3 | 8.7 | 9.6 | 10.1 | 13.3 | 12.9 | 12.9 | 13.4 |
| Scenario # | #89 | #90 | #91 | #92 | #93 | #94 | #95 | #96 |
| '10MHz+G40MHz+10MHz' | 12.9 | 12.9 | 13.3 | 12.9 | 13.3 | 13.3 | 13.3 | 13.4 |

Table 2-26: PSFCH MPR simulation results for SL Non-contiguous CA with 2x23dBm+1LO

| Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| '10MHz+G10MHz+10MHz' | 10.6 | 11.2 | 10.8 | 11.2 | 11.7 | 11.7 | 14.9 | 15.0 |
| Scenario # | #9 | #10 | #11 | #12 | #13 | #14 | #15 | #16 |
| '10MHz+G50MHz+10MHz' | 14.6 | 14.4 | 14.9 | 14.9 | 14.9 | 14.4 | 14.9 | 14.9 |
| Scenario # | #17 | #18 | #19 | #20 | #21 | #22 | #23 | #24 |
| '10MHz+G10MHz+20MHz' | 10.0 | 10.3 | 10.8 | 11.7 | 13.5 | 14.4 | 14.9 | 14.9 |
| Scenario # | #25 | #26 | #27 | #28 | #29 | #30 | #31 | #32 |
| '10MHz+G40MHz+10MHz' | 15.0 | 14.9 | 14.4 | 14.9 | 14.4 | 14.9 | 14.9 | 14.9 |
| Scenario # | #33 | #34 | #35 | #36 | #37 | #38 | #39 | #40 |
| '10MHz+G10MHz+10MHz' | 10.5 | 10.3 | 10.8 | 10.3 | 11.7 | 14.4 | 14.4 | 14.5 |
| Scenario # | #41 | #42 | #43 | #44 | #45 | #46 | #47 | #48 |
| '10MHz+G50MHz+10MHz' | 13.6 | 14.9 | 14.0 | 14.9 | 14.0 | 14.9 | 14.4 | 14.1 |
| Scenario # | #49 | #50 | #51 | #52 | #53 | #54 | #55 | #56 |
| '10MHz+G10MHz+20MHz' | 9.4 | 9.9 | 9.9 | 11.7 | 14.4 | 13.5 | 14.9 | 14.5 |
| Scenario # | #57 | #58 | #59 | #60 | #61 | #62 | #63 | #64 |
| '10MHz+G40MHz+10MHz' | 14.4 | 14.4 | 14.4 | 14.9 | 14.4 | 14.4 | 14.0 | 15.0 |
| Scenario # | #65 | #66 | #67 | #68 | #69 | #70 | #71 | #72 |
| '10MHz+G10MHz+10MHz' | 9.5 | 9.4 | 10.3 | 10.8 | 14.0 | 14.0 | 14.0 | 14.1 |
| Scenario # | #73 | #74 | #75 | #76 | #77 | #78 | #79 | #80 |
| '10MHz+G50MHz+10MHz' | 13.6 | 14.0 | 14.0 | 13.5 | 13.5 | 13.5 | 13.5 | 13.6 |
| Scenario # | #81 | #82 | #83 | #84 | #85 | #86 | #87 | #88 |
| '10MHz+G10MHz+20MHz' | 9.5 | 9.4 | 10.3 | 10.8 | 13.5 | 13.5 | 13.5 | 13.6 |
| Scenario # | #89 | #90 | #91 | #92 | #93 | #94 | #95 | #96 |
| '10MHz+G40MHz+10MHz' | 13.6 | 13.5 | 14.0 | 13.5 | 14.0 | 13.5 | 14.0 | 14.1 |

Table 2-27: PSFCH MPR simulation results for SL Non-contiguous CA with 2x23dBm+2LO

| Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| '10MHz+G10MHz+10MHz' | 4.2 | 4.2 | 5.1 | 4.9 | 5.5 | 5.3 | 11.4 | 12.1 |
| Scenario # | #9 | #10 | #11 | #12 | #13 | #14 | #15 | #16 |
| '10MHz+G50MHz+10MHz' | 11.5 | 11.9 | 11.9 | 11.4 | 11.9 | 11.9 | 11.4 | 12.1 |
| Scenario # | #17 | #18 | #19 | #20 | #21 | #22 | #23 | #24 |
| '10MHz+G10MHz+20MHz' | 4.3 | 5.1 | 5.2 | 5.5 | 11.4 | 9.6 | 11.4 | 12.1 |
| Scenario # | #25 | #26 | #27 | #28 | #29 | #30 | #31 | #32 |
| '10MHz+G40MHz+10MHz' | 11.6 | 11.4 | 11.9 | 11.4 | 11.9 | 11.0 | 11.9 | 11.6 |
| Scenario # | #33 | #34 | #35 | #36 | #37 | #38 | #39 | #40 |
| '10MHz+G10MHz+10MHz' | 4.0 | 3.9 | 3.9 | 4.8 | 4.4 | 10.5 | 11.4 | 11.6 |
| Scenario # | #41 | #42 | #43 | #44 | #45 | #46 | #47 | #48 |
| '10MHz+G50MHz+10MHz' | 11.5 | 10.1 | 11.4 | 11.0 | 11.0 | 10.1 | 11.4 | 11.6 |
| Scenario # | #49 | #50 | #51 | #52 | #53 | #54 | #55 | #56 |
| '10MHz+G10MHz+20MHz' | 4.0 | 3.9 | 3.9 | 4.4 | 11.4 | 9.6 | 9.6 | 11.6 |
| Scenario # | #57 | #58 | #59 | #60 | #61 | #62 | #63 | #64 |
| '10MHz+G40MHz+10MHz' | 11.1 | 10.3 | 11.4 | 10.5 | 11.4 | 11.4 | 11.1 | 11.6 |
| Scenario # | #65 | #66 | #67 | #68 | #69 | #70 | #71 | #72 |
| '10MHz+G10MHz+10MHz' | 4.0 | 3.9 | 4.4 | 4.8 | 9.6 | 9.6 | 9.3 | 9.8 |
| Scenario # | #73 | #74 | #75 | #76 | #77 | #78 | #79 | #80 |
| '10MHz+G50MHz+10MHz' | 9.7 | 9.3 | 9.3 | 9.3 | 9.6 | 10.1 | 9.6 | 9.5 |
| Scenario # | #81 | #82 | #83 | #84 | #85 | #86 | #87 | #88 |
| '10MHz+G10MHz+20MHz' | 4.0 | 3.9 | 4.4 | 4.4 | 9.6 | 9.3 | 9.3 | 9.8 |
| Scenario # | #89 | #90 | #91 | #92 | #93 | #94 | #95 | #96 |
| '10MHz+G40MHz+10MHz' | 9.4 | 9.6 | 9.3 | 9.3 | 9.3 | 9.6 | 9.6 | 9.5 |

Table 2-28 shows the maximum value of simulation results for SL non-contiguous CA considering the frequency of FIM3,low_block,low & FIM3,high_block,high.

Table 2-28 : PC2 PSFCH MPR simulation results for SL non-contiguous CA

|  | MPR (dB) for IM3 frequency |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | SEMfreq_-13 |  |  | SEfreq_-30 |  |  |
|  | SCS=15kHz (B=0.36) | SCS=30kHz (B=0.72) | SCS=60kHz (B=1.44) | SCS=15kHz (B=0.36) | SCS=30kHz (B=0.72) | SCS=60kHz (B=1.44) |
| 1x26dBm + 1LO | 10.6 | 10.6 | 10.1 | 14.8 | 14.7 | 13.4 |
| 2x23dBm + 1LO | 11.2 | 10.8 | 10.8 | 15.0 | 15.0 | 14.1 |
| 2x23dBm + 2LO | 5.2 | 4.8 | 4.8 | 12.1 | 11.6 | 10.1 |

The MPR can be proposed as Table 2-29 based on the simulation results when considering implementation margin.

Table 2-29 : PC2 PSFCH MPR for SL non-contiguous CA

|  | MPR (dB) for IM3 frequency |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | SEMfreq_-13 |  |  | SEfreq_-30 |  |  |
|  | SCS=15kHz (B=0.36) | SCS=30kHz (B=0.72) | SCS=60kHz (B=1.44) | SCS=15kHz (B=0.36) | SCS=30kHz (B=0.72) | SCS=60kHz (B=1.44) |
| 1x26dBm + 1LO | ≤ 15.0 | ≤ 15.0 | ≤ 14.0 | ≤ 20.0 | ≤ 20.0 | ≤ 19.0 |
| 2x23dBm + 1LO | ≤ 15.0 | ≤ 15.0 | ≤ 14.0 | ≤ 20.0 | ≤ 20.0 | ≤ 19.0 |
| 2x23dBm + 2LO | ≤ 9.5 | ≤ 8.5 | ≤ 7.5 | ≤ 16.5 | ≤ 16.0 | ≤ 15.0 |

#### Proposal 8: Consider PSFCH MPR evaluation scenario in Table 2-24.

#### Proposal 9: Consider PC2 PSFCH MPR in Table 2-29 for SL non-contiguous CA.

## PC3 MPR (PSFCH)

### Evaluation scenario

The MPR evaluation scenario in Table 2-24 is considered.

### Simulation results

Table 2-30, Table 2-31, and Table 2-32 show the MPR simulation results for the SL non-contiguous CA with architecture #1-1, #1-2, and #2-2 respectively.

Table 2-30: PSFCH MPR simulation results for SL Non-contiguous CA with 1x23dBm+1LO

| Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| '10MHz+G10MHz+10MHz' | 8.8 | 9.0 | 8.5 | 9.0 | 9.0 | 9.5 | 12.2 | 12.3 |
| Scenario # | #9 | #10 | #11 | #12 | #13 | #14 | #15 | #16 |
| '10MHz+G50MHz+10MHz' | 12.4 | 12.2 | 12.2 | 12.2 | 12.2 | 12.2 | 12.2 | 12.3 |
| Scenario # | #17 | #18 | #19 | #20 | #21 | #22 | #23 | #24 |
| '10MHz+G10MHz+20MHz' | 8.3 | 7.6 | 8.5 | 9.0 | 11.3 | 12.2 | 12.2 | 12.3 |
| Scenario # | #25 | #26 | #27 | #28 | #29 | #30 | #31 | #32 |
| '10MHz+G40MHz+10MHz' | 12.8 | 12.2 | 12.2 | 12.2 | 11.8 | 12.2 | 12.2 | 12.3 |
| Scenario # | #33 | #34 | #35 | #36 | #37 | #38 | #39 | #40 |
| '10MHz+G10MHz+10MHz' | 8.2 | 8.2 | 8.6 | 7.7 | 8.6 | 12.3 | 12.3 | 12.4 |
| Scenario # | #41 | #42 | #43 | #44 | #45 | #46 | #47 | #48 |
| '10MHz+G50MHz+10MHz' | 11.5 | 12.8 | 11.9 | 12.8 | 11.9 | 12.8 | 12.3 | 11.5 |
| Scenario # | #49 | #50 | #51 | #52 | #53 | #54 | #55 | #56 |
| '10MHz+G10MHz+20MHz' | 8.1 | 7.2 | 7.2 | 8.6 | 12.3 | 11.4 | 12.8 | 12.4 |
| Scenario # | #57 | #58 | #59 | #60 | #61 | #62 | #63 | #64 |
| '10MHz+G40MHz+10MHz' | 12.3 | 12.3 | 12.3 | 12.8 | 12.3 | 12.3 | 11.4 | 12.9 |
| Scenario # | #65 | #66 | #67 | #68 | #69 | #70 | #71 | #72 |
| '10MHz+G10MHz+10MHz' | 7.3 | 7.2 | 7.2 | 7.7 | 11.4 | 11.9 | 11.9 | 11.5 |
| Scenario # | #73 | #74 | #75 | #76 | #77 | #78 | #79 | #80 |
| '10MHz+G50MHz+10MHz' | 11.5 | 11.4 | 11.4 | 11.4 | 11.4 | 11.0 | 11.4 | 11.5 |
| Scenario # | #81 | #82 | #83 | #84 | #85 | #86 | #87 | #88 |
| '10MHz+G10MHz+20MHz' | 7.3 | 7.2 | 7.2 | 7.7 | 11.4 | 11.0 | 11.0 | 11.5 |
| Scenario # | #89 | #90 | #91 | #92 | #93 | #94 | #95 | #96 |
| '10MHz+G40MHz+10MHz' | 11.5 | 11.4 | 11.4 | 11.4 | 11.9 | 11.4 | 11.9 | 11.9 |

Table 2-31: PSFCH MPR simulation results for SL Non-contiguous CA with 2x20dBm+1LO

| Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| '10MHz+G10MHz+10MHz' | 9.6 | 9.9 | 9.4 | 9.9 | 10.3 | 10.3 | 13.6 | 13.7 |
| Scenario # | #9 | #10 | #11 | #12 | #13 | #14 | #15 | #16 |
| '10MHz+G50MHz+10MHz' | 13.7 | 13.6 | 13.6 | 13.6 | 13.6 | 13.6 | 13.6 | 13.6 |
| Scenario # | #17 | #18 | #19 | #20 | #21 | #22 | #23 | #24 |
| '10MHz+G10MHz+20MHz' | 8.6 | 8.4 | 9.9 | 10.3 | 12.2 | 13.6 | 13.6 | 13.6 |
| Scenario # | #25 | #26 | #27 | #28 | #29 | #30 | #31 | #32 |
| '10MHz+G40MHz+10MHz' | 13.7 | 13.6 | 13.6 | 13.6 | 13.1 | 13.6 | 13.6 | 13.6 |
| Scenario # | #33 | #34 | #35 | #36 | #37 | #38 | #39 | #40 |
| '10MHz+G10MHz+10MHz' | 8.9 | 8.8 | 9.8 | 8.8 | 9.8 | 13.5 | 13.5 | 13.6 |
| Scenario # | #41 | #42 | #43 | #44 | #45 | #46 | #47 | #48 |
| '10MHz+G50MHz+10MHz' | 12.7 | 14.0 | 13.1 | 14.0 | 13.1 | 14.0 | 13.5 | 12.7 |
| Scenario # | #49 | #50 | #51 | #52 | #53 | #54 | #55 | #56 |
| '10MHz+G10MHz+20MHz' | 8.8 | 7.9 | 7.9 | 9.8 | 13.5 | 12.6 | 14.0 | 13.6 |
| Scenario # | #57 | #58 | #59 | #60 | #61 | #62 | #63 | #64 |
| '10MHz+G40MHz+10MHz' | 13.5 | 13.5 | 13.5 | 14.0 | 14.0 | 13.5 | 12.6 | 14.1 |
| Scenario # | #65 | #66 | #67 | #68 | #69 | #70 | #71 | #72 |
| '10MHz+G10MHz+10MHz' | 7.9 | 7.9 | 8.4 | 8.8 | 12.6 | 13.1 | 13.1 | 12.7 |
| Scenario # | #73 | #74 | #75 | #76 | #77 | #78 | #79 | #80 |
| '10MHz+G50MHz+10MHz' | 12.7 | 12.6 | 12.6 | 12.6 | 12.6 | 12.2 | 12.6 | 12.7 |
| Scenario # | #81 | #82 | #83 | #84 | #85 | #86 | #87 | #88 |
| '10MHz+G10MHz+20MHz' | 8.0 | 7.9 | 7.9 | 8.8 | 12.6 | 12.2 | 12.2 | 12.7 |
| Scenario # | #89 | #90 | #91 | #92 | #93 | #94 | #95 | #96 |
| '10MHz+G40MHz+10MHz' | 12.7 | 12.6 | 12.6 | 12.6 | 13.1 | 12.6 | 13.1 | 13.1 |

Table 2-32: PSFCH MPR simulation results for SL Non-contiguous CA with 2x20dBm+2LO

| Scenario # | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| '10MHz+G10MHz+10MHz' | 3.4 | 3.6 | 4.3 | 4.3 | 9.0 | 9.0 | 10.0 | 11.0 |
| Scenario # | #9 | #10 | #11 | #12 | #13 | #14 | #15 | #16 |
| '10MHz+G50MHz+10MHz' | 9.9 | 10.3 | 10.8 | 9.8 | 10.3 | 10.8 | 10.0 | 11.0 |
| Scenario # | #17 | #18 | #19 | #20 | #21 | #22 | #23 | #24 |
| '10MHz+G10MHz+20MHz' | 3.7 | 4.3 | 4.3 | 9.0 | 9.8 | 9.5 | 9.8 | 10.6 |
| Scenario # | #25 | #26 | #27 | #28 | #29 | #30 | #31 | #32 |
| '10MHz+G40MHz+10MHz' | 10.0 | 9.8 | 10.8 | 9.8 | 10.3 | 10.0 | 10.8 | 10.2 |
| Scenario # | #33 | #34 | #35 | #36 | #37 | #38 | #39 | #40 |
| '10MHz+G10MHz+10MHz' | 3.6 | 3.5 | 3.5 | 4.0 | 9.0 | 10.0 | 10.0 | 10.2 |
| Scenario # | #41 | #42 | #43 | #44 | #45 | #46 | #47 | #48 |
| '10MHz+G50MHz+10MHz' | 10.0 | 9.5 | 10.0 | 10.0 | 10.0 | 9.5 | 10.0 | 10.2 |
| Scenario # | #49 | #50 | #51 | #52 | #53 | #54 | #55 | #56 |
| '10MHz+G10MHz+20MHz' | 3.6 | 3.5 | 3.5 | 9.0 | 10.0 | 9.5 | 9.5 | 10.2 |
| Scenario # | #57 | #58 | #59 | #60 | #61 | #62 | #63 | #64 |
| '10MHz+G40MHz+10MHz' | 10.1 | 9.5 | 9.5 | 10.0 | 10.0 | 10.0 | 9.5 | 10.2 |
| Scenario # | #65 | #66 | #67 | #68 | #69 | #70 | #71 | #72 |
| '10MHz+G10MHz+10MHz' | 3.7 | 3.5 | 3.5 | 4.0 | 9.5 | 9.5 | 9.5 | 9.8 |
| Scenario # | #73 | #74 | #75 | #76 | #77 | #78 | #79 | #80 |
| '10MHz+G50MHz+10MHz' | 9.6 | 9.5 | 9.5 | 9.5 | 9.5 | 9.5 | 9.5 | 9.8 |
| Scenario # | #81 | #82 | #83 | #84 | #85 | #86 | #87 | #88 |
| '10MHz+G10MHz+20MHz' | 3.6 | 3.5 | 3.5 | 8.5 | 9.5 | 9.5 | 9.5 | 9.8 |
| Scenario # | #89 | #90 | #91 | #92 | #93 | #94 | #95 | #96 |
| '10MHz+G40MHz+10MHz' | 9.5 | 9.5 | 9.0 | 9.5 | 9.5 | 9.5 | 9.5 | 9.8 |

Table 2-33 shows the maximum value of simulation results for SL non-contiguous CA with architecture #1-1, #1-2, and #2-2 considering the frequency of FIM3,low_block,low & FIM3,high_block,high.

Table 2-33 : PC3 PSFCH MPR simulation results for SL non-contiguous CA

|  | MPR (dB) for IM3 frequency |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | SEMfreq_-13 |  |  | SEfreq_-30 |  |  |
|  | SCS=15kHz (B=0.36) | SCS=30kHz (B=0.72) | SCS=60kHz (B=1.44) | SCS=15kHz (B=0.36) | SCS=30kHz (B=0.72) | SCS=60kHz (B=1.44) |
| 1x26dBm + 1LO | 9.0 | 8.6 | 7.7 | 12.8 | 12.9 | 11.9 |
| 2x23dBm + 1LO | 9.9 | 9.8 | 8.8 | 13.7 | 14.1 | 13.1 |
| 2x23dBm + 2LO | 4.3 | 4.0 | 4.0 | 11.0 | 10.2 | 9.8 |

The MPR can be proposed as Table 2-34 based on the simulation results when considering implementation margin.

Table 2-34 : PC3 PSFCH MPR for SL non-contiguous CA

|  | MPR (dB) for IM3 frequency |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | SEMfreq_-13 |  |  | SEfreq_-30 |  |  |
|  | SCS=15kHz (B=0.36) | SCS=30kHz (B=0.72) | SCS=60kHz (B=1.44) | SCS=15kHz (B=0.36) | SCS=30kHz (B=0.72) | SCS=60kHz (B=1.44) |
| 1x26dBm + 1LO | ≤ 12.0 | ≤ 12.0 | ≤ 11.0 | ≤ 18.0 | ≤ 18.0 | ≤ 17.0 |
| 2x23dBm + 1LO | ≤ 12.0 | ≤ 12.0 | ≤ 11.0 | ≤ 18.0 | ≤ 18.0 | ≤ 17.0 |
| 2x23dBm + 2LO | ≤ 9.0 | ≤ 8.0 | ≤ 7.0 | ≤ 16.0 | ≤ 15.5 | ≤ 15.0 |

#### Proposal 10: Consider PC3 PSFCH MPR in Table 2-34 for SL non-contiguous CA.

## PC2 MPR (SSSB)

### Evaluation scenario

The MPR evaluation scenario in Table 2-35 is considered.

Table 2-35: SSSB SL non-contiguous CA MPR evaluation scenarios

| Aggregated CBW | Scenario | CC1 | CC2 | Frequency of FIM3,low_block,low & FIM3,high_block,high | SCS1 (= SCS2) |
| --- | --- | --- | --- | --- | --- |
| 10MHz + Gap10MHz+10MHz | 1 | 11RB41 | 11RB0 | SEMfreq_-13 | 15 |
|  | 2 | 11RB34 | 11RB7 | SEMfreq_-13 | 15 |
|  | 3 | 11RB20 | 11RB21 | SEfreq_-30 | 15 |
|  | 4 | 11RB0 | 11RB41 | SEfreq_-30 | 15 |
| 10MHz + Gap50MHz+10MHz | 5 | 11RB41 | 11RB0 | SEfreq_-30 | 15 |
|  | 6 | 11RB34 | 11RB7 | SEfreq_-30 | 15 |
|  | 7 | 11RB20 | 11RB21 | SEfreq_-30 | 15 |
|  | 8 | 11RB0 | 11RB41 | SEfreq_-30 | 15 |
| 10MHz + Gap10MHz+20MHz | 9 | 11RB41 | 11RB0 | SEMfreq_-13 | 15 |
|  | 10 | 11RB34 | 11RB14 | SEMfreq_-13 | 15 |
|  | 11 | 11RB20 | 11RB42 | SEfreq_-30 | 15 |
|  | 12 | 11RB0 | 11RB95 | SEfreq_-30 | 15 |
| 10MHz + Gap40MHz+20MHz | 13 | 11RB41 | 11RB0 | SEfreq_-30 | 15 |
|  | 14 | 11RB34 | 11RB14 | SEfreq_-30 | 15 |
|  | 15 | 11RB20 | 11RB42 | SEfreq_-30 | 15 |
|  | 16 | 11RB0 | 11RB95 | SEfreq_-30 | 15 |
| 10MHz + Gap10MHz+10MHz | 17 | 11RB13 | 11RB0 | SEMfreq_-13 | 30 |
|  | 18 | 11RB12 | 11RB2 | SEMfreq_-13 | 30 |
|  | 19 | 11RB3 | 11RB12 | SEfreq_-30 | 30 |
|  | 20 | 11RB0 | 11RB13 | SEfreq_-30 | 30 |
| 10MHz + Gap50MHz+10MHz | 21 | 11RB13 | 11RB0 | SEfreq_-30 | 30 |
|  | 22 | 11RB12 | 11RB2 | SEfreq_-30 | 30 |
|  | 23 | 11RB3 | 11RB12 | SEfreq_-30 | 30 |
|  | 24 | 11RB0 | 11RB13 | SEfreq_-30 | 30 |
| 10MHz + Gap10MHz+20MHz | 25 | 11RB13 | 11RB0 | SEMfreq_-13 | 30 |
|  | 26 | 11RB12 | 11RB14 | SEfreq_-30 | 30 |
|  | 27 | 11RB3 | 11RB30 | SEfreq_-30 | 30 |
|  | 28 | 11RB0 | 11RB40 | SEfreq_-30 | 30 |
| 10MHz + Gap40MHz+20MHz | 29 | 11RB13 | 11RB0 | SEfreq_-30 | 30 |
|  | 30 | 11RB12 | 11RB14 | SEfreq_-30 | 30 |
|  | 31 | 11RB3 | 11RB30 | SEfreq_-30 | 30 |
|  | 32 | 11RB0 | 11RB40 | SEfreq_-30 | 30 |

### Simulation results

Table 2-36, Table 2-37, and Table 2-38 show the MPR simulation results for the SL non-contiguous CA with architecture #1-1, #1-2, and #2-2 respectively.

Table 2-36: SSSB MPR simulation results for SL Non-contiguous CA with 1x26dBm+1LO

| Scenario # | #1 | #2 | #3 | #4 | #17 | #18 | #19 | #20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| '10MHz+G10MHz+10MHz' | 9.6 | 10.1 | 18.9 | 20.3 | 9.1 | 8.3 | 16.6 | 17.0 |
| Scenario # | #5 | #6 | #7 | #8 | #21 | #22 | #23 | #24 |
| '10MHz+G50MHz+10MHz' | 19.4 | 19.8 | 19.8 | 19.8 | 17.5 | 17.5 | 17.5 | 17.1 |
| Scenario # | #9 | #10 | #11 | #12 | #25 | #26 | #27 | #28 |
| '10MHz+G10MHz+20MHz' | 10.0 | 9.6 | 19.8 | 19.4 | 8.9 | 13.8 | 16.6 | 16.6 |
| Scenario # | #13 | #14 | #15 | #16 | #29 | #30 | #31 | #32 |
| '10MHz+G40MHz+20MHz' | 18.9 | 19.4 | 19.8 | 19.4 | 18.1 | 17.1 | 17.0 | 16.2 |

Table 2-37: SSSB MPR simulation results for SL Non-contiguous CA with 2x23dBm+1LO

| Scenario # | #1 | #2 | #3 | #4 | #17 | #18 | #19 | #20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| '10MHz+G10MHz+10MHz' | 10.7 | 11.6 | 18.3 | 20.1 | 11.0 | 9.5 | 18.2 | 18.3 |
| Scenario # | #5 | #6 | #7 | #8 | #21 | #22 | #23 | #24 |
| '10MHz+G50MHz+10MHz' | 19.7 | 21.1 | 20.6 | 20.6 | 17.9 | 17.8 | 17.3 | 18.3 |
| Scenario # | #9 | #10 | #11 | #12 | #25 | #26 | #27 | #28 |
| '10MHz+G10MHz+20MHz' | 11.6 | 10.4 | 20.5 | 20.1 | 9.6 | 14.6 | 17.4 | 17.8 |
| Scenario # | #13 | #14 | #15 | #16 | #29 | #30 | #31 | #32 |
| '10MHz+G40MHz+20MHz' | 19.7 | 20.6 | 20.6 | 21.0 | 17.6 | 18.7 | 16.9 | 17.4 |

Table 2-38: SSSB MPR simulation results for SL Non-contiguous CA with 2x23dBm+2LO

| Scenario # | #1 | #2 | #3 | #4 | #17 | #18 | #19 | #20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| '10MHz+G10MHz+10MHz' | 2.6 | 3.0 | 11.7 | 12.2 | 4.2 | 5.1 | 7.9 | 7.0 |
| Scenario # | #5 | #6 | #7 | #8 | #21 | #22 | #23 | #24 |
| '10MHz+G50MHz+10MHz' | 12.2 | 12.6 | 12.2 | 12.2 | 7.5 | 7.5 | 8.4 | 7.5 |
| Scenario # | #9 | #10 | #11 | #12 | #25 | #26 | #27 | #28 |
| '10MHz+G10MHz+20MHz' | 2.7 | 5.1 | 12.2 | 12.2 | 4.2 | 7.0 | 7.4 | 8.0 |
| Scenario # | #13 | #14 | #15 | #16 | #29 | #30 | #31 | #32 |
| '10MHz+G40MHz+20MHz' | 12.2 | 12.2 | 12.6 | 12.2 | 7.5 | 7.9 | 7.9 | 7.5 |

Table 2-39 shows the maximum value of simulation results for SL non-contiguous CA considering the frequency of FIM3,low_block,low & FIM3,high_block,high.

Table 2-39 : PC2 SSSB MPR simulation results for SL non-contiguous CA

|  | MPR (dB) for IM3 frequency |  |  |  |
| --- | --- | --- | --- | --- |
|  | SEMfreq_-13 |  | SEfreq_-30 |  |
|  | SCS=15kHz (B=3.96) | SCS=30kHz (B=7.92) | SCS=15kHz (B=3.96) | SCS=30kHz (B=7.92) |
| 1x26dBm + 1LO | 10.1 | 9.1 | 20.3 | 18.1 |
| 2x23dBm + 1LO | 11.6 | 11.0 | 21.1 | 18.7 |
| 2x23dBm + 2LO | 5.1 | 5.1 | 12.6 | 8.4 |

The MPR can be proposed as Table 2-40 based on the simulation results when considering implementation margin.

Table 2-40 : PC2 SSSB MPR for SL non-contiguous CA

|  | MPR (dB) for IM3 frequency |  |  |  |
| --- | --- | --- | --- | --- |
|  | SEMfreq_-13 |  | SEfreq_-30 |  |
|  | SCS=15kHz (B=3.96) | SCS=30kHz (B=7.92) | SCS=15kHz (B=3.96) | SCS=30kHz (B=7.92) |
| 1x26dBm + 1LO | ≤ 13.0 | ≤ 12.0 | ≤ 22.0 | ≤ 20.0 |
| 2x23dBm + 1LO | ≤ 14.0 | ≤ 13.0 | ≤ 23.0 | ≤ 21.0 |
| 2x23dBm + 2LO | ≤ 9.0 | ≤ 8.0 | ≤ 17.0 | ≤ 15.0 |

#### Proposal 11: Consider SSSB MPR evaluation scenario in Table 2-35.

#### Proposal 12: Consider PC2 SSSB MPR in Table 2-40 for SL non-contiguous CA.

## PC3 MPR (SSSB)

### Evaluation scenario

The MPR evaluation scenario in Table 2-35 is considered.

### Simulation results

Table 2-41, Table 2-42, and Table 2-43 show the MPR simulation results for the SL non-contiguous CA with architecture #1-1, #1-2, and #2-2 respectively.

Table 2-41: SSSB MPR simulation results for SL Non-contiguous CA with 1x23dBm+1LO

| Scenario # | #1 | #2 | #3 | #4 | #17 | #18 | #19 | #20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| '10MHz+G10MHz+10MHz' | 8.6 | 8.5 | 16.0 | 16.9 | 7.6 | 7.1 | 14.6 | 14.1 |
| Scenario # | #5 | #6 | #7 | #8 | #21 | #22 | #23 | #24 |
| '10MHz+G50MHz+10MHz' | 16.9 | 16.9 | 17.3 | 16.0 | 14.6 | 15.0 | 14.1 | 14.6 |
| Scenario # | #9 | #10 | #11 | #12 | #25 | #26 | #27 | #28 |
| '10MHz+G10MHz+20MHz' | 8.6 | 9.0 | 16.4 | 16.0 | 6.9 | 10.4 | 15.0 | 14.1 |
| Scenario # | #13 | #14 | #15 | #16 | #29 | #30 | #31 | #32 |
| '10MHz+G40MHz+20MHz' | 16.5 | 16.9 | 16.5 | 15.2 | 14.3 | 14.6 | 14.1 | 14.6 |

Table 2-42: SSSB MPR simulation results for SL Non-contiguous CA with 2x20dBm+1LO

| Scenario # | #1 | #2 | #3 | #4 | #17 | #18 | #19 | #20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| '10MHz+G10MHz+10MHz' | 9.6 | 9.6 | 16.5 | 17.0 | 8.4 | 8.4 | 15.3 | 15.3 |
| Scenario # | #5 | #6 | #7 | #8 | #21 | #22 | #23 | #24 |
| '10MHz+G50MHz+10MHz' | 17.8 | 17.0 | 17.4 | 17.5 | 15.3 | 15.3 | 15.3 | 15.8 |
| Scenario # | #9 | #10 | #11 | #12 | #25 | #26 | #27 | #28 |
| '10MHz+G10MHz+20MHz' | 10.0 | 9.6 | 17.4 | 17.0 | 8.6 | 12.0 | 15.8 | 15.8 |
| Scenario # | #13 | #14 | #15 | #16 | #29 | #30 | #31 | #32 |
| '10MHz+G40MHz+20MHz' | 16.6 | 16.6 | 17.5 | 17.0 | 15.5 | 15.7 | 15.8 | 16.6 |

Table 2-43: SSSB MPR simulation results for SL Non-contiguous CA with 2x20dBm+2LO

| Scenario # | #1 | #2 | #3 | #4 | #17 | #18 | #19 | #20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| '10MHz+G10MHz+10MHz' | 4.0 | 4.0 | 7.9 | 8.0 | 4.8 | 5.7 | 7.0 | 7.5 |
| Scenario # | #5 | #6 | #7 | #8 | #21 | #22 | #23 | #24 |
| '10MHz+G50MHz+10MHz' | 8.4 | 8.4 | 8.4 | 8.4 | 7.5 | 7.0 | 7.0 | 7.0 |
| Scenario # | #9 | #10 | #11 | #12 | #25 | #26 | #27 | #28 |
| '10MHz+G10MHz+20MHz' | 4.1 | 5.2 | 7.9 | 8.0 | 4.8 | 6.5 | 6.5 | 7.1 |
| Scenario # | #13 | #14 | #15 | #16 | #29 | #30 | #31 | #32 |
| '10MHz+G40MHz+20MHz' | 7.9 | 7.9 | 7.9 | 8.0 | 7.0 | 7.0 | 7.0 | 7.0 |

Table 2-44 shows the maximum value of simulation results for SL non-contiguous CA considering the frequency of FIM3,low_block,low & FIM3,high_block,high.

Table 2-44 : PC3 SSSB MPR simulation results for SL non-contiguous CA

|  | MPR (dB) for IM3 frequency |  |  |  |
| --- | --- | --- | --- | --- |
|  | SEMfreq_-13 |  | SEfreq_-30 |  |
|  | SCS=15kHz (B=3.96) | SCS=30kHz (B=7.92) | SCS=15kHz (B=3.96) | SCS=30kHz (B=7.92) |
| 1x23dBm + 1LO | 9.0 | 7.6 | 17.3 | 15.0 |
| 2x20dBm + 1LO | 10.0 | 8.6 | 17.8 | 16.6 |
| 2x20dBm + 2LO | 5.2 | 5.7 | 8.4 | 7.5 |

The MPR can be proposed as Table 2-45 based on the simulation results when considering implementation margin.

Table 2-45 : PC3 SSSB MPR for SL non-contiguous CA

|  | MPR (dB) for IM3 frequency |  |  |  |
| --- | --- | --- | --- | --- |
|  | SEMfreq_-13 |  | SEfreq_-30 |  |
|  | SCS=15kHz (B=3.96) | SCS=30kHz (B=7.92) | SCS=15kHz (B=3.96) | SCS=30kHz (B=7.92) |
| 1x23dBm + 1LO | ≤ 12.0 | ≤ 10.5 | ≤ 20.0 | ≤ 18.0 |
| 2x20dBm + 1LO | ≤ 13.0 | ≤ 11.5 | ≤ 21.0 | ≤ 20.0 |
| 2x20dBm + 2LO | ≤ 8.5 | ≤ 7.5 | ≤ 15.0 | ≤ 14.0 |

#### Proposal 13: Consider PC3 SSSB MPR in Table 2-45 for SL non-contiguous CA.

## Configured output power

The existing configured output power of SL intra-band contiguous CA can be reused for SL intra-band non-contiguous CA. For it, some wording is necessary to be updated.

For example,

- intra-band contiguous SL CA operation  intra-band contiguous SL CA operation and intra-band non-contiguous SL CA operation

#### Proposal 14: Update wording to apply the existing requirement of configured output power  to SL non-contiguous CA.

# Conclusion

In this contribution, we provided our view on Tx requirements of SL intra-band non-contiguous CA. Based on those, we propose as follows.

### PC2 & PC3 PSSCH MPR

#### Proposal 1: Consider MPR evaluation scenario in Table 2-3.

#### Proposal 2: Consider PC2 MPR in Table 2-10 for SL non-contiguous CA with 1x26dBm PA + 1LO.

Table 2-10 : PSSCH/PSCCH MPR for SL non-contiguous CA with 1x26dBm PA + 1LO

| B | MPR (dB) for IM3 frequency |  |
| --- | --- | --- |
|  | SEMfreq_-13 | SEfreq_-30 |
| 0 ≤ B < 5.04 | ≤ 11.0 | ≤ 18.5 |
| 5.04 ≤ B < 10.08 | ≤ 10.0 | ≤ 17.0 |
| 10.08 ≤ B |  | ≤ 16.0 |

#### Proposal 3: Consider PC2 MPR in Table 2-11 for SL non-contiguous CA with 2x23dBm PA + 1LO.

Table 2-11 : PSSCH/PSCCH MPR for SL non-contiguous CA with 2x23dBm PA + 1LO

| B | MPR (dB) for IM3 frequency |  |
| --- | --- | --- |
|  | SEMfreq_-13 | SEfreq_-30 |
| 0 ≤ B < 5.04 | ≤ 11.5 | ≤ 19.0 |
| 5.04 ≤ B < 10.08 | ≤ 11.0 | ≤ 17.5 |
| 10.08 ≤ B |  | ≤ 16.5 |

#### Proposal 4: Consider PC2 MPR in Table 2-12 for SL non-contiguous CA with 2x23dBm PA + 2LO.

Table 2-12 : PSSCH/PSCCH MPR for SL non-contiguous CA with 2x23dBm PA + 2LO

| B | MPR (dB) for IM3 frequency |  |
| --- | --- | --- |
|  | SEMfreq_-13 | SEfreq_-30 |
| 0 ≤ B < 5.04 | ≤ 7.0 | ≤ 15.0 |
| 5.04 ≤ B < 10.08 |  | ≤ 13.0 |
| 10.08 ≤ B < 20.16 |  | ≤ 12.0 |
| 20.16 ≤ B |  | ≤ 10.0 |

#### Proposal 5: Consider PC3 MPR in Table 2-21 for SL non-contiguous CA with 1x23dBm PA + 1LO.

Table 2-21 : PSSCH/PSCCH MPR for SL non-contiguous CA with 1x23dBm PA + 1LO

| B | MPR (dB) for IM3 frequency |  |
| --- | --- | --- |
|  | SEMfreq_-13 | SEfreq_-30 |
| 0 ≤ B < 5.04 | ≤ 10.0 | ≤ 16.5 |
| 5.04 ≤ B < 10.08 | ≤ 9.0 | ≤ 15,5 |
| 10.08 ≤ B |  | ≤ 15.0 |

#### Proposal 6: Consider PC3 MPR in Table 2-22 for SL non-contiguous CA with 2x20dBm PA + 1LO.

Table 2-22 : PSSCH/PSCCH MPR for SL non-contiguous CA with 2x20dBm PA + 1LO

| B | MPR (dB) for IM3 frequency |  |
| --- | --- | --- |
|  | SEMfreq_-13 | SEfreq_-30 |
| 0 ≤ B < 5.04 | ≤ 10.5 | ≤ 17.0 |
| 5.04 ≤ B < 10.08 | ≤ 9.5 | ≤ 16.0 |
| 10.08 ≤ B |  | ≤ 15.5 |

#### Proposal 7: Consider PC3 MPR in Table 2-23 for SL non-contiguous CA with 2x20dBm PA + 2LO.

Table 2-23 : PSSCH/PSCCH MPR for SL non-contiguous CA with 2x20dBm PA + 2LO

| B | MPR (dB) for IM3 frequency |  |
| --- | --- | --- |
|  | SEMfreq_-13 | SEfreq_-30 |
| 0 ≤ B < 5.04 | ≤ 6.5 | ≤ 13.5 |
| 5.04 ≤ B < 10.08 | ≤ 6.5 | ≤ 12.5 |
| 10.08 ≤ B < 20.16 |  | ≤ 11.5 |
| 20.16 ≤ B |  | ≤ 10.5 |

### PC2 & PC3 PSFCH MPR

#### Proposal 8: Consider PSFCH MPR evaluation scenario in Table 2-24.

#### Proposal 9: Consider PC2 PSFCH MPR in Table 2-29 for SL non-contiguous CA.

Table 2-29 : PC2 PSFCH MPR for SL non-contiguous CA

|  | MPR (dB) for IM3 frequency |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | SEMfreq_-13 |  |  | SEfreq_-30 |  |  |
|  | SCS=15kHz (B=0.36) | SCS=30kHz (B=0.72) | SCS=60kHz (B=1.44) | SCS=15kHz (B=0.36) | SCS=30kHz (B=0.72) | SCS=60kHz (B=1.44) |
| 1x26dBm + 1LO | ≤ 15.0 | ≤ 15.0 | ≤ 14.0 | ≤ 20.0 | ≤ 20.0 | ≤ 19.0 |
| 2x23dBm + 1LO | ≤ 15.0 | ≤ 15.0 | ≤ 14.0 | ≤ 20.0 | ≤ 20.0 | ≤ 19.0 |
| 2x23dBm + 2LO | ≤ 9.5 | ≤ 8.5 | ≤ 7.5 | ≤ 16.5 | ≤ 16.0 | ≤ 15.0 |

#### Proposal 10: Consider PC3 PSFCH MPR in Table 2-34 for SL non-contiguous CA.

Table 2-34 : PC3 PSFCH MPR for SL non-contiguous CA

|  | MPR (dB) for IM3 frequency |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | SEMfreq_-13 |  |  | SEfreq_-30 |  |  |
|  | SCS=15kHz (B=0.36) | SCS=30kHz (B=0.72) | SCS=60kHz (B=1.44) | SCS=15kHz (B=0.36) | SCS=30kHz (B=0.72) | SCS=60kHz (B=1.44) |
| 1x26dBm + 1LO | ≤ 12.0 | ≤ 12.0 | ≤ 11.0 | ≤ 18.0 | ≤ 18.0 | ≤ 17.0 |
| 2x23dBm + 1LO | ≤ 12.0 | ≤ 12.0 | ≤ 11.0 | ≤ 18.0 | ≤ 18.0 | ≤ 17.0 |
| 2x23dBm + 2LO | ≤ 9.0 | ≤ 8.0 | ≤ 7.0 | ≤ 16.0 | ≤ 15.5 | ≤ 15.0 |

### PC2 & PC3 S-SSB MPR

#### Proposal 11: Consider SSSB MPR evaluation scenario in Table 2-35.

#### Proposal 12: Consider PC2 SSSB MPR in Table 2-40 for SL non-contiguous CA.

Table 2-40 : PC2 SSSB MPR for SL non-contiguous CA

|  | MPR (dB) for IM3 frequency |  |  |  |
| --- | --- | --- | --- | --- |
|  | SEMfreq_-13 |  | SEfreq_-30 |  |
|  | SCS=15kHz (B=3.96) | SCS=30kHz (B=7.92) | SCS=15kHz (B=3.96) | SCS=30kHz (B=7.92) |
| 1x26dBm + 1LO | ≤ 13.0 | ≤ 12.0 | ≤ 22.0 | ≤ 20.0 |
| 2x23dBm + 1LO | ≤ 14.0 | ≤ 13.0 | ≤ 23.0 | ≤ 21.0 |
| 2x23dBm + 2LO | ≤ 9.0 | ≤ 8.0 | ≤ 17.0 | ≤ 15.0 |

#### Proposal 13: Consider PC3 SSSB MPR in Table 2-45 for SL non-contiguous CA.

Table 2-45 : PC3 SSSB MPR for SL non-contiguous CA

|  | MPR (dB) for IM3 frequency |  |  |  |
| --- | --- | --- | --- | --- |
|  | SEMfreq_-13 |  | SEfreq_-30 |  |
|  | SCS=15kHz (B=3.96) | SCS=30kHz (B=7.92) | SCS=15kHz (B=3.96) | SCS=30kHz (B=7.92) |
| 1x23dBm + 1LO | ≤ 12.0 | ≤ 10.5 | ≤ 20.0 | ≤ 18.0 |
| 2x20dBm + 1LO | ≤ 13.0 | ≤ 11.5 | ≤ 21.0 | ≤ 20.0 |
| 2x20dBm + 2LO | ≤ 8.5 | ≤ 7.5 | ≤ 15.0 | ≤ 14.0 |

### Configured output power

#### Proposal 14: Update wording to apply the existing requirement of configured output power  to SL non-contiguous CA.

# References

[1] R4-2414312, “WF on NR sidelink for intra-band non-contiguous CA”, OPPO

[2] R4-2410586, “WF on NR_SL_ intraB_CA_ITS_part1”, OPPO, Meta Ireland

[3] R4-2406611, “WF on NR_SL_ intraB_CA_ITS”, OPPO, Meta Ireland

[4] RP-240840, “New WID on NR Sidelink: Intra-band Carrier Aggregation in ITS band”, Moderator(Apple)