3GPP TSG-RAN WG4 Meeting #112                                                       	R4-2416067

# Hefei, China, 14th – 18th Oct, 2024

Agenda item:			6.9.4

Source:	LG Electronics

Title:	Discussion on NR NTN Ku-Band SCS and CBW

Document for:	Approval

# Introduction

The General requirements of the Rel-19 NR NTN Ku band work item were discussed in RAN4#112. The WF in [1] captures the result of the discussion in the meeting. This document discusses the System parameter and Numerology related Issues 3-3, 3-4 and 3-5 listed there.

#### Issue 3-2: Numerology - Decision criteria

The following technical criteria will be used to determine the feasibility of FR1 and FR2 numerology

- UL timing synchronization

- Phase noise

- Efficient channel utilization

- Beam hopping granularity

- Link budget UL/DL – as a function of SCS and channel bandwidth

#### Issue 3-3: Numerology – timing aspects

The impact of SCS on timing is critical to reliable performance therefore decisions on FR1 and FR2 numerology feasibility should not be made without a thorough analysis of the issues.

#### Issue 3-4: Numerology – phase noise

The impact of SCS on susceptibility to phase noise is critical to reliable performance therefore decisions on FR1 and FR2 numerology feasibility should not be made without a thorough analysis of the issues.

#### Issue 3-5: Numerology – efficient channel utilization

The ability to efficiently support in future the required aggregate bandwidths [list] of NTN services for different SCS choices based on existing FR1 and FR2 channel bandwidths needs to be fully studied.

# Discussion

## Issue 3-3: Numerology – timing aspects

The impact of SCS on timing is critical to reliable performance therefore decisions on FR1 and FR2 numerology feasibility should not be made without a thorough analysis of the issues.

The SCS, which directly also sets the duration of the symbol and cyclic prefix (CP) is important for the timing accuracy of the uplink. For OFDMA receiver it is important that received symbols are time-aligned, with channel delay spread included, within the CP to avoid leakage between the symbols in the FFT process. This then requires the timing advance of different uplink users to be adjusted in a way that above mentioned time alignment is achieved at the receiver side.

The duration of the CP with different Sub-carrier spacing in shown in table below

| NTN FR1/FR2 | SCS (kHz) | Short CP duration (us) | Long CP duration (us) |
| --- | --- | --- | --- |
| FR1 | 15 | 4.68 | 5.21 |
| FR1 | 30 | 2.34 | 2.34 |
| FR1 and FR2 | 60 | 1.17 | 1.69 |
| FR2 | 120 | 0.586 | 1.107 |

As can be seen from the table above the duration of the CP changes from about 5us down to under 1us with different SCS between 15 and 120kHz.

The distance and the propagation delay between the UE and satellite depends of the height of the satellite orbit. These are analysed on high level using the simplified diagram shown in Figure 1. For LEO with height of 600km the propagation delay is roughly 2ms but for GEO with height of 35785km it is more than 100ms. The propagation delay differences that need to be adjusted with timing advance between two UEs at 1 degree apart from one another (UE-UE distance is about 10km for LEO and 600km for GOE), when looking from the satellite, are about 0.3us and 18us.

The delay adjustment is also needed due to movement of the satellites at LEO orbit, which travel with the speed of about 8km/s, which causes the delay to drift at rate of about 0.5us/s. In the case of mobile VSAT UE the movement of the UE is so slow that it would cause only negligible additional impact.

Figure 1. Simplified diagram for defining the 1degree distance difference

Based on there we can make following observations:

Observation 1: Uplink timing control is necessary for both LEO and GEO and that use of lower SCS and longer symbols results in more relaxed requirement for the controlling accuracy.

Observation 2: The 60 and 120kHz SCS are already defined for NR FR2 and therefore operation also with these wider SCS used at Ka band has been considered feasible and should therefore also apply for Ku-band.

## Issue 3-4: Numerology – phase noise

The impact of SCS on susceptibility to phase noise is critical to reliable performance therefore decisions on FR1 and FR2 numerology feasibility should not be made without a thorough analysis of the issues.

Phase noise performance of the base-station and UE implementations for different frequency ranges has been analyzed in several occasions in RAN4. The conclusions here have changed over time as expected when improvements in the technology are made and cost and power efficient solutions can be used with good performance level also at higher frequencies. For example, the frequency range for Ku-band was analyzed in 2018-2019 in Study on the 7 to 24 GHz frequency range for NR with results documented in TR38.820 [2].

Observation 3: Based on the results already shown in [2] and other publically available information it seems that phase-noise performance of the integrated PLLs enable also use of at least 30kHz (if not even 15kHz) SCS on Ku-band even if modulations up to 64QAM are considered. On the other hand, larger SCS like 60kHz or 120kHz still leads to more relaxed phase-noise requirements for a given signal quality requirement level. This means that phase-noise can occupy smaller part of the total error budget allowing higher contribution for other error terms like compression, which may then allow use of higher output power and more efficient transmitters.

## Issue 3-5: Numerology – efficient channel utilization

The ability to efficiently support in future the required aggregate bandwidths [list] of NTN services for different SCS choices based on existing FR1 and FR2 channel bandwidths needs to be fully studied.

The supported Maximum transmission bandwidth configurations for FR1-NTN and FR-2 NRN are shown in 2.3.1 below with FR1-NTN supporting widths from 5 to 30MHz and FR2-NTN from 50 up to 400MHz.

Based on the discussion in the RAN4#112 the FR2-NTN bandwidths are considered too wide to be effectively used in the existing satellites and narrower bandwidth options would be necessary for better match with the existing satellite RF configurations to achieve optimum implementation efficiency.

Proposal 1: RAN4 to study how narrower bandwidths below 50MHz could also be introduced to FR2-NTN, like 20 and 30MHz as shown in table below.

| SCS | 20 | 30 | 50 | 100 | 200 | 400 |
| --- | --- | --- | --- | --- | --- | --- |
| 60 | 24 | 38 | 66 | 132 | 264 | N/A |
| 120 | 12 | 18 | 32 | 66 | 132 | 264 |

Observation 4: It is also important on ensure that all needed channels and signals can be transmitted within the narrower bandwidths like SS/PBCH (SSB). For FR2-NTN the SSB is only defined for 120kHz SCS and SSB occupies 240 SCs i.e. 20 RBs and has a width of 28.8MHz and therefore the 20 and 30MHz bandwidths for 120kHz SCS in Proposal 1 are not compatible with existing RAN1 specifications [3].

However, RAN1 has already defined in 38.211 sub-clause7.4.3 that for cell search on a carrier with a channel bandwidth of 3 MHz, the UE is not expected to receive subcarriers 0 to 47 and 192 to 239 in any of the 4 OFDM symbols of the SS/PBCH block, where the remaining 12 resource blocks form the SS/PBCH block after puncturing.

Furthermore, the 38.213 clause 13 already includes the definition for the CORESET 0 occupying only 12 resource blocks.

Observation 5: Utilizing these same approach for FR2-NTN with 20 and 30MHz CBW as already used for 3MHz channel bandwidth, could enable the FR2 numerology based solution also to support channel bandwidths that are narrower than 50MHz as in existing FR2-NRN specification.

Proposal 2: RAN4 to study and request RAN1 feedback for using similar approach for 20 and 30MHz CBW FR2-NTN as already used for 3MHz channel bandwidth, where 12 resource block wide SS/BPBCH and CORESET 0 are used.

### Extract from TS38.101-5

### 5.3.2	Maximum transmission bandwidth configuration

The maximum transmission bandwidth configuration NRB for each UE channel bandwidth and subcarrier spacing is specified in Table 5.3.2-1 for FR1-NTN and table 5.3.2-2 for FR2-NTN.

Table 5.3.2-1: Maximum transmission bandwidth configuration NRB for FR1-NTN

| SCS (kHz) | 5MHz | 10MHz | 15MHz | 20MHz | 30MHz |
| --- | --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB | NRB |
| 15 | 25 | 52 | 79 | 106 | 160 |
| 30 | 11 | 24 | 38 | 51 | 78 |
| 60 | N/A | 11 | 18 | 24 | 38 |

Table 5.3.2-2: Maximum transmission bandwidth configuration NRB for FR2-NTN

| SCS (kHz) | 50MHz | 100MHz | 200MHz | 400MHz |
| --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB |
| 60 | 66 | 132 | 264 | N/A |
| 120 | 32 | 66 | 132 | 264 |

# Conclusions

The General requirements of the Rel-19 NR NTN Ku band work item were discussed in RAN4#112. The WF in [1] captures the result of the discussion in the meeting. This document discusses the System parameter and Numerology related Issues 3-3, 3-4 and 3-5 listed there.

Observation 1: Uplink timing control is necessary for both LEO and GEO and that use of lower SCS and longer symbols results in more relaxed requirement for the controlling accuracy.

Observation 2: The 60 and 120kHz SCS are already defined for NR FR2 and therefore operation also with these wider SCS used at Ka-band has been considered feasible and should therefore also apply for Ku-band.

Observation 3: Based on the results already shown in [2] and other publically available information it seems that phase-noise performance of the integrated PLLs enable also use of at least 30kHz (if not even 15kHz) SCS on Ku-band even if modulations up to 64QAM are considered. On the other hand, larger SCS like 60kHz or 120kHz still leads to more relaxed phase-noise requirements for a given signal quality requirement level. This means that phase-noise can occupy smaller part of the total error budget allowing higher contribution for other error terms like compression, which may then allow use of higher output power and more efficient transmitters.

Proposal 1: RAN4 to study how narrower bandwidths below 50MHz could also be introduced to FR2-NTN, like 20 and 30MHz as shown in table below.

| SCS | 20 | 30 | 50 | 100 | 200 | 400 |
| --- | --- | --- | --- | --- | --- | --- |
| 60 | 24 | 38 | 66 | 132 | 264 | N/A |
| 120 | 12 | 18 | 32 | 66 | 132 | 264 |

Observation 4: It is also important on ensure that all needed channels and signals can be transmitted within the narrower bandwidths like SS/PBCH (SSB). For FR2-NTN the SSB is only defined for 120kHz SCS and SSB occupies 240 SCs i.e. 20 RBs and has a width of 28.8MHz and therefore the 20 and 30MHz bandwidths for 120kHz SCS in Proposal 1 are not compatible with existing RAN1 specifications [3].

Observation 5: Utilizing these same approach for FR2-NTN with 20 and 30MHz CBW as already used for 3MHz channel bandwidth, could enable the FR2 numerology based solution also to support channel bandwidths that are narrower than 50MHz as in existing FR2-NRN specification.

Proposal 2: RAN4 to study and request RAN1 feedback for using similar approach for 20 and 30MHz CBW FR2-NTN as already used for 3MHz channel bandwidth, where 12 resource block wide SS/BPBCH and CORESET 0 are used.

# References

[1] R4-2413519, Way Forward for [112][314] NR_NTN_Ku_Band_General, Moderator (Eutelsat Group)

[2] TR38.820, Study on the 7 to 24 GHz frequency range for NR (v1.0.0. RAN#86, January 2020)

[3] TS38.211, Physical channels and modulation

[3] TS38.213, Physical layer procedures for control