3GPP TSG-RAN WG4 Meeting #112bis	R4-2416070

Hefei, Anhui, China, 14 – 18 October, 2024

Source:	Ericsson

Title:	On UE Capability/UE behavior and network signaling for Type 4 EN-DC/NR-CA

Agenda item:	6.5.2.2

Document for:	Approval

# 1	Introduction

In RAN4#112 we agreed the following WF [1]:

… ## 1.2 Sub-topic 2-3: UE Capability(s)/UE behavior and BS signalling for Type 4 EN-DC/NR-CA ### 1.2.1 < Issue 2-3-1: New UE Capability for Type 4a(6Rx/UE) and 4b(8Rx/UE) for EN-DC/NR-CA > Way forward: - Define one capability for NR CA and one UE capability for EN-DC - FFS on how to differentiate type 4a and 4b. … ### 1.2.3 < Issue 2-3-4: New BS Signaling to switch between Type 4a/4b and Type 1 4L/CC(collocated)> Way Forward: - First, conclude discussions of whether to add new UE capability(s) for Type 4. - And then, continue further discussion on BS signalling in the next meeting. … ### 1.2.4 < Issue 2-3-5: New BS Signaling to switch between Type 4a/4b and Type 1 6L/8L/CC(collocated) > Way Forward: - First, conclude discussions of whether to add new UE capability(s) for Type 4. - And then, continue further discussion on BS signalling in the next meeting. … ### 1.2.6 < Issue 2-3-7: UE behavior during initial connection between Type 1, Type 2 and Type 4a/4b > Way Forward: - First, conclude discussions of whether to add new UE capability(s) for Type 4 and BS signaling. - And then, continue further discussion on UE behavior in the next meeting. …

In this contribution we discuss the open issues and make proposals.

# 2	Discussion

## 2.1	< Issue 2-3-1:  New UE Capability for Type 4a(6Rx/UE) and 4b(8Rx/UE) for EN-DC/NR-CA >

The WF [1] states agreement that RAN4 shall define one capability for NR CA and one UE capability for EN-DC and that it is FFS on how to differentiate type 4a and 4b.

The UE types are defined in R4-2217734 (WF on NonCol_intraB_ENDC_NR_CA for New Type UE [2]):

Table 1: UE Types [2]

| UE Type | CC# | antenna / LNA |  | Mixer | Analog BB | #Rx | Frequency Separation between 2cc | NRCA/ENDC | power imbalance | comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 4 shared |  | 4 shared | 4 shared | 4Rx | ≤ X MHz | NRCA,ENDC | 6dB full range | Baseline architecture (i.e. legacy architecture) |
|  | 2 |  |  |  |  | 4Rx |  |  |  |  |
| 2 | 1 | 2 | 4 total | 2 | 2 | 2Rx | No limitation or ≤ X MHz | NRCA,ENDC | 25dB full range | Reuse of baseline architecture restricted to 2Rx/band but need 2LO frequencies |
|  | 2 | 2 |  | 2 | 2 | 2Rx |  |  |  |  |
| 4a | 1 | 4 | 6 total | 4 | 4 | 4Rx | No limitation or ≤ X MHz | ENDC | 25dB full range | Requires 6 antennas and LNA => is it compatible with smartphone? (for which frequency range), FWA only |
|  | 2 | 2 |  | 2 | 2 | 2Rx |  |  |  |  |
| 4b | 1 | 4 | 8 total | 4 | 4 | 4Rx | No limitation or ≤ X MHz | NRCA,ENDC | 25dB full range | Requires 8 antennas and LNA => is it compatible with smartphone? (for which frequency range), FWA only |
|  | 2 | 4 |  | 4 | 4 | 4Rx |  |  |  |  |

The type 4a and 4b UE RF architectures are also illustrated in [3]:

Figure 1: Architecture of type 4a(left) and type 4b(right) (Fig. 2.1 in [3])

From table 1 and figure 1, we can conclude that type 4a ⊂ 4b , i.e. all type 4a properties, #CC, antennas/LNA, mixer,  BB, #RX, NRCA/EBDC-feature support and power imbalance, are a subset of type 4b. Moreover the difference between the type 4a and 4b are small.

Observation 1: 	All type 4a properties, #CC, antennas/LNA, mixer,  BB, #RX, NRCA/EBDC-feature support and power imbalance, are a subset of type 4b. Moreover the difference between the type 4a and 4b are small.

Proposal 1: 	We support to add one new UE capability for Type 4 support indication (no distinction between Type 4a and 4b). How to configure a Type 4 UE is up to UE capability and subsequent RRC configuration.

## 2.2	< Issue 2-3-4: New BS Signaling to switch between Type 4a/4b and Type 1 4L/CC(collocated)>

We have defined rel-18 procedures for switching between Type 2 and Type 1. Our preference is to extend the existing rel-18 signal for switching between Type 2 and Type 1 to also include switching between Type 4 and Type 1.

Observation 2: 	We have defined rel-18 procedures for switching between Type 2 and Type 1.

Proposal 2: 	Our preference is to extend the existing rel-18 signal for switching between Type 2 and Type 1 to also include switching between Type 4 and Type 1. How to configure a Type 4 UE is up to UE capability and subsequent RRC configuration.

## 2.3	< Issue 2-3-5:  New BS Signaling to switch between Type 4a/4b and Type 1 6L/8L/CC(collocated) >

Proposal 3: 	Our preference is to extend the existing rel-18 signal for switching between Type 2 and Type 1 to also include switching between Type 4 and Type 1. How to configure a Type 4 UE is up to UE capability and subsequent RRC configuration.

## 2.4	< Issue 2-3-7:  UE behavior during initial connection between Type 1, Type 2 and Type 4a/4b >

When a UE connects it reports its capabilities. How to operate these capabilities, once reported, is up to UE.

Observation 3: 	When a UE connects it reports its capabilities. How to operate these capabilities, once reported, is up to UE.

Proposal 4: 	The UE can reports its capabilities during initial connection, including supported Types.

# 3 	Summary

Observation 1: 	All type 4a properties, #CC, antennas/LNA, mixer,  BB, #RX, NRCA/EBDC-feature support and power imbalance, are a subset of type 4b. Moreover the difference between the type 4a and 4b are small.

Proposal 1: 	We support to add one new UE capability for Type 4 support indication (no distinction between Type 4a and 4b). How to configure a Type

Observation 2: 	We have defined rel-18 procedures for switching between Type 2 and Type 1.

Proposal 2: 	Our preference is to extend the existing rel-18 signal for switching between Type 2 and Type 1 to also include switching between Type 4 and Type 1. How to configure a Type 4 UE is up to UE capability and subsequent RRC configuration.

Observation 3: 	When a UE connects it reports its capabilities. How to operate these capabilities, once reported, is up to UE.

Proposal 4: 	The UE can reports its capabilities during initial connection, including supported Types.

# References

[1] 		R4-2414453, WF on Rel-19 non-collocated scenario, KDDI