3GPP TSG-RAN WG4 Meeting #112bis	R4-2416071

Hefei, Anhui, China, 14 – 18 October, 2024

Source:	Ericsson

Title:	On	RRM core requirements

Agenda item:	6.5.3

Document for:	Discussion

# 1	Introduction

In RAN4#112 a WF was approved which collects agreements and open issues [1]. The open issues are listed below.

… <Way Forward> Issue 1-3: Whether to introduce MRTD/MTTD requirements for non-collocated FR1 intra-band asynchronous NR-CA for Type 4 UE in Rel-19 - Proposals - Option 1: Not to introduce MRTD/MTTD requirements for non-collocated FR1 intra-band asynchronous NR-CA for Type 4 UE in Rel-19. <Way Forward> Issue 1-4: Other RRM requirements (apart from MRTD/MTTD requirements) for Type 4 UE - Proposals - Option 1(Apple, Nokia, Huawei, ZTE, Ericsson, Samsung, KDDI): The following RRM requirements (apart from MRTD/MTTD requirements) are also impacted due to Type 4 UE - Interruption - SCell activation delay - Scheduling availability - SSB based and CSI-RS based BFD (Beam failure detection) and CBD (candidate beam detection) … <Way Forward> Issue 1-6: Interruption requirements for Type 4 capable UE when UE operates with shared RF chains in collocated scenario - Proposals - Option 1(Apple, Huawei, Samsung, KDDI): when Type 4 capable UE operates with shared RF chains in collocated scenario, interruption requirements (below listed) defined for type 2 UE in collocated scenario in R18 can be applied - Interruption at SCell addition/release - Interruptions at SCell activation/deactivation - Interruptions during measurements on deactivated SCC Note: Some clarifications are needed with regard to the UE capability reporting signalling and network indication - Option 2 (Nokia): wait for RF progress on UE type transition and design of new UE capabilities/NW signalling. … <Way Forward> Issue 1-8: SCell activation delay requirements for Type 4 capable UE when UE operates with shared RF chains in collocated condition - Proposals - Option 1(Apple, Huawei, Samsung, KDDI): When Type 4 capable UE operates with shared RF chain in collocated condition, SCell activation delay defined for type 2 UE in collocated condition in R18 can be applied. Note: Some clarifications are needed with regard to the UE capability reporting signalling and network indication - Option 2 (Nokia): wait for RF progress on UE type transition and design of new UE capabilities/NW signalling. … <Way Forward> Issue 1-10: Scheduling availability for Type 4 capable UE when UE operates with shared RF chain in collocated condition - Proposals - Option 1(Apple, Huawei, Samsung, KDDI): When type 4 capable UE operates with shared RF chain in collocated condition, scheduling availability defined for type 2 UE in collocated condition in R18 can be applied Note: Some clarifications are needed with regard to the UE capability reporting signalling and network indication - Option 2 (Nokia): wait for RF progress on UE type transition and design of new UE capabilities/NW signalling. <Way Forward> Issue 1-11: SCell BFD/CBD requirements for Type 4 capable UE - Proposals - Option 1(Apple, Huawei, ZTE): Type 4 capable UE when operating in non-collocated condition can be configured with BFD/CBD measurements on up to two serving cells per band. …

In this contribution we discuss the open issues.

# 2	Discussion

Issue 1-3: Whether to introduce MRTD/MTTD requirements for non-collocated FR1 intra-band asynchronous NR-CA for Type 4 UE in Rel-19

The option stated was: “Option 1, Not to introduce MRTD/MTTD requirements for non-collocated FR1 intra-band asynchronous NR-CA for Type 4 UE in Rel-19”. 
   Asynchronous operation means that we have no predefined relation between slot timing of different carriers to be aggregated and the handling at least a relative receive timing difference given by ±1/2 slot duration.

Observation 1:	Asynchronous operation means that we have no predefined relation between slot timing of different carriers to be aggregated and the handling at least a relative receive timing difference given by ±1/2 slot duration.

Proposal 1:	We are open to not to state MRTD/MTTD requirements for non-collocated FR1 intra-band asynchronous NR-CA and have this implicit. Max MRTD will be ±1/2 slot duration, regardless.

Issue 1-4: Other RRM requirements (apart from MRTD/MTTD requirements) for Type 4 UE

- Proposals

  - Option 1(Apple, Nokia, Huawei, ZTE, Ericsson, Samsung, KDDI): The following RRM requirements (apart from MRTD/MTTD requirements) are also impacted due to Type 4 UE

- Interruption

- SCell activation delay

- Scheduling availability

- SSB based and CSI-RS based BFD (Beam failure detection) and CBD (candidate beam detection)

The release-18 WI worked out Scheduling restriction/availability and SCell operation related delay and interruption for non colocated intra band ENDC and NRCA. We still have the same 2 CC non-colocated case in release 19, as in release 18. The only difference is that we had RANK2 MIMO per CC in release-18 and we have up to RANK 4 MIMO per CC in release-19.

Observation 2: 	We still have the same 2 CC non-colocated case in release 19, as in release 18. The only difference is that we had RANK2 MIMO per CC in release-18 and we have up to RANK 4 MIMO per CC in release-19.

Proposal 2:	The development for release-19 non-colocated intra band ENDC and NRCA requirements can use release-18 Scheduling restriction/availability, SCell operation related delay and interruptions as starting points for analysis.

Issue 1-6: Interruption requirements for Type 4 capable UE when UE operates with shared RF chains in collocated scenario

- Proposals

  - Option 1(Apple, Huawei, Samsung, KDDI): when Type 4 capable UE operates with shared RF chains in collocated scenario, interruption requirements (below listed) defined for type 2 UE in collocated scenario in R18 can be applied

- Interruption at SCell addition/release

- Interruptions at SCell activation/deactivation

- Interruptions during measurements on deactivated SCC

Note: Some clarifications are needed with regard to the UE capability reporting signalling and network indication

  - Option 2 (Nokia): wait for RF progress on UE type transition and design of new UE capabilities/NW signalling.

We believe it is a fair assumption that when Type 4 capable UE operates with shared RF chains in collocated scenario, interruption requirements (below listed) defined for type 2 UE in collocated scenario in R18 can be applied for Interruption at SCell addition/release, Interruptions at SCell activation/deactivation and Interruptions during measurements on deactivated SCC, since we are dealing with a shared receiver, also in type 4 (with different MIMO rank capability).

Observation 3: 	We are dealing with a shared receiver, also in type 4 (with different MIMO rank capability).

Proposal 3:	It is a fair assumption that when Type 4 capable UE operates with shared RF chains in collocated scenario, interruption requirements (below listed) defined for type 2 UE in collocated scenario in R18 can be applied for Interruption at SCell addition/release, Interruptions at SCell activation/deactivation and Interruptions during measurements on deactivated SCC.

Issue 1-8: SCell activation delay requirements for Type 4 capable UE when UE operates with shared RF chains in collocated condition

- Proposals

  - Option 1(Apple, Huawei, Samsung, KDDI): When Type 4 capable UE operates with shared RF chain in collocated condition, SCell activation delay defined for type 2 UE in collocated condition in R18 can be applied.

Note: Some clarifications are needed with regard to the UE capability reporting signalling and network indication

  - Option 2 (Nokia): wait for RF progress on UE type transition and design of new UE capabilities/NW signalling.

Observation 4: 	We are dealing with a shared receiver, also in type 4 (with different MIMO rank capability).

Proposal 4:	When Type 4 capable UE operates with shared RF chain in collocated condition, SCell activation delay defined for type 2 UE in collocated condition in R18 can be applied.

Issue 1-10: Scheduling availability for Type 4 capable UE when UE operates with shared RF chain in collocated condition

- Proposals

  - Option 1(Apple, Huawei, Samsung, KDDI): When type 4 capable UE operates with shared RF chain in collocated condition, scheduling availability defined for type 2 UE in collocated condition in R18 can be applied

Note: Some clarifications are needed with regard to the UE capability reporting signalling and network indication

  - Option 2 (Nokia): wait for RF progress on UE type transition and design of new UE capabilities/NW signalling.

Observation 5: 	We are dealing with a shared receiver, also in type 4 (with different MIMO rank capability).

Proposal 5:	Scheduling availability defined for type 2 UE in collocated condition in R18 can be applied for type 4.

Issue 1-11: SCell BFD/CBD requirements for Type 4 capable UE

- Proposals

  - Option 1(Apple, Huawei, ZTE): Type 4 capable UE when operating in non-collocated condition can be configured with BFD/CBD measurements on up to two serving cells per band.

Observation 6: 	BFD/CBD measurements on up to two serving cells per band is in existing specification for rel-18 Type 2 UE.

Proposal 6:	Type 4 capable UE when operating in non-collocated condition can be configured with BFD/CBD measurements on up to two serving cells per band.

# 3 	Summary

Observation 1:	Asynchronous operation means that we have no predefined relation between slot timing of different carriers to be aggregated and the handling at least a relative receive timing difference given by ±1/2 slot duration.

Proposal 1:	We are open to not to state MRTD/MTTD requirements for non-collocated FR1 intra-band asynchronous NR-CA and have this implicit. Max MRTD will be ±1/2 slot duration, regardless.

Observation 2: 	We still have the same 2 CC non-colocated case in release 19, as in release 18. The only difference is that we had RANK2 MIMO per CC in release-18 and we have up to RANK 4 MIMO per CC in release-19.

Proposal 2:	The development for release-19 non-colocated intra band ENDC and NRCA requirements can use release-18 Scheduling restriction/availability, SCell operation related delay and interruptions as starting points for analysis.

Observation 3: 	We are dealing with a shared receiver, also in type 4 (with different MIMO rank capability).

Proposal 3:	It is a fair assumption that when Type 4 capable UE operates with shared RF chains in collocated scenario, interruption requirements (below listed) defined for type 2 UE in collocated scenario in R18 can be applied for Interruption at SCell addition/release, Interruptions at SCell activation/deactivation and Interruptions during measurements on deactivated SCC.

Observation 4: 	We are dealing with a shared receiver, also in type 4 (with different MIMO rank capability).

Proposal 4:	When Type 4 capable UE operates with shared RF chain in collocated condition, SCell activation delay defined for type 2 UE in collocated condition in R18 can be applied.

Observation 5: 	We are dealing with a shared receiver, also in type 4 (with different MIMO rank capability).

Proposal 5:	Scheduling availability defined for type 2 UE in collocated condition in R18 can be applied for type 4.

Observation 6: 	BFD/CBD measurements on up to two serving cells per band is in existing specification for rel-18 Type 2 UE.

Proposal 6:	Type 4 capable UE when operating in non-collocated condition can be configured with BFD/CBD measurements on up to two serving cells per band.

# References

[1] 		R4-2413973, WF on RRM requirements for supporting intra-band non-collocated EN-DC/NR-CA deployment Phase2: new receiver type(s), Huawei, HiSilicon.