3GPP TSG-RAN WG4 Meeting #112bis	R4-2416076

Hefei, CN, October 14th-18th, 2024

Source: 	Ericsson

Title:  	Discussion on RedCap RRM requirements for NTN for NR Phase 3

Agenda Item:	6.26.4.1

Document for:	Discussion

# Introduction

In the light of the WI NR Non-Terrestrial Networks (NTN) for NR Phase 3, the outcomes of the last meeting's debate, encompassing consensuses and unresolved issues, were documented in WF [1].

This contribution outlines our point of view on on the underlying impacts to RRM requirements mandated by the issues summarized in the WF.

# Discussion

## Topic #6: Support of (e)RedCap UEs with NR FR1-NTN

Issue 6-2-2: What RRM requirements are defined for (e)RedCap UE with FR1-NTN

<Way Forward> - FFS for the following requirements: - Minimization of Drive Tests (MDT) in RRC_IDLE state and RRC_INACTIVE state - NR Rel-17/18 Conditional Handover, and RACH-less Handover - Note: further check whether they are supported for RedCap NTN in RAN1/2 spec. - FFS: Send LS to RAN1/2 to ask the status. RAN4 does not trigger the RAN1/2 discussion to support these features. - Configured Grant based Small Data Transmissions (CG-SDT) - Random access based Small Data Transmissions (RA-SDT)

To our understanding, the SDT procedure shall be supported by RedCap for NTN. But TA validation based on the RSRP change criterion for RedCap in TN shall be removed and the requirements may refer to the SDT requirements for IoT in NTN.

Proposal 1: Support SDT for RedCap for NTN. TA validation based on the RSRP change criterion for RedCap in TN shall be removed and the requirements may refer to the SDT requirements for IoT for NTN.

Since MDT isn’t supported in RedCap for TN, we don’t see the necessity to introduce the feature for RedCap for NTN, which conflict with the lower-cost, lower-complexity and energy-efficient design target of RedCap.

Proposal 2: Not support MDT for RedCap for NTN.

In NTN Rel-17 and Rel-18, conditional handover plays a key role to optimize the mobility in NTN by adding time, location-based events which contain some specific parameters acquirable in NTN framework. Supporting such features may bring observable benefits to RedCap for NTN, e.g., signaling saving and measurement efficiency, efficient resource utilization, adaptation to satellite movement. In this manner, conditional handover is expected to be enabled for RedCap for NTN, at least the fundamental Rel-17 conditional handover for NTN shall be supported.

Proposal 3: Support conditional handover for RedCap for NTN, at least conditional handover for NTN in Rel-17.

In the last meeting, the below agreements were obtained, but it’s unclear if NTN specific cell reselection also is applicable to RedCap for NTN. The issue shall be clarified.

- The requirements defined for both NTN and (e)RedCap UE requirements should be defined for (e)RedCap UE with FR1-NTN bands, including the following: - Cell Re-selection for RRC_IDLE state mobility - Cell Re-selection for RRC_INACTIVE state mobility

Proposal 4: Support NTN specific time/location assisted cell reselection for RedCap for NTN.

RACH-less handover was introduced in Rel-18 for NTN. One of the advantages of this procedure is that the UE can access the target cell without exchanging Msg1 and Msg2 of the RACH procedure and therefore reducing the signaling overhead. But we don’t see the urgent necessity to introduce to for RedCap in NTN. Actually, the application of RACH-less handover may not so common in satellite constellation cases.

Proposal 5: Deprioritize RACH-less handover for RedCap for NTN.

Issue 6-2-3: Features to be considered for (e)RedCap UE with FR1-NTN

<Way Forward> - RAN4 to analyze the RRM requirement impacts from following features for (e)RedCap UE with FR1-NTN case by case: - 1Rx - FFS: NCD-SSB - HD-FDD - FFS: R17 relaxed measurement - FFS: RRC_IDLE/RRC_INACTIVE mode eDRX enhancement - FFS: Determine if CSSF and concurrent gap is supported in Redcap NTN - FFS: FH in PRS measurement - Single cell search and measurement engine

Regarding one objective of this WI, ‘Rel-17 RedCap and Rel-18 eRedCap UEs with NR NTN operating in FR1-NTN bands’, it appears that RedCap shall support FR1-NTN bands. That’s the reason that at least one company in the last meeting proposed to preclude NCD-SSB support, since of enough bandwidth for RedCap for NTN.

However, even for NR, NCD-SSB has been introduced in Release 18, for providing several important benefits, e.g., simplified receiver design with power-saving and cost reduction, which may bring significant benefits to RedCap. By same reason, we shall talk applicability of NCD-SSB for RedCap for NTN into account.

Proposal 6: Support NCD-SSB for RedCap for NTN.

It appears that eDRX has been supported by IoT for NTN, even NR NTN doesn’t support it. RedCap for NTN may follow same rule for serving the purpose of power saving.

Proposal 7: Support eDRX for RedCap for NTN.

Regarding R17 relaxed measurement, the target scenario applying such relaxion is the stationary scenario, or the not-at-cell-edge scenario, low mobility or combined of them. However, it conflicts with the NTN scenario since of the large satellite cell footprint and the satellite movement. Despite of that, RedCap in NTN can apply the relaxation requirements for GEO.

Proposal 8: Not apply R17 relaxed measurement for RedCap for NTN. Besides, relaxation for GEO as done in NR NTN is applicable for RedCap in NTN.

Regarding FH in PRS measurement, because NTN only supports NW verified UE location, which allows for greater than 2km precision, the accuracy introduced by FH doesn’t bring observable effect.

Proposal 9: Not support FH in PRS measurement for RedCap for NTN.

The issue of the number of SMTC to be supported was recognized in the last meeting, which concerns the capability of RedCap UE.

In R15, dual-SMTC configurations are allowed where the offset and duration of the two SMTCs are the same and only the periodicity is different. Apparently, it’s not suitable in NTN scenario.

Later, in NTN Rel-17, to measure neighbour satellites and the serving satellites, a UE shall at the least be capable of 2 SMTCs that support different offsets while the same periodicity. It’s noted that in the case that the UE capable of 2 SMTCs doesn’t support parallelMeasurementWithoutRestriction, the UE only can measure one NGSO satellite while measuring the serving satellite. It could be regarded as the minimal requirement to an NTN UE to be able to perform inter-satellite mobility while keeping the measurement on the serving cell/satellite. Considering the complexity limitation of RedCap UE, we don’t pursue more SMTCs for RedCap UE. In a consistent manner, 1 MG may be adequate for RedCap for NTN.

Proposal 10: RedCap for NTN supports (at the least) 2 SMTC.

Proposal 11: RedCap for NTN supports (at the least) 1 MG.

For Issue 6-4-1: The specific impact of HD-FDD for (e)Redcap UE with FR1-NTN, the WF has an agreement [1]:

... Online agreement <Agreement> - RAN4 to consider the impact of HD-FDD for (e)Redcap UE with FR1-NTN. - The TN HD-FDD related requirements and applicable conditions defined for (e)RedCap UE can be used as a starting point, and RAN4 take into account NTN specific aspects if any. ...

In the WID [2] we have:

... - For HD-FDD RedCap UEs and eRedCap UEs, specify enhancements for mitigating issues caused by TA mismatch between actual TA used by the UE and assumed TA for the UE at the gNB [RAN1] - Enhancements are targeted for the following HD collisions cases (cases 3 and 4): - Semi-statically configured DL reception collides with semi-statically configured UL transmission - Dynamically scheduled DL reception collides with dynamic scheduled UL transmission - Note: enhancements for other HD collision cases are not targeted. Enhancements for HD collision cases 3 and 4 that may improve other HD collision cases are not precluded. - Note: potential work in RAN2, if any, starts after RAN1 has decided on the solution. ...

There is ongoing work in RAN1 regarding the mechanism to handle HD-FDD (e)RedCap collision cases in NTN for collision cases 3 & 4. RAN1 does not yet have a solution ready. However, we prefer to restate our RAN1 view here, for RAN4 reference [3].

In RAN1# 118, the following agreement was reached related to collision case 3:

Agreement The collision case 3 (Semi-statically configured DL reception collides with semi-statically configured UL transmission) can’t be assumed as error case. When the collision happens at UE side for collision case 3, one of the following options on priority rules is supported. - Option A: the priority of keeping one direction and overriding another direction is based on different use cases (e.g. the collision happening in different channel types or different scheduling cases) without network signalling - It is not precluded that some or all use cases are left to UE implementation regarding whether DL reception or UL transmission is dropped - Note: it is not precluded that the same direction is prioritized for all different use cases - Option B: network indicates the priority configuration to the UE - It is not precluded that some use cases are left to UE implementation regarding whether DL reception or UL transmission is dropped - It is not precluded to define default rules, if necessary

In RAN1, we make the following observations regarding case 3:

In our understanding, the intention behind defining priorities for collision case 3 is eliminating the current uncertainty of those cases being treated as error cases. Thus, adding a fixed rule in the specification seems to be a suitable approach (just as it has been done for most of the existing collision cases).

For the approach of leaving up to the UE implementation to decide the priority: In our view, this approach does not eliminate the uncertainty, since it is as per the UE’s discretion what is to be prioritized which may vary from time-to-time. This this approach is not preferred.

For the approach of introducing signalling for the network to decide the priority: The information to be potentially prioritized could take place during both initial access and connected mode. Thus, using signalling to define priority rules for collision case 3 likely will end-up impacting “common control information” for initial access and “UE specific control information” for when the UE has established RRC connection with the network. Thus, this approach is not preferred.

And in RAN1, we propose Option A. Which does not require that network signalling is supported.

Furthermore regarding case 4 we observe that the first step for collision case 4 is reaching an agreement similar to the one reached for collision case 3.

The first step for collision case 4 is reaching an agreement similar to one reached for collision case 3 in RAN1# 118.

# Conclusion

Proposal 1: Support SDT for RedCap for NTN. TA validation based on the RSRP change criterion for RedCap in TN shall be removed and the requirements may refer to the SDT requirements for IoT for NTN.

Proposal 2: Not support MDT for RedCap for NTN.

Proposal 3: Support conditional handover for RedCap for NTN, at least conditional handover for NTN in Rel-17.

Proposal 4: Support NTN specific time/location assisted cell reselection for RedCap for NTN.

Proposal 5: Deprioritize RACH-less handover for RedCap for NTN.

Proposal 6: Support NCD-SSB for RedCap for NTN.

Proposal 7: Support eDRX for RedCap for NTN.

Proposal 8: Not apply R17 relaxed measurement for RedCap for NTN. Besides, relaxation for GEO as done in NR NTN is applicable for RedCap in NTN.

Proposal 9: Not support FH in PRS measurement for RedCap for NTN.

Proposal 10: RedCap for NTN supports (at the least) 2 SMTC.

Proposal 11: RedCap for NTN supports (at the least) 1 MG.

- In our understanding, the intention behind defining priorities for collision case 3 is eliminating the current uncertainty of those cases being treated as error cases. Thus, adding a fixed rule in the specification seems to be a suitable approach (just as it has been done for most of the existing collision cases).

For the approach of leaving up to the UE implementation to decide the priority: In our view, this approach does not eliminate the uncertainty, since it is as per the UE’s discretion what is to be prioritized which may vary from time-to-time. This this approach is not preferred.

For the approach of introducing signalling for the network to decide the priority: The information to be potentially prioritized could take place during both initial access and connected mode. Thus, using signalling to define priority rules for collision case 3 likely will end-up impacting “common control information” for initial access and “UE specific control information” for when the UE has established RRC connection with the network. Thus, this approach is not preferred.

The first step for collision case 4 is reaching an agreement similar to one reached for collision case 3 in RAN1# 118.

# References

- R4-2414037, The WF on RRM requirements for Rel-19 NR NTN phase3, CATT

- RP-241667, “Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3”, 3GPP TSG RAN Meeting #104, Shanghai, China, June 17-20, 2024.

- R1-24XXXXX, On HD-FDD Redcap UEs for NTN, Ericsson