3GPP TSG-RAN WG4 Meeting #112bis	R4-2416077

Hefei, CN, October 14th-18th, 2024

Source: 	Ericsson

Title:  	Discussion on other RRM requirements for NTN for NR Phase 3

Agenda Item:	6.26.4.2

Document for:	Discussion

# Introduction

In the light of the WI NR Non-Terrestrial Networks (NTN) for NR Phase 3, the outcomes of the last meeting's debate, encompassing consensuses and unresolved issues, were documented in WF [1].

This contribution outlines our point of view on on the underlying impacts to RRM requirements mandated by the issues listed in the WF.

# Discussion

## Topic #1: Downlink coverage enhancements

#### Issue 2-1-1: RRM impact of SSB periodicity enhancement

Recall RAN1 and RAN2 agreements in last meetings as follows:

RAN1:

Excerpted from RAN1 #116 meeting: For system level study based on analytical evaluation: - N1 beam footprints are in state “off” - These beam footprints are not served by any signal (no satellite service in this area) - N2 beam footprints are in state “common messages only” - These beam footprints do not have any active user traffic, and are served the necessary information for cell discovery and initial access. - Optionally, companies may consider user arrival (e.g. RACH access) in this type of cell, and should describe how this is taken into account in the analytical evaluation - N3 beam footprints are in state “active traffic” - These beam footprints have X active (e.g. VoNR) users each. - These beam footprints are also served the necessary information for cell discovery and initial access

As part of the NTN DL coverage enhancements at both system level and link level, RAN1 to consider: - Extending the periodicity of the half frames with SS/PBCH blocks assumed by UE during initial access. - Default value[s] with extended periodicity assumed by NTN UE for initial access can be: - One [or more] values from the list {40ms, 80 ms, 160 ms, 320ms, 640ms} - Potential enhancements for transmitting the DL common channels using a wider beam footprint, while DL/UL dedicated channels (incl. PRACH) may be transmitted using a narrower beam footprint

RAN2:

Agreements: 1. From RAN2 point of view, if the SSB periodicity is no larger than 160ms, there is no RAN2 impact on SSB configuration (there might still be impacts on DTX aspects) 2. From RAN2 point of view, If the SSB periodicity is larger than 160ms, for example ssb-PeriodicityServingCell, measurement gap periodicity, SMTC configuration, ssb-Periodicity-r17 for NonCellDefiningSSB-r17 may need to be extended. And the field description of nAndPagingFrameOffset may need to be enhanced to consider the SSB periodicity higher than 160ms. 3. RAN2 can further consider SMTC impacts due to beam-hopping / larger SSB periodicity 4. If there is a need to bar pre-Rel19 NTN UEs from accessing a cell operating with DL coverage enhancement (e.g. because of extreme SSB periodicity) the existing NTN bar bit can be used. FFS about the behaviour for Rel-19 UEs not supporting DL coverage enhancement when the existing NTN bar bit is set.

Accordingly, RAN1 has agreed to study the SSB periodicity extension and further the SIB periodicity extension for initial access, which may be referred to N2 beam footprints.

From RRM perspective, the initial access feature isn’t in the scope. Though, the requirements concerning cell reselection and interruption time in paging reception are relevant to the SSB periodicity and the SIB periodicity associated with N2 beam footprints, which may be scaled up with respect to the extension of the SSB/SIB periodicity.

On other hand, some RRM requirements are relevant with the default SSB periodicity, which is referred to N3 beam footprints, as shown below.

- Interruption time for handover and conditional handover:

- Trs is the SMTC periodicity of the target NR SAN cell if the UE has been provided with an SMTC configuration for the target cell in the handover command, otherwise Trs is the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing. If the UE is not provided SMTC configuration or measurement object on this frequency, the requirement in this clause is applied with Trs=5ms assuming the SSB transmission periodicity is 5ms. There is no requirement if the SSB transmission periodicity is not 5ms. If the UE has been provided with higher layer in TS 38.331 [2] signaling of smtc2 prior to the handover command, Trs follows smtc1 or smtc2 according to the physical cell ID of the target cell.

- RRC Connection Release with Redirection:

- Trs is the SMTC periodicity of the target NR cell if the UE has been provided with an SMTC configuration for the target cell in the redirection command, otherwise Trs is the SMTC periodicity configured in the measObjectNR having the same SSB frequency and subcarrier spacing configured for the RRC connection release with redirection. If the UE is not provided with SMTC configuration or measurement object for the frequency which is also configured for the RRC connection release with redirection then: - the requirement in this clause is applied with Trs = 20 ms if the SSB transmission periodicity is not larger than 20 ms; otherwise, - there is no requirement if the SSB transmission periodicity is larger than 20ms.

A valid question arises: shall we revise or maintain the present definitions of Trs for handover, conditional handover and RRC Connection Release with Redirection?

Proposal 1: RAN4 to study whether to revise or maintain the present definitions of Trs defined for handover and conditional handover.

Proposal 2: RAN4 to study whether to revise or maintain the present definitions of Trs defined for RRC Connection Release with Redirection.

In the same manner, the requirements: transmit timing requirement, timing advance adjustment accuracy also may be influenced by the extension of the SSB periodicity. As the content quoted in the below, Te is applicable provided that at least one SSB is available at the UE during the last 160 ms.

When the UL SCS is 120 kHz or smaller, the UE shall meet the Te requirement for an initial transmission provided that at least one SSB is available at the UE during the last 160 ms. When the UL SCS is 480 kHz the UE shall meet the Te requirement for an initial transmission provided that at least one SSB is available in the last 80 ms. When the UL SCS is 960 kHz the UE shall meet the Te requirement for an initial transmission provided that at least one SSB is available in the last 40 ms.

Proposal 3: RAN4 to study whether to revisit or maintain the present 160ms defined for Te requirement if SSB periodicity is extended beyond 160ms.

It appears that, due to the extension of the SSB periodicity and the underlying extension of SMTC and/or DRX, the measurement time delay is scaled up, which may subsequently degrade the measurement accuracy for the requirements including cell reselection, L1/L3 measurements, especially for the earth-moving cell case.

Proposal 4: RAN4 to study if RRM requirements, e.g., measurement accuracy, is impacted by the scaled measurement time delay due to the extension of SSB/SMTC/DRX periodicity. The measurement herein may comprise cell reselection, signaling, L1/L3 measurements.

The definition of the N1/N2/N3 beam footprint, as a study reference, isn’t concrete until now.

Observed by a UE, the periodicity of N2 beam footprints (common control channels or broadcasting, including SSB and SIB) may be same as or different from N3 beam footprints periodicity (UE specific traffic) which are demonstrated in Figure 1. In one solution, N2 beam footprints receive a wide beam with a short periodicity, while N3 beam footprints receive a narrow beam with a long periodicity.

Figure 1 Beam footprints

It appears that such variants of beam footprints, if they are agreed, may influent the definition of SSB periodicity and consequently may influent the measurement configuration.

Observation 1: Beam footprint definition may impact measurement configuration.

## Topic #5: Support of regenerative payload

In RAN2#127 the following LS was forwarded to RAN4 and RAN1 [2]:

… # 1. Overall Description: In RAN2#127 meeting, the setting of common TA and Kmac for regenerative payload with full gNB on board was discussed, and a question related to the value setting was raised. Related to this, RAN2 kindly asks RAN1 and RAN4: Question: Whether in a regenerative payload scenario it would be a problem to stick to 0 as the minimum possible value for TA-Common or whether we should e.g. introduce negative values for ta-Common. Additionally, RAN2 understands in any case legacy UEs would have to rely on existing signaling and then for legacy UEs, minimum value of Common TA is equal to 0. # 2. Actions: # To RAN1 and RAN4: # RAN2 kindly requests RAN1 and RAN4 to provide feedback on above question. …

In this contribution we discuss the question and make proposals.

### Background

In TS 38.211 the NTN UL TA mechanism is stated in section 4.2:

Uplink frame number  for transmission from the UE shall start TTA=NTA+NTA,offset+NTA,adjcommon+NTA,adjUETc before the start of the corresponding downlink frame at the UE where

- NTA and NTA,offset are given by clause 4.2 of [5, TS 38.213], except for msgA transmission on PUSCH where NTA=0 shall be used;

-	NTA,adjcommon given by clause 4.2 of [5, TS 38.213] is derived from the higher-layer parameters ta-Common, ta-CommonDrift, and ta-CommonDriftVariant if configured, otherwise NTA,adjcommon=0;

-	NTA,adjUE given by clause 4.2 of [5, TS 38.213] is computed by the UE based on UE position and serving-satellite-ephemeris-related higher-layers parameters if configured, otherwise NTA,adjUE=0.

Figure 4.3.1-1: Uplink-downlink timing relation.

In TS 38.213 section 4.2	Transmission timing adjustments, we have:

Using higher-layer ephemeris parameters for a serving satellite, if provided, a UE pre-compensates the two-way transmission delay on the service link based on NTA,adjUE that the UE determines using the serving satellite position and its own position. To pre-compensate the two-way transmission delay between the uplink time synchronization reference point and the serving satellite, the UE determines NTA,adjcommon [4, TS 38.211] based on one-way propagation delay Delaycommont that the UE determines as:

Delaycommont= TACommon2+ TACommonDrift2×t-tepoch+TACommonDriftVariant2×t-tepoch2

where TACommon, TACommonDrift, and TACommonDriftVariant are respectively provided by ta-Common, ta-CommonDrift, and ta-CommonDriftVariant and tepoch is provided by epochTime which is the epoch time of ta-Common, ta-CommonDrift, and ta-CommonDriftVariant [12, TS 38.331]. Delaycommon(t) provides a distance at time t between the serving satellite and the uplink time synchronization reference point divided by the speed of light. The uplink time synchronization reference point is the point where DL and UL are frame aligned with an offset given by NTA,offset.

Furthermore, in TS 38.331 we have, for the three components TACommon, TACommonDrift and TACommonDriftVariant

In Rel-19 non-transparent architecture then TACommonDrift=0 and TACommonDriftVariant=0 and we get:
Delaycommont= TACommon2

Since NTA,adjcommon=2∙Delaycommont we get NTA,adjcommon= TACommon.  In release 17 and 18 we have TACommon>0 and NTA,adjcommon>0.

### Analysis

#### 0 as the minimum possible value for TA-Common

Regarding the question from RAN2:

Whether in a regenerative payload scenario it would be a problem to stick to 0 as the minimum possible value for TA-Common or whether we should e.g. introduce negative values for ta-Common.

No information is included in the LS on what potential problems that could be associated with sticking to 0 as the minimum value. Based on company input to RAN2 [2][3], it is our understanding that the main concern is that the UE may overestimate the TA pre-compensation of Msg1 (e.g., due to a GNSS position error) and the preamble may therefore arrive too early at the gNB. The initial transmit timing error for NR NTN is given in clause 7.1c.2 of TS 38.133 [4] and can be up to Te_NTN = 29*Ts (depending on SCS). The scenario is illustrated in Figure 1 (note that the figure is not to scale – the maximum timing error is actually less than 0.1% of the slot length for SCS=15 kHz).

Figure 1: Preamble reception with overestimated TA pre-compensation.

Two aspects of sticking to 0 as the minimum possible value for ta-Common are discussed below:

- If TA pre-compensation is overestimated, the received preamble may be partially outside the gNB receive window.

- If TA pre-compensation is overestimated, the received preamble may collide with a received signal (e.g. from another UE) in the preceding slot.

Potential problem 1: The received preamble is outside the gNB receive window

The placement of the receive window is up to gNB implementation. By advancing the receive window by e.g. Te_NTN, problem 1 can be avoided. This is illustrated in Figure 2. In principle, this has the same effect as delaying the transmitted signal by using a negative ta-Common.

Figure 2: Advanced RX window

Observation 2: A preamble with overestimated TA pre-compensation that falls outside the receive window can be avoided by advancing the gNB receive window by e.g. Te_NTN. The placement of the receive window is up to gNB implementation. Potential problem 2: The received preamble collides with a received signal in the preceding slot

Since ta-Common is applied equally by all UEs, including the one transmitting in the preceding slot, it has no impact on the collision probability. The limitation to 0 as the minimum value for ta-Common is not a problem in this regard, nor would delaying the transmitted signals by a negative ta-Common avoid collisions. To avoid collisions, the transmitted signal in the preceding slot (e.g. a PUSCH) may be advanced by e.g. Te_NTN using NTA, e.g., through a positive TAC in Msg2. This is up to gNB implementation.

Figure 3: Advanced signal in the preceding slot though closed-loop TAC.

Observation 3: A preamble with overestimated TA pre-compensation may collide with a received signal in the preceding slot. That is not impacted by ta-Common, since ta-Common is applied equally by all UEs, including the one transmitting in the preceding slot. Collisions can be avoided by gNB implementation.

#### Negative values for ta-Common

In TS 38.213 section 4.2, the definition of the uplink time synchronization reference point is given:

…the uplink time synchronization reference point is the point where DL and UL are frame aligned with an offset given by NTA,offset.

Observation 4: the uplink time synchronization reference point is the point where DL and UL are frame aligned with an offset given by NTA,offset.

The purpose of NTA,adjcommon in TTA=NTA+NTA,offset+NTA,adjcommon+NTA,adjUETc is to pick the reference point to be somewhere along the feeder link, between the GW and the satellite, if NTA,adjcommon > 0 or along the service link if NTA,adjcommon<0.

Observation 5: The reference point will be somewhere along the feeder link, between the GW and the satellite, if NTA,adjcommon > 0 or along the service link if NTA,adjcommon<0.

The TA mechanism will align all UL connections at the uplink time synchronization reference point. However, the BS RX have to consider all individual NTN-UE connections and decide how to demodulate the uplink. The UL timing has uncertainties like:

- TA-step quantization errors

- TA-adjustment error allowance

- Radio channel dispersion

- NTN GNSS positioning uncertainty

The latter term is specific for NTN. This means that the UL symbols will arrive with some arrival time spread around the uplink time synchronization reference point (RP17,18). This is shown in Figure 1.

Figure 1:BS RX reception of symbols and CP

It is important to keep the variation of UL timing to a minimum. That is the point of the whole TA mechanism. However, BS RX demodulation will be fine as long as we can handle a symbol separation within a CP.

Observation 6: It is important to keep the variation of UL timing to a minimum. That is the point of the whole TA mechanism.

The LS [2] states an important precondition:

Additionally,  RAN2 understands in any case legacy UEs would have to rely on existing signaling and then for legacy UEs, minimum value of Common TA is equal to 0.

This means that if we allow negative values for TACommon then we would have two groups of UE. One legacy group consisting of all release-17 and release-18 UE with TACommon=0 (RP17,18) and another group of release-19 UE (RP19) with TACommon< 0

Figure 2:BS RX reception of symbols and CP, two groups

A rel-17 and rel-18 group of UE with TACommon=0 and another group of Rel-19 with TACommon<0 would increase the spread UL timing at BS RX. This could affect demodulation performance if the separation is larger than a CP.

Observation 7: A rel-17 and rel-18 group of UE with TACommon=0 and another group of Rel-19 with TACommon<0 would mean that we have two different reference points, where DL and UL are frame aligned. This could affect demodulation performance if the separation is larger than a CP.

Proposal 5: Send LS to RAN2 that it is preferred to stick to 0 as the minimum possible value for TA_Common. An LS draft can be found in annex to this contribution.

# Conclusion

Proposal 1: RAN4 to study whether to revise or maintain the present definitions of Trs defined for handover and conditional handover.

Proposal 2: RAN4 to study whether to revise or maintain the present definitions of Trs defined for RRC Connection Release with Redirection.

Proposal 3: RAN4 to study whether to revisit or maintain the present 160ms defined for Te requirement if SSB periodicity is extended beyond 160ms.

Proposal 4: RAN4 to study if RRM requirements, e.g., measurement accuracy, is impacted by the scaled measurement time delay due to the extension of SSB/SMTC/DRX periodicity. The measurement herein may comprise cell reselection, signaling, L1/L3 measurements.

Proposal 5: Send LS to RAN2 that it is preferred to stick to 0 as the minimum possible value for TA_Common. An LS draft can be found in annex to this contribution.

Observation 1: Beam footprint definition may impact measurement configuration.

Observation 2: A preamble with overestimated TA pre-compensation that falls outside the receive window can be avoided by advancing the gNB receive window by e.g. Te_NTN. The placement of the receive window is up to gNB implementation. Potential problem 2: The received preamble collides with a received signal in the preceding slot

Observation 3: A preamble with overestimated TA pre-compensation may collide with a received signal in the preceding slot. That is not impacted by ta-Common, since ta-Common is applied equally by all UEs, including the one transmitting in the preceding slot. Collisions can be avoided by gNB implementation.

Observation 4: the uplink time synchronization reference point is the point where DL and UL are frame aligned with an offset given by NTA,offset.

Observation 5: The reference point will be somewhere along the feeder link, between the GW and the satellite, if NTA,adjcommon > 0 or along the service link if NTA,adjcommon<0.

Observation 6: It is important to keep the variation of UL timing to a minimum. That is the point of the whole TA mechanism.

Observation 7: A rel-17 and rel-18 group of UE with TACommon=0 and another group of Rel-19 with TACommon<0 would mean that we have two different reference points, where DL and UL are frame aligned. This could affect demodulation performance if the separation is larger than a CP.

# References

- R4-2414037, The WF on RRM requirements for Rel-19 NR NTN phase3, CATT

- R2-2407623, LS on common TA in a regenerative payload scenario, CMCC

====== Annex: Draft LS =======

Title:	Reply LS on common TA in a regenerative payload scenario

Response to:	-

Release:	Release 19

Work Item:	NR_NTN_Ph3-Core

Source:	RAN4

To:	RAN2

Cc:	RAN1

Contact person:	Magnus Larsson

Magnus dot k dot Larsson at Ericsson dot com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

# 1. Background:

In RAN2#127 an LS was forwarded to RAN 4 and RAN2 [1]. In the LS RAN2 asks:

Question:  Whether in a regenerative payload scenario it would be a problem to stick to 0 as the minimum possible value for TA-Common or whether we should e.g. introduce negative values for ta-Common.

Additionally,  RAN2 understands in any case legacy UEs would have to rely on existing signaling and then for legacy UEs, minimum value of Common TA is equal to 0.

# 2. Analysis and reply

<Editor’s note: Add Stefan Ericsson (RAN) analysis here. Repeat the RAN1 message why to 0 as the minimum possible value for TA-Common is ok>

The uplink time synchronization reference point is the point where DL and UL are frame aligned with an offset given by NTA,offset. The term TA-Common decides where the reference point is. 
   To introduce negative values for ta-Common, for release-19 UE, while release 17 and 18 legacy UE would have ta-Common = 0, would create two separate uplink time synchronization reference points. This would defeat the purpose of UL TA mechanism, which is to keep all UE UL connections aligned at a single reference point, with a spread that is less than CP. 
   It is therefore preferred to stick to 0 as the minimum possible value for TA-Common.

# 3. Actions:

# To RAN2

RAN4 kindly requests RAN2 to consider the feedback in subsequent analysis and work.

[1] 		R2-2407623, LS on common TA in a regenerative payload scenario, CMCC