3GPP TSG-RAN WG4 Meeting #112bis	R4-2416078

Hefei, CN, October 14th-18th, 2024

Source: 	Ericsson

Title:  	Discussion on FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor

Agenda Item:	6.15.2.1

Document for:	Discussion

# Introduction

In the light of the WI of NR Radio Resource Management (RRM) enhancements (WID in [1]) for Rel-19, the outcomes of the last meeting's debate, encompassing consensuses and unresolved issues, were documented in WF [2].

This contribution outlines our point of view on FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor as follows.

# Discussion

## Topic #2-1: FR2-1 SSB based L3 measurement delay reduction for connected mode

Applicability requirement:

Updated Agreement in online session: Baseline: L3 delay enhancements in Rel-19 by optimizing Rx BSF for UE supporting multi-rx simultaneous reception are applicable provided that: - the target carrier(s) to be measured: only one carrier in the single FR2-1 band is configured for L3 SSB measurement and - UE serving carrier(s): UE is configured with single carrier on FR2-1 band, i.e. FR2-1 PCell without CA/DC. Note: Target and serving carrier frequency can be the same or different. Note: The ‘other UE CA/DC modes (e.g., 1 or 2 FR2-1 bands CA, or FR1+FR2 CA/DC, or EN-DC)’ and/or the ‘other number of target to-be-measured carrier(s) on FR2-1 band’ can be FFS after concluding the baseline above. These extra FFS parts will NOT delay the WI completion.

Since the agreements do not hinder the exploration or examination of CA/DC mode, to ensure consistency of applying FBS between the case of single carrier and the case of multiple CCs, the clarification concerning the below questions is necessary.

- The FBS for different CCs

Considering a use case comprising 2 carriers: PCC with FBS, i.e., scaling factor = N1 and SCC with the legacy beam sweeping, i.e., scaling factor = N2, a question arises: shall N1 equals to N2 mandatorily or N1 may be different from N2?  Regarding this question, one straightforward approach is assuming N1=N2 mandatorily, another approach is the beam sweeping (FBS/legacy beam sweeping) is defined per carrier.

- SMTC between CCs

In legacy, limited by Rx beam sweeping scheme, a UE is assumed to use the same Rx beam when multiple CCs are measured at the same time. Meanwhile, it’s noted that measurements on multiple CCs are limited to having the same SMTC offset, as indicated below. However, we still can analyze the case having SMTC offset.

The requirements in this clause apply provided that - The SMTC on all CCs and inter-frequency layers without measurement gap in FR2 have the same offset, and one of following conditions is met - If smtc2 is configured on any FR2 CC, - All CCs have the same configuration for smtc1, and - All CCs configured with smtc2 have the same configuration for smtc2 - If smtc2 is not configured on any FR2 CC, - The total number of different SMTC periodicities on all serving CCs and inter-frequency layers without measurement gap does not exceed 4

The following figures, utilizing the configuration of two CCs (PCC and SCC) as a point of reference, illustrate the above questions.

Figure 1 Same beam sweeping with same SMTC offset between CCs

Figure 2 Different beam sweeping with same SMTC offset between CCs

According to Figure 1 and Figure 2, in the case of same SMTC offset between CCs, differentiating beam sweeping, e.g., applying FBS on the PCC while applying normal beam sweeping on the SCC, doesn’t make sense provided the UE receives the PCC and the SCC with the identical RX beams.

Figure 3 Same beam sweeping with different SMTC offset between CCs

Figure 4 Different beam sweeping with different SMTC offset between CCs

According to Figure 3 and Figure 4, in the case of different SMTC offset between CCs, distinguishing beam sweeping, e.g., applying FBS on the PCC and the normal beam sweeping on the SCC may gain power saving when the SCC is measured while no PCC is measured.

Figure 5 Same beam sweeping with same SMTC offset between CCs

Figure 6 Different beam sweeping with same SMTC offset between CCs

According to Figure 5 and Figure 6, in the case of the same SMTC offset between CCs, distinguishing beam sweeping, e.g., applying FBS on the PCC and the normal beam sweeping on the SCC may achieve power saving when the SCC is measured while PCC isn’t measured. Considering further expansion of SCCs other than only one SCC, differentiating Rx beam sweeping schemes among carriers to enable power savings is becoming more apparent.

Another reason for distinguishing Rx beam sweeping scheme among carriers is, since one of the most important target scenarios of FBS is mobility, for the SCC(s) without neighbor cell measurement, FBS isn’t a critical feature and could be configured with normal beam sweeping.

Proposal 1: For CA/DC configuration, Rx beam sweeping scheme on multiple CCs shall be studied.

Proposal 2: For CA/DC configuration, Rx beam sweeping scheme (i.e., FBS or the normal beam sweeping) is configurable per carrier or carrier group, at least for the configuration that PCC and at least one SCC are not measured simultaneously.

Issue 1-2-3: FFS: RRM measurement with two panels activated, two searchers are occupied by this single carrier:

[Way forward]: FFS: - Option A/1a: Two searchers are occupied by this single carrier to the requirements of enhanced BSF for single carrier. Discuss CA case later. (HW, CATT, Apple, OPPO, LGE, Xiaomi) - Option B: Consider one searcher for single carrier. (MTK, ZTE, OPPO, QC, Nokia, Intel, vivo, Ericsson, Apple, Xiaomi, CATT)

In the case of single carrier configuration, apparently, one searcher supporting multi-Rx reception is achievable, which is the minimal request to the number of searchers, from implementation standpoint. If we suppose that multi-Rx reception always uses a single searcher, the following observations could be obtained.

- Pro: It follows the legacy searcher assumption for CA/DC configuration, then the CSSF rule remains unaffected, regardless of any future support for CA/DC by FBS.

- Con: As some companies proposed, it results in extra processing time.

As a result, we recommend not limiting the number of working searchers for a single carrier configuration.

On the other hand, the searcher numbers for single carrier and for CA/DC need not be identical anyhow, this aspect pertains to UE implementation. For example, when only single carrier is configured with multi-Rx reception, two searchers are occupied by the single carrier in multi-Rx reception; while when more than one carrier is configured, one searcher for PCC and one searcher for all SCCs in multi-Rx reception. We do assume that the UE is able to utilize all available searchers for measurement in multi-Rx reception without breaking the legacy CSSF rule.

Given above, we intend to define two separate sets of processing time requirements with respect to single carrier and CA/DC configuration respectively. Since RRM requirements don’t explicitly list or limit any UE implementation, e.g., searcher, as follows, only processing time is specified differently for such two configurations.

- For single carrier configuration in multi-Rx reception, extra processing time=0.

- For CA/DC configuration in multi-Rx reception, extra processing time is FFS.

Through this approach, we can concentrate on the implementation of single carrier setup and examine the associated solution at present. If time is left for the WI permits, we could recap CA/DC configuration later. The FFS in CA/DC configuration does not obstruct the consensus of the FBS solution for the WI.

Proposal 3: Reflecting different assumptions of searcher number, processing time is specified differently for such two configurations.

- For single carrier configuration in multi-Rx reception, extra processing time=0.

- For CA/DC configuration in multi-Rx reception, if applicable, extra processing time could be reviewed after completing single carrier requirements.

Issue 1-2-4: FFS: SSB processing delay/time for processing multiple beams received in a SMTC

[Way forward]: FFS: - Option 1 (CATT, vivo): No need to add additional processing time due to multiple SSBs within one SMTC. - Option 2 (Apple): allow additional processing time for UE supporting multiple-Rx simultaneous reception for L3 delay enhancement if there is only one searcher available for the processing - Option 3 (Ericsson): Extra SSB post-processing time in several ms, as side effect of BSF enhancement, may be acceptable, but it depends on how much we can gain with BSF enhancement - Option 4 (Intel): - Consider UE baseband processing capabilities when specifying the L3 delay reduction for simultaneous receptions on multiple FR2 SSB-s. - Different (or whether or not) delay reduction applies when the ratio of number of SSB within a burst and time duration of the measurement periodicity varies. - Option 5 (Nokia): - If additional time for SSB processing is needed when UE is measuring multiple beams in one SMTC, RAN4 to consider measurement delay with SSB processing as - a. Tidentify_intra_without_index = (TPSS/SSS_sync_intra + T SSB_measurement_period_intra+TSSB_processing) ms - b. Tidentify_inter_without_index = (TPSS/SSS_sync_inter + T SSB_measurement_period_inter+TSSB_processing) ms - c. where TSSB_processing = 2 ms

A UE may store the SSB samples from the SMTC window and post-process them before the next SMTC window. Depending on UE capability, a UE may or may not need an extra processing time for processing SSB samples under the multi-Rx reception configuration.

In legacy, a processing time serving its function called as ﻿Tmargin  is allocated in the equation defining handover interruption time, which is instantly after the time tracking time. Tmargin can reach a maximum of 2 milliseconds.  To the contrary, the definitive duration for the processing time is not allocated in the time delay of cell identification.

6.1.1.4 NR FR2- NR FR2 Handover The requirements in this clause are applicable to both intra-frequency and inter-frequency handovers from NR FR2 cell to NR FR2 cell. 6.1.1.4.1 Handover delay When the UE receives a RRC message implying handover the UE shall be ready to start the transmission of the new uplink PRACH channel within Dhandover ms from the end of the last TTI containing the RRC command. Where: Dhandover equals the applicable RRC procedure delay defined in clause 12 in TS 38.331 [2] plus the interruption time stated in clause 6.1.1.4.2. 6.1.1.4.2 Interruption time The interruption time is the time between end of the last TTI containing the RRC command on the old PDSCH and the time the UE starts transmission of the new PRACH, excluding the RRC procedure delay. When intra-frequency or inter-frequency handover is commanded, the interruption time shall be less than Tinterrupt Tinterrupt = Tsearch + TIU + Tprocessing + T∆ + Tmargin ms Where: Tsearch is the time required to search the target cell when the handover command is received by the UE. Regardless of whether DRX is in use by the UE, Tsearch shall still be based on non-DRX target cell search times. - If the target cell is a known intra-frequency cell, then Tsearch = 0ms. - If the target cell is an unknown intra-frequency cell and the target cell Es/Iot≥-2 dB, then Tsearch = N* Trs ms. - If the target cell is a known inter-frequency cell, then - if the measured SSB is the target SSB for handover of the target cell, Tsearch = 0ms; - if the measured SSB and the target SSB for handover belong to the same NR target cell and if the UE supports ncd-SSB-BWP-Wor-r18, Tsearch = Trs ms provided any one of the following conditions is satisfied: - CD-SSB in initial DL BWP is the measured SSB and NCD-SSB in first active DL BWP is the target SSB for handover - NCD-SSB in a DL BWP is the measured SSB and CD-SSB in initial DL BWP is the target SSB for handover - Both measured SSB and the target SSB for handover are NCD-SSB within different DL BWPs - Otherwise, the target cell is an unknown inter-frequency cell and Tsearch = N*3* Trs ms if the target cell Es/Iot≥-2 dB. Where N = 8 when the target cell is in FR2-1, and N = 12 when the target cell is in FR2-2. Tprocessing is time for UE processing. Tprocessing can be up to 20ms. Tmargin is time for SSB post-processing. Tmargin can be up to 2ms. T∆ is time for fine time tracking and acquiring full timing information of the target cell. T∆ = Trs for both known and unknown target cell.

9.2.5 Intrafrequency measurements without measurement gaps 9.2.5.1 Intrafrequency cell identification The UE shall be able to identify a new detectable intra-frequency cell within Tidentify_intra_without_index if the UE is not indicated to report SSB based RRM measurement result with the associated SSB index(reportQuantityRsIndexes or maxNrofRSIndexesToReport is not configured), or the UE is indicated that the neighbour cell is synchronous with the serving cell (deriveSSB-IndexFromCell is enabled). Otherwise UE shall be able to identify a new detectable intra frequency cell within Tidentify_intra_with_index. The UE shall be able to identify a new detectable intra frequency SS block of an already detected cell within Tidentify_intra_without_index. It is assumed that deriveSSB-IndexFromCell is always enabled for FR1 TDD and FR2 with SCS smaller or equal to 480 kHz. Tidentify_intra_without_index = (TPSS/SSS_sync_intra + T SSB_measurement_period_intra) ms Tidentify_intra_with_index = (TPSS/SSS_sync_intra + T SSB_measurement_period_intra + TSSB_time_index_intra) ms Where: TPSS/SSS_sync_intra: it is the time period used in PSS/SSS detection - For UE supporting power class 6 with highSpeedMeasFlagFR2-r17 configured, if SMTC <= 40ms, TPSS/SSS_sync_intra is given in Table 9.2.5.1-11; otherwise, TPSS/SSS_sync_intra is given in Table 9.2.5.1-2. - For UE indicating no-gap-no-interruption, TPSS/SSS_sync_intra is given in Table 9.2.5.1-1 for FR1 and Table 9.2.5.1-2 for FR2. For UE indicating no-gap-with-interruption, TPSS/SSS_sync_intra is given in Table 9.2.5.1-17 for FR1 and Table 9.2.5.1-18 for FR2. - Otherwise, TPSS/SSS_sync_intra is given in table 9.2.5.1-1, 9.2.5.1-2, 9.2.5.1-4 (deactivated SCell) or 9.2.5.1-5 (deactivated SCell) or 9.2.5.1-9 (deactivated SCell) or 9.2.5.1-11 or 9.2.5.1-12 (deactivated PSCell) or 9.2.5.1-13 (deactivated PSCell). TSSB_time_index_intra: it is the time period used to acquire the index of the SSB being measured

In a consistent way, the significance of enlarging the value of Tmargin in handover interruption time shall be exploited in terms of the practical demand of the UE.  It is conceivable to disregard the processing time involved in cell identification delay, as cell identification consumes multiple SMTC/DRX occasions and intervals and may provide adequate opportunity for processing incoming SSBs.

Proposal 4: For single carrier configuration, no extra processing time. For CA/DC configuration, if applicable, RAN4 to check the necessity of extra processing time for handover interruption time.

Proposal 5: No extra processing time for cell identification delay.

Issue 1-2-5: FFS: Power consumption issue (including conditions to trigger UE using FBS for L3 measurement)

- Option 1 (Apple, LGE): For power saving purpose, there is a need to have a mechanism to activate/de-activate L3 fast beam sweeping. The R18 mechanism (i.e., multi-RX operation definition and UAI indication of preference) can be considered as a baseline, while other conditions are not precluded. - Option 2 (HW): - RAN4 shall firstly identify the promising scenario(s) for L3 measurement delay reduction enabled by multi-Rx with clear/significant benefits, which could help to converge the discussion. - One possible scenario to be considered is when there is strong demand of mobility performance (e.g. UE at cell edge or the link is about to break). - It shall not be assumed that UE supporting this feature shall activate multiple panels all the time for all L3 measurement. RAN4 shall discuss the entering and/or exiting conditions for enhanced L3 measurement enabled by multi-Rx taking the targeting scenario into account. - Option 3 (Ericsson/Nokia): NW indicates UE enabling/disabling FBS through L3 or lower layers signaling. - Option 4 (vivo): Power consumption issue is important and BSF reduction of L3 measurement will not trigger UE to activate multi-Rx - Option 5 (Intel): Power consumption is not an issue in the scope since the total power consumption for a handover stays roughly the same even delay is reduced. Whether UE has prior knowledge or cell centre/edge conditions do not affect reduction in BSF but they are addressed in legacy side conditions. - Option 6a (Nokia): Mobility Event triggering BSF reduction - Option 6b (Nokia): FBS is triggered by conditional Handover configuration - Option 6c (Nokia): BSF reduction is always enabled, but used for reduced measurement delay in cell edge and used for reduced scheduling restrictions in cell center (e.g. by extending T_SMTC). - Option 7 (QC): RAN4 to adopt the following framework for the fast UE Rx beam sweeping based L3 measurement and mobility requirements: - NW provides the following criteria for fast beam sweeping application, and the signaling details are FFS - Cell edge condition: Threshold value of absolute L3 SSB-RSRP of SpCell - High speed condition: Threshold value of L3 SSB-RSRP variation on SpCell over a time period T - When the condition of not cell-edge (and not high-speed, if configured) is met, the UE is allowed to fallback Rx beam sweeping factor to the existing N value - Note: the existing criteria defined for the relaxed idle/inactive mode measurement and/or RLM/BFD evaluation can be reused or served as a baseline - Report configuration for the status of fast beam sweeping factor application, and the signaling details are FFS - A TTT-like time window or N310-like timer, which starts running or counting upon the first satisfaction of the condition “not cell-edge (and not high-speed)” is observed by the UE, can be configured to avoid frequent status transitions and reports - Note: the existing report defined for RLM/BFD relaxation status can be reused or served as a baseline - Besides, other explicit signaling (e.g. FR2 CHO, FR HO, GBBR, etc.) may disallow the fallback of UE Rx beam sweeping factor to the existing N value until the signaled configuration is no longer in effect or the relevant task has been completed. - FFS on the application delay of UE Rx beam sweeping factor switch. - Option 8 (MTK): Activating multi-Rx for L3 measurements (intra/inter-frequency) may or may not be always necessary, depending on UE current conditions: - UE location (cell centre or cell edge) - UE mobility (stationary or moving) - Both above - Option 8a (MTK): - On UE mobility status, RAN4 should consider low/medium speed mobility of the UEs as one of the conditions to activate multi-Rx for L3 measurement delay reduction. - RAN4 to discuss UE indication capability to the NW whenever UE requires to deactivate multi-Rx for FR2-1 SSB based L3 measurement delay reduction (e.g., indication due to overheating resulting from activating multiple panels for long time). - Option 9 (ZTE): Due to L3 measurement is long-term operation, power consumption issue could be considered, which may lead to some interaction signalling. But which would not be the applicability condition of applying fast beam sweeping. [Way forward]: discussion can be mainly focus on 3 directions: (1)“additional triggering for this R19 L3 measurement with FBS” (use option 8 for discussion) or (2)“L3 measurement with FBS can be activated/deactivation following on R18 mechanism” (use option 1 for discussion) or (3)“up to network indication” (use option 3 for discussion). Discuss option 1/3/8 together, and then add details from other options if needed.

Apparently, the NW serving a UE is responsible for any operation/procedure conducted by the UE in RRC_CONNECTED state, by sending the dedicated signaling/command/configuration to the UE, based on the reports from the UE.

The main application scenario of FBS is facilitating the rapid assessment of mobility, particularly when a UE experiences a low level or poor signal from the serving cell, necessitating a transition to a different serving cell. Given that, the NW is already aware of the necessity for a handover based on the received measurement report, such that the NW is capable of determining the appropriate timing for enabling or disabling FBS accordingly.

At least one company expressed the concern that the NW may still instruct a UE to enable FBS even when the UE is overheating or running on low battery, such that they suggested the UE is able to report the NW of a request to deactivate FBS. Another concern is that a UE shall determine enabling/disabling FBS with respect to the UE location/mobility.

Regarding the former concern, it is understood that the NW may prioritize obtaining measurement results via FBS to meet the urgent handover demand, depending on the measurement result, e.g., the measurement result with FBS can’t fulfill handover criteria, then the NW may command the UE to disable FBS since as the frequent measurement result is no longer essential to the handover process. In such FBS enabled time period, since the time span only cover the handover time, which normally is less than several seconds, the impact to the UE’s power consumption isn’t significant.

Regarding the latter concern, defining the conditions with respect to UE location/mobility is complex and inherently requires designing the configuration of UE location/mobility parameters along with the associated criteria. Additionally, establishing such criteria may necessitate the introduction of extra RRC Information Elements (IEs) in the signalings provided by the NW. However, if the criteria are deemed to be based on the comparison between the RSRP measurement result and a configurable RSRP threshold, not only the UE, but the NW also can evaluate them based on the RSRP measurement report.

Given that, between the two options:

- Option 1: NW provides the criteria of UE location/mobility and UE determines FBS in terms of the criteria

- Option 2: NW determines triggering UE of FBS based on RSRP report

We prefer Option 2 which doesn’t introduce any new parameters and meet the current rule managing UE in RRC_CONNECTED state.

Observation 1: NW has the knowledge based on measurement result for indicating UE of enabling or disabling FBS.

As a compromise to address such concern of power consumption of FSB in a long time period, the UE may determine to enable FBS in a predefined time period or for a predefined measurement samples/occasions upon receiving FBS command by the NW. After that, the UE is allowed to disable FBS and back to normal beam sweeping. The simplified timeline is depicted in Figure 7. From the application perspective, in the FBS period, the NW is capable of discerning which procedure the UE should carry out, e.g., handover, based on the measurement results.

Figure 7 FBS timeline

One merit of the solution is that there’s no need to define the criteria of UE location/mobility, another merit is that non-continued FBS doesn’t drain tremendous UE battery.

Since handover is the first priority scenario being served by the WI, we must consider how the NW indication is applied to handover. In interruption time of handover, Tsearch is the time required to search the target cell when the handover command is received by the UE. To speed up Tsearch with FBS, the handover command or CHO configuration may also be regarded as a indication indicating the UE of enabling FBS. Once handover is completed or handover is failed, the UE may fall back to the normal beam sweeping.

Proposal 6: NW indicates UE enabling/disabling FBS through L3 or lower layers signaling.

Proposal 7: NW indicates UE enabling FBS through L3 or lower layers signaling. To save power consumption, after receiving the indication, UE may enable FBS in a predefined time period or for predefined measurement samples. After that, the UE is allowed to disable FBS or fall back to normal beam sweeping.

Proposal 8: As a variant of NW indication, handover command or CHO configuration message indicates UE enabling FBS as well.

We also observe the indication may be indicated in an implicit way. For example, relaxed Radio Link Monitoring (RLM) is designed for a UE to be able to operate these relaxation features when it is under good radio conditions and/or at a low mobility state. In other words, it means the radio conditions are stable and no need to demand the UE to perform fast beam sweeping or to shorten the measurement delay time on the neighbor cells. In this sense, there could be some relationships between BFD and/or RLM and FBS.

It appears that if the UE meets the criteria of relaxed BFD and/or RLM, it indeed implicitly indicates the UE hasn’t the demand to conduct FBS. On the contrary, if the UE doesn’t meet the criteria of relaxed BFD and/or RLM, it indeed implicitly indicates the UE may have the demand to conduct FBS.

Proposal 9: BFD and/or RLM status may indicate UE of enabling FBS. If the UE meets the criteria to operate relaxed BFD and/or RLM, it indeed implicitly indicates the UE hasn’t the demand to conduct FBS. On the contrary, if the UE doesn’t meet the criteria to operate relaxed BFD and/or RLM, it indeed implicitly indicates the UE may have the demand to conduct FBS.

In the last meeting, at least one company brought a use case that FBS shall be naturally enabled without any loss if GBBR is enabled.  In this way, the use case leads to the following signaling schemes.

- GBBR is configured -> L1 measurement enhancement in Rel-18 +FBS

- New indication -> FBS

- GBBR is configured + New indication -> L1 measurement enhancement in Rel-18 +FBS

However, if GBBR refrains from FBS feature, which is our preference, then it leads to the following signaling mechanisms:

- GBBR is configured -> L1 measurement enhancement in Rel-18

- New indication -> FBS

- GBBR is configured + New indication -> L1 measurement enhancement in Rel-18 +FBS

From the comparison of the two mechanisms, GBBR triggering FBS results in an issue that a UE is unable to operate solely with the L1 measurement enhancement introduced in Rel-18 without the use of FBS. We don’t expect the two features always being intertwined.

Observation 2: GBBR triggering FBS results in an issue for the UE being unable to operate solely with the L1 measurement enhancement introduced in Rel-18 without the use of FBS.

We believe that L1 measurement enhancement in Rel-18 and FBS (L3 measurement enhancement in Rel-19) shall be two irrelative features. Thus, we don’t agree that FBS shall be always enabled if enabling L1 measurement enhancement in Rel-18. As we mentioned several times, there is no direct relationship between L1 measurement enhancement in Rel-18 and FBS (L3 measurement enhancement in Rel-19). They may occur at the same time or at a different time depending on variant conditions on beam management, mobility, etc.

Furthermore, the new indication indicating UE enabling FBS, if it’s agreed to be introduced in RAN4, may be a L3 signaling or lower layers signaling which shall be studied further. A problematic consequence is that if the new indication is decided to be a DCI or MAC-CE message in the end, at least that is our preference, it is inappropriate to employ GBBR, which is a RRC IE, as the configuration triggering enabling FBS since of complex interplay between the L3 signaling and the lower layers signaling.

Proposal 10: GBBR shall not trigger FBS (L3 measurement enhancement in Rel-19).

Issue 1-2-9: FFS: DRX is configured or not

[Way forward]: FFS: - Option 1 (Xiaomi): SSB based L3 measurement delay reduction with DRX shall be deprioritized - Option 2 (Ericsson): FBS may cover DRX cases, no need to deprioritize DRX case.

According to some discussions arising in the last meeting, to our understanding, there are at least two interpretations on DRX on/off in this issue:

- The feature of L3 measurement enhancement in Rel-19 doesn’t apply to a UE configured with DRX on.

- The UE ignores the DRX configuration during measurement, i.e., the measurement delay requirement ignores DRX periodicity, if L3 measurement enhancement in Rel-19 is enabled.

Regarding the former option, we do not deem it essential. The NW can adjust DRX settings with flexibility consideration and directing a UE to work with FBS for certain DRX situations. There’s no strong reason to manually ban all DRX configurations completely.

Regarding the latter option, it leads to the shorter measurement delay, which is anyhow beneficial to the purpose of L3 measurement enhancement in Rel-19. However, the advantage depends on SMTC and DRX configurations, the gain isn’t significant in some cases.

Proposal 11: L3 measurement enhancement is applicable to UE configured with DRX. It can be studied if some DRX cases really impact performance.

Proposal 12: When L3 measurement enhancement is enabled, DRX can be ignored during the fast L3 measurement from tech. perspective. We’re open to the study.

Issue 1-4: Solutions to apply/specify L3 measurement delay reduction by optimizing Rx BSF

[Way forward]: FFS: - Proposal 1 (CATT): - For UE supporting Multi-Rx operation for L3 measurement, the Rx BSF can be reduced to 2, 4 or 6 according to the UE capability. - Proposal 2 (Apple, CTC): - For UE supporting multiple-Rx simultaneous reception, it is proposed to reduce L3 measurement delay by reducing Rx BSF, and Rel-18 Rx BSF reduction in L1 measurement can be used as baseline. - Proposal 3 (Xiaomi): - In order to shorten the overall L3 measurements delay, the smaller RX beam sweeping factor for SSB index acquiring and SSB measurement can be used in comparison with that for PSS/SSS detection. - Proposal 4 (LGE): - If UE supports multi-Rx reception simultaneously for L3 measurement and signal quality for serving cell is low, L3 measurement delay reduction by optimizing Rx BSF could be applied to scenario 1 to 4. - RAN4 to discuss how to reduce M values for SSB based intra- / inter-frequency measurements for UE supporting multi-Rx simultaneous reception - Proposal 5 (Ericsson): - As a result of FBS, RX beam sweep factor can be defined as [4]. - Proposal 6 (Ericsson): - RAN4 to study the requirements relevant to switching between different operations, including: - Transition time period when enabling and disabling FBS in Rel-19. - Transition time period when switching between FBS in Rel-19 and multi-Rx in Rel-18. - Proposal 7 (Samsung): - To define the delay requirements for intra-frequency and inter-frequency measurement w/o MG for L3 multi-Rx measurement, suggest to take N=4, M=20 as the baseline - Whether to/How to define the other values can be based on SLS results (if necessary)

Recall the definition of beam sweeping factor for multi-Rx operation in Rel-18 in TS38.133 as follows, the beam sweeping factor may be 2, 4 or 6 with respect to UE capability.

3.6.19 Applicability of requirements for multi-Rx operation in FR2-1 The requirements related to the support of [reducedRxBeamNum and 30-1] are applicable to a PCell, PSCell, or SCell, provided the UE is configured with a single serving cell (PCell, PSCell, or SCell) in FR2-1. The requirements related to the support of [reducedRxBeamNum] is applicable when the network configures the UE with a CSI report containing groupBasedBeamReporting-v1710.

- 8.1.2.2 Minimum requirement UE shall be able to evaluate whether the downlink radio link quality on the configured RLM-RS resource estimated over the last TEvaluate_out_SSB [ms] period becomes worse than the threshold Qout_SSB within TEvaluate_out_SSB [ms] evaluation period. UE shall be able to evaluate whether the downlink radio link quality on the configured RLM-RS resource estimated over the last TEvaluate_in_SSB [ms] period becomes better than the threshold Qin_SSB within TEvaluate_in_SSB [ms] evaluation period. TEvaluate_out_SSB and TEvaluate_in_SSB are defined in Table 8.1.2.2-1 for FR1. TEvaluate_out_SSB and TEvaluate_in_SSB are defined in Table 8.1.2.2-2 for FR2 with scaling factor N, where N=2, 4, or 6 for FR2-1 for UE supporting [TBD - multi-rx fast beam switching capability] according to the conditions in clause 3.6.x. N=8 for other cases in FR2-1, and N=12 for FR2-2, …

Hence, some companies proposed to reuse the solution of [TBD - multi-rx fast beam switching capability] for L3 measurement enhancement in Rel-19.

However, it’s worth noting that the application/use case difference between L3 measurement enhancement in Rel-19 and L1 measurement enhancement in Rel-18 may result in the invalid reusage between each other.

In L1 measurement enhancement in Rel-18, from beam management perspective, variants of the beam sweeping factor by narrow beams may be reasonable for dealing with different and time varying AoAs in mTRP scenario, e.g., by applying different sets of beams and beam sweepings. As shown in Figure 8, the varying Rx beams on each panel when time passes are applied to deal with the changing directions of serving cell(s) or propagation channels where the UE is monitoring L1 beam.

Figure 8 Variants of L1 beam sweeping on multiple panels

By contrast, L3 measurement mainly deals with the cell-level measurement on the neighbor cell(s) which may be located at any possible direction(s) to panels/Rx beams by broad beams, and then based on the averaged measurement results, the NW determines the most underlying handover target. In this sense, multi-rx fast beam switching capability, which is caple of high directional beam, isn’t critical, instead the L3 measurement shall be able to cover all directions without any missing. To our understanding, as Figure 9 demostrated, the change from the legacy L3 beam sweeping to FBS is simultaneous reception from at least two panels but with the same beam coverage/directions.

Figure 9 L3 beam sweeping on multiple panels (left: legacy, right: FBS)

Observation 3: It’s not rational for Rx beam sweeping factor to reuse [TBD - multi-rx fast beam switching capability] which is introduced in Rel-18, since the applications/use cases are different between L3 measurement enhancement in Rel-19 and L1 measurement enhancement in Rel-18.

Proposal 13: In order to cover all possible directions of neighbor cells, RX beam sweep doesn’t dynamically change, the factor can be defined as a single number, e.g., [4].

Issue 1-5: feature capability of L3 measurement delay reduction by optimizing Rx BSF

[Way forward]: FFS: - Option 1(Apple, vivo, Nokia, Samsung): - RAN4 to introduce a new individual optional capability for L3 BSF reduction due to multi-Rx operation in R19. - Option 1a(Apple): The detailed definition can be postponed to the end of the core part discussion. - Option 2(Eircsson): - To support FBS, RAN4 to check if those capabilities for multi-Rx in Rel-18, e.g., faster RX beam sweeping, enhanced scheduling and measurement restrictions and multi-Rx preference indication, can be used directly. If the NW indicates FBS and multi-RX in Rel-18 is not enabled parallelly, we can observe the possibility of reusing at least part of such capabilities.

We have no concern on Option 1a, the detailed definition can be postponed after the ending of the core part study. Even though, we’re open to discuss the issue by this chance. At the least, we think the capability indicating supporting FBS shall be included. Besides, we don’t observe other capabilities which shall be introduced.

Proposal 14: RAN4 to postpone discussion on capability until fixing FBS solution.

# Conclusion

Observation 1: NW has the knowledge based on measurement result for indicating UE of enabling or disabling FBS.

Observation 2: GBBR triggering FBS results in an issue for the UE being unable to operate solely with the L1 measurement enhancement introduced in Rel-18 without the use of FBS.

Observation 3: It’s not rational for Rx beam sweeping factor to reuse [TBD - multi-rx fast beam switching capability] which is introduced in Rel-18, since the applications/use cases are different between L3 measurement enhancement in Rel-19 and L1 measurement enhancement in Rel-18.

Proposal 1: For CA/DC configuration, Rx beam sweeping scheme on multiple CCs shall be studied.

Proposal 2: For CA/DC configuration, Rx beam sweeping scheme (i.e., FBS or the normal beam sweeping) is configurable per carrier or carrier group, at least for the configuration that PCC and at least one SCC are not measured simultaneously.

Proposal 3: Reflecting different assumptions of searcher number, processing time is specified differently for such two configurations.

- For single carrier configuration in multi-Rx reception, extra processing time=0.

- For CA/DC configuration in multi-Rx reception, if applicable, extra processing time could be reviewed after completing single carrier requirements.

Proposal 4: For single carrier configuration, no extra processing time. For CA/DC configuration, if applicable, RAN4 to check the necessity of extra processing time for handover interruption time.

Proposal 5: No extra processing time for cell identification delay.

Proposal 6: NW indicates UE enabling/disabling FBS through L3 or lower layers signaling.

Proposal 7: NW indicates UE enabling FBS through L3 or lower layers signaling. To save power consumption, after receiving the indication, UE may enable FBS in a predefined time period or for predefined measurement samples. After that, the UE is allowed to disable FBS or fall back to normal beam sweeping.

Proposal 8: As a variant of NW indication, handover command or CHO configuration indicates UE enabling FBS as well.

Proposal 9: BFD and/or RLM status may indicate UE of enabling FBS. If the UE meets the criteria to operate relaxed BFD and/or RLM, it indeed implicitly indicates the UE hasn’t the demand to conduct FBS. On the contrary, if the UE doesn’t meet the criteria to operate relaxed BFD and/or RLM, it indeed implicitly indicates the UE may have the demand to conduct FBS.

Proposal 10: GBBR shall not trigger FBS (L3 measurement enhancement in Rel-19).

Proposal 11: L3 measurement enhancement is applicable to UE configured with DRX. It can be studied if some DRX cases really impact performance.

Proposal 12: When L3 measurement enhancement is enabled, DRX can be ignored during the fast L3 measurement from tech. perspective. We’re open to the study.

Proposal 13: In order to cover all possible directions of neighbor cells, RX beam sweep doesn’t dynamically change, the factor can be defined as a single number, e.g., [4].

Proposal 14: RAN4 to postpone discussion on capability until fixing FBS solution.

# References

- RP-240830, New WID: WI resulting from discussion on RRM enhancements, vivo (Moderator)

- R4-2414026, WF for NR_RRM_Ph5, Apple