3GPP TSG-RAN WG4 Meeting #112bis	R4-2416079

Hefei, CN, October 14th-18th, 2024

Source: 	Ericsson

Title:  	Discussion on FR2-1 L3 measurement delay by optimizing CSSF outside gap in CA/DC

Agenda Item:	6.15.2.2

Document for:	Discussion

# Introduction

In the light of the WI of NR Radio Resource Management (RRM) enhancements (WID in [1]) for Rel-19, the outcomes of the last meeting's debate, encompassing consensuses and unresolved issues, were documented in WF [2].

This contribution outlines our point of view on FR2-1 L3 measurement delay by optimizing CSSF outside gap in CA/DC as follows.

# Discussion

## Topic #2-2: L3 measurement delay reduction by optimizing CSSF for UE not in multiple-Rx simultaneous reception mode

Issue 2-1: Clarification on the bullets in WID for this CSSF optimization

[Way forward]: FFS: - Option a (ZTE, CMCC, Intel, CATT, MTK): - Rel-19 CSSF optimization applies for the both cases: (1)UE is not capable of Rel-18 multi-Rx simultaneous reception, (2)UE is capable of Rel-18 multi-Rx simultaneous reception but not work in multiple-Rx reception mode currently. - Note: if option a is agreeable, then work on wording polishing based on option 1a/1b. - Option b (HW, MTK, Samsung, E///): - Rel-19 CSSF optimization applies for case that UE is not capable of Rel-18 multi-Rx simultaneous reception. - Option c (Apple, Nokia, QC): - Rel-19 CSSF optimization applies for CA/DC scenarios with at least two FR2 serving cells, independently of the UE support of multi-Rx capabilities. - Option d (newly added): - Same Rel-19 CSSF optimization applies for the both cases: (1) UE is not capable of Rel-18 and Rel-19 multi-Rx simultaneous reception, (2) UE is capable of Rel-18 or Rel-19 multi-Rx simultaneous reception but not work in multiple-Rx reception mode currently.

Firstly, we summarize all the options in the last meeting as follows.

We understand the kay concern on the issue is that, is there any difference from CSSF requirement perspective between two cases:

- Case 1: UE is not capable of Rel-18 multi-Rx simultaneous reception

- Case 2: UE is capable of Rel-18 multi-Rx simultaneous reception but not work in multiple-Rx reception mode currently.

We haven’t gotten any answer from any proponent supporting Option a, and that’s the reason we proposed Option b as a conservative approach which doesn’t introduce diverse use cases and specially different solutions.

Option c gives a workaround solution to remove the support of multi-Rx simultaneous reception in Rel-18 by precluding the configuration of a single FR2 serving cells.

Option d seems like an enhancement of Option a, as it includes Rel-19 multi-Rx simultaneous reception.

In light of the information provided, we may update our viewpoint to encourage companies to concentrate on a common CSSF solution which can be applied generally under a wide range of situations.

Proposal 1: Rel-19 CSSF optimization applies for the below cases. The CSSF solution for Case 1 is the baseline and the same CSSF solution applies for Case2 without any extra enhancement.

- Case 1: UE is not capable of Rel-18 or Rel-19 multi-Rx simultaneous reception.

- Case 2: UE is capable of Rel-18 or Rel-19 multi-Rx simultaneous reception but not work in multiple-Rx reception mode currently.

1.1.4	Issue 2-4: Searcher assumption to apply L3 measurement delay reduction by optimizing CSSF

[Way forward]: FFS: - Option 1 (CATT): Not to preclude the solutions based on 3 searchers assumption in current stage. - Option 1a (CMCC, HW): it is proposed to consider both the CSSF optimization based on 2 searchers and the CSSF optimization based on 3 searchers. - Option 1b (QC): RAN4 to consider introducing a new UE optional capability regarding the number of cell search/L3-measurement engines for CSSF enhancement, with the details to be finalized (FFS). - Option 2 (Apple, CTC, Nokia, Ericsson): RAN4 only consider the enhancement based on 2 searchers, i.e., same as previous release, for L3 measurement delay reduction by optimizing CSSF. - Option 2a (Apple): If companies cannot achieve consensus, RAN4 work can start with the baseline assumption in WID, i.e., 2 searchers. - Option 2b (Nokia) RAN4 to confirm if one of the searchers is assumed for PCC/PSCC measurement and the other is assumed for the measurements on all SCCs.

Proposal 2: Prioritize CSSF solution based on 2 searchers. Keep the CSSF solution as common as possible so that it may be applied to the cases involving more than two searchers.

Issue 2-5: Solutions to apply/specify L3 measurement delay reduction by optimizing CSSF outside gap in CA/DC

[Way forward]: FFS: - Option 1 (Apple, CTC. Ericsson, Intel, Samsung, ZTE): UE only needs to measure one serving CC per band if multiple serving CCs are in the same band - Option 2 (CATT, Apple, CTC, ZTE): UE can change the searcher occupancy ratio of PCC or PSCC measurement to speed up SCC measurement for some conditions - Option 4 (CATT, HW): - For UE supporting per FR gap, when all MOs are to be measured outside gap, the searcher used for within gap can be leveraged for outside gap. CSSFoutsidegap can be optimized, as these MOs can share totally three searchers. (HW, CATT) - We are open to further discuss the applicable condition of the three searcher solution and whether a new capability is needed. (HW)

One solution of CSSF optimization is skipping measurement on one or more than one serving CC.

Considering the similarity between CCs, the measured CC(s) and the not-measured CC(s) shall be in the same band. Recap the table below which is quoted from TS 38.133.

#### 9.2.3.2 Requirements for FR2 For one single intra-frequency layer in a band, during each layer 1 measurement period, the UE shall be capable of performing SS-RSRP, SS-RSRQ, and SS-SINR measurements for at least: - 6 identified cells, and - 24 SSBs with different SSB index and/or PCI, where this single intra-frequency layer shall be: - PCC when UE is configured with SA NR operation mode with PCC in the band; or - PSCC when UE is configured with EN-DC with PSCC in the band; or - PSCC when UE is configured with NR-DC with PSCC in the band; or - One of the SCCs on which UE is configured to report SSB based measurements when neither PCC nor PSCC is in the same band, so that the selected SCC shall be an SCC where the UE is configured with SS-RSRP measurement reporting if such SCC exists, otherwise the selected SCC is determined by UE implementation. The UE shall also be capable of performing SS-RSRP, SS-RSRQ, and SS-SINR measurements for at least 2 SSBs on serving cell for each of the other intra-frequency layer(s) in the same band.

Such a solution for UE was raised that a UE only needs to measure the CCs with respect to the rules highlighted in blue and the CC(s) to be skipped from measurement by the CSSF optimization solution in this WI exactly is the CCs highlighted in yellow. From NW perspective, we observe a critical drawback that if MOs are configured for all CCs, then the NW cannot decide the CC(s) to be measured even the NW knows which CC is measured.

In this way, the NW couldn’t acquire the entire information of the overall channel quality on the whole band or all CCs in the band, if the UE doesn’t report SSBs on all intra-frequency layers (one particular example is only one frequency layer is to be measured) in the serving cells and the similarity of channel quality on the whole band isn’t valid.

Then, for the purpose of determining the CCs to be measured and acquiring the overall channel quality of the band, the NW has to reconfigure/update MOs and/or measurement report configurations one or more than one time for the UE. Apparently, these actions instead add more and unneglectable signaling load and delay, which is not expected by the WI.

Another advantage of the NW determining the CCs to be measured is that the UE cannot change the CCs to be measured once the rule is fixed, on the contrary, the NW may update the CCs to be measured based on the reports from the UE, which improves the scheduling process.

Given MOs are configured for all CCs, we prefer that the NW shall be in charge of measuring the CCs. Moreover, we consider it’s beneficial to give the UE of some flexibilities. For example, the UE may decide the CC(s) to be measured instead if the NW doesn’t offer any configuration or indication for identifying the CC(s) to be measured.

However, it shall be noted that some questions shall be clarified for the case of UE determining the CC (s) to be measured:

- Question 1: Is the UE allowed to independently (up to UE implementation) measure only one CC or more than one CC over other serving CCs?

- Question 2: Shall there be any condition/rule for the UE determining the CC(s) to be measured?

As a reference point, in the case of 2 CCs, if a CC measurement result is stationary while the other CC measurement result varies with time, the measurements on the two CCs both shall be kept. On other hand, if a CC measurement result is stationary while another CC measurement result also is stationary and keeps similarity to the former CC, then only measuring one CC is adequate.

Proposal 3: NW configuration/indication enables/disables measurement on particular CC(s), if multiple serving CCs are allocated in the same band.

Proposal 4: In case of no NW configuration/indication, UE is allowed to determine the CC(s) to be measured based on rules which shall be studied, if multiple serving CCs are allocated in the same band.

Observation 1: Even UE is allowed to determine the CC(s) to be measured, a clear rule shall be specified.

Proposal 5: RAN4 to study the rule (including condition) for the UE determining the CC(s) to be measured.

Among all SCCs, to reduce CSSF, the measurement on particular CC(s) may be disabled, and it’s reflected in  NSCC_SSB in CSSF definition table in TS 38.133 which could be reduced eventually. It’s noted that NSCC_SSB in that CSSF table is for accounting the SCC(s) on which neighbor cell measurement isn’t required, we may focus on such case.

Proposal 6: CSSF reduction by disabling measurement on particular CC(s) covers the below case:

- CSSF for SCCs where neighbor cell measurement isn’t required, e.g., reducing NSCC_SSB.

And it doesn’t cover the below cases:

- CSSF on SCCs where neighbor cell measurement is required

- CSSF on SCCs for inter-frequency without gap.

Another solution of CSSF optimization is adjusting measurement ratio among CCs.

Recall the CSSF rule From Rel-15 as follows. Based on 2 searchers assumption,

- One searcher is dedicated for PCC,

- The other searcher is shared by all SCCs.

  - For DC, the cell search/L3-measurement engine is dedicated for PSCC with 50% of the overall time, and for the other 50%, the engine is equally shared by the rest of the CCs.

  - For CA, half of the other searcher (50%) is dedicated for SCC where neighbour cell measurement is required, which is prioritized than other SCC(s), the remaining half of the searcher is shared among FR1 SCCs and FR2 SCCs where neighbour cell measurement is not required.

It appears that the CSSF can be further optimized through changing the measurement ratio among CCs. It is important to recognize, however, that various alternatives are available for implementing this technique. Our points of view are offered below, along with a series of questions on the measurement ratio solution from numerous viewpoints.

From the carrier on which measurement ratio is to be adjusted perspective, there’re at least the below options:

- Option 1: Adjust measurement ratio between PCC/PSCC and SCC.

- Option 2: Adjust measurement ratio among SCCs.

- Option 3: Option 1 and Option 2 both are applied.

We understand that Option 1 and Option 2 both could be applied in theory. But reducing the measurement ratio of PCC/PSCC and consequently delaying the measurement on PCC/PSCC may leads to the network performance degradation. In a similar manner, reducing the measurement ratio of SCC too much may affect the network performance as well.

From the procedure enabling the adjustment of measurement ratio perspective, there’re at least the below options:

- Option 1: The NW configures the measurement ratio among CCs.

- Option 2: The UE determines the measurement ratio among CCs.

To our opinion, the NW is able to configure the measurement ratio among CCs based on the reports on those.

From the condition of applying adjustment of the measurement ratio perspective, there’re at least the below options:

- Option 1: The NW configures the measurement ratio among CCs. When the UE receives it, the UE applies the indicated measurement ratio among CCs.

- Option 2: It’s up to the UE to determine when to apply the adjustment of measurement ratio among CCs.

- Option 3: The UE applies the measurement ratio among CCs if certain condition (predefined or configured) is fulfilled.

To avoid the unexpected performance degradation what we mentioned in the above paragraph, the measurement ratio among CCs can’t be adjusted unconditionally. Either the NW signals the UE of adjusting the measurement ratio or the UE adjusts the measurement ratio only when certain condition is fulfilled.

We may think about what condition can trigger the adjustment of measurement ratio, the below are some examples.

- In one example of mobility, PCC may be prioritized over SCCs. On the contrary, if the UE is not at cell-edge and mobility isn’t expected to happen, PCC priority may be decreased a little to an acceptable level. However, fully relying on the UE determination is risky and inefficient, instead, the NW shall be in charge of the CSSF allocation since the NW manages SCCs including activating/deactivating and addition/release.

- In another example, in the case of 2 CCs, if a CC measurement result is stationary while the other CC measurement result is varying with time, maybe the stationary CC can be allocated with the lower measurement ratio and the varying CC can be allocated with the higher measurement ratio.

Moreover, the ratio number shall be determined, it may be one from a candidate ratio list. There’re at least some options as follows:

- Option 1: The ratio is configured by the NW.

- Option 2: The ratio is determined by the UE.

Proposal 7: RAN4 needs to address the following questions for the solution of adjusting the measurement ratio among CCs.

Question 1: Which CCs shall be applied with measurement ratio adjustment?

- Option 1: Adjust measurement ratio between PCC/PSCC and SCC.

- Option 2: Adjust measurement ratio among SCCs or SCC groups.

- Option 3: Option 1 and Option 2 both are applied.

- Preference: Option 2.

Question 2: Who determines measurement ratio adjustment?

- Option 1: NW configures measurement ratio.

- Option 2: UE determines measurement ratio.

- Preference: Option 1.

Question 3: When to apply measurement ratio adjustment?

- Option 1: NW configures measurement ratio. When UE receives it, UE applies the indicated measurement ratio.

- Option 2: It’s up to UE to determine when to apply the adjustment of measurement ratio.

- Option 3: UE applies the adjustment of measurement ratio if certain condition (predefined or configured) is met.

- Preference: Option 1.

Question 4: What is the ratio number? It may be a ratio number out of a candidate ratio number list.

- Option 1: The ratio applied is configured by NW.

- Option 2: The ratio applied is determined by UE.

- Preference: Option 1.

Eventually, it’s able to design various measurement ratio solutions concerned by different views, use cases and scenarios. But for the sake of simplification and keeping generality on most of cases, our preferences are also listed as follows.

Recap the measurement delay of the activated SCell and the deactivated Scell, the measurement ratio of the deactivated Scell in fact is reduced by filling various values in RRC IE measCycleSCell. It may be regarded as a particular measurement ratio between the activated SCell and the deactivated Scell to some level.

To our understanding, the WI doesn’t enhance the measurement ratio between activated SCell and deactivated Scell, but it needs confirmation.

Proposal 8: The WI doesn’t enhance measurement ratio between activated SCell and deactivated Scell.

Moreover, regarding the case of multiple component carriers, in the current specification, the typical assumption is that a UE measures all component carriers with same priority and then reports the measurement results after all measurements are completed. However, the NW may expect to know the coverage of certain carriers more urgently than other carriers. For this purpose, the NW may prioritize the carrier(s) to be measured and reported above the other carrier(s), which enables the NW to obtain the information on such prioritized carrier(s) earlier. By this way, the target of this WI, reducing FR2-1 L3 measurement delay on certain carrier(s) could be achieved without or with the CSSF solution changing measure chance/ratio on CCs. To our understanding, it’s desirable to take the approach into account.

Proposal 9: NW may prioritize the carrier(s) to be measured and reported above other carrier(s), which enables the NW to obtain information on such prioritized carrier(s) earlier than others.

Issue 2-6: feature capability of L3 measurement delay reduction by optimizing CSSF

[Way forward]: FFS: - Option 1 (Apple, Nokia): RAN4 to introduce a new individual capability for CSSF reduction in R19. - Option 1a(Nokia):The reduced CSSF shall be applied to the UE supporting the capability and starting from R19 - Option 2 (CATT, Apple): delay the capability discussion to the end of the core part.

Since there’s no concrete solution yet, we agree to delay the capability discussion to the end of the core part.

Proposal 10: Postpone the discussion about capability to the end of the core part.

Issue 2-7: Other WID scope discussion

[Way forward]: FFS: - Proposal 1(CMCC, CTC): after FR2-1 L3 measurement delay reduction by optimizing CSSF is concluded, the technical solutions can be extended to FR1 if applicable.

We haven't been concerned about Proposal 1, which is essentially our understanding.

Proposal 11: After FR2-1 L3 measurement delay reduction by optimizing CSSF is concluded, the technical solutions can be extended to FR1 if applicable.

# Conclusion

Observation 1: Even UE is allowed to determine the CC(s) to be measured, a clear rule shall be specified.

Proposal 1: Rel-19 CSSF optimization applies for the below cases. The CSSF solution for Case 1 is the baseline and the same CSSF solution applies for Case2 without any extra enhancement.

- Case 1: UE is not capable of Rel-18 or Rel-19 multi-Rx simultaneous reception.

- Case 2: UE is capable of Rel-18 or Rel-19 multi-Rx simultaneous reception but not work in multiple-Rx reception mode currently.

Proposal 2: Prioritize CSSF solution based on 2 searchers. Keep the CSSF solution as common as possible so that it may be applied to the cases involving more than two searchers.

Proposal 3: NW configuration/indication enables/disables measurement on particular CC(s), if multiple serving CCs are allocated in the same band.

Proposal 4: In case of no NW configuration/indication, UE is allowed to determine the CC(s) to be measured based on rules which shall be studied, if multiple serving CCs are allocated in the same band.

Proposal 5: RAN4 to study the rule (including condition) for the UE determining the CC(s) to be measured.

Proposal 6: CSSF reduction by disabling measurement on particular CC(s) covers the below case:

- CSSF for SCCs where neighbor cell measurement isn’t required, e.g., reducing NSCC_SSB.

And it doesn’t cover the below cases:

- CSSF on SCCs where neighbor cell measurement is required

- CSSF on SCCs for inter-frequency without gap.

Proposal 7: RAN4 needs to address the following questions for the solution of adjusting the measurement ratio among CCs.

Question 1: Which CCs shall be applied with measurement ratio adjustment?

- Option 1: Adjust measurement ratio between PCC/PSCC and SCC.

- Option 2: Adjust measurement ratio among SCCs or SCC groups.

- Option 3: Option 1 and Option 2 both are applied.

- Preference: Option 2.

Question 2: Who determines measurement ratio adjustment?

- Option 1: NW configures measurement ratio.

- Option 2: UE determines measurement ratio.

- Preference: Option 1.

Question 3: When to apply measurement ratio adjustment?

- Option 1: NW configures measurement ratio. When UE receives it, UE applies the indicated measurement ratio.

- Option 2: It’s up to UE to determine when to apply the adjustment of measurement ratio.

- Option 3: UE applies the adjustment of measurement ratio if certain condition (predefined or configured) is met.

- Preference: Option 1.

Question 4: What is the ratio number? It may be a ratio number out of a candidate ratio number list.

- Option 1: The ratio applied is configured by NW.

- Option 2: The ratio applied is determined by UE.

- Preference: Option 1.

Eventually, it’s able to design various measurement ratio solutions concerned by different views, use cases and scenarios. But for the sake of simplification and keeping generality on most of cases, our preferences are also listed as follows.

Proposal 8: The WI doesn’t enhance measurement ratio between activated SCell and deactivated Scell.

Proposal 9: NW may prioritize the carrier(s) to be measured and reported above other carrier(s), which enables the NW to obtain information on such prioritized carrier(s) earlier than others.

Proposal 10: Postpone the discussion about capability to the end of the core part.

Proposal 11: After FR2-1 L3 measurement delay reduction by optimizing CSSF is concluded, the technical solutions can be extended to FR1 if applicable.

# References

- RP-240830, New WID: WI resulting from discussion on RRM enhancements, vivo (Moderator)

- R4-2414026, WF for NR_RRM_Ph5, Apple