3GPP TSG-RAN WG4 Meeting #112bis	R4-2416080

Hefei, CN, October 14th-18th, 2024

Source: 	Ericsson

Title:  	Discussion on RRM core requirements for beam management

Agenda Item:	6.17.4.1

Document for:	Discussion

# Introduction

In last meeting 3GPP TSG RAN WG4 Meeting #112, RAN4 discussed RRM requirements for AI/ML based beam management and the discussion summary is listed in [1]. Following the summary in the WF, we herein in this contribution give our views and proposals on the remaining open issues related to core requirements.

# Discussion

## Sub-topic 2-1 RSRP Prediction accuracy

Issue 2-1: RSRP Prediction accuracy

- Proposals - Option 1: The predicted L1-RSRP difference is similar to the legacy, the absolute RSRP accuracy and the relative RSRP accuracy can be: - Absolute RSRP accuracy= predicted L1-RSRP of beam index i – ideal measured L1-RSRP of beam index I (ground truth as agreed in RAN4#111) - Relative RSRP accuracy= predicted L1-RSRP of beam index i – predicted L1-RSRP of beam index n, where the beam index n owns the largest predicted value. - Accuracy requirements should apply to all predicted beams that satisfy the predefined side conditions, including SNR. - Option 2: others

Regarding online discussions in the last meeting, most of companies agreed on the absolute RSRP accuracy in Option 1. At least one company proposed that absolute RSRP accuracy may refer to the definition in TR38.843. Below we analyze the reasons supporting Option 1 instead of reusing the definition in TR38.843.

We suppose that the mentioned definition in TR38.843 is the content that is cited in the below.

Beam prediction accuracy related KPIs, including: - Top-1 genie-aided Tx beam considers the following definitions: - Option A (baseline), the Top-1 genie-aided Tx beam is the Tx beam that results in the largest L1-RSRP over all Tx and Rx beams - Option B (optional), the Top-1 genie-aided Tx beam is the Tx beam that results in the largest L1-RSRP over all Tx beams with specific Rx beam(s) - Specific Rx beam(s) are to be reported. Note: specific Rx beams are a subset of all Rx beams. - Top-1 genie-aided Tx-Rx beam pair considers the following definitions: - Option A: The Tx-Rx beam pair that results in the largest L1-RSRP over all Tx and Rx beams - Other options not precluded and can be reported - Average L1-RSRP difference of Top-1 predicted beam: - The difference between the ideal L1-RSRP of Top-1 predicted beam and the ideal L1-RSRP of the Top-1 genie-aided beam - Beam prediction accuracy (%): - Top-1 (%): the percentage of "the Top-1 genie-aided beam is Top-1 predicted beam" - Top-K/1 (%): the percentage of "the Top-1 genie-aided beam is one of the Top-K predicted beams" - Top-1/K (%) (Optional): the percentage of "the Top-1 predicted beam is one of the Top-K genie-aided beams" - Where K >1 and values can be reported - CDF of L1-RSRP difference for Top-1 predicted beam - Beam prediction accuracy (%) with 1dB margin for Top-1 beam - The beam prediction accuracy (%) with 1dB margin is the percentage of the Top-1 predicted beam "whose ideal L1-RSRP is within 1dB of the ideal L1-RSRP of the Top-1 genie-aided beam" - Other beam prediction accuracy related KPIs are not precluded and can be reported

It’s noted that the first paragraph quoted indicates that the KPIs represented by the subsequent bullets is to indicate ‘beam prediction accuracy’. It’s worth noting that the two concepts: ‘RSRP prediction accuracy’ and ‘beam prediction accuracy’ are different and shall not be messed up between each other, wherein, as per our understanding,

- The beam prediction accuracy aims at the evaluation (system/link level simulation) of AI/ML model performance, wherein, the ideal L1-RSRP and the genie-aided beam are difficult to be acquired by RAN4 test.

- The RSRP prediction accuracy is an evaluation from RAN4 perspective, which is derived from the measurable RSRP acquired by RAN4 test.

Furthermore, TR38.843 was already aware of RSRP prediction accuracy in content, as evidenced by the content 'predicted L1-RSRP difference' quoted below. Where, ‘predicted L1-RSRP’ is used as the term, but the definition is same.

For AI/ML models, which provide L1-RSRP as the model output, the accuracy of predicted L1-RSRP is to be evaluated. Companies optionally report average (absolute value)/CDF of the predicted L1-RSRP difference, where the predicted L1-RSRP difference is defined as the difference between the predicted L1-RSRP of Top-1 predicted beam and the ideal L1-RSRP of the same beam.

Observation 1: It does not make sense to use average L1-RSRP difference of Top-1 predicted beam (referring to TS38.843) for defining RSRP prediction accuracy. ‘RSRP prediction accuracy’ and ‘beam prediction accuracy’ are different concepts, which is exposed in TS38.843 as well.

Proposal 1: Absolute RSRP accuracy= predicted L1-RSRP of beam index i – ideal measured L1-RSRP of beam index i. i may be any beam index in top-K predicted beams.

Regarding the relative RSRP accuracy, firstly, recap the legacy relative RSRP accuracy definition in Clause 10.1.20.1.2 in TS38.133, the definition is copied as follows:

﻿The relative accuracy of SSB based L1-RSRP is defined as the L1-RSRP measured from one SSB compared to the largest measured value of L1-RSRP among all SSBs of the serving cell.

To define the relative accuracy for the predicted RSRP, it really looks straightforward to replace the measured RSRP in the legacy definition with the predicted RSRP, i.e., relative RSRP accuracy= predicted L1-RSRP of beam index i – predicted L1-RSRP of beam index n, where the beam index n owns the largest predicted value.

However, before endorsing such a solution, we must start from understanding the fundamental demand to the relative accuracy requirement. The intention of the requirement is to guarantee the accuracy of the relative L1-RSRP, which allows the network to compare the received signal power from different beams (or cells) more accurately by comparing the relative differences among them. This aids the UE and the gNB in determining the optimal beam or cell. Hence, the reference L1-RSRP of relative accuracy shall be assessed.

In legacy, there is no doubt that the reference L1-RSRP is the largest measured value of L1-RSRP.

Differently, in AI/ML based BM, regarding the RAN1 agreements duplicated in the below, the RSRP of a beam, which is reported by a UE, may be the predicted RSRP (if no measurement configured for the beam) or the measured RSRP.

Agreement For report content of inference results for UE-sided model for BM-Case 1, for the RSRP of predicted Top K beam(s) in the report of inference results, when applicable, further study the following options: - Option A: Predicted RSRP - Option B: Predicted RSRP, if the beam is not configured for corresponding measurement, and measured L1-RSRP if the beam is configured for corresponding measurement - Where the predicted RSRP is based on AI/ML output - Note: Support both Option A and Option B is not precluded.

From the practical use case point of view, it can be observed that the relative RSRP accuracy may be predicted or measured values depending on whether the largest value of reported RSRP is derived from prediction or measurement.

To our opinion, the definition of the relative RSRP accuracy may be:

- Relative RSRP accuracy= predicted L1-RSRP of beam index i – predicted L1-RSRP of beam index n, where the beam index n owns the largest value (including predicted and measured beams), if beam index n is not configured for corresponding measurement.

- Relative RSRP accuracy= predicted L1-RSRP of beam index i – measured L1-RSRP of beam index n, where the beam index n owns the largest value (including predicted and measured beams), if beam index n is configured for corresponding measurement.

By this, the definition in this sense encompasses all potential conditions.

Proposal 2: The definition of relative RSRP accuracy shall be:

- Relative RSRP accuracy= predicted L1-RSRP of beam index i – predicted L1-RSRP of beam index n, where the beam index n owns the largest value (including predicted and measured beams), if beam index n is not configured for corresponding measurement.

- Relative RSRP accuracy= predicted L1-RSRP of beam index i – measured L1-RSRP of beam index n, where the beam index n owns the largest value (including predicted and measured beams), if beam index n is configured for corresponding measurement.

## Sub-topic 2-6 Measurement error impact evaluation

Multiple companies brought up the need to evaluate the impact that the UE measurement error and the error in the training data have on inference accuracy Issue 2-6: Measurement error impact - Proposals - Option 1: RAN4 should study the impact of measurement error, companies should bring proposals for the next meeting on how to proceed with such a study - Option 2: RAN4 should postpone the discussion on the impact of the measurement error for now - Option 3: No need to evaluate the measurement error impact - Recommended WF - To be discussed

Depending on the type of output of AI/ML model, the output type, such as label (classifier network) or RSRP (CNN or MLP), the request to the measurement error is different.  Some examples show that the AI/ML model with label (classifier network) output may be rather less impacted by measurement error impact on the prediction than the AI/ML model with RSRP output.

Observation 2: In case of AI/ML model with label (classifier network) output, measurement error may have less impact to the prediction than the AI/ML model with RSRP output, if the reporting of the UE only relies on the output of AI/ML model without explicit or implicit assistance of measurement result.

Proposal 3: Measurement error shall be evaluated with respect to the type of output of AI/ML model, e.g., label (classifier network) or RSRP (CNN or MLP).

Proposal 4: Measurement error impact to prediction shall be evaluated for BM-case 1 and BM-case 2 respectively.

Proposal 5: Measurement error impact to prediction shall be verified (e.g. by simulation) and the same methodology in Rel-18 RAN1 studies can be referred with the consideration of to guarantee an acceptable prediction performance.

### Link-level evaluation methodology:

Based on previous RAN1’s study in Rel-18, the main source of measurement error includes RF impairments in transmitter and receiver (mainly EVM), fading channel and additive thermal AWGN noise at receiver. Specifically, the RF impairment inaccuracy can be set as +-4dB, which was based on reported value of UE vendors. The effect of noise and fading can be evaluated via different SINR levels and typical channels during the link level simulations. The obtained accuracy requirement should be set by adding up the value ranges.

The errors are independently selected for each gNB beam, according to the following:

- During training: Errors were applied to model input as well as to targeted model output (ground truth).

- During inference: Errors were applied to model input but not to targeted model output (ground truth).

Proposal 6: RAN4 to consider the L1-RSRP values for generated channel as the metric in the link-level simulations.

Proposal 7: If tightening measurement error under current side condition is difficult from UE implementation perspective, RAN4 can add another side condition for tightened measurement error for enabling AI-ML model.

Proposal 8: RAN4 to consider the following parameters in Table 1 in the link level evaluation.

Table 1 summary of link level evaluation assumptions

| Parameters | Value |
| --- | --- |
| Frequency range | FR2@30GHz; SCS 120kHz |
| Data allocation | 8 RBs First 2 OFDM symbols for PDCCH, and following 12 OFDM symbols for data channel |
| PDCCH decoding | Ideal or non-ideal |
| Channel model | LOS channel: CDL-D extension, DS = 100ns NLOS channel: CDL-A/B/C extension, DS = 100ns |
| The measurement inaccuracy | Zero mean Gaussian distribution with 95% of density be within (-6.5, 6.5) dB |
| Impairment types affecting set B measurements and set A measurements | 4dB RF impairments with normal distribution, 1dB fading condition. RF impairment model is composed of different elements, including I/Q imbalance, Quantization noise, Phase noise, Filters/Ripple noise, RF PA distortion noise |

### System-level evaluation methodology:

There are multiple scenarios and configurations for generalization verification may consider the AI model inference node at either UE or gNB side and BM-Case1 / BM-Case2. However, for the sake of simplicity, it is preferred that RAN4 consider unifying the baseline simulation assumption based on the existing RAN1 studies.

Proposal 9: RAN4 to consider the following parameters in Table 2 in the system level evaluation.

Table 2 Summary of system level evaluation assumptions

| Parameters | Value |
| --- | --- |
| Frequency range | FR2@30GHz; SCS 120kHz |
| Deployment | 200m ISD, 2-tier model with wrap-around (7 sites, 3 sectors/cells per site) |
| Scenario | Dense urban (macro-layer only) |
| System level channel model | UMa with distance-dependent LoS probability function defined in Table 7.4.2-1 in TR 38.901. |
| Number of set -B/ set-A beams, | 8 SSB /32 CSI-RS beams |
| Antenna port layout | Antenna setup and port layouts at gNB: (4, 8, 2, 1, 1, 1, 1), (dV, dH) = (0.5, 0.5) λ Antenna setup and port layouts at UE: (1, 4, 2, 1, 2, 1, 1), 2 panels (left, right) |
| Best Rx beam selection algorithm | Baseline 1: Select the best beam within Set A of beams based on the measurement of all RS resources or all possible beams of beam Set A (exhaustive beam sweeping) Baseline 2: Select the best beam within Set B of beams based on the measurement of all RS resources or all possible beams of beam Set B |
| Transmission Power | Maximum Power and Maximum EIRP for base station and UE as given by corresponding scenario in 38.802 (Table A.2.1-1 and Table A.2.1-2) |
| Traffic Model | Full buffer |

## Sub-topic 2-8 Beam prediction and TCI state related issues

Some companies brought up the issue that the UE will predict the gNB Tx beam but it might know the UE Rx beam. This could affect the known/unknown TCI state conditions Issue 2-8: Beam prediction and TCI state related issues - Proposals - Option 1: Tx beam prediction does not imply that UE knows the best UE Rx beam to receive the predicted beam - TCI state for the predicted beam is unknown unless it has been explicitly measured within 1280ms - Option 2: UE knows the Rx beam paired with the predicted Tx beam - FFS whether the TCI state associated with a predicted Tx beam is known or unknown - Option 3: UE knows the Rx beam paired with the predicted Tx beam - TCI state should be considered known, FFS whether side conditions for known TCI state should be modified (relaxed) for this case - Option 6: others - Recommended WF - To be discussed

First of all, the issue actually raises a basic question: when a UE predicts a Tx beam, whether it always knows the associated Rx beam for receiving the TX beam?

As per our understanding, the output type of AI/ML model may be label (beam ID) & RSRP or label only.

- For the label type, the AI/ML model typically is a classifier network.

- For label & RSRP type, the AI/ML model typically is a CNN or MLP network.

For the former one, beam prediction is derived by the valid beam detection in training, and for the later one, beam prediction is derived by the valid beam detection and RSRP measurement in training.

In the training phase, for beam detection, the UE shall be able to receive the SSB and identify the beam ID and other IDs, e.g., associated ID, with respect to the given Set A and Set B. In same manner, for RSRP measurement, the UE shall be able to receive the SSB and identify the beam ID and further measure its RSRP. In order to get the above information, Rx beam is enabled for the beam identification and measurement, such that the UE aware of the association between the Tx beam and Rx beam, even if it is not specified how the UE conduct the Rx beam.

It's worth noting that Set A and Set B remain unchanged in training phase and inference phase with respect to the consistency between training phase and inference phase. Besides, we understand that if a TX beam has not been detected or measured in the training phase, it’s not rational for the NW to configure the TX beam to the UE for prediction since such prediction couldn’t be performance guaranteed.

Then, in inference phase, for label prediction, the UE shall be able to predict the SSB and beam ID with respect to Set A.  In same manner, for label &RSRP prediction, the UE shall be able to predict the SSB and beam ID and further measure its RSRP. In order to predict the above information, the Rx beam shall be enabled and operatable to those Tx beams and such that the UE is aware of the predicted association between the Tx beam and RX beam, even if it is not specified how the UE assumes Rx beam. Otherwise, for example, if the UE doesn’t know its Rx beam associated with a Tx beam, it may wrongly use 0dB Rx gain for Tx beam 1, and 6dB Rx gain for Tx beam 2 which is 3dB lower than Tx beam 1 at transmitter side, the prediction then always determines the best Tx beam is Tx beam 2, but it isn’t true, a test or monitoring can easily detect the fault and eventually the AI/ML model prediction is invalid. Only when the UE knows the Rx beam and knows the association between the Tx beam and Rx beam by prediction, as a prerequisite, the AI/ML model prediction is valid.

Observation 3: UE always knows the Rx beam paired with the predicted Tx beam provided the predicted Tx beam is detectable in training phase and inference phase.

One possible concern is when the prediction happens, the corresponding training phase has passed a while, if the time interval between the training phase and inference phase is long enough, the UE may lose or abandon the information of Rx beam. In our opinion, it appears that at least in the inference phase, the UE shall retain all information relevant to the Rx beam, which is learned from the training phase. Otherwise, the UE must detect and measure the predicted beam to acquire Rx beam again, which indeed opposes the objective of prediction, reducing latency.

Observation 4: In inference phase, UE is able to retain all information relevant to Rx beam (including the association between Tx beam and Rx beam), which is learned from training phase.

It’s observed that a UE may not know the Rx beam associated with a Tx beam in the below two cases:

- Case Set A: the UE has not detected/measured the Tx Set A beam in training phase, but as we mentioned in above paragraph, if the TX beam isn’t trained, no reason to predict it directly in inference phase.

- Case Set B: the Tx beam is not in Set B. Same concern to the case, if the TX beam isn’t trained, no reason to predict it directly in inference phase.

Observation 5: UE needs not know the Rx beam associated with a Tx beam, if the Tx beam belongs to the below two cases:

- Case Set A: the UE has not detected/measured the Tx beam in train phase.

- Case Set B: the Tx beam is not in Set B.

Different from the legacy way as quoted in the below, defining an exact delay, i.e., 1280ms in the known conditions is not suitable for an AI/ML based known TCI state, i.e., a TCI state QCLed with a predicted Tx beam. Instead, the criteria for known conditions based on the AI/ML model behavior must be constructed while taking training and interference into consideration.

The TCI state is known if the following conditions are met: - During the period from the last transmission of the RS resource used for the L1-RSRP measurement reporting for the target TCI state to the completion of active TCI state switch, where the RS resource for L1-RSRP measurement is the RS in target TCI state or QCLed to the target TCI state - TCI state switch command is received within 1280 ms upon the last transmission of the RS resource for beam reporting or measurement - The UE has sent at least 1 L1-RSRP report for the target TCI state before the TCI state switch command - The TCI state remains detectable during the TCI state switching period - The SSB associated with the TCI state remain detectable during the TCI switching period - SNR of the TCI state ≥ -3dB

Proposal 10: For prediction-based TCI state switch, not to reuse 1280ms as one of known conditions, instead, we shall take training/ inference phase, or training/ inference validity as benchmark in the known conditions.

As listed in the following table, in the inference phase, the known conditions of TCI state have below relationships with the RS in the target TCI state.

| Known TCI state | Unknown TCI state |
| --- | --- |
| Detectable RS in Set A (even the RS isn’t in the prediction report by UE) and the RS in Set A is trained | Undetected RS in Set A, or Detected RS in Set A, but the RS isn’t trained |
| Detectable RS in Set B | Undetectable RS in Set B |

Proposal 11: The known conditions of the predicted TCI state shall take below conditions into account:

- The TCI state switch command is received during inference phase of AI/ML mode.

- The predicted beam in Set A is the RS in target TCI state.

- The predicted beam in Set A is detectable during training and inference phase, which implies the beam has been trained successfully and available in inference phase.

Proposal 12: During training phase, measurement reporting on Set A and Set B also may enable the NW to configure a TCI state associated with a RS out of the measured Set A or Set B. However, it is not relevant to AI/ML model. Hence, the known conditions for Set A or Set B during training phase are not in scope.

Proposal 13: During inference phase, it may occur that the TCI state is QCLed with a RS belonging to Set B. The legacy known conditions are applied. Hence, the known conditions for Set B during inference phase are not in scope.

## Sub-topic 2-10 KPIs for beam prediction

KPIs for beam prediction Different metrics/KPIs have been discussed and were captured in the TR: For metrics for beam management requirements/tests, the following test metrics are identified and could be considered - Option 1: RSRP accuracy - Option 2: Beam prediction accuracy - Top-1 (%): the percentage of "the Top-1 strongest beam is Top-1 predicted beam" - Top-K/1 (%): the percentage of "the Top-1 strongest beam is one of the Top-K predicted beams" - Top-1/K (%): the percentage of "the Top-1 predicted beam is one of the Top-K strongest beams" - Option 3: The successful rate for the correct prediction which is considered as maximum RSRP among top-K predicted beams is larger than the RSRP of the strongest beam – x dB, - Related measurement accuracy can be considered to determine x - Option 4: combinations of above options KPIs and metrics were discussed in the previous meeting without any agreement, RAN4 should continue to discuss what metrics are more appropriate and how they impact requirement definition and testing Issue 2-10: Metrics/KPIs for BM Proposals - Option 1: Use Option 1 - Option 2: Use Option 2 - Option 3: Use Option 1,2,3 depending on use case - Option 4: Neither Option 1, 2, 3 is appropriate, a new metric is needed - Option 5: Consider reliability metric based on relative difference in between the predictions due to change in the conditions/inputs. - Option 6: Other - Recommended WF - To be discussed Companies suggesting to use different metrics(new) should come up with a concrete proposal

We consider the Metrics/KPIs for beam management requirements/tests as follows with the below options separately:

- In the case of label (beam ID) output type of AI/ML model, the Metrics/KPIs for beam management requirements/tests must be derived by beam prediction accuracy.

- In the case of label (beam ID) + RSRP output type of AI/ML model, the Metrics/KPIs for beam management requirements/tests must take both of beam prediction accuracy and RSRP accuracy into account, i.e., the UE shall fulfil beam prediction accuracy and RSRP accuracy simultaneously.

Proposal 14: Metrics/KPIs for beam management requirements/tests are separated in below options:

- In the case of label (beam ID) output type of AI/ML model

  - Beam prediction accuracy shall be fulfilled.

- In the case of label (beam ID) + RSRP output type of AI/ML model

  - Beam prediction accuracy and RSRP accuracy simultaneously shall be fulfilled.

Besides, another possible issue may be valuable to be identified as well. According to RAN1 agreements to date, for BM-Case 1, if a beam in Set A is configured for measurement, then the AI/ML model may produce the measured RSRP not the predicted RSRP. In this case, if the beam information and RSRP both are in the content in the report of AI/ML model inference results, the Metrics/KPIs for beam management requirements/tests must take both of the measured RSRP accuracy and beam information into account, where the measured RSRP accuracy shall follow the legacy requirements.

Proposal 15: For BM-Case 1, if a beam in Set A is configured for being measured as well, then the AI/ML model may produce the measured RSRP instead of the predicted RSRP. In this case, if beam information and RSRP both are in the content in the report of AI/ML model inference results, the Metrics/KPIs for beam management requirements/tests must take both of the measured RSRP accuracy and beam information into account, where measured RSRP accuracy shall follow the legacy requirements.

# Conclusion

Observation 1: It does not make sense to use average L1-RSRP difference of Top-1 predicted beam (referring to TS38.843) for defining RSRP prediction accuracy. ‘RSRP prediction accuracy’ and ‘beam prediction accuracy’ are different concepts, which is exposed in TS38.843 as well.

Observation 2: In case of AI/ML model with label (classifier network) output, measurement error may have less impact to the prediction than the AI/ML model with RSRP output, if the reporting of the UE only relies on the output of AI/ML model without explicit or implicit assistance of measurement result.

Observation 3: UE always knows the Rx beam paired with the predicted Tx beam provided the predicted Tx beam is detectable in training phase and inference phase.

Observation 4: In inference phase, UE is able to retain all information relevant to Rx beam (including the association between Tx beam and Rx beam), which is learned from training phase.

Observation 5: UE needs not know the Rx beam associated with a Tx beam, if the Tx beam belongs to the below two cases:

- Case Set A: the UE has not detected/measured the Tx beam in train phase.

- Case Set B: the Tx beam is not in Set B.

Proposal 1: Absolute RSRP accuracy= predicted L1-RSRP of beam index i – ideal measured L1-RSRP of beam index i. i may be any beam index in top-K predicted beams.

Proposal 2: The definition of relative RSRP accuracy shall be:

- Relative RSRP accuracy= predicted L1-RSRP of beam index i – predicted L1-RSRP of beam index n, where the beam index n owns the largest value (including predicted and measured beams), if beam index n is not configured for corresponding measurement.

- Relative RSRP accuracy= predicted L1-RSRP of beam index i – measured L1-RSRP of beam index n, where the beam index n owns the largest value (including predicted and measured beams), if beam index n is configured for corresponding measurement.

Proposal 3: Measurement error shall be evaluated with respect to the type of output of AI/ML model, e.g., label (classifier network) or RSRP (CNN or MLP).

Proposal 4: Measurement error impact to prediction shall be evaluated for BM-case 1 and BM-case 2 respectively.

Proposal 5: Measurement error impact to prediction shall be verified (e.g. by simulation) and the same methodology in Rel-18 RAN1 studies can be referred with the consideration of to guarantee an acceptable prediction performance.

Proposal 6: RAN4 to consider the L1-RSRP values for generated channel as the metric in the link-level simulations.

Proposal 7: If tightening measurement error under current side condition is difficult from UE implementation perspective, RAN4 can add another side condition for tightened measurement error for enabling AI-ML model.

Proposal 8: RAN4 to consider the following parameters in Table 1 in the link level evaluation

Table 1 summary of link level evaluation assumptions

| Parameters | Value |
| --- | --- |
| Frequency range | FR2@30GHz; SCS 120kHz |
| Data allocation | 8 RBs First 2 OFDM symbols for PDCCH, and following 12 OFDM symbols for data channel |
| PDCCH decoding | Ideal or non-ideal |
| Channel model | LOS channel: CDL-D extension, DS = 100ns NLOS channel: CDL-A/B/C extension, DS = 100ns |
| The measurement inaccuracy | Zero mean Gaussian distribution with 95% of density be within (-6.5, 6.5) dB |
| Impairment types affecting set B measurements and set A measurements | 4dB RF impairments with normal distribution, 1dB fading condition RF impairment model is composed of different elements, including I/Q imbalance, Quantization noise, Phase noise, Filters/Ripple noise, RF PA distortion noise |

Proposal 9: RAN4 to consider the following parameters in Table 2 in the system level evaluation

Table 2 Summary of system level evaluation assumptions

| Parameters | Value |
| --- | --- |
| Frequency range | FR2@30GHz; SCS 120kHz |
| Deployment | 200m ISD, 2-tier model with wrap-around (7 sites, 3 sectors/cells per site) |
| Scenario | Dense urban (macro-layer only) |
| System level channel model | UMa with distance-dependent LoS probability function defined in Table 7.4.2-1 in TR 38.901. |
| Number of set -B/ set-A beams, | 8 SSB /32 CSI-RS beams |
| Antenna port layout | Antenna setup and port layouts at gNB: (4, 8, 2, 1, 1, 1, 1), (dV, dH) = (0.5, 0.5) λ Antenna setup and port layouts at UE: (1, 4, 2, 1, 2, 1, 1), 2 panels (left, right) |
| Best Rx beam selection algorithm | Baseline 1: Select the best beam within Set A of beams based on the measurement of all RS resources or all possible beams of beam Set A (exhaustive beam sweeping) Baseline 2: Select the best beam within Set B of beams based on the measurement of all RS resources or all possible beams of beam Set B |
| Transmission Power | Maximum Power and Maximum EIRP for base station and UE as given by corresponding scenario in 38.802 (Table A.2.1-1 and Table A.2.1-2) |
| Traffic Model | Full buffer |

Proposal 10: For prediction-based TCI state switch, not to reuse 1280ms as one of known conditions, instead, we shall take training/ inference phase, or training/ inference validity as benchmark in the known conditions.

Proposal 11: The known conditions of the predicted TCI state shall take below conditions into account:

- The TCI state switch command is received during inference phase of AI/ML mode.

- The predicted beam in Set A is the RS in target TCI state.

- The predicted beam in Set A is detectable during training and inference phase, which implies the beam has been trained successfully and available in inference phase.

Proposal 12: During training phase, measurement reporting on Set A and Set B also may enable the NW to configure a TCI state associated with a RS out of the measured Set A or Set B. However, it is not relevant to AI/ML model. Hence, the known conditions for Set A or Set B during training phase are not in scope.

Proposal 13: During inference phase, it may occur that the TCI state is QCLed with a RS belonging to Set B. The legacy known conditions are applied. Hence, the known conditions for Set B during inference phase are not in scope.

Proposal 14: Metrics/KPIs for beam management requirements/tests are separated in below options:

- In the case of label (beam ID) output type of AI/ML model

  - Beam prediction accuracy shall be fulfilled.

- In the case of label (beam ID) + RSRP output type of AI/ML model

  - Beam prediction accuracy and RSRP accuracy simultaneously shall be fulfilled.

Proposal 15: For BM-Case 1, if a beam in Set A is configured for being measured as well, then the AI/ML model may produce the measured RSRP instead of the predicted RSRP. In this case, if beam information and RSRP both are in the content in the report of AI/ML model inference results, the Metrics/KPIs for beam management requirements/tests must take both of the measured RSRP accuracy and beam information into account, where measured RSRP accuracy shall follow the legacy requirements.

# References

- R4-2414323 WF on AI/ML, Qualcomm