3GPP TSG-RAN WG4 Meeting #112bis	R4-2416091

# Hefei, China, October 14th – 18th 2024

Source:	Ericsson

Title:	Discussion on Rel-19 MMSE-IRC BS demodulation requirements

Agenda item:	6.16.3

Document for:	Discussion

# Introduction

In RAN4#112 meeting, companies agreed to reuse LTE signal/interference model and receiver model as baseline for Rel-19 NR MMSE-IRC BS requirement discussion [1].

MMSE-IRC Receiver assumption

Reuse the same receiver model for MMSE-IRC in TR36.884.

Baseline receiver assumption (for performance gain study only)

MMSE receiver as defined in TR36.884.

Signaling model

Reuse the same signaling model in TR36.884 with agnostic to the waveform.

In this contribution, the open issues like interference profile definition, link level simulation configurations will be furtherly discussed.

# Discussion

## 2.1	General

Test scenarios

- Proposals:

  - Proposal 1: Cover HomNet and HetNet scenarios, i.e., DIP set1 and set2 in TS36.104

  - Proposal 2: Consider the scenario with different TDD patterns between BS and discuss how to model the corresponding interference profile

Network type

- Only synchronous scenario

There is an open issue on test scenario for NR MMSE-IRC BS requirements. In corresponding LTE BS requirements, both HomNet and HetNet are considered and also both the synchronization and asynchronization networks are considered. But in Rel-19 WID, it clearly limit the scope to synchronization network deployment only. It is strange to discuss asynchronization network scenario in RAN4 meeting. Furthermore, there is agreement in interference model discussion that only synchronous scenario is considered. In that case, it is not proper to consider network deployment with different TDD pattern in Rel-19 discussion.

Proposal 1	Don’t consider network deployment with different TDD patterns for Rel-19 NR MMSE-IRC BS demodulation requirements.

As for HomNet and HetNet, the interference profile could be different and HetNet could have higher level for the strongest interferer but lower level for the 2nd strongest interferer. Considering both deployment scenarios can keep a good requirement coverage, and extra test effort can be avoided by introducing proper applicability rules. In LTE requirements, the HomNet requirements won’t be applied to Local Area BS and Home BS by adding a note in the requirement table. In NR requirements, the similar approach could be followed but the applicability rule could depend on manufactory declaration on BS class (i.e. D.2 in 38.141-1 Table 4.6).

Proposal 2	Consider both HomNet and HetNet deployment requirements and introduce applicability rules based on BS class (i.e. D.2 in 38.141-1 Table 4.6).

## 2.2	Interference profile

Interference modelling criteria

- Candidate options on interference modelling criteria:

  - Option 1: Use INR-based modelling + SNR-based requirement definition criteria and transfer the DIP value into INR value

  - Option 2: DIP-based modelling

- Candidate options on DIP definition:

  - Option 1: Reuse definition of DIP in sub-clause B.6.1 of TS 36.104

As for modeling criteria selection between DIP and INR, the INR definition would be easier for link level simulation. The DIP method could be better to compare system level simulation results due to no changing of DIPx even the number of interferers is changed.

It is also possible to convert DIP value to INR value based on proper assumptions. In NR UE side MMSE-IRC demodulation requirement definition, INR values are used rather than DIP values used in LTE requirements. Following sections will deliver a mathematical derivation for conversion.

The DIPx is defined by the ratio of the power of xth strongest interference to the power of all interference and noise. If we assume there are NUE in total UE transmitting simultaneously and the received signal power of wanted UE is I0,  I1, I2, …, INUE are interferences. The thermal noise power of a certain channel bandwidth for all UE is the same as σ2. Then,

DIPi=IiIoc, where Ioc=i=1NUE-1Ii+σ2 	       	             (2.2-1)

The INR model is also widely used for interference level definition.

INRi=j=1NRxIor(i)(j)j=1NRxNoc(j) 	 	                                         (2.2-2)

Where, Ior(i)(j) is the average received power spectral density from the i-th strongest interfering cell involved in the requirement scenario on the j-th antenna connector and Noc(j) is the average power spectral density of a white noise source on the j-th antenna connector. In 3GPP NR MMSE-IRC PDSCH inter-cell interference requirements, INR is used for interference model definition.

If we assume interference and wanted signal have same channel bandwidth, we compare (2.2-1 and 2.2-2) and we can have Ii=j=1NRxIor(i)(j) and σ2= j=1NRxNoc(j), then we can have

DIPi=Iiσ2Iocσ2=INRij=1NUE-1Ijσ2+1=INRij=1NUE-1INRj+1    	              (2.2-3)

On the other direction, we can also express INR by DIP by evaluation on (2.2-3). By cumulate all UEs’ DIPx, we could have:

j=1NUE-1DIPj=j=1NUE-1INRjj=1NUE-1INRj+1=1-1j=1NUE-1INRj+1                        (2.2-4)

Compare (2.2-3) and (2.2-4), removing  j=1NUE-1INRj and align the indicator in accumulation, we can get:

INRi=DIPi*(i=1NUE-1INRi+1)=DIPi1-i=1NUE-1DIPi                       (2.2-5)

Based on (2.2-5), we can get INR values from DIP values under different interferer scenarios.

- For 1 interferer only scenario, INR1=DIP11-DIP1

- For 2 interferers scenario, INR1=DIP11-DIP1-DIP2,  INR2=DIP21-DIP1-DIP2

DIP interference definition is better for system level simulation result comparison and interference profile derivation for different number of interferer cases.

NR UE MMSE-IRC demodulation requirements are defined based on INR values rather than DIP values used in corresponding LTE requirements.

It is possible to convert DIP values to INR values based on different number of interferers.

Proposal 3 	Use INR values for NR BS MMSE-IRC demodulation requirement definition.

Proposal 4	Use following formula to convert DIP values to INR values.

INRi=DIPi1-i=1NUE-1DIPi, where NUE is the total number of interferers.

Interference profile

- Candidate options on interference modelling:

  - Option 1: Reuse the same interference modelling as captured in B.6.2 in TS36.104, to transmit random 16QAM symbol for the neighbour PUSCH

  - Other options not precluded

- On interference power level:

  - Reuse the LTE interference power level as a start point, and check performance gain of MMSE-IRC over baseline MMSE.

  - It is not precluded for companies to bring system level simulation on other profiles.

  - FFS how to account for HPUE in existing LTE profiles.

Number of interferers

- Candidate options:

  - Option 1: Consider 1 interferer case for 2 Rx antenna and 2 interferers case for 4/8 Rx antenna

  - Other options not precluded

Regarding the large effort of system level simulations, companies seem prefer not to run system simulations but reuse output from LTE system simulations. One of the concerns is about the interference caused by HPUE deployed in NR network.

Table 2.2-1 Simulation results summary

| Scenario | Tx/Rx, MCS | SCS, CBW | (INR1, INR2) [dB] | IRC [dB] | MRC [dB] | Gain [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| HetNet 1 interferer | 1T/2R MCS 2 | 15k, 5M | (9.83, NA) | -1.19 | 2.53 | 3.72 |
|  |  | 15k, 50M |  | -1.47 | 2.05 | 3.52 |
|  |  | 30k, 10M |  | -0.83 | 2.55 | 3.38 |
|  |  | 30k, 100M |  | -1.48 | 1.95 | 3.43 |
|  | 1T/4R MCS 16 | 15k, 5M |  | 5.73 | 11.13 | 5.4 |
|  |  | 15k, 50M |  | 6.3 | 11.57 | 5.27 |
|  |  | 30k, 10M |  | 6.24 | 11.39 | 5.15 |
|  |  | 30k, 100M |  | 6.39 | 11.45 | 5.06 |
|  | 1T/8R MCS 20 | 15k, 5M |  | 5.56 | 10.6 | 5.04 |
|  |  | 15k, 50M |  | 5.86 | 11.05 | 5.15 |
|  |  | 30k, 10M |  | 5.75 | 10.61 | 4.86 |
|  |  | 30k, 100M |  | 6.26 | 11.08 | 4.82 |
| HetNet 2 interferers | 1T/2R MCS 2 | 15k, 5M | (12.38, -0.97) | 1.17 | 5.09 | 3.92 |
|  |  | 15k, 50M |  | 0.9 | 4.59 | 3.49 |
|  |  | 30k, 10M |  | 1.60 | 5.06 | 3.46 |
|  |  | 30k, 100M |  | 0.93 | 4.48 | 3.53 |
|  | 1T/4R MCS 16 | 15k, 5M |  | 7.55 | 13.65 | 6.1 |
|  |  | 15k, 50M |  | 8.27 | 14.13 | 5.86 |
|  |  | 30k, 10M |  | 8.13 | 13.94 | 5.81 |
|  |  | 30k, 100M |  | 8.35 | 14.0 | 5.65 |
|  | 1T/8R MCS 20 | 15k, 5M |  | 6.67 | 13.17 | 6.5 |
|  |  | 15k, 50M |  | 7.08 | 13.56 | 6.48 |
|  |  | 30k, 10M |  | 7.0 | 13.14 | 6.14 |
|  |  | 30k, 100M |  | 7.47 | 13.57 | 6.1 |
| HomNet 1 interferer | 1T/2R MCS 2 | 15k, 5M | (5.36, NA) | -1.77 | 0.02 | 1.79 |
|  |  | 15k, 50M |  | -2.31 | -0.53 | 2.84 |
|  |  | 30k, 10M |  | -1.56 | 0.05 | 1.61 |
|  |  | 30k, 100M |  | -2.37 | -0.59 | 1.78 |
|  | 1T/4R MCS 16 | 15k, 5M |  | 6.39 | 9.02 | 2.63 |
|  |  | 15k, 50M |  | 6.44 | 9.11 | 2.67 |
|  |  | 30k, 10M |  | 6.54 | 9.13 | 2.59 |
|  |  | 30k, 100M |  | 6.33 | 8.89 | 2.56 |
|  | 1T/8R MCS 20 | 15k, 5M |  | 6.08 | 8.43 | 2.35 |
|  |  | 15k, 50M |  | 6.23 | 8.67 | 2.44 |
|  |  | 30k, 10M |  | 6.11 | 8.43 | 2.32 |
|  |  | 30k, 100M |  | 6.32 | 8.64 | 2.32 |
| HomNet 2 interferers | 1T/2R MCS 2 | 15k, 5M | (7.29, -2.5) | -0.01 | 1.93 | 1.94 |
|  |  | 15k, 50M |  | -0.54 | 1.38 | 1.92 |
|  |  | 30k, 10M |  | 0.27 | 2.02 | 1.75 |
|  |  | 30k, 100M |  | -0.60 | 1.31 | 1.91 |
|  | 1T/4R MCS 16 | 15k, 5M |  | 7.84 | 10.97 | 3.13 |
|  |  | 15k, 50M |  | 7.95 | 11.02 | 3.07 |
|  |  | 30k, 10M |  | 8.0 | 11.07 | 3.07 |
|  |  | 30k, 100M |  | 7.78 | 10.8 | 3.02 |
|  | 1T/8R MCS 20 | 15k, 5M |  | 7.0 | 10.29 | 3.29 |
|  |  | 15k, 50M |  | 7.23 | 10.55 | 3.32 |
|  |  | 30k, 10M |  | 7.06 | 10.34 | 3.28 |
|  |  | 30k, 100M |  | 7.29 | 10.54 | 3.25 |
| HetNet 1 interferer | 1T/2R MCS 2 | 15k, 5M | (16.83, NA) | 1.84 | 8.5 | 6.66 |
|  | 1T/4R MCS 16 | 15k, 5M |  | 7.68 | 17.16 | 9.48 |
|  | 1T/8R MCS 20 | 15k, 5M |  | 6.54 | 16.38 | 9.83 |
| HetNet 2 interferer | 1T/2R MCS 2 | 15k, 5M | (19.38, -0.97) | 4.37 | 11.0 | 6.63 |
|  | 1T/4R MCS 16 | 15k, 5M |  | 9.88 | 19.69 | 9.81 |
|  | 1T/8R MCS 20 | 15k, 5M |  | 7.85 | 18.94 | 11.09 |

According to our simulations [2] and results summary in Table 2.2-1, the performance gain of IRC to MRC is around 2~3dB for HomNet scenario and 3~6dB for HetNet scenario with LTE interference profile. From the receiver algorithm perspective, the LTE interference profile could be enough to check the MMSE-IRC implementation comparing to baseline MMSE receiver.

LTE interference profile can give gain of IRC to MRC as 2dB for HomNet scenario and 3~4dB for HetNet scenario.

As for HPUE deployment, not all UEs are HPUE and only small partial of UE could transmit higher power during limited time. In that case, even we consider some HPUE (i.e., UE with 26dBm or even 30dBm) on top of the LTE system simulation assumptions, it would be very low possibility to have 3dB or 7dB higher value for the strongest interferer (DIP1) than LTE profile.

It would low possibility to have 3dB higher DIP from PC2 UE or 7dB higher DIP from PC1 UE under the similar assumptions as LTE system level simulations.

From the algorithm perspective, increasing interference power of course will give receiver a more challenging condition. To verify this, we run some simulations with 7dB higher INR1 value for HetNet deployment to check the PC1 UE impact under an extremely worse condition, see Table 2.2-13 and 2.2-14 in [2]. Comparing to legacy INR1 value results in Table 2.2-1 and 2.2-2, we can have following observations:

- For both 1 interferer and 2 interferers cases, the degradation of IRC performance are 3dB for 2Rx, 2dB for 4Rx and <1dB for 8Rx. The degradation of MRC performance is 6dB for 2/4/8 Rx.

- For the gain of IRC to MRC, HPUE cases will have more gain due to MRC performance will be much degraded.

All of these observations are not surprising and aligned with the expectation. Note that, this is an extreme condition, and it is not typical in real network. According to these simulations, if 3dB higher INR1 is consider, the degradation of IRC and MRC will be smaller, and the benefit of considering extra case is limited.

Proposal 5	Reuse LTE interference profile for NR MMSE-IRC BS demodulation requirements.

The number of interferer could use same configuration as LTE requirements because the 2nd strongest interferer DIP/INR value is much smaller than the 1st strongest interferer based on system simulation. In that case, the 3rd strongest interferer power level would be even smaller than the 2nd one which would not have much impact on the performance. Thus, it is enough to use up to 2 interferers for the requirements. The 2 interferers scenario can be used for 4/8 Rx antenna configuration which is the typical implementation in real network.

Proposal 6	Define NR MMSE-IRC BS demodulation requirements for both 1 interferer case for 2 Rx antenna and 2 interferers case for 4/8 Rx antenna.

## 2.3	IRC simulation assumptions

For the simulation assumptions, there are some candidate options are captured in the WF as proposed by companies. In this chapter, detailed analysis is delivered.

Test scope

- Candidate options:

  - Option 1: Cover the following SCS and TDD patterns

- 15kHz SCS FDD and TDD with pattern 3D1S1U, S=10D:2G:2U

- 30 kHz SCS TDD with pattern 7D1S2U, S=6D:4G:4U

  - Other options not precluded

Channel bandwidth

- Candidate options for CP-OFDM:

  - Option 1:

- For 15kHz SCS, cover 5MHz, 10MHz, 20MHz

- For 30kHz SCS, cover 10MHz, 20MHz, 40MHz, 100MHz

  - Option 2: 10MHz/15kHz and 40MHz/30kHz for initial simulation purpose

  - Option 3: Minimum channel bandwidth for each SCS, i.e., 5MHz for 15kHz SCS and 10MHz for 30kHz

  - Option 4: Minimum/Maximum channel bandwidth for each SCS, i.e., 5/50MHz for 15kHz SCS and 10/100MHz for 30kHz

- Candidate options for DFT-s-OFDM:

  - Option 1:

- For 15kHz SCS, 5MHz

- For 30kHz SCS, 10MHz

The FDD and TDD pattern could directly use typical configuration as normal PUSCH requirement and the final IRC requirements could be applied for different TDD pattern configurations.

For channel bandwidth per SCS, the minimum channel bandwidth should be considered as high priority regarding existing applicability rule. As for larger channel bandwidth, the key issue is the performance difference from the minimum bandwidth. According to our simulation results [2], the both IRC and MRC performance difference between channel bandwidth is <1dB. Thus, we propose only consider the minimum channel bandwidth per SCS.

The performance difference between channel bandwidth is <1dB.

Proposal 7	Only consider minimum channel bandwidth per SCS for MMSE-IRC PUSCH requirements.

Antenna configuration

- Candidate options:

  - Option 1: Cover 2/4/8 Rx with 1Tx

  - Option 2: Consider only 4RX and 8RX gNB

  - Option 3: Cover 2/4/8 Rx with 1/2/4 Tx

  - Option 4: Prioritize 4Rx case especially in the case of 2 explicit interferers

  - Option 5: Prioritize 1Tx

In NR BS demodulation requirements, different Tx antenna configurations means corresponding transmission layer numbers. For inter-cell interference cancelation, the 1 layer transmission should be typical scenario. The received SINR threshold of scheduling 2 or 4 layers transmission for target UE should be much higher than 1 layer case otherwise increasing MCS could be more reliable than increase layers. In that case, LTE interference profile can’t be directly applied for 2/4 layers cases and new interference profile should be evaluated. The corresponding INRx is expected lower than 1 layer condition. From the checking receiver algorithm perspective, 1 layer case could be the worst case and no need to consider 2/4 Tx cases.

As for Rx antenna configuration, 2/4/8 could be considered for the requirements to have good coverage for different BS antenna implementations. In that case, existing applicability rule for antenna configuration can be reused.

The interference power level in 2/4 Tx cases could be much lower than 1 Tx case due to typical scheduling mechanism.

Including 2/4/8 Rx antenna requirements could give better test coverage and use existing applicability rule on different antenna configurations.

Proposal 8	Only consider 1 Tx with 2/4/8 Rx test cases for NR MMSE-IRC BS demodulation requirements.

Propagation condition

- Candidate options:

  - Option 1:

- For the serving PUSCH, use TDLA30-10 and TDLC300-100 propagation channel model for HetNet and HomNet respectively

- Introduce new propagation channel for testing with desired delay spread 1000ns for the interference PUSCH, i.e., TDLC1000.

  - Option 2: Select among legacy TDLA30, TDLB100, TDLC300

- Option 2A:

- Cover TDLA30-10, TDLB100-400 and TDLC300-100

- Same propagation condition for serving and interference PUSCH

- Option 2B:

- For the serving PUSCH, use TDLA30-10 for HetNet and TDLB100-100 for HomNet

- For the interference PUSCH, use TDLC300-100

- Option 2C:

- Cover TDLB100-400, TDLC300-100 and TDLA30-10 low for study purpose, select one for requirement definition

- For the interference PUSCH, use TDLC300-100

For serving UE channel condition, all companies agree to use legacy channel models but how to consider channel model for interferer is quite controversial.

Regarding the typical scenario of inter-cell interference, the delay for the interfering UE should be longer than serving UE and the interfering UE could be close to the victim cell edge to deliver higher interference. It would be better to use different channel models for serving UE and interfering UE to estimate the real network condition. But it seems no necessary to consider a new channel model with very large delay spread for interferers because the interfering signal won’t be demodulated and irrelative to serving signal.

In LTE MMSE-IRC BS demodulation requirements, two pairs of channel models are considered for serving UE and interferer UE under HomNet and HetNet deployment separately, see table 2.3-1 below. No new channel model is introduced for IRC requirements specially.

Table 2.3-1 Channel model in LTE MMSE-IRC PUSCH requirement

| Deployment scenario | MCS | Propagation condition (Serving, interferers) | Antenna configuration for serving and interferers |
| --- | --- | --- | --- |
| Heterogenous | 6 | (EPA5, ETU5) | 1x2 Low |
|  | 15 | (EPA5, ETU5) | 1x4 Low |
|  | 20 | (EPA5, ETU5) | 1x8 Low |
| Homogenous | 6 | (EVA70, ETU70) | 1x2 Low |
|  | 15 | (EVA70, ETU70) | 1x4 Low |
|  | 20 | (EVA70, ETU70) | 1x8 Low |

For NR, similar logic could be reused that take different channel models for serving UE and interferers, but interferers should have larger delay spread. As for Doppler, HetNet should consider low Doppler configuration due to small cell range while HomNet should consider relative larger Doppler. To simplify the simulation assumptions, same Doppler value could be configured for both serving UE and interferer as LTE requirements.

MCS

- Candidate options:

  - Option 1: Cover QPSK/16QAM/64QAM

- Option 1A: Use MCS 5(QPSK, R=379/1024), MCS 13(16QAM, R=490/1024) and MCS 24(64QAM, R=772/1024)

- Option 1B: MCS index 2, 12, 20 in MCS table 1

- Option 1C: Cover MCS 2, 16, 20

The modulation of the wanted signal could be different in real network. Regarding the typical inter-cell interference scenario, the target SNR should not be very high. To have a better test coverage, one MCS per modulation level could be considered. Furthermore, the modulation level could be limited to 64QAM to cover typical interference limit scenarios. UL 256QAM transmission would happen in the cell center with very good channel condition and its inter-cell interference should be very small.

The specific MCS could start from typical values used in normal PUSCH demodulation requirements, e.g., MCS index 2, 16, 20 in MCS table 1 for QPSK, 16QAM and 64QAM separately. Furthermore, different MCS could be applied to different number of Rx antenna to have feasible target SNR and good test coverage.   Lower MCS could be also considered if simulation results show feasibility issue.

Proposal 8	Take MCS index 2, 16, 20 in MCS table for QPSK, 16QAM and 64QAM simulations separately.

The table 2.3-2 capture some possible channel model configuration based on summary above and could be the start point for further discussion.

Table 2.3-2 Channel model for NR MMSE-IRC PUSCH requirement

| Deployment scenario | MCS | Propagation condition (Serving, interferers) | Antenna configuration for serving and interferers |
| --- | --- | --- | --- |
| Heterogenous | 2 | (TDLA30-10, TDLC300-10) | 1x2 Low |
|  | 16 | (TDLA30-10, TDLC300-10) | 1x4 Low |
|  | 20 | (TDLA30-10, TDLC300-10) | 1x8 Low |
| Homogenous | 2 | (TDLB100-100, TDLC300-100) | 1x2 Low |
|  | 16 | (TDLB100-100, TDLC300-100) | 1x4 Low |
|  | 20 | (TDLB100-100, TDLC300-100) | 1x8 Low |

Proposal 9	Use channel model configuration in Table 2.3-2 for serving signal and interferer for NR MMSE-IRC BS demodulation requirements.

Waveform

- Candidate options:

  - Option 1: Cover both CP-OFDM and DFT-s-OFDM waveform

  - Option 2: Cover CP-OFDM only

From receiver IRC algorithm perspective, there should be no much performance difference between CP-OFDM and DFT-s-OFDM. Only one waveform type could be enough for the requirement. The DFT-s-OFDM implementation have been tested by normal PUSCH requirements, so it seems no need to double check it again in IRC requirements.

Proposal 10 	Only consider CP-OFDM for NR MMSE-IRC PUSCH requirements.

# Conclusions

In the previous sections we made the following observations:

Observation 1	DIP interference definition is better for system level simulation result comparison and interference profile derivation for different number of interferer cases.

Observation 2	NR UE MMSE-IRC demodulation requirements are defined based on INR values rather than DIP values used in corresponding LTE requirements.

Observation 3	It is possible to convert DIP values to INR values based on different number of interferers.

Observation 4	LTE interference profile can give gain of IRC to MRC as 2dB for HomNet scenario and 3~4dB for HetNet scenario.

Observation 5	It would low possibility to have 3dB higher DIP from PC2 UE or 7dB higher DIP from PC1 UE under the similar assumptions as LTE system level simulations.

Observation 6	The performance difference between channel bandwidth is <1dB.

Observation 7	The interference power level in 2/4 Tx cases could be much lower than 1 Tx case due to typical scheduling mechanism.

Observation 8	Including 2/4/8 Rx antenna requirements could give better test coverage and use existing applicability rule on different antenna configurations.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Don’t consider network deployment with different TDD patterns for Rel-19 NR MMSE-IRC BS demodulation requirements.

Proposal 2	Consider both HomNet and HetNet deployment requirements and introduce applicability rules based on BS class (i.e. D.2 in 38.141-1 Table 4.6).

_Toc178342519

Proposal 3 	Use INR values for NR BS MMSE-IRC demodulation requirement definition.

Proposal 4	Use following formula to convert DIP values to INR values.

INRi=DIPi1-i=1NUE-1DIPi, where NUE is the total number of interferers.

Proposal 5	Reuse LTE interference profile for NR MMSE-IRC BS demodulation requirements.

Proposal 6	Define NR MMSE-IRC BS demodulation requirements for both 1 interferer case for 2 Rx antenna and 2 interferers case for 4/8 Rx antenna.

Proposal 7	Only consider minimum channel bandwidth per SCS for MMSE-IRC PUSCH requirements.

Proposal 8	Only consider 1 Tx with 2/4/8 Rx test cases for NR MMSE-IRC BS demodulation requirements.

Proposal 8	Take MCS index 2, 16, 20 in MCS table for QPSK, 16QAM and 64QAM simulations separately.

Proposal 9	Use channel model configuration in Table 2.3-2 for serving signal and interferer for NR MMSE-IRC BS demodulation requirements.

Proposal 10 	Only consider CP-OFDM for NR MMSE-IRC PUSCH requirements.

# References

[1] 	R4-2413570, ‘WF on NR demodulation performance: Phase 5’, China Telecomm

[2]	R4-2416092, Simulation results for NR MMSE-IRC PUSCH requirements, Ericsson

[3]