3GPP TSG-RAN WG4 Meeting #112bis                                      R4-2416117

Hefei, Anhui, China, 14th – 18th October, 2024

Source:	Samsung

Title:               Discussion on L3 measurement delay by optimizing CSSF

Agenda item:			6.15.2.2

Document for:	Discussion

1 Introduction

In last meeting, RAN4 had some plentiful discussions on FR2-1 L3 measurement delay by optimizing CSSF outside gap in CA/DC. Finally, the UE measurement procedure and applicability requirement for L3 measurement enhancement CSSF were confirmed. In this meeting, we will give some further discussions for some open issues captured in the agreed WF [1].

2 Discussion

2.1 Clarification on the bullets in WID for this CSSF optimization

In RANP #105, the WID was revised with the following clarifications:

<RP #105 RP-242380> - FR2-1 SSB based L3 measurement delay reduction for connected mode - For UE supporting multiple-Rx simultaneous reception on single carrier: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - Rx beam sweeping factor - The CSSF requirement optimization is defined assuming a UE not operating with multiple-Rx based simultaneous L3 measurement: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - CSSF outside gap in CA/DC scenarios - Baseline assumption on number of searchers is 2 - Note: Rel-19 CSSF optimization applies for CA/DC scenario independently of the UE support of multi-Rx capabilities.

The principle of the updated clarifications is fine for us. If we combine them together, from our understanding, the relationship of L3 CSSF and multi-Rx simultaneous reception can be understood as:

Rel-19 CSSF optimization and multi-Rx simultaneous reception enhancement are independent features, while the two features can be supported independently or together. It can say, for feature group definitions, Rel-19 CSSF optimization and multi-Rx simultaneous reception capabilities are independent but can coexist. More specifically,

- It is likely to introduce (new) capabilities/signallings to support Rel-19 multi-RX simultaneous reception and Rel-19 CSSF enhancement features, separately and define new corresponding UE behaviours.

- Enhancements in the Rel-19 CSSF should be independent of the UEs support of the multi-Rx based simultaneous measurement features/capabilities.

  - Dedicate configurations for Rel-19 CSSF and multi-RX reception enhancements are required.

  - No need to take multi-Rx based simultaneous measurement/reception as prerequisite for Rel-19 enhancement to CSSF based L3 measurement delay.

- If the UE is capable of multi-Rx simultaneous reception features/capabilities, the Rel-19 CSSF enhancement requirement is applied only when the conditions to trigger UE using FBS for L3 measurement are not met

In summary, regarding the clarification on the bullets in WID for this CSSF optimization, we have the following views (including the text revision Alt):

Proposal 1: Rel-19 CSSF optimization and multi-Rx simultaneous reception enhancement are independent features, while the two features can be supported independently or together. For feature group definitions, Rel-19 CSSF optimization and multi-Rx simultaneous reception capabilities are independent but can coexist

Proposal 2:

- Rel-19 CSSF optimization applies for CA/DC scenario independently of the UE support of multi-Rx simultaneous reception features/capabilities

- If the UE is capable of multi-Rx simultaneous reception features/capabilities, the Rel-19 CSSF enhancement requirement is applied only when the conditions to trigger UE using FBS for L3 measurement are not met

2.2 Feature capability of L3 measurement delay reduction by optimizing CSSF

Based on the discussions in issue 2.1.1, from our understanding, to make everything clear and simple, a reasonable way is to define a new individual feature for CSSF optimization in Rel-19, and a new capability is required to support the feature, accordingly. And, we also agree that it is too early to discuss the details.

Proposal 3:

- Define a new capability to support the new Rel-19 CSSF enhancement feature.

- Delay the capability discussion to the end of the core part

2.3 Searcher assumption to apply L3 measurement delay reduction by optimizing CSSF

- When multiple CCs are configured, how many CCs the UE can measure simultaneously, it is a main question of buffering capability and the amount of searchers supported in UE. No matter in NR-CA or EN-DC, 2 searchers are assumed

- When measuring multiple carriers (intra-/inter-frequency), especially when the number of CCs increases, based on the searcher limitation, in order to share the searcher (s) among the carriers, the scaling is needed, that is the rationale of CSSF from our understanding. The existing CSSF scaling factors are defined based on the necessity and the priority of the serving carriers, it can control the measurement opportunity, and which also impacts how frequently the to-be SCC would be measured

- Now, from the very beginning, if the UE has 2 searchers and is measuring two carriers, the scaling however not be necessary in this case, and the two carriers can be measured simultaneously. While, when the UE is measuring more than 2 carriers (depends on UE’s strong measurement capability), the additional processing capability is needed, and the scaling related to Ncc and the amount of searchers shall be considered, the measurement delays grow relatively long, accordingly.

- Of course, it is very likely that UEs will be provisioned with more searcher hardware, e.g., the searcher is from 2 to 3, it provides a possibility that UE can measure 3 carriers in parallel without any scaling introduced, and the measurement delay can be reduced compared to the case when searcher = 2 at least Ncc=3. But in this case, the searcher occupancy/sharing ratio among PCC/PSCC/SCC shall be re-defined based on the assumption of 3 searchers for all the cases, and the computation equations/rules on how to scale the measurement period might be significantly changed.

Observation 1: 3 searchers provide a possibility that UE can measure 3 carriers in parallel without any scaling unnecessarily done.

Observation 2: When 3 searchers are considered, the searcher occupancy/sharing ratio among PCC/PSCC/SCC shall be re-defined based on the assumption of 3 searchers for all the cases, and the rules on how to scale the measurement period might be significantly changed

And follow the WID,

Proposal 4: Prefer that RAN4 only consider the enhancement based on 2 searchers, i.e., same as previous release, for L3 measurement delay reduction by optimizing CSSF at current stage.

2.4 Solutions to apply/specify L3 measurement delay reduction by optimizing CSSF outside gap in CA/DC

1. Optimize NSCC_SSB for FR2 intra-band CA

For FR2 intra-band CA (collocated), from measurement perspective, UE only needs to measure the serving SCell in the same band as PCell according to the existing principle specified in 9.2.3.2

- 9.2.3.2 Requirements for FR2 [omit] where this single intra-frequency layer shall be: - PCC when UE is configured with SA NR operation mode with PCC in the band; or ..[omit] The UE shall also be capable of performing SS-RSRP, SS-RSRQ, and SS-SINR measurements for at least 2 SSBs on serving cell for each of the other intra-frequency layer(s) in the same band

In this sense, theoretically, there is no need to define the scaling factor for SCC(s) as. e.g., 2×Number of configured SCell(s), shown below

Proposal 5: It is possible for RAN4 to discuss the method of reducing NSCC_SSB to optimize the CSSF for FR2 intra-band CA

2. Optimize searcher occupancy/sharing ratio among PCC/PSCC/SCC.

From our understanding, it might be a possible way, however, based on the previous discussions, the existing CSSF scaling factors are defined based on the necessity and the priority of the serving carriers. For example, in Rel-15, based on 2 searchers assumption, for CA mode:

- One searcher is dedicated for PCC, the other searcher is shared by all the SCCs.

  - Half of the other searcher (50%) is dedicated to SCC where neighbour cell measurement is required, which is prioritized than other SCC(s)

  - The remaining half of the searcher is shared among FR1 SCCs and FR2 SCCs where neighbour cell measurement is not required.

From this, if RAN4 to optimize the CSSF by changing the searcher sharing ratio among all serving carriers, the condition and the scenario should be confirmed. Besides, evaluation on whether the change will bring extra complexity at UE side is also needed.

Proposal 6: If RAN4 agree to take: Optimize the CSSF by changing the searcher sharing ratio among all serving carriers as the solution to specify L3 measurement delay reduction

- RAN4 should to discuss the corresponding applicable condition and the scenario

3 Conclusion

In this contribution, we provided our viewpoints on L3 measurement delay by optimizing CSSF. The following observations and proposals are obtained:

Observation 1: 3 searchers provide a possibility that UE can measure 3 carriers in parallel without any scaling unnecessarily done.

Observation 2: When 3 searchers are considered, the searcher occupancy/sharing ratio among PCC/PSCC/SCC shall be re-defined based on the assumption of 3 searchers for all the cases, and the rules on how to scale the measurement period might be significantly changed

Proposal 1: Rel-19 CSSF optimization and multi-Rx simultaneous reception enhancement are independent features, while the two features can be supported independently or together. For feature group definitions, Rel-19 CSSF optimization and multi-Rx simultaneous reception capabilities are independent but can coexist

Proposal 2:

- Rel-19 CSSF optimization applies for CA/DC scenario independently of the UE support of multi-Rx simultaneous reception features/capabilities

- If the UE is capable of multi-Rx simultaneous reception features/capabilities, the Rel-19 CSSF enhancement requirement is applied only when the conditions to trigger UE using FBS for L3 measurement are not met

Proposal 3:

- Define a new capability to support the new Rel-19 CSSF enhancement feature.

- Delay the capability discussion to the end of the core part

Proposal 4: Prefer that RAN4 only consider the enhancement based on 2 searchers, i.e., same as previous release, for L3 measurement delay reduction by optimizing CSSF at current stage.

Proposal 5: It is possible for RAN4 to discuss the method of reducing NSCC_SSB to optimize the CSSF for FR2 intra-band CA

Proposal 6: If RAN4 agree to take: Optimize the CSSF by changing the searcher sharing ratio among all serving carriers as the solution to specify L3 measurement delay reduction

- RAN4 should to discuss the corresponding applicable condition and the scenario

4 References

[1] R4-2414026, “WF for NR_RRM_Ph5_Part1”, Apple