# 3GPP TSG-RAN WG4 Meeting #112bis	R4-2416122

# Hefei, Anhui, China, 14th – 18th October, 2024

Title: 	On RRM requirements of SSB adaptation

Source: 	OPPO

Agenda item:	6.22.2.2

Document for:	Discussion

Introduction

In this contribution, we will provide discussion on RRM requirements for SSB adaptation which is with high priority as agreed in last meeting.

Discussion

### Sub-topic 3-1: SSB Adaptation requirements Issue 3-1-1: SSB adaptation in IDLE/CONNECTED mode - Agreement - RAN4 to prioritize the SSB adaptation discussion in CONNECTED mode. - Whether to discuss SSB adaptation in IDLE mode depends on RAN1 progress Issue 3-1-2: SSB adaptation impact in RRM requirement - Agreement: SSB adaptation impact in RRM requirement - RAN4 to discuss the L1/L3 measurement requirement impact as starting point. - Wayforward: - RAN4 to discuss SSB periodicity adaptation in time-domain as starting point. - Other SSB adaptation mechanism (s) and requirements are not precluded depending on RAN1 progress.

RAN4 has agreed to discuss L1/L3 measurement requirement impact as starting point. There are different kinds of measurement to be considered by Rel-18, including L1 serving cell measurement, L1 neighbor cell measurement and L3 neighbor cell measurement.

For L1 measurement on serving cells (e.g., RLM, BFD, CBD and L1-RSRP measurement), the legacy assumption can be reused, e.g., measurements outside gap, the time domain overlapping between L1 and L3 measurement needs to be considered with scaling factor P. Especially for FR2, the SSB/CSI-RS resource, SMTC occasion and GAP instance need to be considered in the overlapping cases. L1 measurement needs to avoid the colliding with SMTC.

In our view, L1 measurement on serving cells can be enhanced due to SSB adaptation. TSSB or Max(TDRX,TSSB) can be reused. And for deactivated PSCell, measCyclePSCell or Max(TDRX, measCyclePSCell) can be reused. We don’t pursue SMTC-based measurement before SCell activation.

Observation 1: For the impact of SSB adaptation on L1 measurement on serving cells, SSB periodicity can be reused.

For L3 measurement on neighbor cells (e.g., intra-frequency and inter-frequency L3 measurements), the SMTC is always considered for measurements either with gap or without gap. The sharing factor Kp is also defined as the scaling factor for an SSB frequency layer to be measured without GAP. Kp = Ntotal / Navailable,

Ntotal is the total number of SMTC occasions within the window, including those overlapped with GAP and MUSIM gap occasions within the window, and

Navailable is the number of SMTC occasions that are not overlapped with any non-dropped GAP or non-dropped MUSIM gap occasions within the window W, after accounting for measurement GAP and MUSIM gap collisions by applying the collision rules for GAP and MUSIM gap in section 9.1.8.3, 9.1.10.4, 9.1.10.5, 9.1.12.3, and 9.1.13.3, respectively.

Kp = 1 when Navailable = 0.

The assumption of SMTC based L3 measurements cannot be changed due to SSB adaptation, as all SSBs from neighbor cells are considered.

Observation 2: For the impact of SSB adaptation on L3 measurement on neighbor cells, SMTC periodicity can be used as baseline.

For L1 measurement on neighbor cell (e.g., LTM, and L1-RSRP measurements of configured measurement resources from a cell with different PCI in addition to serving cell), P value for SSB resource to be measured is defined as Ntotal / Noutside_MG in FR1, and 	P = PL1_sharing factor * Psharing factor * Ntotal / Noutside_MG in FR2, if Navailable = 0 or P is defined as below based on P2, where P2 = Ntotal / Navailable in FR2, if Navailable > 0. And the window W of duration max(TL1,  MGRP_max) is introduced.

-	Ntotal is the total number of SSB resource occasions within the window, including those overlapped with measurement gap occasions, MUSIM gap occasions or SMTC occasions within the window, and

-	Noutside_MG is the number of SSB resource occasions that are not overlapped with any non-dropped measurement gap occasion nor non-dropped MUSIM gap occasion within the window W, and

-	Navailable is the number of SSB resource occasions that are not overlapped with any non-dropped measurement gap occasion, non-dropped MUSIM gap occasion nor any SMTC occasion within the window W.

-	an SSB or an SMTC occasion is considered to be overlapped with the MUSIM gap if it overlaps a MUSIM gap occasion.

-	TL1 is periodicity of the target SSB.

The windows W of duration max(TL1,  MGRP_max) have already considered the periodicity of the target SSB.

Observation 3: For the impact of SSB adaptation on L1 measurement on neighbor cells, current time windows W can be reused, which already considers the periodicity of the target SSB.

Even for OD-SSB based SCell activation with SSB adaptation, we don’t pursue SMTC-based L3 measurement for SCell activation, and don’t support L3 measurement after the SCell has been activated, without relying on SMTC. Because the OD-SSB before SCell activation is mainly for fast SCell activation, it is hard to restrict the L3 measurement to SMTC window, considering the SMTC window is more designed for static SSB transmission, while now the OD-SSB can be dynamically (de)activated with various periodicity (FFS other parameters), so it is hard to always restrict the serving cell L3 measurement into a configured SMTC window (with measurement cycle), i.e., cannot wait for a coming SMTC window for L3 measurement, which may further delay the SCell activation. On the other hand, after the SCell has been activated, it seems OK to limit serving cell measurement as intra-frequency layers to the SMTC window as legacy.

Proposal 1: Don’t pursue SMTC-based L3 measurement for SCell activation, and don’t support L3 measurement after the SCell has been activated without relying on SMTC.

In summary, we propose RAN4 not to introduce new SMTC due to SSB adaptation.

Proposal 2: RAN4 not to introduce new SMTC due to SSB adaptation.

RAN1 #116 Agreement For the adaptation mechanisms of SSB in time-domain, study further following mechanisms: - Adaptation mechanism indicated or configured by gNB without UE trigger - Adaptation triggered by UE (if any) FFS: Details of associated signaling/indication/configuration

There may exist network controlled SSB adaptation and UE autonomous SSB adaptation. From our side, we are ok to focus on network controlled SSB adaptation first.
Proposal 3: RAN4 to specify requirements for network-controlled adaptation mechanism.

Conclusions

In this paper we provided our views on the impact on RRM requirements due to SSB adaptation .

Observation 2: For the impact of SSB adaptation on L3 measurement on neighbor cells, SMTC periodicity can be used as baseline.

Observation 2: For the impact of SSB adaptation on L3 measurement on neighbor cells, SMTC periodicity can be used as baseline.

Observation 3: For the impact of SSB adaptation on L1 measurement on neighbor cells, current time windows W can be reused, which already considers the periodicity of the target SSB.

Proposal 1: Don’t pursue SMTC-based L3 measurement for SCell activation, and don’t support L3 measurement after the SCell has been activated without relying on SMTC.

Proposal 2: RAN4 not to introduce new SMTC due to SSB adaptation.

Proposal 3: RAN4 to specify requirements for network-controlled adaptation mechanism.

Reference

- RP-240170, Revised WID for Enhancements of network energy savings for NR

- R4-2413885, WF on Netw_Energy_NR_enh, Ericsson