3GPP TSG-RAN WG4 Meeting #112bis	R4-2416123

# Hefei, Anhui, China, 14th – 18th October, 2024

Agenda item:	6.23.4.2

Source:	OPPO

Title:	Discussion on core requirements for LP-WUS/WUR

Document for:	Approval

- Introduction

In previous meeting, RAN4 discussed and reached some agreements regarding LP-WUS/WUR in [1]. This contribution will provide our further considerations on the open issues captured in [2].

- Discussion

Issue 1-1-2: Cases/states to be considered for RRM relaxation Agreement: Discuss the RAN4 requirements first for the following case #1, and FFS for case #2 to #5. RRM measurement case index; MR serving cell measurement; MR neighboring cell measurement; LR measurement. #1 Fully offloading case; Off; Off: FFS the condition and the details; ON. RAN4 to further discuss case #2 to #4: RRM measurement case index; MR serving cell measurement; MR neighboring cell measurement; LR measurement. #2 Relaxed case a; On with relaxation measurement; Off; ON. #3 Relaxed case b; On with relaxation measurement; On with relaxation measurement; ON. #4 Relaxed case c; Off; On, FFS the condition and the details; ON.

Issue 1-1-1: Cases/states to be considered for RRM relaxation and serving cell measurement offloading Agreement: - Support case #3 - For serving cell measurement, further discuss: - Option 1: Further discuss whether to combine the measurements across the two radios, i.e., MR and WUR - Option 2: From RAN4 requirement perspective, not consider combining the measurements across the two radios, i.e., MR and WUR - Option 3: It is up to UE implementation whether to combine the measurement across the two radios. - Other options are not precluded. RRM measurement case index; MR serving cell measurement; MR neighboring cell measurement; LR measurement. #3 Relaxed case b; On with relaxation measurement; On with relaxation measurement; ON. - Note: In RAN4 understanding, the discussion of the related measurement criteria (i.e., whether to use the legacy or new criteria) is out of RAN4 responsibility. Recommendations: Suggest to discuss issues related to higher priority frequency layers under 1-1-11. Continue discuss case 2 and other issues.

As shown above, 4 cases for RRM relaxation and serving cell measurement offloading were proposed in the previous RAN4 meetings where case #1 and case #3 were agreed. For case #3, UE will perform measurement on serving cell by both MR and LR. Whether the results measured from the two radios could be combined needs further discussion. In our view, it is better to not combine the measurements across the two radios when defining RRM requirements. The structure of target reference signals, minimum measurement samples and calibration margin are quite different for MR and LR and therefore it is not reasonable to combine them together. Besides, it is hard to specify a unified RRM requirements considering a lot of combinations which could be implemented by the UE, e.g. 4 LR samples + 1 MR samples, or 3 LR samples + 2 MR samples. To define RRM requirements in case #3, RAN4 should assume the worst case and only consider the results from the target radio. Of course, it is up to UE implementation to combine the results from the two radios, for example to accelerate period or improve accuracy of LR measurement by utilizing some of measurement results by MR. But the RRM spec should not be impacted by such the UE implementation.

Proposal 1: For case #3, not consider combining the measurements across the two radios when defining RRM requirements.

Another open issue is whether to consider case #2. UE will turn on the MR in both case #2 and case #3 but the measurement purposes are different. Comparing the two cases, we think case #3 is more reasonable. Firstly, if the UE already turns on the MR for serving cell measurement, it could also perform neighboring cell measurement with MR without much extra power consumption. Secondly, working in relaxed cases rather than fully offloading case #1 means that the serving cell quality is not good enough and is likely to further deteriorate, e.g. does not meet the criterion S any longer. To make sure the UE could reselect to a suitable neighboring cell in time, even with equal or lower priority, neighboring cell measurement is necessary. Therefore, we propose to deprioritize case #2.

Proposal 2: Case #2 should be deprioritized.

Issue 1-1-2: Measurement requirements to be specified for LP-WUR - Proposals - P1: RAN4 suspends study on measurement requirements for OFDM based LP-WUR based on LP-SS signal at idle/inactive state until more RAN1 conclusions; RAN4 shall focus on following cases; (Apple Docomo CMCC vivo, ZTE): - Measurement requirements for OOK-based LP-WUR serving cell measurement based on LP-SS at Idle/Inactive state - Measurement requirements for OFDM-based LP-WUR serving cell measurement based on existing PSS/SSS at Idle/Inactive state - P2: RAN4 to define requirements for OFDM-based LR serving cell measurement based on LP-SS. (Huawei) - P3: RAN4 discuss LP-WUR measurement requirements starting from SSS based measurement and wait for more progress from RAN1 for LP-SS based measurement. For SSS based LP-WUR measurement, the existing intra-frequency cell reselection requirements can be used as baseline with the side condition to be further discussed. (CATT) - P4：RAN4 shall define the serving cell measurement requirements based on LP-SS. However, we deem that the time window like SMTC may not be introduced since LP-SS will not perform neighbor cell measurement. (ZTE)

Agreement in RAN1 #116 meeting For the overlaid OFDM sequence(s) for LP-SS, consider the following options for further down-selection: - Option 1: Do not specify the overlaid OFDM sequences(s) - Option 2: Specify the overlaid OFDM sequence(s) targeting for OOK waveform generation without targeting for sync and RRM measurement for OFDM-based LP-WUR using the overlaid sequence of LP-SS. - Option 3: Specify the overlaid OFDM sequence(s) targeting for OOK waveform generation and also targeting for sync and RRM measurement for OFDM-based LP-WUR using the overlaid sequence of LP-SS. - For Option 3, it is up to RAN4 to make decision on whether/how to define the RRM measurement requirement for OFDM-based LP-WUR using the overlaid sequence of LP-SS.

Working Assumption in RAN1 #117 meeting From RAN1 perspective, for the RRM measurement metrics based on SSS for OFDM-based LP-WUR, use the same definition of SS-RSRP and SS-RSRQ for LP-SSS-RSRP and LP-SSS-RSRQ, respectively. - Above is applicable for both time-domain processing or frequency-domain processing - Above does not imply that RAN1 will introduce LP-SSS-RSRP and LP-SSS-RSRQ in the specifications

[H][FL1] Proposal 4.2-1 in RAN1 #118 meeting: Support overlaid OFDM sequence(s) for LP-SS - LP-SS reuse the overlaid OFDM sequence(s) specified for LP-WUS.

Different measurement types could be supported by LP-WUR and whether to define the corresponding RRM requirements for each type needs to be discussed. Comparing P1 and P2 for issue 1-1-2 above, the main different part is which reference signal, LP-SS or the existing PSS/SSS, could be measured for OFDM based LP-WUR. Obviously, OFDM based LP-WUR is able to receive and measure the existing SSS and RAN1 also reached a working assumption to reuse the definition of SS-RSRP and SS-RSRQ. However, whether to specify the overlaid OFDM sequence(s) for LP-SS is still open and therefore whether the OFDM based LP-WUR could perform RRM measurement using the overlaid sequence cannot be determined yet. From this perspective, we prefer P1 and give the following proposal.

Proposal 3: RAN4 suspends study on measurement requirements for OFDM based LP-WUR based on LP-SS signal at idle/inactive state until more RAN1 conclusions; RAN4 shall focus on following cases:

- Measurement requirements for OOK based LP-WUR serving cell measurement based on LP-SS at Idle/Inactive state

- Measurement requirements for OFDM based LP-WUR serving cell measurement based on existing SSS at Idle/Inactive state

Issue 1-1-3: Core requirements to be specified for MR RRM relaxation - Proposals - P1: RAN4 specifies MR relaxation requirements for both serving cell and neighbor cell measurements at idle/inactive mode for UE supporting LP-WUR. (Apple oppo xiaomi Docomo CT LG Huawei, ZTE, CATT) - RAN4 can investigate the mobility performance to quantify the relaxation, e.g., scaling factor for measurement period. (Apple) - If both LR and MR are ON, RAN4 to discuss whether UE uses LR measurement or UE uses both LR and MR measurement to check with the relaxation/offloading/LP-WUS-monitoring criteria (especially in case#3) (Apple). - P2: RAN4 needs to further evaluate serving cell measurement relaxation and neighbouring cell measurement relaxation by MR (Samsung) - P3: Further discuss MR RRM measurement relaxation after Case#2 and Case#3 are agreed by RAN4 or other work groups (CMCC) - P4: When UE stays in MR+LR state, RAN4 shall consider the MR serving cell measurement relaxation and also study the mobility performance to quantify the relaxation such as scaling factor for time period (ZTE) Recommendations: Suggest to agree P1 following the WI, details on requirements are encouraged to be purposed and are FFS

Since RRM relaxation case #3 was agreed, where both serving cell and neighboring cells are expected to be measured by MR in relaxation approach, it is straightforward to define the corresponding RRM requirements and support P1. How to use the relaxed measurement results especially in case #3 (e.g. for cell-reselection, or offloading or LP-WUR monitoring) can be further discussed.

Proposal 4: Specify MR relaxation requirements for both serving cell and neighbor cell measurements in RRC idle/inactive mode.

Issue 1-2-2: Possible interruption related requirements for LP-WUR - Proposals - P1: Interruption requirements could be defined for the time between LP-WUS received paging indication to MR ready for Paging monitoring (Samsung xiaomi Docomo CMCC) - In addition, between LP-WUR measurement result fulfills exit criteria and MR to start measurement (CMCC) - In addition, between MR measurement result fulfills entry criteria and LP-WUR to start measurement. (CMCC ZTE) - P2: RAN4 shall clarify the timeline from decoding LP-WUS to MR wake-up in order to monitor legacy PO. (ZTE)

Agreement in RAN1 #118 meeting For the wake-up delay, consider the following options: Option 2: UE reports one value from X candidate values for the wake-up delay via UE capability reporting. - FFS: X is 2, 3, 4 Above applies for IDLE/INACTIVE mode. Definition of wake-up delay: Minimum gap time between LP-WUS reception and MR to start PDCCH monitoring

Issue 1-2-2 proposed to define the possible interruption between LP-WUS with paging indication and MR ready for paging monitoring. It is reasonable to define such the requirements. In our understanding, this interruption time is the wake-up delay discussed in RAN1, which is also applicable for RRC idle/inactive modes. As shown above, X candidate values will be defined in RAN1 and UE could report one value via capability reporting.

Proposal 5: The interruption time between LP-WUR reception and MR ready for paging monitoring could be defined based on the wake-up delay determined in RAN1.

Issue 1-2-4: LP-WUR operating carrier frequency - Proposals - P1: RAN4 assumed LR and MR are operating on the same carrier frequency as baseline, and FFS on the other scenarios if RAN1 has agreement. (Apple QC)

LS from RAN1, R1-2407559 In RAN1, the common understanding is that UE may not support LP-WUS reception on all the bands supported by the UE. RAN1 respectfully asks RAN2 and RAN4 to check if there is any issue and specification support needed for IDLE/INACTIVE UEs.

Whether to support LR and MR operating on different carriers/bands is discussed in RAN1. Although the common understanding in RAN1 is that UE may not support LP-WUS reception on all the bands, further check by RAN2 and RAN4 is needed [3]. Therefore, we agree to assume LR and MR are operating on the same carrier frequency when defining RRM requirements and further discuss cross-band scenarios after confirmed by RAN2 and RF session.

Proposal 6: Consider LR and MR on the same carrier frequency as the baseline to define RRM requirements.

- Conclusion

This contribution gave the following proposals on the core requirements for LP-WUR/WUS.

Proposal 1: For case #3, not consider combining the measurements across the two radios when defining RRM requirements.

Proposal 2: Case #2 should be deprioritized.

Proposal 3: RAN4 suspends study on measurement requirements for OFDM based LP-WUR based on LP-SS signal at idle/inactive state until more RAN1 conclusions; RAN4 shall focus on following cases:

- Measurement requirements for OOK based LP-WUR serving cell measurement based on LP-SS at Idle/Inactive state

- Measurement requirements for OFDM based LP-WUR serving cell measurement based on existing SSS at Idle/Inactive state

Proposal 4: Specify MR relaxation requirements for both serving cell and neighbor cell measurements in RRC idle/inactive mode.

Proposal 5: The interruption time between LP-WUR reception and MR ready for paging monitoring could be defined based on the wake-up delay determined in RAN1.

Proposal 6: Consider LR and MR on the same carrier frequency as the baseline to define RRM requirements.

References

[1] R4-2413899, WF for RRM core requirements for LP-WUSWUR, vivo, RAN4 #112

[2] R4-2411818, Topic summary for [112][223] NR_LPWUS, vivo, RAN4 #112

[3]R1-2407559, LS on LP-WUS operation in IDLE/INACTIVE mode, RAN1 #118