# 3GPP TSG-RAN WG4 Meeting #112bis	R4-2416124

# Hefei, Anhui, China, 14th – 18th October, 2024

Title: 	Discussion on measurements related enhancements for LTM

Source: 	OPPO

Agenda item:	6.24.2.1

Document for:	Discussion

Introduction

In last meeting, a WF was approved on RRM requirements for R19 mobility.

- Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1] - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] - RAN1 and RAN2 to progress independently on the event triggered measurements objectives of their respective MIMO and Mobility enhancement WIs. Review progress at RAN#105 to see if any modification of objectives is required to avoid/manage any overlap in the work - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management, and/or other necessary physical layer operations on candidate cells before LTM [RAN1] - Specify RRM requirements related to the above objectives as necessary [RAN4]

In this contribution, we provide our views on measurements related enhancements for LTM, including event trigger L1 measurement reporting and CSI-RS based L1 measurement.

Discussion

Event triggered L1 measurement reporting

Issue 2-2-1: RRM scope of Event triggered L1 measurement reporting - Candidate solutions: - Option 1: RAN4 need to discuss the absolute threshold for beam of serving cell and candidate cell in L1 LTM measurement events, including the measurement matrix and the value. RAN4 can wait for more RAN2 input. (CATT) - Option 2: (Apple) - Requirements of Capabilities for Support of Event Triggering and Reporting Criteria specified in TS38.133 section 9.1.4 need to be updated to cover event triggered L1 measurement reporting. Details are FFS. - Existing framework in L3 event triggered reporting RRM requirements can be reused to cover L1 event triggered reporting. - Option 3: define measurement reporting requirements for event triggered L1 measurement reporting, e.g. the L1 measurement reporting delay. (CMCC, Apple, Samsung, HW, ZTE, Nokia, E///, vivo,CTC) - Option 3a: “Event Triggered Reporting” for LTM L1 measurement is to be introduced in section 9.14.3 and section 9.15.3. - Option 3b: RAN4 can start the discussion based on SSB firstly. Further RAN1/RAN2 progress are needed. (Samsung) - Option 3c: RAN4 to define event-triggered L1 reporting delay requirement for SSB based and CSI-RS based measurements. (Nokia, E///, vivo, CMCC) - Option 4: RAN4 should discuss the accuracy requirement for event triggered L1 measurement reporting. (ZTE) - Option 5: RAN4 to discuss potential impact of event-triggered L1 reporting to other LTM requirements, for example (Nokia) - Known TCI state condition for early TCI state activation and cell switch - Applicability of cell switch delay requirements - early DL sync (Tfirst-RS = 0 conditions) - early ASN.1 decoding and validity check - LTM L1 measurement requirements - Option 6: For the topic of ‘Measurements related enhancements for the purpose of supporting LTM,’ RAN4 to wait for further progress to be made in other working groups under this WI . (QC, vivo) - Recommended WF - Continue discussion.

RAN2 has achieved the agreements on events related to LTM. The RRM impact due to event triggered L1 measurement reporting could include the followings:

- Capabilities for Support of Event Triggering and Reporting Criteria

- Event-triggered L1 reporting delay requirement

- The accuracy requirement for event-triggered L1 reporting delay requirement

- Other potential impact of event-triggered L1 reporting to other LTM requirements, e.g., known TCI state condition for early TCI state activation and cell switch

In our view, we can start from the first 2 bullets which received the majority support.

For example, the requirements of “Capabilities for Support of Event Triggering and Reporting Criteria” specified in TS38.133 section 9.1.4 need to be updated to cover event triggered L1 measurement reporting. The “Event Triggered Reporting” can be introduced in section 9.14.3 and section 9.15.3 for event triggered L1 measurement reporting delay requirements as well.

Proposal 1: RAN4 to introduce requirements of “Capabilities for Support of Event Triggering and Reporting Criteria” and “event triggered L1 measurement reporting delay”.

CSI-RS based L1 measurements

RAN4 has agreed to consider the following requirements need to be discussed for CSI-RS based L1 RSRP measurement on LTM candidate cell(s):

-	Measurement delay

-	Measurement reporting requirements

-	Measurement restrictions

-	Scheduling availability

o	FFS on other potential impact.

Issue 2-3-2: definition of intra-frequency and inter-frequency for CSI-RS based L1 measurement - Candidate solutions: - Option 1: following CSI-RS based L3 measurement, (Apple) - A measurement is defined as a CSI-RS based intra-frequency L1 measurement provided that: - the SCS of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the SCS of the CSI-RS resource on the serving cell indicated for L1 measurement, and - the CP type of the CSI-RS resource of neighbour cell configured for L1 measurement is the same as the CP type of the CSI-RS resource of the serving cell indicated for L1 measurement, and - It is applied for SCS = 60KHz - the centre frequency of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the centre frequency of the CSI-RS resource of the serving cell indicated for L1 measurement - Option 2: Categorize CSI-RS based L1-RSRP measurement into CSI-RS based L1-RSRP measurement within active BWP and outside active BWP for further discussion. (MTK) - Option 3: For CSI-RS based L1 measurement, RAN4 not to introduce definition of intra-frequency/inter-frequency candidate cell measurement. (vivo) - Recommended WF - Continue discussion. Issue 2-3-3: supported measurement types - Candidate solutions: - Option 1: (CATT) - Intra-frequency - Inter-frequency without gap - Inter-frequency with gap - Option 2: (Apple) - Intra-frequency without gap - Inter-frequency with gap - Option 3: deprioritize CSI-RS based L1-RSRP measurement outside active BWP in R19. (MTK) - Option 4: (vivo) - For CSI-RS based L1 measurement on candidate cell with periodic reporting, RRM requirements are specified for the following cases: - CSI-RS based L1 measurement without gap - Recommended WF - Continue discussion.

About the definition of intra-frequency and inter-frequency for CSI-RS based L1 measurement, we prefer to follow CSI-RS based L3 measurement and support Option 1.

Proposal 1: The definition of intra-frequency and inter-frequency for CSI-RS based L1 measurement follows CSI-RS based L3 measurement.

About the measurement types, we also agree to support intra-frequency without gap and inter-frequency with gap. For option 3, if companies are ok to deprioritize CSI-RS based L1-RSRP measurement outside active BWP, we are also supportive. But it will impact the definition of intra-frequency measurement which cannot base on the center frequency of CSI-RS resource.  If option 3 was agreed, we further suggest to only define the requirements for CSI-RS based L1-RSRP measurement within active BWP and without gap.

Proposal 2: Suggest to discuss measurement type together with the definition of intra- and inter-frequency measurement. Slightly prefer option 2 (intra-frequency without gap and inter-frequency with gap) and also fine with option 3 (CSI-RS based L1-RSRP measurement within active BWP and without gap).

Issue 2-3-4: supported frequency range - Candidate solutions: - Option 1: only FR1 and FR2-1 (Apple) - Recommended WF - Continue discussion. Issue 2-3-5: RTD assumption - Candidate solutions: - Option 1: RTC<CP is taken as baseline. For the case of RTD>CP between serving cell and neighbour cell on the same carrier, UE capability should be introduced. (Apple) - Option 2: Further study whether to support RTD>CP case for CSI-RS based L1-RSRP measurement. (MTK) - Recommended WF - Continue discussion.

Proposal 3: Support only FR1 and FR2-1.

Proposal 4: Support to firstly consider RTD<CP case for CSI-RS based L1-RSRP measurement.

Issue 2-3-6: CSI-RS resource configuration - Candidate solutions: - Option 1: the same CSI-RS resource configuration {D=3, PRB≧48} as for CSI-RS based L3 measurement is considered as a starting point. FFS on resource location limitation. (Apple) - Option 1a: For CSI-RS based L1-RSRP measurement on neighbor cell, define the requirements for Density=3 with (≥) 48 PRBs and not to define tighter requirements for more 48 PRBs. (MTK) - Recommended WF - Continue discussion.

CSI-RS resource configuration can follow the logic for CSI-RS based L3 measurement. Either option 1 or 1a is fine.

Proposal 5: Support the same CSI-RS resource configuration {D=3, PRB≧48}.

Issue 2-3-8: known cell definition in CSI-RS based L1 measurement - Candidate solutions: - Option 1: (Apple) - The cell is considered as known if the following conditions are met: - The UE has performed L3 measurement on the target cell during the last 5 seconds, and - The CSI-RS from the target cell configured for L1 measurement remains detectable. - Recommended WF - Continue discussion. Issue 2-3-9: whether to cover both known and unknown cell for CSI-RS L1 measurement - Candidate solutions: - Option 1: only consider known cell in CSI-RS based L1-RSRP measurement. (Apple, MTK) - Recommended WF - Continue discussion.

We support to reused the side conditions of current known cell definition. Option 1 is fine. And whether to cover both known and unknown cell for CSI-RS L1 measurement,  we also support to consider known case only.

Proposal 6: Support only consider known cell in CSI-RS based L1-RSRP measurement.

Conclusion

In the contribution we provide the following proposals:

Proposal 1: The definition of intra-frequency and inter-frequency for CSI-RS based L1 measurement follows CSI-RS based L3 measurement.

Proposal 2: Suggest to discuss measurement type together with the definition of intra- and inter-frequency measurement. Slightly prefer option 2 (intra-frequency without gap and inter-frequency with gap) and also fine with option 3 (CSI-RS based L1-RSRP measurement within active BWP and without gap).

Proposal 3: Support only FR1 and FR2-1.

Proposal 4: Support to firstly consider RTD<CP case for CSI-RS based L1-RSRP measurement.

Proposal 5: Support the same CSI-RS resource configuration {D=3, PRB≧48}.

Proposal 6: Support only consider known cell in CSI-RS based L1-RSRP measurement.

Reference

- R4-2414060, WF on NR mobility enhancements Phase 4, Apple