3GPP TSG-RAN WG4 Meeting #112bis	R4-2416125

# Hefei, Anhui, China, 14th – 18th October, 2024

Agenda Item:	6.25.2

Source: 	OPPO

Title: 	Discussion on RRM core requirements for XR

Document for:	Approval

1	Introduction

In last meeting RAN4 had an initial discussion on the RRM core requirements for XR_Ph3 WI [1]. Based on the approved workplan, this meeting is supposed to further discuss the scenarios for measurement skipping, and we will provide our considerations on this topic.

2	Discussion

### Issue 1-1: Workscope < Agreement>: - As starting point, RAN4 to discuss the Tx/Rx in occasions of L3 measurement. - The progress in RAN1/2 should be taken into account. - FFS whether/when to start the discussion of Tx/Rx in occasions of scheduling restriction due to L1 operation. - RAN4 to discuss - The potential impact on measurement requirements due to measurement cancellation - The corresponding solution to address the impact if needed.

As shown above, the Tx/Rx in occasions of L3 measurements is agreed to be considered as the starting point in RAN4. However, whether/when to discuss Tx/Rx in occasions of scheduling restriction due to L1 operation needs further study. From our view, L1 operation could be deprioritized. Firstly, the scheduling restriction due to L1 operation applies to a few OFDM symbols, which is quite less compared with the MGL for L3 measurements. The throughput gains to enable XR data Tx/Rx on the restricted symbols due to L1 operation is limited. Secondly, L1 operation such as RLM, BFD, CBD and L1-RSRP measurement is critical to monitor the radio link quality of the serving cell(s) and should be prioritized over XR data Tx/Rx. Otherwise, the UE cannot realize that the current serving link is getting worse timely and then the XR data Tx/Rx will fail either, for example when OOS occurs.

Proposal 1: Deprioritize Tx/Rx on the scheduling restricted symbols due to L1 measurement.

### Issue 1-4: Types of measurements <Way forward>: - Discuss types of measurements that may potentially be impacted by the XR operation. - FFS: measurements without gaps

For L3 measurement within gaps, the consensus during last meeting is to consider it. In this case, some gap occasions will be skipped and the whole MGL (1.5ms ~ 20ms) could be released for XR data Tx/Rx. For L3 measurement without gaps, the scheduling restrictions may apply to partial or all the symbols within SMTC window duration. Similar to L1 measurement, the benefit to skip L3 measurement occasion without gap is limited and could be deprioritized. Besides scheduling restrictions, L3 measurement without gaps may also cause short interruption around measurement occasions, which is discussed and specified in Rel-18 MG enhancements WI. However, the exact interruption location is not defined and it may occur [4ms] before SMTC occasions depending on UE implementation. The timeline to skip such the SMTC occasion with scheduling restriction and uncertain interruption will be complicated.

Proposal 2: Deprioritize Tx/Rx on the scheduling restricted symbols due to L3 measurement without gaps.

When some measurement occasions are skipped for XR data Tx/Rx, the following two options could be considered to address the potential RRM impacts basically:

- Option 1: extend the delay and keep the accuracy requirement

- Option 2: keep the delay and enhance the accuracy requirement

Option 1 has been widely used with different implementation methods, for example, by introducing scaling factor Kgap to extend delay requirement due to dropped gap occasions in concurrent gap, or by introducing extra samples like Lpss/sss to extend delay requirement due to unavailable SSB occasions due to LBT failure in NR-U. We slightly prefer to take option 1 as the starting point, and can further discuss whether the existing approaches could be reused or a new approach is needed. The workload of option 2 is heavier than option 1. Keeping the delay requirement will lead to less available measurement samples and accuracy loss. Generally, a lot of simulations are required to evaluate how much accuracy requirements could be reached with less measurement samples. It is hard and unadvisable to re-simulate all the impacted measurement types.

Proposal 3: To reflect the RRM impacts due to measurement cancellation/skipping, extend the delay and keep the accuracy requirements could be considered as the baseline.

### Issue 1-3: Types of measurement gaps to consider In this issue we discuss the types of measurement gaps to be considered in this WID. In order to help the discussion, we reuse the definition of Type-1 and Type-2 measurement gaps as agreed in Rel-18 NR_MG-enh2: - Type-1 MG: Gap(s) configured via GapConfig without suffix - Type-2 MG: Gap(s) configured via GapConfig-r17 without preConfigInd-r17 or ncsgInd-r17 - Rel-18 Concurrent Gap: gap with assigned priority which can be also NCSG or Pre-MG <Way forward>: Please discuss the priorities for RAN4 to work based on the following table: Cap types; First prio; Second prio; Don’t include. Type 1; Nokia, MTK, QC, Vivo, ZTE, Apple, CMCC, HW, Ericsson, OPPO, CATT, IDC. Type 2; QC, vivo, Apple, CMCC, HW, OPPO, CATT; Nokia; ZTE. NCSG; Apple, CMCC, HW, OPPO, MTK; QC, Nokia, CATT, IDC; vivo. MUSIM; HW; Vivo, Apple, Nokia, Ericsson, CATT. Positioning; QC, vivo, HW; ZTE, Apple, Nokia, Ericsson, CATT, IDC. Rel 18 concurrent gap; QC, Apple, CMCC, HW, CATT, MTK; Vivo, ZTE, Nokia, Ericsson. PreConfigured; Apple, QC, CATT, CMCC, HW, OPPO, MTK.

So far, several types of measurement gaps have been introduced in RRM spec as listed above and we will discuss which types of measurement gaps should be considered in this WI case by case.

- Type-1 gap is the basic gap considered since Rel-15 and is widely supported for all types of UEs and all types of reference signals. It is obvious to consider type-1 gaps as first priority in Rel-19 XR WI.

- Type-2 gap enables Rel-17 concurrent gaps, where each gap is expected to be configured with priority and associated measurement objects. We don’t see any strong technical concern to include type-2 gap in Rel-19 XR WI but one small issue regarding the execution order between Rel-17 priority-based gap collision rule and Rel-19 gap skipping rule. As mentioned in our contribution R1-2406248 [3], different execution orders will lead to different available gap occasions and therefore the corresponding UE behaviour need to be clarified.

- NCSG contains ML and two VILs before/after ML. Scheduling restrictions may be required during ML and up to 1ms interruption is required during each VIL. If one NCSG occasion is skipped, both scheduling restriction and interruption could be avoided and UE can be scheduled during the whole NCSG occasion. The performance gain is noticeable and the related spec impact is similar as that of type-1 gap in our view. So, we suggest to consider NCSG in Rel-19 XR WI.

- Pre-MG could be (de)activated by either UE autonomous mechanism or NW controlled mechanism. The procedure is similar as gap skipping in XR and we think it is better to include Pre-MG.

- MUSIM gap is only applicable for MUSIM capable UE performing for measurements, paging monitoring, SIB acquisition, and/or on-demand SI request of the target cell. It is an optional feature and could be replaced by other high-level solutions, e.g. UE temporary capability restriction or leaving RRC CONNECTED from the source cell. Therefore, we prefer to exclude MUSIM gaps from Rel-19 XR WI.

- Positioning gap (PosGapConfig) was introduced from Rel-17 and could be activated by MAC-CE to achieve fast and flexible gap adaptation for positioning measurement. Jointly considering positioning gap activated by MAC-CE and gap skipping indicated by DCI is quite complicated and should be excluded in our view.

- Rel-18 concurrent gaps are further MG enhancements on the top of Rel-17 features and whether to considered such gaps could be postponed after we finish the work related Rel-17 gaps. For example, if Rel-17 type-2 gap or Pre-MG is excluded, naturally Rel-18 concurrent gaps with Pre-MG should be excluded either. If both Rel-17 type-2 gap and Pre-MG are considered in Rel-19 XR WI and no much additional work effort is needed to include Rel-18 concurrent gaps with Pre-MG, then we are fine to include it.

Proposal 4: In Rel-19 XR WI,

- Consider type-1 gap, type-2 gap (Rel-17 concurrent gap), NCSG and pre-MG with first priority

- Postpone Rel-18 concurrent gaps

- Exclude MUSIM gap and positioning gap

Proposal 5: If type-2 gap (Rel-17 concurrent gap) is considered, the execution order between Rel-17 priority-based gap collision rule and Rel-19 gap skipping rule should be discussed.

Working Assumption For solutions based on triggering/enabling by network signaling to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements select the following option: - Alt. 1: Dynamic indication to enable Tx/Rx in particular gap/restriction that are caused by RRM measurements. - Alt 1-1: Explicit indication by DCI to skip a particular gap/restriction; - Indication is included as part of scheduling DCI: - Bit-field size is one bit; - The bit in the DCI is used to indicate whether to skip the first gap/restriction occasion after a minimum time offset required between the last symbol of the PDCCH carrying the DCI format and the start of corresponding skipped gap/restriction occasion indicated by the DCI. Send an LS to RAN4 to inform them of the above working assumption and ask them if there is any issue with it. Agreement If Alt. 1 from RAN1#117 agreement is supported, minimum time offset(s) X between indication to skip and skipped measurement occasion is up to RAN4 to discuss and decide on particular value(s). To RAN4 ACTION: RAN1 respectfully asks RAN4 to check the working assumption and inform RAN1 if there are concerns that RAN1 should address.

Last RAN1 meeting discussed the solutions to enable Tx/Rx in gaps/restrictions caused by RRM and reached the above working assumption. Alt 1-1 dynamic explicit indication in DCI was agreed. The LS R1-2407561 [4] was sent to RAN4 to check if there is any issue with Alt 1-1, and to further discuss the minimum time offset(s) X between the DCI indication to skip and the skipped measurement occasion. Rel-17 MG_enh WI has introduced Pre-MG and it could be (de)activated upon DCI-based BWP switch. This procedure is quite similar as Alt 1-1 in our understanding and no technical issue to support Alt 1-1 in Rel-19 XR WI is identified from RAN4 perspective.

Observation 1: Alt 1-1 in the LS R1-2407561 is similar as Pre-MG deactivation upon DCI-based BWP switch and no technical issue is identified in RAN4.

Proposal 6: Alt 1-1 in the LS R1-2407561 could be confirmed from RAN4 perspective.

As for the minimum time offset X between DCI indication to skip and the skipped measurement occasion, we think the principle to define Pre-MG (de)activated delay could be reused. For example, the Pre-MG (de)activated delay upon DCI/timer-based BWP switch is defined as the legacy BWP switch delay + 5ms time margin for UE to re-arrange the following measurement or data scheduling due to Pre-MG status change. Therefore, in this case when measurement occasion is skipped by DCI indication, both DCI decoding time defined in TS 38.214 spec and additional time margin (e.g. [5]ms) should be considered when defining the minimum time offset X.

Proposal 7: The minimum time offset X between DCI indication to skip and the skipped measurement occasion should be DCI decoding time defined in TS 38.214 + [5]ms time margin.

3	Conclusion

This contribution discussed the RRM impacts in Rel-19 XR WI and gave the following proposals:

Observation 1: Alt 1-1 in the LS R1-2407561 is similar as Pre-MG deactivation upon DCI-based BWP switch and no technical issue is identified in RAN4.

Proposal 1: Deprioritize Tx/Rx on the scheduling restricted symbols due to L1 measurement.

Proposal 2: Deprioritize Tx/Rx on the scheduling restricted symbols due to L3 measurement without gaps.

Proposal 3: To reflect the RRM impacts due to measurement cancellation/skipping, extend the delay and keep the accuracy requirements could be considered as the baseline.

Proposal 4: In Rel-19 XR WI,

- Consider type-1 gap, type-2 gap (Rel-17 concurrent gap), NCSG and pre-MG with first priority

- Postpone Rel-18 concurrent gaps

- Exclude MUSIM gap and positioning gap

Proposal 5: If type-2 gap (Rel-17 concurrent gap) is considered, the execution order between Rel-17 priority-based gap collision rule and Rel-19 gap skipping rule should be discussed.

Proposal 6: Alt 1-1 in the LS R1-2407561 could be confirmed from RAN4 perspective.

Proposal 7: The minimum time offset X between DCI indication to skip and the skipped measurement occasion should be DCI decoding time defined in TS 38.214 + [5]ms time margin.

4	Reference

- R4-2414043, WF on RRM requirements for XR_Ph3, Nokia

- R4-2411820, Topic summary for [112][225] NR_XR_Ph3, Nokia

- R1-2406248, Enhancements to enable Tx/Rx for XR during RRM measurements, OPPO

- R1-2407561, LS on gaps/restrictions that are caused by RRM measurements, RAN1