3GPP TSG-RAN WG4 Meeting #112bis	R4-2416128

Hefei, China, Oct 14th – 18th, 2024

Title: 	Further discussion on power domain enhancements for single carrier

Agenda Item:	6.1.1.2.1

Source: 	CATT

Document for:	Discussion

# 1 Introduction

In RAN4#112, power domain enhancements for single carrier was discussed with some progress [1], while there are still open issues identified as shown below:

#### Issue 1-2-1: Clarification of BS CBW - Proposals - Proposal 1: Clarify that the BS channel bandwidth means BS RF bandwidth that covers single carrier, multi-carriers and multi-RATs scenarios. (Huawei) - WF - FFS in next meeting #### Issue 1-2-5: Boundary to apply ACLR and SEM - Proposals - Proposal 1: ACLR and SEM should be applicable from the edge of extended UE CBW instead of the BS CBW. (Samsung, Sony, ZTE, vivo, LGE) - Proposal 2: ACLR, SEM and spurious emissions would be defined as of today but based on BS channel bandwidth. (Nokia, Ericsson) - WF - FFS in next meeting #### Issue 1-2-7: Which CBW is utilized as the basis for the integral region of OOBE - Proposals - Option 1: The integral region and the boundary of OOBE should be based on UE CBW. (Samsung, vivo, ZTE, China Telecom) - Option 2: The integral region and boundary of OOBE is based on extended UE CBW. (Sony) - WF - FFS in next meeting #### Issue 1-2-8: Ratio size of extended CBW between UE CBW and larger BS channel BW - Proposals - Proposal 1: (Apple) - If the goal is to convert all outer RB allocations into inner RB allocations, then the minimum BS channel needs to be at least twice the size of the UE channel. - If the sole goal is to obtain output power improvements, then it is possible to have a more convenient ratio between UE and BS channel such as two-third. - Proposal 2: (Qualcomm) - If minimum excess BW equal to half of the original UE BW can be placed on both sides of the original UE BW this extended UE BW can be treated as the new UE BW and all performance metrics can be based on this new BW. - If the minimum extended BW on both sides of original UE BW is less than half the original UE BW then the original UE BW should not be extended, and all performance metrics should be based on the original UE BW. - Proposal 3: The required extended bandwidth of the both sides could be set to 2/5 UE CBW as a starting point. (vivo) - Proposal 4: Consider extended UE CBW approach to improve MPR starting from 1.5*CBW (OPPO) - Proposal 5: (Huawei) - For the shifted start point of ΔfOOB, 1/2 UE CBW can be the default value for UE indication. - Consider to introduce shorter shifted frequency e.g. 1/3, 1/4 UE CBW in conjunction with the requirement that will be introduced to the shifted frequency. - For instance, the modified general part of IBE requirements which is further relaxed by replacing the EVM corresponding to the scheduling modulation order with the one for BPSK - WF - FFS with consideration of above proposals in next meeting

In this contribution, we present our views on these open issues.

# 2 Discussion

Clarification of BS CBW

In the current specs, BS channel bandwidth is defined very clearly for single carrier [2]:

And channel bandwidth for single carrier is the basis for many RF requirements. If its definition is extended to cover multiple carriers, impacts on all channel bandwidth based RF requirements need to be checked. Actually, there is an existing term, i.e., BS RF bandwidth, which may fit for the purpose and covers multiple carriers or even multi-RATs:

Hence, there is no need to make any further clarification of BS CBW.

Proposal 1: RAN4 not to make any further clarification on BS CBW, and use “BS RF bandwidth” instead if needed.

Boundary to apply ACLR and SEM

Of the two options for the boundary to apply ACLR and SEM, where one is based on extended UE CBW, the other on BS CBW. If BS CBW serves as the extended UE CBW, then these two options are equal. However, no matter how extended UE CBW is defined, its hard limit is BS CBW, which means that extended UE CBW shall not exceed the boundary of BS CBW, therefore, it may be simpler to apply ACLR and SEM based on BS CBW.

Proposal 2: ACLR and SEM can be defined as of today but based on BS CBW.

CBW for the integral region of OOBE

Of the two options for CBW for the integral region of OOBE, where one is based on UE CBW, and the other on extended UE CBW. If the extended UE CBW is used as the basis for the integral region of OOBE, the boundary may exceed some regulatory requirements, hence it better to use UE CBW instead of extended UE CBW for OOBE.

Proposal 3: Use UE CBW as the basis for OOBE.

Ratio size of extended CBW between UE CBW and larger BS channel BW

One key approach for enhancing output power involves converting the outer region into the inner region, where a reduction in MPR can be achieved. To guarantee that all outer regions can be converted, the BS CBW would need to be at least double that of the UE CBW. However, this requirement could limit the range of practical applications. A more favourable ratio might be for the BS CBW to be 1.5 times the UE CBW.

Proposal 4: To enable power domain enhancements, the minimum BS CBW should be set to 1.5 times the UE CBW.

# 3 Conclusion

In this contribution, we have the following proposals for power domain enhancements for single carrier:

Proposal 1: RAN4 not to make any further clarification on BS CBW, and use “BS RF bandwidth” instead if needed.

Proposal 2: ACLR and SEM can be defined as of today but based on BS CBW.

Proposal 3: Use UE CBW as the basis for OOBE.

Proposal 4: To enable power domain enhancements, the minimum BS CBW should be set to 1.5 times the UE CBW.

# Reference

[1] R4-2414441, “WF on power domain enhancements”, Huawei

[2] TS 38.104 v18.7.0