3GPP TSG-RAN WG4 Meeting #112bis	R4-2416129

Hefei, China, Oct 14th – 18th, 2024

Title: 	Further discussion on MPR applicability for FR1 intra-band UL CA

Agenda Item:	6.1.1.2.2

Source: 	CATT

Document for:	Discussion

# 1 Introduction

In RAN4#112, open issues were identified for MPR applicability for FR1 intra-band UL CA [1], and some of these open issues are listed below:

#### Issue 2-1-1: Applicable MPR for intra-band contiguous CA with single activated cell - Proposals - Proposal 1: For PC3/PC2 intra-band contiguous carrier aggregation with single CC with activated cell, the single CC MPR requirements can apply. (Samsung) - Proposal 2: For PC3/PC2 intra-band contiguous carrier aggregation with single CC with activated cell, the following MPR requirements are applied (Samsung, CATT,, Nokia, Huawei, LGE) - MPR defined in Table 6.2.2-1 applies for UE power class 3 CA bandwidth classes B and C; - MPR defined in Table 6.2D.2-1 applies for power class 2 CA bandwidth classes B and C when TxD capability is indicated; - MPR defined in Table 6.2.2-2 applies for power class 2 CA bandwidth classes B and C when TxD capability is absent. - Proposal 3: For PC3 and PC2 contiguous UL CA use the corresponding single CC MPR when only 1 CC is activated and refer to the following MPR tables and corresponding configured Tx power requirements (Qualcomm). - MPR defined in Table 6.2.2-1 applies for UE power class 3 CA bandwidth classes B and C, along with configured Tx power requirements for 6.2.4 - MPR defined in Table 6.2D.2-1 applies for power class 2 CA bandwidth classes B and C when TxD capability is indicated, along with configured Tx power requirements for 6.2D.4 - MPR defined in Table 6.2.2-2 applies for power class 2 CA bandwidth classes B and C when TxD capability is absent, along with configured Tx power requirements for 6.2.4 - WF - FFS in next meeting #### Issue 2-1-2: Single CC CBW or aggregated CBW for applying requirements of ACLR/SEM/SE - Proposals - Proposal 1: Integral region and boundary of spurious emissions/ACLR/SEM should be based on the activated CC CBW instead of aggregated CBW. (Samsung) - Proposal 2: When only 1 CC is activated the aggregated BW for CA should be used for evaluating metrics such as spurious emissions, SEM and ACLR. (Qualcomm, Huawei) - WF - FFS in next meeting #### Issue 2-1-3: Single CC or CA requirements of ACLR/SEM/SE applied for single activated cell - Proposals - Proposal 1: Even when transmitted RBs are allocated in only one of the CC, the emissions requirement based on the configured intra-band contiguous ULCA are used (SEM, ACLR, spurious emissions) and all allocations can use the single CC MPR for inner. FFS if a similar approach is applicable to some additional emission requirements. (Skyworks) - Proposal 2: Apply single carrier spurious emission/ACLR/SEM requirements for contiguous UL CA with only 1 CC transmitted. (CATT, Samsung, ZTE, vivo) - Proposal 3: For Rel-19 MPR applicability for FR1 intra-band contiguous UL CA, the spurious emissions/ACLR/SEM are kept for aggregated/configured CBW. (Huawei, Ericsson) - WF - FFS in next meeting

In this contribution, we present our views on these open issues.

# 2 Discussion

Applicable MPR for intra-band contiguous CA with single activated cell

When comparing PC3 MPR values for single carrier and intra-band contiguous CA, a few dBs difference could be observed. If a UE in RRC_CONNECTED state is reconfigured from a single cell to two cells with intra-band contiguous CA, but UL transmission is still limited to PCell, then due to MPR difference between single carrier and intra-band contiguous CA, the UE has to lower the UL transmission, in this case, the UL quality becomes worse with the addition of an SCell even though UL transmission is the same on PCell only. This is not reasonable in the real field, and the issue was created arbitrarily and should be corrected. The solution is to apply the single carrier MPR for intra-band contiguous CA when only single UL is transmitted.

Observation: Even with the same single UL transmission on one carrier, the UL performance will be degraded with the addition of an intra-band contiguous SCell due to MPR increase from single carrier to intra-band contiguous CA.

Hence, we propose to apply single carrier MPR for intra-band contiguous CA with single activated cell.

Proposal 1: Apply single carrier MPR for intra-band contiguous CA with single activated cell.

ACLR/SEM/SE for intra-band contiguous CA with single activated cell

For spurious emissions, ACLR, and SEM, the same logic applies for uplink contiguous CA (bandwidth class B and C), e.g., for ACLR, tables below specify CA ACLR, where aggregated channel bandwidth acts as a ”virtual” larger channel bandwidth.

It is reasonable to apply spurious emission, ACLR and SEM for single UL CC transmission for CA bandwidth class B and C based on the aggregated bandwidth.

Proposal 2: Apply aggregated bandwidth as the basis for spurious emission/ACLR/SEM requirements for contiguous UL CA with only 1 CC transmitted.

Proposal 3: Apply single carrier spurious emission/ACLR/SEM requirements for contiguous UL CA with only 1 CC transmitted.

# 3 Conclusion

In this contribution, we have the following observation and proposals for MPR applicability for FR1 intra-band UL CA with 1 CC transmitted:

Observation: Even with the same single UL transmission on one carrier, the UL performance will be degraded with the addition of an intra-band contiguous SCell due to MPR increase from single carrier to intra-band contiguous CA.

Proposal 1: Apply single carrier MPR for intra-band contiguous CA with single activated cell.

Proposal 2: Apply aggregated bandwidth as the basis for spurious emission/ACLR/SEM requirements for contiguous UL CA with only 1 CC transmitted.

Proposal 3: Apply single carrier spurious emission/ACLR/SEM requirements for contiguous UL CA with only 1 CC transmitted.

# Reference

[1] R4-2414441, “WF on power domain enhancements”, Huawei