3GPP TSG-RAN WG4 Meeting #112bis                                     R4-2416130

Hefei, Anhui, China, 14th – 18th October, 2024

Source:	Samsung, KDDI

Title:            General discussion on intra-band non-collocated EN-DC/NR-CA type 4

Agenda item:			6.5.3

Document for:	Discussion

1. Introduction

In last meeting, RAN4 had some initial discussions on intra-band non-collocated EN-DC/NR-CA deployment; Phase2: new receiver type(s). In this meeting, we will give some further discussions for some open issues captured in the agreed WF [1].

2. Rel-19 RAN4 Progress on UE RF aspects

In the previous meetings, RF achieved the following agreements

Topic #2: Rel-19 Type 4 Issue 2-1: Revised WID for Type 4 Agreement: - RAN4 suggests to revise WID to clarify the max number of CCs for Type 4 EN-DC/NR-CA. Issue 2-2-2: Minimum DL frequency separation Agreement: - Assume that Center of BWanother relative to edge of BWwanted is at least 80MHz+BWanother/2 away from the edge of the wanted CC. - For Type 4 UE, minimum DL separation is not reflected in RAN4. Issue 2-3-1: New UE Capability for Type 4a(6Rx/UE) and 4b(8Rx/UE) for EN-DC/NR-CA Tentative agreement: - Define one capability for NR CA and one capability for EN-DC - FFS on how to differentiate type 4a and 4b. Issue 2-3-3: New BS Signalling to switch between Type 4a/4b and Type 2 Agreement: - No new NW signalling needed from Type-4 to Type-2 switch and maxMIMO-layer can be used. Issue 2-4-1: The number of B42 CCs for Type 4 EN-DC Agreement: - B42: multiple contiguous CCs, all collocated - Based on the requested band combinations (see R4-2406628) Issue 2-4-2: The number of NR CCs for Type 4 EN-DC and NR-CA Agreement: - Prioritize the following number of NR CCs same as Type 2. - Non-collocated Type 4 EN-DC - n77/n78: one CC - Non-collocated Type 4 NR-CA - n77/n78: Non-contiguous two CCs, non-collocated - If TU is remained in Rel-19, discuss other scenario(s) of the number of CCs and contiguous cases later.

The critical agreements are: A. From UE perspective, two capabilities are defined for NR CA and EN-DC, respectively, while, the detailed capability signalling design is still suspending. B. From BS perspective, 1) No new BS signalling needed from Type-4 to single carrier fallback; 2) No new NW signalling needed from Type-4 to Type-2 switch and maxMIMO-layer can be used. But how to define the Rel-19 NW signalling was not discussed extensively

3 Discussion on RRM requirements impact

3.1 Whether to introduce MRTD/MTTD requirements for non-collocated FR1 intra-band asynchronous NR-CA for Type 4 UE in Rel-19

It was agreed in RRM that not to introduce MRTD/MTTD requirements for non-collocated intra-band asynchronous NR-CA Type 2 UE in Rel-18, one of the observations they gave is that RAN4 did not discuss any about synchronous or asynchronous NR CA, what we assume when to discuss NR-CA is synchronous operation, the definition of async NR-CA is not clear enough. As a continuous work of Rel-18 noncol, there is no need to introduce non-collocated FR1 intra-band asynchronous NR-CA for Type 4 UE in Rel-19.

Proposal 1: Not to introduce MRTD/MTTD requirements for non-collocated FR1 intra-band asynchronous NR-CA for Type 4 UE in Rel-19

3.2 Other RRM requirements (apart from MRTD/MTTD requirements) for Type 4 UE

In Rel-18, in addition to MRTD/MTTD requirements, the following requirements were also discussed: Scheduling restriction requirements; Interruption requirements (at SCell addition/release/activation/deactivation, during measurements on deactivated SCC); SCell activation delay requirements; BFD/CBD requirements.

From our understanding, the essential reasons to define such requirements for non-collocated intra-band CA are that: 1) The aggregated CCs will be received using separate Rx chain; 2) SCell addition/release/activation/deactivation may impact on not only the active serving cells are in the same band but also the active serving cells are in the overlapping DL bands (i.e., inter-band CA shall be considered)

The same logic, since the type 4 UE has the similar architecture with type 2 UE except the number of Rx chain/layer per CC, and still focus on overlapping or partially overlapping bands, we believe

Proposal 2: The impact on the following RRM requirements for type 4 UE can be the same as for type 2 UE in non-collocated condition

- Scheduling restriction

- Interruption

- SCell activation delay

- SSB/CSI-RS based BFD

- SSB/CSI-RS based CBD

3.3 Other RRM requirements (apart from MRTD/MTTD requirements) for Type 4 UE operates with shared RF chains in collocated scenario

The same principle as previous issues. Since Type 4 UE has the similar architecture with type 2 UE, not only the requirements for Type 2 capable UE operates with separate RF chain in non-collocated deployment, but also the ones for Type 2 capable UE operates with shared RF chains in collocated deployment can be applied to Type 4 capable UE (no matter 8 L or 6 L).The other impacted RRM requirements contain :

- Interruptions at SCell addition/release/activation/deactivation/ during measurements on deactivated SCC

- SCell activation delay requirement for Deactivated Scell

- Scheduling availability

Proposal 3: The following requirements for Type 2 capable UE when it operates with shared RF chain in collocated condition in Rel 18 can be applied to Type 4 capable UE when it operates with the same condition:

- Interruptions at SCell addition/release/activation/deactivation/ during measurements on deactivated SCC

- SCell activation delay requirement for Deactivated SCell

- Scheduling availability

3.4 SCell BFD/CBD requirements for Type 4 capable UE

From Rel-15, the BFD/CBD RRM requirements were specified for the cases that when UE is required to perform beam failure detection/candidate beam detection on no more than 1 serving cell per band with the assumption of collocated deployment. While, non-collocated deployment was considered in intra-band later, under which the discrepancy between the channel qualities, RTD, side conditions of the two cells in the same band can not be ignored. Getting benefits from using separate Rx chain, the type 2 capable UE is capable of performing BFD/CBD on no more than 2 serving cells per band if these 2 serving cells are in non-contiguous carriers. Type 4 capable UE as well.

Proposal 4: Type 4 capable UE when operating in non-collocated condition can be configured with BFD/CBD measurements on up to two serving cells per band

3.5 NW signalling and UE capability signalling

- Rel-19 NW signalling.

In Rel-18, in conjunction with the value of IE maxMIMO-Layers no more than 2, two new BS signallings were defined for inter-band non-collocated EN-DC (i.e, nonCollocatedTypeMRDC-r18) and intra-band non-collocated NR-CA (i.e, nonCollocatedTypeNR-CA-r18), separately. The two signallings to be used for indicating the switching between type 2 (2 Rx/CC)and type 1 (4 Rx/CC), and if the signallings indicate, the Type 1 requirement shall be applied.

In Rel-19, the same logic are re-visited, and seems that it would be optimistic to define new NW signalling (s) to indicate the switching from Type 4 to Type 1 with 6Rx/8Rx, however, how to define the new signallings are not decided yet in RF session.

- UE capability signalling.

In last meeting, companies agreed to define one capability for NR CA and one capability for EN-DC, the mainstream solutions discussed in RF are summarized as below:

- Define separate capabilities for Type 4a and Type 4b

- Define a single UE capability for type 4 but with reporting/indicating different maximum number of layers that the UE can support per CC (i.e., maxNumberMIMO-LayersPDSCH)

- RRM requirements definition

Recap Rel-18 non-collocated discussions, the complete requirements were defined by combining the NW signalling and UE capability signalling together, e.g.,

8.1.7 Scheduling availability of UE during radio link monitoring When intra-band non-contiguous carrier aggregation is configured for a UE indicating intraBandNR-CA-non-collocated-r18, there are no scheduling restrictions on FR1 serving cell(s) configured on the non-contiguous CC(s) in the same band if nonCollocatedTypeNR-CA-r18 is not provided. Otherwise, the scheduling restrictions on FR1 serving PCell or PSCell applies to all serving cells in the same band on the symbols that fully or partially overlap with the restricted symbols if nonCollocatedTypeNR-CA-r18 is provided.

However, since in RF no conclusions regarding UE capability reporting signalling and network signalling achieved for the time being, how to introduce/specify the impacted RRM requirements for Rel-19 type 4 UE cannot be concluded at current stage.

Proposal 5: How to specify the impacted RRM requirements for Rel-19 type 4 UE need to wait for RF progress on the design of new UE capabilities/NW signallings

4 Conclusion

In this contribution, we provided our initial viewpoints to RRM requirement impact analyses on intra-band non-collocated EN-DC/NR-CA type 4. The following observations and proposals are obtained:

Proposal 1: Not to introduce MRTD/MTTD requirements for non-collocated FR1 intra-band asynchronous NR-CA for Type 4 UE in Rel-19

Proposal 2: The impact on the following RRM requirements for type 4 UE can be the same as for type 2 UE in non-collocated condition

- Scheduling restriction

- Interruption

- SCell activation delay

- SSB/CSI-RS based BFD

- SSB/CSI-RS based CBD

Proposal 3: The following requirements for Type 2 capable UE when it operates with shared RF chain in collocated condition in Rel 18 can be applied to Type 4 capable UE when it operates with the same condition:

- Interruptions at SCell addition/release/activation/deactivation/ during measurements on deactivated SCC

- SCell activation delay requirement for Deactivated SCell

- Scheduling availability

Proposal 4: Type 4 capable UE when operating in non-collocated condition can be configured with BFD/CBD measurements on up to two serving cells per band

Proposal 5: How to specify the impacted RRM requirements for Rel-19 type 4 UE need to wait for RF progress on the design of new UE capabilities/NW signallings

5 References

[1] R4-2413973, “WF on RRM requirements for supporting intra-band non-collocated EN-DC/NR-CA deployment Phase2: new receiver type(s)”, Huawei, HiSilicon