# 3GPP TSG-RAN WG4 Meeting #112bis	R4-2416143

# Hefei, Anhui, China, 14th – 18th October, 2024

Source:			Ericsson

Title:			NTN Ku-band – VSAT RF

Agenda item:		6.9.4

Document for:		Discussion

# Introduction

In RAN#104, a new WI ([1]) has been approved to specify new NTN Ku band(s).

RAN4 had initial discussion in last RAN4#112 meeting and a way forward ([2]) has been agreed. This contribution discusses further the VSAT RF aspect, and more specifically the regulatory information for the agreed NTN KU-band for CEPT countries.

# Discussion

## ECC Regulation and Harmonized Standards

In last RAN4#112 meeting, we shared our findings ([3]) about relevant ECC Decision (Table 1) and Harmonized Standards (Figure 1) that should be considered when specifying the new NTN Ku-band as agreed in last meeting ([4]).

Table 1: Relevant ECC Decisions to be considered for the NTN KU-band

| ECC Decision | Frequency Range | Title |
| --- | --- | --- |
| ECC Decision(18)04 | 10.7 - 12.75 GHz (S-to-E) 14.0 – 14.5 GHz (E-to-S) | The harmonised use, exemption from individual licensing and free circulation and use of land based Earth Stations In-Motion (ESIM) operating with GSO FSS satellite systems in the frequency bands 10.7-12.75 GHz and 14.0- 14.5 GHz |
| ECC Decision(18)05 | 10.7 - 12.75 GHz (S-to-E) 14.0 – 14.5 GHz (E-to-S) | The harmonised use, exemption from individual licensing and free circulation and use of Earth Stations In-Motion (ESIM) operating with NGSO FSS satellite systems in the frequency bands 10.7-12.75 GHz and 14.0-14.5 GHz |
| ECC Decision(19)04 | 10.7 - 12.75 GHz (S-to-E) 12.75 – 13.25 GHz (E-to-S) | The harmonised use of spectrum, free circulation and use of earth stations on-board aircraft operating with GSO FSS networks and NGSO FSS systems in the frequency bands 12.75-13.25 GHz (Earth-to-space) and 10.7-12.75 GHz (space-to-Earth) |
| ECC Decision(05)10 | 10.7 – 11.7 GHz and 12.5 – 12.75 GHz (S-to-E) 14.0 - 14.5 GHz (E-to-S) | The free circulation and use of Earth Stations on board Vessels operating in fixed satellite service networks in the frequency bands 14-14.5 GHz |
| ECC Decision(17)04 | 10.7 - 12.75 GHz (S-to-E) 14.0 – 14.5 GHz (E-to-S) | The harmonised use and exemption from individual licensing of fixed earth stations operating with NGSO FSS satellite systems in the frequency bands 10.7-12.75 GHz and 14.0-14.5 GHz |
| ECC Decision(03)04 | 10.7 – 11.7 GHz (S-to-E) 14.25 – 14.5 GHz (E-to-S) | Exemption from Individual Licensing of Very Small Aperture Terminals (VSAT) operating in the frequency bands 14.25-14.50 GHz Earth-to-space and 10.70-11.70 GHz space-to-Earth |
| ECC Decision(06)03 | 10.7 – 12.75 GHz (S-to-E) 14.0 – 14.25 GHz (E-to-S) | Exemption from Individual Licensing of high e.i.r.p. satellite terminals (HEST) operating with geostationary satellites and in the frequency bands 10.70-12.75 GHz or 19.70-20.20 GHz space-to-Earth and 14.00-14.25 GHz or 29.50-30.00 GHz Earth-to-space |
| ERC Decision(00)08 | 10.7 – 12.5 GHz (S-to-E) | ERC Decision of 19 October 2000 on the use of the band 10.7 – 12.5 GHz by the fixed service and Earth stations of the broadcasting-satellite and fixed-satellite service (space-to-Earth) |

Figure 1: Relevant ETSI Harmonized Standards for the NTN Ku-band

Proposal1: Consider the ECC Decisions listed in Table 1 and the ETSI Harmonized Standards listed in Figure 1 when specifying VSAT RF requirement for the NTN Ku band (10.7 – 12.75 GHz (E-to-S) and 14.0 - 14.5 GHz (E-to-S)).

## FR choice and TS 38.101-5 impacts

According to the WID (), only NTN UEs types 1 to 5 shall be considered in the scope of this WI, no handheld device is expected in this NTN Ku-band.

Observation1: Only VSAT (NTN UE Type 1-5) shall be considered for the NTN Ku-band.

Looking at TS 38.101-5, only FR2-NTN bands are bands with radiated requirements, TN UE type 1-5 are currently only supported in FR2-NTN bands.

Observation2: Currently, radiated requirements and NTN UE type 1-5 are only supported in FR2-NTN bands in TS 38.101-5.

If RAN4 decides that the NTN Ku-band should belong to FR1-NTN frequency range, some preliminary udpates would be needed to ensure NTN UE type 1-5 could easily be also supported in FR1-NTN.

For example:

- Subclauses in section 5.2 (5.2.2 Operating bands with conducted requirements  and 5.2.3 Operating bands with radiated requirements) need to be rearranged or their name should be updated.

- In sub-clause 9.2.2.1, the following sentence needs to be updated so that FR1-NTN VSAT are included:

We suggest doing this clean-up phase before adding any requirement for the NTN Ku-band in TS 38.101-5.

Proposal2: If RAN4 decides that the NTN Ku-band(s) should belong to FR1-NTN, RAN4 should first review and update TS 38.101-5 to facilitate the support of NTN UE type 1-5 for the NTN Ku-band.

# Conclusion

In this contribution, we made an initial analysis of the impacts of adding NTN Ku-band in TS 38.101-5, looking also at CEPT regulatory and ETSI Harmonized Standards context. We made the following proposals and observations:

Proposal1: Consider the ECC Decisions listed in Table 1 and the ETSI Harmonized Standards listed in Figure 1 when specifying VSAT RF requirement for the NTN Ku band (10.7 – 12.75 GHz (E-to-S) and 14.0 - 14.5 GHz (E-to-S)).

Observation1: Only VSAT (NTN UE Type 1-5) shall be considered for the NTN Ku-band.

Observation2: Currently, radiated requirements and NTN UE type 1-5 are only supported in FR2-NTN bands in TS 38.101-5.

Proposal2: If RAN4 decides that the NTN Ku-band(s) should belong to FR1-NTN, RAN4 should first review and update TS 38.101-5 to facilitate the support of NTN UE type 1-5 for the NTN Ku-band.

# References

- RP-241690, New WID on Introduction of Ku Band for NR NTN, Intelsat, Eutelsat Group, Thales, CHTTL, Hispasat

- R4-2413525, Way Forward for [112][315] NR_NTN_Ku_Band_UE_SAN_RF

- R4-2411190, NTN Ku-band – Regulatory aspects and band(s) definition, Ericsson

- R4-2413519, Way Forward for [112][314] NR_NTN_Ku_Band_General