# 3GPP TSG-RAN WG4 Meeting # 112bis      					R4-2416144

# Hefei, China, 14-18 October, 2024

Source:			Ericsson

Title:			New NTN S-band –UE RF requirements

Agenda item:		5.17.2

Document for:		Approval

# Introduction

In RAN#104, a new WI ([1]) has been approved to specify new NTN S-band.

In last RAN4#112 meeting, a way forward ([3]) was agreed capturing the next steps related to UE-UE coexistence issues. This contribution is further discussion those aspects.

# Discussion

## Background

The new NTN S-band would occupy the 2000-2020 MHz frequency range in UL and 2180-2200 MHz in DL. The adjacent and overlapping bands are shown in Figure 1.

Figure 1: Adjacent and overlapping bands to the new NTN S-band

Regarding UE-UE coexistence requirement for this new S-band, it was agreed ([3]) in last RAN4#112 meeting to focus on coexistence with bands n2/n25 and clarify in the TR that there is no good solution for the coexistence with bands n66/n70.

The following next steps were captured in this WF:

## NTN-TN coexistence background for FR1-NTN

When RAN4 ran coexistence studies (TR 38.863) for the NTN S-band, using 2 GHz as reference frequency, an issue raised with case 1 (TN DL aggressor vs NTN DL victim) where the required ACIR value was too high to derive acceptable NTN UE ACS value.

To solve this, RAN4 agreed to consider an isolation distance between the edge of a TN network and the location when NTN UE could be connected to a satellite, assuming TN connection will always be prioritized over NTN. For the simulation’s purpose, an isolation of 1500m was then considered.

Observation1: For FR1-NTN, 1500m isolation distance was assumed between TN edge and a connected NTN UE to address TN-NTN coexistence issues.

Based on this isolation distance, RAN4 checked that NTN and TN could coexist and derived the corresponding NTN UE ACS. The suggested NTN UE ACS value was 28dBc ([5]) but, to align with TN UE ACS, it was finally agreed to specify NTN UE ACS with 33 dBc value ([5]).

Nevertheless, RAN4 didn’t check back how this isolation distance could be reduced with this agreed NTN UE ACS value. In other words, an NTN UE might still connect to a satellite with very good performance even if this isolation distance is shorter. In the scope of this investigation to reduced NTN S-band UE coexistence requirement with bands n2/n25, it would make sense that RAN4 makes such investigation and evaluate, for the agreed NTN ACS value (33dB), what would be the minimum isolation distance between TN and NTN.

Observation2: RAN4 didn’t check what is the minimum isolation distance for TN and NTN to coexist considering the specified NTN UE ACS value (33dB).

It should also be noted that no requirement was specified to address this minimum isolation distance and no such priority mechanism was specified. This was deemed acceptable as, if this isolation distance is not respected, any legacy TN networks won’t be impacted, only the NTN performance would be degraded. Still, even if it could be challenging to keep connected, the NTN UE would still be able to connect to the satellite.

Observation3: No requirement has been specified to capture this assumed isolation distance and no priority mechanism (TN over NTN) exists.

Observation4: If this isolation distance is not respected, the NTN network performance might be impacted, not the TN performance. Also, the NTN should still be able to connect to the satellite.

## Less stringent coexistence limits

As captured in the way forward ([3]), limits higher than the -50dBm/MHz value already exists in TS 36.101-1 for some bands to coexist with other ones. The following examples were given in last RAN4#112 meeting:

- n1 protection of n39. The following levels were specified:

| Frequency range | 1880 | - | 1895 | -40 |
| --- | --- | --- | --- | --- |
| Frequency range | 1895 | - | 1915 | -15.5 |
| Frequency range | 1915 | - | 1920 | +1.6 |

Nevertheless, for the 1895-1915 MHz and 1915-1920 MHz frequency ranges, the note 26 was also added. This note states that “For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band”.

Similar notification exists for the coexistence between other bands, e.g. n7 and n38.

- n106 coexistence with n5/n26. The protection limit for was specified to -30 dBm/MHz for n106 to protect those 2 bands. But this was agreed since band n106 is only supporting 3 MHz channel BW and that the n106 channel raster adjacent to those bands are excluded from the specification.

Observation5: Less stringent (than -50dBm/MHz) coexistence limits exist for some bands in TS 38.101-1 but allowing then interference in the adjacent band(s) or considering an usage restriction of the considered band.

## UE-UE coexistence with n2/n25

According to the RAN4 contribution mentioned in the WF ([4]), the -50dBm/MHz coexistence limit would guarantee the UE-UE coexistence when UEs are separated with ~1m distance (MCL~37dB), even if the transmitting UE would use 23 dBm power.

With a separation distance of 1500m, the path loss (free space) would be 102dB. With such PL value, if this separation distance (between NTN S-band UEs and TN n2/n25 UEs) could be guaranteed, relaxation could then be considered for the new S-band UE coexistence requirement with TN bands n2/n25. Nevertheless, as we explained previously, this 1500m isolation distance was an assumption for the NTN-TN coexistence study but RAN4 didn’t evaluate what would be the minimum isolation distance for NTN and TN to coexist considering the specified NTN UE ACS (33dBc).

Also, to better progress on this topic and converge on some potential relaxation, input from filter vendors would be useful to determine what attenuation could be expected at 1990 MHz and 1995 MHz. Still, it would be tough to get any major rejection 5 MHz away from the band edge.

To facilitate new NTN S-band coexistence with n2/n25, RAN4 should also further discuss any possible scheduling restriction.

Proposal: Continue investigating possible UE coexistence requirement with the bands n2 and n25 by:

- Evaluate the minimum isolation distance for NTN and TN to coexist assuming an NTN UE ACS of 33 dBc.

- Collect inputs from filter performance to determine an achievable attenuation at 1990 MHz and 1995 MHz for such new S-band (2000-2020 MHz) filter.

- Consider possible scheduling restriction in the new NTN S-band to facilitate coexistence.

# Conclusion

In this contribution, we further the new S-band UE coexistence requirement with TN bands n2 and n25. We made the following observations and proposal:

Observation1: For FR1-NTN, 1500m isolation distance was assumed between TN edge and a connected NTN UE to address TN-NTN coexistence issues.

Observation2: RAN4 didn’t check what is the minimum isolation distance for TN and NTN to coexist considering the specified NTN UE ACS value (33dB).

Observation3: No requirement has been specified to capture this assumed isolation distance and no priority mechanism (TN over NTN) exists.

Observation4: If this isolation distance is not respected, the NTN network performance might be impacted, not the TN performance. Also, the NTN should still be able to connect to the satellite.

Observation5: Less stringent (than -50dBm/MHz) coexistence limits exist for some bands in TS 38.101-1 but allowing then interference in the adjacent band(s) or considering an usage restriction of the considered band.

Proposal: Continue investigating possible UE coexistence requirement with the bands n2 and n25 by:

- Evaluate the minimum isolation distance for NTN and TN to coexist assuming an NTN UE ACS of 33 dBc.

- Collect inputs from filter performance to determine an achievable attenuation at 1990 MHz and 1995 MHz for such new S-band (2000-2020 MHz) filter.

- Consider possible scheduling restriction in the new NTN S-band to facilitate coexistence.

# References

- RP-241661, New WID on Introduction of NR-NTN S-band (MSS band 2000-2020 MHz UL and 2180-2200 MHz DL), DISH Network, EchoStar, TerreStar, OmniSpace, Telus

- TR 36.811, Adding 2 GHz band LTE Frequency Division Duplex (FDD) (Band 23) for Ancillary Terrestrial Component (ATC) of Mobile Satellite Services (MSS) in North America

- R4-2414287, WF on Rel-19 FR1 NTN bands

- R4-080710, TS36.101: TP for UE Spurious emission limits, Motorola

- TR 38.863, Non-terrestrial networks (NTN) related RF and co-existence aspects