# 3GPP TSG-RAN WG4 Meeting #112bis	R4-2416161

# Hefei, Anhui, China, 14th – 18th October, 2024

Source:			Ericsson

Title:			mmWave in NR: EESS protection – UE RF aspects

Agenda item:		5.21.2

Document for:		Discussion

# Introduction

In last RAN#105 meeting, a new WI was adopted to consider China and EC Regulation related to the 24GHz band updates and the EESS protection.

This contribution is discussing the impacts on UE RF aspects.

# Discussion

## Background

When RAN4 specified the mmWave NR bands n257 and n258, ITU-R regulation and WRC-19 Resolution 750 was considered.

On UE side, this was captured partly in TS 38.101-2, with the following additional spurious requirements handled via NS_202 and NS_203 as shown below:

Only the first stage limit (-29dBW/200MHz) from WRC-19 Resolution 750 is captured, the second stage is not mentioned.

Observation: TS 38.101-2 only captures the first stage limit from WRC-19 Resolution 750 (valid until 1 September 2027 when following ITU-R RR).

## Europe and China situation

In April 2020, the EU Decision 2019/784 ([2]) was amended with the Commission Implementing Decision 2020/590 ([3]). Based on CEPT recommendation ([4]), the EU decided to set the transition to the second stage limit to an earlier date (1 January 2024) , this to better protect EESS in the 23.6-24.0 GHz frequency range while early enabling 26GHz deployment. Such decision is binding to all EU countries.

In China, the more stringent limit (-5dBm/200 MHz) applies as well.

## TS 38.101-2 impacts

As mentioned earlier, the second stage limit (more stringent) should be captured in TS 38.101-2.

NS_202 was initially specified to handle ERC 74-02 spurious emission limits ([6]). It was then updated ([7]) to consider potential EU emission limits of +1dBm/200MHz discussed in RSCOM but, as far as we investigated, no EC Decision was published based on such discussion. Rather, as we mentioned previously, the amended EU Decision specified a limit of -5dBm/200 MHz which was not considered in TS 38.101-2.

Considering that this -5dBm/200MHz is applicable from 1 January 2024 for any device sold on the EU market (even if it was produced earlier), our proposal is to update NS_202, changing the 1dBm/200MHz to -5dBm/200 MHz limit.

Proposal1: Update NS_202 (and CA_NS_202) changing the 1dBm/200MHz limit to -5dBm/200MHz.

NS_203 was specified to handle the WRC-19 Resolution 750 ([8]). It specifies only the first stage limit of 1dBm/200MHz. To address China’s situation and anticipate the second stage limit which will be appliable from 1 September 2027, we would make the following proposal:

Proposal2: Specify a new NS_205 to manage the second stage limit of -5dBm/200 MHz in the 23.6-24.0 GHz frequency range.  A corresponding new NS CA_NS_205 for CA should also be specified.

It should be noted that NS_203 would become obsolete from 1 September 2027 but should still be used to handle legacy UEs already in the market, only supporting this NS_203 and not NS_205. To properly handle new and legacy UEs, the network will have to send NS_205 and NS_203 in this order.

Also, to address the handover issue where only one NS could be indicated in the RRC configuration, RAN4 should adopt the same approach that was used for NS_203, indicating NS_205 support in the modifiedMPR-behavior field.

Corresponding A-MPR simulations should be made to evaluate if any backoff would be needed on UE side to handle this more stringent limit for the updated NS_202 and the new NS_205.

Our companion draft CR ([5]) proposes an initial draft of expected changes. Note that this draft CR is made on Rel-18 specifications to highlight all possible changes but they should be done to earlier release as well.

# Conclusion

In this contribution, we analyzed the impacts of Chinese and European regulation related to the EESS protection in the 23.6-24.0 GHz frequency range. We made the following observation and proposal:

Observation: TS 38.101-2 only captures the first stage limit from WRC-19 Resolution 750 (valid until 1 September 2027 when following ITU-R RR).

Proposal1: Update NS_202 (and CA_NS_202) changing the 1dBm/200MHz limit to -5dBm/200MHz.

Proposal2: Specify a new NS_205 to manage the second stage limit of -5dBm/200 MHz in the 23.6-24.0 GHz frequency range. This NS CA_NS_205 should only be used from 1 September 2027. A corresponding new NS for CA should also be specified.

# References

- RP-242371, WID on mmWave UE spurious emssion, China Unicom, Deutsche Telekom, Vodafone, CATT, Huawei, HiSilicon, OPPO, Qualcomm, Xiaomi, ZTE

- EU Decision 2019/784, Commission Implementing Decision (EU) 2019/784 of 14 May 2019 on harmonisation of the 24,25-27,5 GHz frequency band for terrestrial systems capable of providing wireless broadband electronic communications services in the Union.

- EU Decision 2020/590, Commission Implementing Decision (EU) 2020/590 of 24 April 2020 amending Decision (EU) 2019/784 as regards an update of relevant technical conditions applicable to the 24,25-27,5 GHz frequency band.

- ECC(20) 055 Annex 18, CEPT letter dated 6 March 2020 ‘CEPT response on additional input regarding the impact of the WRC-19 outcome on the harmonised technical conditions for the 26 GHz band’

- R4-2416163, draft CR to TS 38.101-2 – EESS protection in the 23.6-24.0 GHz frequency range, Ericsson

- R4-1910261, dCR to 38.101-2: FR2 A-MPR updates, including ERC 74-01 changes, Qualcomm

- R4-2006785, CR to 38.101-2: NS_202 update after changes to EU regulations, Qualcomm

- R4-2016785, EESS protection related requirements for FR2 bands, Nokia, Nokia Shanghai Bell, NTT DOCOMO, INC