3GPP TSG-RAN WG4 Meeting #112bis                                                             R4-2416165

# Hefei, Anhui, China, 14th – 18th October, 2024

Agenda item:	6.12.2.3

Source:	CMCC

Title:		Reference values for FR1 dynamic channel models validation

Document for:	Approval

- Introduction

In the last RAN4 #112 meeting, some companies have agreed that 3GPP adopted the work done by CTIA on FR1 MIMO OTA dynamic testing, and some channel model parameters have been given. In this contribution, based on the channel model parameters and the post processing of parameters from different routes, we present the reference values of PDP, TC in Uma scenario, which we hope is a good starting point of alignment among different CE vendors.

- Details

In this contribution, we give the BS and UE layout firstly to have a overview of dynamic scenario geometry, then the channel model parameters along the driving route are presented according to the interpolated method depicted in [1], finally we give the reference values of PDP, TC for later dynamic channel model validation.

- Layout

The BS and UE driving route for UMa scenario are shown in Fig.1. The BS is located at the original of the X-Y plane and the UE driving route is composed of 11 mid waypoints, lasting for 122.7s. Each waypoint is assigned a different 3GPP CDL model, together with orientations, speed and direction of travel (DoT) of the DUT.

Fig.1 BS and UE driving route for UMa scenario

- Channel parameters along the route

The channel parameters along the driving route are provided in following figures, where the dotted lines denote the mid waypoints.

Fig.2 UE speed and max Doppler along the route

Fig.3 UE LoS AOA in LCS along the route

Fig.4 UE DoT along the route

Fig.5 K-factor along the route

Fig.6 XPR along the route

Fig.7 Pathloss along the route

- PDP reference values

The theoretical PDP results at the centers of each segment are provided here for achieving a good alignment. It is noted that the following PDP results correspond to the X2V channel, i.e., ranging from the superposition of two orthogonally polarized co-located gNB antennas to the Dipole antennas at the center of test area, and the start and end time of each segment is reference to the Table 4.2.1.2- 1 in [1].

Fig.8 PDP reference value for segment 1

| Cluster Index | Delay [ns] | Power [dB] |
| --- | --- | --- |
| 1 | 0 | 0.0000 |
| 2 | 0 | -13.0456 |
| 3 | 3 | -37.3299 |
| 4 | 58 | -39.5299 |
| 5 | 132 | -18.0188 |
| 6 | 167 | -25.6402 |
| 7 | 170 | -20.2188 |
| 8 | 244 | -22.0188 |
| 9 | 380 | -35.5075 |
| 10 | 747 | -24.9978 |
| 11 | 887 | -31.3318 |

Fig.9 PDP reference value for segment 2

| Cluster Index | Delay [ns] | Power [dB] |
| --- | --- | --- |
| 1 | 0 | 0.0000 |
| 2 | 0 | -13.0347 |
| 3 | 132 | -18.6559 |
| 4 | 167 | -27.3587 |
| 5 | 170 | -20.8559 |
| 6 | 244 | -22.6559 |
| 7 | 380 | -32.2678 |
| 8 | 747 | -23.3364 |
| 9 | 887 | -33.9614 |

Fig.10 PDP reference value for segment 3

| Cluster Index | Delay [ns] | Power [dB] |
| --- | --- | --- |
| 1 | 0.0 | -10.1243 |
| 2 | 108.8 | 0.0000 |
| 3 | 114.4 | -1.7718 |
| 4 | 126.3 | -6.2313 |
| 5 | 142.8 | -8.1784 |
| 6 | 220.0 | -7.5531 |
| 7 | 224.1 | -3.3816 |
| 8 | 241.6 | -9.2661 |
| 9 | 259.3 | -8.5574 |
| 10 | 430.1 | -11.6823 |
| 11 | 503.2 | -9.1739 |
| 12 | 574.9 | -16.4985 |
| 13 | 636.1 | -10.2638 |
| 14 | 701.0 | -13.5217 |
| 15 | 855.1 | -12.2272 |
| 16 | 1053.1 | -12.8622 |
| 17 | 1517.1 | -29.3780 |
| 18 | 1647.9 | -29.0347 |
| 19 | 1824.5 | -16.6798 |
| 20 | 1888.4 | -19.1286 |
| 21 | 2049.7 | -16.7504 |
| 22 | 2163.9 | -32.1592 |
| 23 | 3060.3 | -30.0068 |

Fig.11 PDP reference value for segment 4

| Cluster Index | Delay [ns] | Power [dB] |
| --- | --- | --- |
| 1 | 0.0 | -15.1643 |
| 2 | 129.4 | 0.0000 |
| 3 | 136.6 | -2.0157 |
| 4 | 154.7 | -9.9660 |
| 5 | 179.1 | -12.0825 |
| 6 | 212.5 | -14.9898 |
| 7 | 217.2 | -3.8816 |
| 8 | 243.2 | -13.6463 |
| 9 | 271.4 | -6.8384 |
| 10 | 518.6 | -22.2269 |
| 11 | 631.2 | -6.4352 |
| 12 | 722.2 | -15.9795 |
| 13 | 754.6 | -20.1899 |
| 14 | 842.1 | -17.6036 |
| 15 | 895.9 | -12.2950 |
| 16 | 1094.6 | -11.2528 |
| 17 | 1495.9 | -28.2581 |
| 18 | 1630.9 | -29.2025 |
| 19 | 1715.1 | -23.0873 |
| 20 | 1792.0 | -18.1911 |
| 21 | 1895.2 | -17.2400 |
| 22 | 2005.2 | -30.3826 |
| 23 | 3371.6 | -27.9113 |

Fig.12 PDP reference value for segment 5

| Cluster Index | Delay [ns] | Power [dB] |
| --- | --- | --- |
| 1 | 0.0 | -8.2798 |
| 2 | 84.1 | -0.7479 |
| 3 | 87.8 | -3.6222 |
| 4 | 92.2 | -3.3845 |
| 5 | 99.3 | -5.1432 |
| 6 | 229.0 | 0.0000 |
| 7 | 232.3 | -1.6110 |
| 8 | 239.6 | -3.7007 |
| 9 | 244.8 | -10.3065 |
| 10 | 323.9 | -9.8928 |
| 11 | 349.5 | -13.3548 |
| 12 | 398.1 | -22.4587 |
| 13 | 493.8 | -7.7517 |
| 14 | 531.7 | -13.5244 |
| 15 | 806.1 | -16.1241 |
| 16 | 1003.4 | -19.0477 |
| 17 | 1542.6 | -23.0765 |
| 18 | 1668.3 | -21.4887 |
| 19 | 1955.8 | -23.7322 |
| 20 | 2004.0 | -22.4565 |
| 21 | 2235.1 | -20.2711 |
| 22 | 2354.4 | -26.5130 |
| 23 | 2686.8 | -33.0613 |

Fig.13 PDP reference value for segment 6

| Cluster Index | Delay [ns] | Power [dB] |
| --- | --- | --- |
| 1 | 0 | -11.5640 |
| 2 | 76 | -4.0651 |
| 3 | 79 | -8.5674 |
| 4 | 81 | -6.3651 |
| 5 | 85 | -8.0651 |
| 6 | 232 | 0.0000 |
| 7 | 235 | -2.2000 |
| 8 | 239 | -3.9000 |
| 9 | 240 | -7.1218 |
| 10 | 289 | -18.3911 |
| 11 | 299 | -11.1074 |
| 12 | 340 | -31.9934 |
| 13 | 447 | -14.4210 |
| 14 | 476 | -18.3379 |
| 15 | 790 | -23.7525 |
| 16 | 987 | -16.4108 |
| 17 | 1551 | -16.2732 |
| 18 | 1675 | -14.8267 |
| 19 | 1999 | -38.2988 |
| 20 | 2042 | -18.9111 |
| 21 | 2296 | -17.4961 |
| 22 | 2417 | -19.1721 |
| 23 | 2564 | -27.8109 |

Fig.14 PDP reference value for segment 7

| Cluster Index | Delay [ns] | Power [dB] |
| --- | --- | --- |
| 1 | 0 | 0.0000 |
| 2 | 0 | -21.1407 |
| 3 | 47 | -18.7029 |
| 4 | 49 | -21.0029 |
| 5 | 49 | -25.5636 |
| 6 | 51 | -22.7029 |
| 7 | 65 | -20.4918 |
| 8 | 173 | -16.9062 |
| 9 | 175 | -19.1062 |
| 10 | 178 | -20.9062 |
| 11 | 240 | -20.3780 |
| 12 | 337 | -24.2656 |
| 13 | 495 | -19.2460 |
| 14 | 1091 | -32.1996 |
| 15 | 1876 | -32.1379 |

Fig.15 PDP reference value for segment 8

| Cluster Index | Delay [ns] | Power [dB] |
| --- | --- | --- |
| 1 | 0.0 | 0.0000 |
| 2 | 0.0 | -22.3567 |
| 3 | 105.6 | -1.3327 |
| 4 | 111.4 | -3.5374 |
| 5 | 124.7 | -7.3177 |
| 6 | 143.3 | -11.1518 |
| 7 | 156.7 | -12.3719 |
| 8 | 199.1 | -5.6099 |
| 9 | 218.9 | -10.9026 |
| 10 | 241.0 | -11.1927 |
| 11 | 443.7 | -17.7234 |
| 12 | 562.3 | -9.2238 |
| 13 | 683.4 | -19.5894 |
| 14 | 912.2 | -21.4233 |
| 15 | 1259.9 | -19.2463 |
| 16 | 915.0 | -22.7276 |
| 17 | 1114.0 | -23.9646 |
| 18 | 1486.0 | -19.1889 |
| 19 | 1623.0 | -21.6725 |
| 20 | 1664.0 | -22.3623 |
| 21 | 1747.0 | -29.4530 |
| 22 | 1823.0 | -31.9750 |
| 23 | 1931.0 | -23.6173 |
| 24 | 3517.0 | -34.8736 |

Fig.16 PDP reference value for segment 9

| Cluster Index | Delay [ns] | Power [dB] |
| --- | --- | --- |
| 1 | 0 | 0.0000 |
| 2 | 0 | -13.0439 |
| 3 | 3 | -33.4289 |
| 4 | 58 | -35.6289 |
| 5 | 128 | -37.4289 |
| 6 | 132 | -17.4491 |
| 7 | 167 | -24.1259 |
| 8 | 170 | -19.6491 |
| 9 | 244 | -21.4491 |
| 10 | 380 | -38.3250 |
| 11 | 747 | -26.4342 |
| 12 | 887 | -29.0313 |
| 13 | 1178 | -38.2448 |

- TC reference values

Moreover, the theoretical TC results at the center of each segment for the X2V channel are also provided.

Fig.17 TC reference value for segment 1

| Time [lambda] | TC |
| --- | --- |
| 0 | 1.0000 |
| 0.1 | 0.9919 |
| 0.2 | 0.9711 |
| 0.3 | 0.9464 |
| 0.4 | 0.9277 |
| 0.5 | 0.9206 |
| 0.6 | 0.9239 |
| 0.7 | 0.9311 |
| 0.8 | 0.9354 |
| 0.9 | 0.9333 |
| 1 | 0.9263 |
| 1.1 | 0.9184 |
| 1.2 | 0.9139 |
| 1.3 | 0.9143 |
| 1.4 | 0.9180 |
| 1.5 | 0.9220 |
| 1.6 | 0.9241 |
| 1.7 | 0.9241 |
| 1.8 | 0.9233 |
| 1.9 | 0.9230 |
| 2 | 0.9236 |
| 2.1 | 0.9240 |
| 2.2 | 0.9228 |
| 2.3 | 0.9201 |
| 2.4 | 0.9172 |
| 2.5 | 0.9160 |
| 2.6 | 0.9174 |
| 2.7 | 0.9205 |
| 2.8 | 0.9232 |
| 2.9 | 0.9234 |
| 3 | 0.9205 |
| 3.1 | 0.9162 |
| 3.2 | 0.9132 |
| 3.3 | 0.9137 |
| 3.4 | 0.9176 |
| 3.5 | 0.9230 |
| 3.6 | 0.9269 |
| 3.7 | 0.9278 |
| 3.8 | 0.9257 |
| 3.9 | 0.9222 |
| 4 | 0.9192 |
| 4.1 | 0.9176 |
| 4.2 | 0.9169 |
| 4.3 | 0.9163 |
| 4.4 | 0.9153 |
| 4.5 | 0.9144 |
| 4.6 | 0.9148 |
| 4.7 | 0.9172 |
| 4.8 | 0.9208 |
| 4.9 | 0.9241 |
| 5 | 0.9254 |

Fig.18 TC reference value for segment 2

| Time [lambda] | TC |
| --- | --- |
| 0 | 1.0000 |
| 0.1 | 0.9918 |
| 0.2 | 0.9717 |
| 0.3 | 0.9500 |
| 0.4 | 0.9366 |
| 0.5 | 0.9349 |
| 0.6 | 0.9402 |
| 0.7 | 0.9454 |
| 0.8 | 0.9458 |
| 0.9 | 0.9418 |
| 1 | 0.9363 |
| 1.1 | 0.9317 |
| 1.2 | 0.9282 |
| 1.3 | 0.9248 |
| 1.4 | 0.9214 |
| 1.5 | 0.9192 |
| 1.6 | 0.9194 |
| 1.7 | 0.9216 |
| 1.8 | 0.9234 |
| 1.9 | 0.9229 |
| 2 | 0.9198 |
| 2.1 | 0.9161 |
| 2.2 | 0.9143 |
| 2.3 | 0.9155 |
| 2.4 | 0.9185 |
| 2.5 | 0.9215 |
| 2.6 | 0.9236 |
| 2.7 | 0.9250 |
| 2.8 | 0.9265 |
| 2.9 | 0.9279 |
| 3 | 0.9283 |
| 3.1 | 0.9269 |
| 3.2 | 0.9244 |
| 3.3 | 0.9226 |
| 3.4 | 0.9231 |
| 3.5 | 0.9259 |
| 3.6 | 0.9292 |
| 3.7 | 0.9311 |
| 3.8 | 0.9310 |
| 3.9 | 0.9295 |
| 4 | 0.9280 |
| 4.1 | 0.9266 |
| 4.2 | 0.9248 |
| 4.3 | 0.9222 |
| 4.4 | 0.9192 |
| 4.5 | 0.9173 |
| 4.6 | 0.9172 |
| 4.7 | 0.9186 |
| 4.8 | 0.9197 |
| 4.9 | 0.9193 |
| 5 | 0.9177 |

Fig.19 TC reference value for segment 3

| Time [lambda] | TC |
| --- | --- |
| 0 | 1.0000 |
| 0.1 | 0.9381 |
| 0.2 | 0.7674 |
| 0.3 | 0.5310 |
| 0.4 | 0.3035 |
| 0.5 | 0.2252 |
| 0.6 | 0.3064 |
| 0.7 | 0.3666 |
| 0.8 | 0.3598 |
| 0.9 | 0.3055 |
| 1 | 0.2465 |
| 1.1 | 0.2185 |
| 1.2 | 0.2100 |
| 1.3 | 0.1862 |
| 1.4 | 0.1365 |
| 1.5 | 0.0898 |
| 1.6 | 0.1090 |
| 1.7 | 0.1603 |
| 1.8 | 0.1913 |
| 1.9 | 0.1890 |
| 2 | 0.1570 |
| 2.1 | 0.1072 |
| 2.2 | 0.0541 |
| 2.3 | 0.0177 |
| 2.4 | 0.0353 |
| 2.5 | 0.0544 |
| 2.6 | 0.0638 |
| 2.7 | 0.0667 |
| 2.8 | 0.0688 |
| 2.9 | 0.0761 |
| 3 | 0.0895 |
| 3.1 | 0.1035 |
| 3.2 | 0.1118 |
| 3.3 | 0.1123 |
| 3.4 | 0.1083 |
| 3.5 | 0.1057 |
| 3.6 | 0.1067 |
| 3.7 | 0.1071 |
| 3.8 | 0.1021 |
| 3.9 | 0.0928 |
| 4 | 0.0873 |
| 4.1 | 0.0931 |
| 4.2 | 0.1064 |
| 4.3 | 0.1179 |
| 4.4 | 0.1226 |
| 4.5 | 0.1206 |
| 4.6 | 0.1146 |
| 4.7 | 0.1075 |
| 4.8 | 0.1010 |
| 4.9 | 0.0961 |
| 5 | 0.0927 |

Fig.20 TC reference value for segment 4

| Time [lambda] | TC |
| --- | --- |
| 0 | 1.0000 |
| 0.1 | 0.9491 |
| 0.2 | 0.8125 |
| 0.3 | 0.6397 |
| 0.4 | 0.5113 |
| 0.5 | 0.4932 |
| 0.6 | 0.5425 |
| 0.7 | 0.5720 |
| 0.8 | 0.5405 |
| 0.9 | 0.4479 |
| 1 | 0.3157 |
| 1.1 | 0.1737 |
| 1.2 | 0.0540 |
| 1.3 | 0.0589 |
| 1.4 | 0.1073 |
| 1.5 | 0.1251 |
| 1.6 | 0.1219 |
| 1.7 | 0.1111 |
| 1.8 | 0.0999 |
| 1.9 | 0.0868 |
| 2 | 0.0676 |
| 2.1 | 0.0419 |
| 2.2 | 0.0240 |
| 2.3 | 0.0434 |
| 2.4 | 0.0715 |
| 2.5 | 0.0923 |
| 2.6 | 0.1013 |
| 2.7 | 0.0972 |
| 2.8 | 0.0810 |
| 2.9 | 0.0556 |
| 3 | 0.0295 |
| 3.1 | 0.0326 |
| 3.2 | 0.0600 |
| 3.3 | 0.0837 |
| 3.4 | 0.0955 |
| 3.5 | 0.0927 |
| 3.6 | 0.0776 |
| 3.7 | 0.0611 |
| 3.8 | 0.0616 |
| 3.9 | 0.0785 |
| 4 | 0.0928 |
| 4.1 | 0.0949 |
| 4.2 | 0.0851 |
| 4.3 | 0.0724 |
| 4.4 | 0.0692 |
| 4.5 | 0.0749 |
| 4.6 | 0.0766 |
| 4.7 | 0.0670 |
| 4.8 | 0.0500 |
| 4.9 | 0.0467 |
| 5 | 0.0681 |

Fig.21 TC reference value for segment 5

| Time [lambda] | TC |
| --- | --- |
| 0 | 1.0000 |
| 0.1 | 0.9553 |
| 0.2 | 0.8443 |
| 0.3 | 0.7283 |
| 0.4 | 0.6673 |
| 0.5 | 0.6577 |
| 0.6 | 0.6429 |
| 0.7 | 0.5876 |
| 0.8 | 0.5059 |
| 0.9 | 0.4434 |
| 1 | 0.4271 |
| 1.1 | 0.4302 |
| 1.2 | 0.4173 |
| 1.3 | 0.3869 |
| 1.4 | 0.3636 |
| 1.5 | 0.3616 |
| 1.6 | 0.3655 |
| 1.7 | 0.3568 |
| 1.8 | 0.3386 |
| 1.9 | 0.3277 |
| 2 | 0.3284 |
| 2.1 | 0.3250 |
| 2.2 | 0.3062 |
| 2.3 | 0.2832 |
| 2.4 | 0.2793 |
| 2.5 | 0.2972 |
| 2.6 | 0.3129 |
| 2.7 | 0.3092 |
| 2.8 | 0.2924 |
| 2.9 | 0.2833 |
| 3 | 0.2910 |
| 3.1 | 0.3006 |
| 3.2 | 0.2962 |
| 3.3 | 0.2802 |
| 3.4 | 0.2689 |
| 3.5 | 0.2712 |
| 3.6 | 0.2758 |
| 3.7 | 0.2687 |
| 3.8 | 0.2505 |
| 3.9 | 0.2357 |
| 4 | 0.2338 |
| 4.1 | 0.2360 |
| 4.2 | 0.2297 |
| 4.3 | 0.2162 |
| 4.4 | 0.2098 |
| 4.5 | 0.2178 |
| 4.6 | 0.2297 |
| 4.7 | 0.2335 |
| 4.8 | 0.2305 |
| 4.9 | 0.2318 |
| 5 | 0.2420 |

Fig.22 TC reference value for segment 6

| Time [lambda] | TC |
| --- | --- |
| 0 | 1.0000 |
| 0.1 | 0.9388 |
| 0.2 | 0.7901 |
| 0.3 | 0.6515 |
| 0.4 | 0.6177 |
| 0.5 | 0.6513 |
| 0.6 | 0.6480 |
| 0.7 | 0.5714 |
| 0.8 | 0.4678 |
| 0.9 | 0.4255 |
| 1 | 0.4593 |
| 1.1 | 0.4909 |
| 1.2 | 0.4795 |
| 1.3 | 0.4512 |
| 1.4 | 0.4528 |
| 1.5 | 0.4830 |
| 1.6 | 0.5011 |
| 1.7 | 0.4911 |
| 1.8 | 0.4785 |
| 1.9 | 0.4934 |
| 2 | 0.5231 |
| 2.1 | 0.5308 |
| 2.2 | 0.5041 |
| 2.3 | 0.4681 |
| 2.4 | 0.4582 |
| 2.5 | 0.4722 |
| 2.6 | 0.4752 |
| 2.7 | 0.4504 |
| 2.8 | 0.4182 |
| 2.9 | 0.4117 |
| 3 | 0.4280 |
| 3.1 | 0.4332 |
| 3.2 | 0.4098 |
| 3.3 | 0.3763 |
| 3.4 | 0.3664 |
| 3.5 | 0.3827 |
| 3.6 | 0.3951 |
| 3.7 | 0.3882 |
| 3.8 | 0.3776 |
| 3.9 | 0.3857 |
| 4 | 0.4052 |
| 4.1 | 0.4095 |
| 4.2 | 0.3890 |
| 4.3 | 0.3646 |
| 4.4 | 0.3650 |
| 4.5 | 0.3860 |
| 4.6 | 0.3974 |
| 4.7 | 0.3854 |
| 4.8 | 0.3662 |
| 4.9 | 0.3648 |
| 5 | 0.3783 |

Fig.23 TC reference value for segment 7

| Time [lambda] | TC |
| --- | --- |
| 0 | 1.0000 |
| 0.1 | 0.9901 |
| 0.2 | 0.9639 |
| 0.3 | 0.9301 |
| 0.4 | 0.8986 |
| 0.5 | 0.8758 |
| 0.6 | 0.8630 |
| 0.7 | 0.8577 |
| 0.8 | 0.8571 |
| 0.9 | 0.8591 |
| 1 | 0.8641 |
| 1.1 | 0.8733 |
| 1.2 | 0.8882 |
| 1.3 | 0.9080 |
| 1.4 | 0.9287 |
| 1.5 | 0.9440 |
| 1.6 | 0.9488 |
| 1.7 | 0.9415 |
| 1.8 | 0.9251 |
| 1.9 | 0.9058 |
| 2 | 0.8892 |
| 2.1 | 0.8787 |
| 2.2 | 0.8740 |
| 2.3 | 0.8734 |
| 2.4 | 0.8747 |
| 2.5 | 0.8775 |
| 2.6 | 0.8825 |
| 2.7 | 0.8910 |
| 2.8 | 0.9036 |
| 2.9 | 0.9180 |
| 3 | 0.9298 |
| 3.1 | 0.9345 |
| 3.2 | 0.9299 |
| 3.3 | 0.9175 |
| 3.4 | 0.9018 |
| 3.5 | 0.8881 |
| 3.6 | 0.8798 |
| 3.7 | 0.8779 |
| 3.8 | 0.8806 |
| 3.9 | 0.8852 |
| 4 | 0.8901 |
| 4.1 | 0.8949 |
| 4.2 | 0.9005 |
| 4.3 | 0.9075 |
| 4.4 | 0.9153 |
| 4.5 | 0.9211 |
| 4.6 | 0.9220 |
| 4.7 | 0.9161 |
| 4.8 | 0.9047 |
| 4.9 | 0.8914 |
| 5 | 0.8807 |

Fig.24 TC reference value for segment 8

| Time [lambda] | TC |
| --- | --- |
| 0 | 1.0000 |
| 0.1 | 0.9340 |
| 0.2 | 0.7800 |
| 0.3 | 0.6603 |
| 0.4 | 0.6710 |
| 0.5 | 0.7312 |
| 0.6 | 0.7183 |
| 0.7 | 0.6015 |
| 0.8 | 0.4405 |
| 0.9 | 0.3604 |
| 1 | 0.3926 |
| 1.1 | 0.4053 |
| 1.2 | 0.3276 |
| 1.3 | 0.1753 |
| 1.4 | 0.0510 |
| 1.5 | 0.1492 |
| 1.6 | 0.1936 |
| 1.7 | 0.1717 |
| 1.8 | 0.2010 |
| 1.9 | 0.3277 |
| 2 | 0.4469 |
| 2.1 | 0.5014 |
| 2.2 | 0.4983 |
| 2.3 | 0.5009 |
| 2.4 | 0.5635 |
| 2.5 | 0.6514 |
| 2.6 | 0.6989 |
| 2.7 | 0.6834 |
| 2.8 | 0.6386 |
| 2.9 | 0.6242 |
| 3 | 0.6537 |
| 3.1 | 0.6767 |
| 3.2 | 0.6481 |
| 3.3 | 0.5709 |
| 3.4 | 0.4908 |
| 3.5 | 0.4532 |
| 3.6 | 0.4430 |
| 3.7 | 0.4094 |
| 3.8 | 0.3300 |
| 3.9 | 0.2235 |
| 4 | 0.1327 |
| 4.1 | 0.0874 |
| 4.2 | 0.0500 |
| 4.3 | 0.0305 |
| 4.4 | 0.1250 |
| 4.5 | 0.2179 |
| 4.6 | 0.2814 |
| 4.7 | 0.3156 |
| 4.8 | 0.3504 |
| 4.9 | 0.4119 |
| 5 | 0.4854 |

Fig.25 TC reference value for segment 9

| Time [lambda] | TC |
| --- | --- |
| 0 | 1.0000 |
| 0.1 | 0.9939 |
| 0.2 | 0.9806 |
| 0.3 | 0.9701 |
| 0.4 | 0.9663 |
| 0.5 | 0.9644 |
| 0.6 | 0.9584 |
| 0.7 | 0.9491 |
| 0.8 | 0.9429 |
| 0.9 | 0.9429 |
| 1 | 0.9450 |
| 1.1 | 0.9436 |
| 1.2 | 0.9390 |
| 1.3 | 0.9365 |
| 1.4 | 0.9387 |
| 1.5 | 0.9421 |
| 1.6 | 0.9414 |
| 1.7 | 0.9366 |
| 1.8 | 0.9332 |
| 1.9 | 0.9351 |
| 2 | 0.9397 |
| 2.1 | 0.9415 |
| 2.2 | 0.9395 |
| 2.3 | 0.9375 |
| 2.4 | 0.9394 |
| 2.5 | 0.9434 |
| 2.6 | 0.9449 |
| 2.7 | 0.9424 |
| 2.8 | 0.9393 |
| 2.9 | 0.9387 |
| 3 | 0.9394 |
| 3.1 | 0.9378 |
| 3.2 | 0.9328 |
| 3.3 | 0.9284 |
| 3.4 | 0.9283 |
| 3.5 | 0.9313 |
| 3.6 | 0.9329 |
| 3.7 | 0.9312 |
| 3.8 | 0.9289 |
| 3.9 | 0.9297 |
| 4 | 0.9332 |
| 4.1 | 0.9354 |
| 4.2 | 0.9342 |
| 4.3 | 0.9317 |
| 4.4 | 0.9314 |
| 4.5 | 0.9334 |
| 4.6 | 0.9343 |
| 4.7 | 0.9322 |
| 4.8 | 0.9295 |
| 4.9 | 0.9296 |
| 5 | 0.9327 |

Observation 1: PDP and TC reference values of each segment are given in our contribution, and there are some gap between CMCC’s results and reference values in [1].

Proposals 1: RAN4 can organize the dynamic channel model reference value alignment among CE vendors before the pass/fail limits can be determined.

3	Conclusions

Observation 1: PDP and TC reference values of each segment are given in our contribution, and there are some gap between CMCC’s results and reference values in [1].

Proposals 1: RAN4 can organize the dynamic channel model reference value alignment among CE vendors before the pass/fail limits can be determined.

4 References

[1] CTIA 01.42 Test Methodology, MIMO, Multi-Probe Anechoic Chamber, Dynamic Channel Model with Link Adaptation