3GPP TSG-RAN WG4 Meeting #112bis	R4-2416169

Hefei, China, October 14th-18th 2024

Agenda Item:	6.2.4.2

Source:	Ericsson

Title:	On 15 GHz BS-UE radio and antenna parameters

Document for:	Approval

# 1	Introduction

ITU-R WP5D sent a LS [1] to 3GPP RAN seeking information on terrestrial component IMT-2030 parameters to be used for sharing and compatibility studies in preparation for WRC-27, for frequency ranges 4400 to 4800 MHz, 7125 to 8400 MHz, and 14800 to 15350 MHz.

In the previous 3GPP RAN4#110-bis meeting, the Work Plan [2] for delivery of information to ITU-R LS for those respective frequency ranges was agreed, following a phase approach to send information in 3 phases as below -

- Track 1: 4400 to 4800 MHz, the estimated date for completion is latest by May 2024 (RAN4#111).

- Track 2: 7125 to 8400 MHz, the estimated date for completion is latest by August 2024 (RAN4#112).

- Track 3: 14800 to 15350 MHz, the estimated date for completion is latest by November 2024 (RAN4#113).

WF [3] [4] and TR 38.922 v0.2.0 [4] captured initial agreements on co-existence simulation assumptions for this frequency range.

This contribution provides further discussion points on radio and antenna parameters relevant for 14800 to 15350 MHz frequency range.

# 2	Discussion

## 2.1	BS antenna characteristics

### 2.1.1	BS array and sub-array antenna size

In the previous RAN4 meeting, following BS array and sub-array sizes are recommended:

A larger sub-array would limit the coverage angular range (CAR) due to the narrow sub-array radiation pattern along the vertical axis. A narrow vertical range limits operation to deployment scenarios where UE only are spread out along the horizontal axis. In addition, a large sub-array will also introduce high transmission losses in the Radio Distribution Network (RDN) at 15 GHz. For sub-arrays larger than 4 elements, the loss assumed to derive the peak element gain GE,max need to be reconsidered. As a result, we propose to consider sub-arrays of 4x1 for Urban Macro, while 8x1 would be more adequate for a rural deployment where UEs are on the same elevation plane.

In Urban Macro scenario, RAN4 to consider sub-array size of 4x1 and drop 8x1 as a larger sub-array would limit the coverage angular range due to narrow sub-array radiation pattern in vertical axis.

In our companion contribution [7], we have proposed a couple of combinations for BS-UE antenna configurations by selecting the one that provides the most restrictive requirements for Urban Macro scenario.

The rationale is to go for a liberal approach with larger antenna arrays for higher antenna again. It however comes with its own design and implementation challenges but cannot be ruled out at this stage.

RAN4 to consider following BS array antenna architectures:

- Urban Macro: 4096 AEs with 16x32 array and 4x1 sub-array

- Indoor Hotspot: 4x4

- Dense Urban: Same HW as Urban Macro

### 2.1.2	BS array antenna parameters

Table 2.1.1: Antenna parameters relevant for 14800 to 15350 MHz

| Parameter | Macro suburban | Macro urban | Micro urban | Indoor |
| --- | --- | --- | --- | --- |
| Am | 30 dB | 30 dB | 30 dB | 30 dB |
| SLAv | 30 dB | 30 dB | 30 dB | 30 dB |
| 3dB | 90 deg. | 90 deg. | 90 deg. | 90 deg. |
| 3dB | 65 deg. | 65 deg. | 65 deg. | 90 deg. |
| GE,max | 6.4 dBi | 6.4 dBi | 6.4 dBi | 5 dBi |
| Msub | 4 | 4 | 4 | N/A |
| dv,sub | 0.7l m | 0.7l m | 0.7l m | 0.5l m |
| subtilt | 3 deg. | 3 deg. | 3 deg. | N/A |
| M | 16 | 16 | 16 | 4 |
| N | 32 | 32 | 32 | 4 |
| dh | 0.5l m | 0.5l m | 0.5l m | 0.5l m |
| dv | 2.8l m | 2.8l m | 2.8l m | 0.5l m |
| etilt | 90≤θescan≤100 deg. | 90≤θescan≤100 deg. | 90≤θescan≤100 deg. | 0≤θescan≤180 deg. |
| escan | -60≤φescan≤60 deg. | -60≤φescan≤60 deg. | -60≤φescan≤60 deg. | -90≤φescan≤90 deg. |
| r | 1 | 1 | 1 | 1 |
| Ptx | 46 dBm | 46 dBm | 36 dBm | 23 dBm |
| mech | 6 deg. | 6 deg. | 6 deg. | 90 deg. |

## 2.2	UE antenna characteristics

In the previous RAN4 meeting, following UE types were recommended:

Some companies have raised concerns over choice for a particular UE RF architecture as a trade-off between implementation feasibility for the handheld devices and anticipated performance or coverage. On that basis, UE antenna assumptions were categorized on FR1 and FR2 UE type. 

In our opinion, the choice of UE antenna configuration cannot be independently decided without considering the choice of possible BS antenna configuration, to converge on the combination with the most stringent requirements. As discussed in our companion contribution [7], the most restrictive requirements are derived from the Combination 1 – 4096 Antenna Elements (AE) (in total) (16x32 and 4x1 sub-array) and FR1-like UE - 1TX assumption with omnidirectional UE antenna.

If we had to propose two different UE antenna configurations to ITU-R depending on the use-case, this would only lead to further confusion and non-alignment between the proposed parameter sets.

RAN4 to converge on the combination of the BS-UE antenna configuration that seems the most viable based on implementation feasibility aspects, to achieve a reasonable coverage.

Considering the given timeline to agree on the parameters, RAN4 to consider the combination 4096 Antenna Elements (AE) (in total) (16x32 and 4x1 sub-array) and FR1-like UE - 1TX assumption with omnidirectional UE antenna as baseline.

In general, we observe dependency of parameters on other, e.g. the antenna architectures at both BS and UEs, the power levels in UL and DL, the ISD, the channel bandwidth, etc. As a result, to make progress, RAN4 should start by tentatively agreeing on some system parameters and then confirm them only after validating the overall system feasibility.

Most of the system parameters are dependent on other, so to make progress RAN4 should start by tentatively agreeing on parameters, and then confirm them only after verifying the overall system feasibility.

## 2.3	General parameters

#### 2.3.1	Duplex mode

In the previous RAN4#111 and RAN4#112 meetings, it has been agreed on TDD as baseline duplexing for the 14800 – 15350 MHz frequency range for the LS response to WP5D, but also capture additional information related to SBFD in the TR.

Following additional information has been suggested to be captured in the TR:

“There is no defined 3GPP band for the 14800 - 15350 MHz frequency range. Similar to the 4400 – 4800 MHz and 7125 – 8400 MHz frequency ranges, SBFD can be a candidate duplexing method for this frequency range. The core requirements for Rel-19 SBFD work item can be tracked through the list of impacted specs captured in [6]. To provide a timely response to WP5D regarding the requested RF parameters, RAN4 assumed TDD as a baseline duplexing for the 14800 – 15350 MHz frequency range.”

To remain consistent with the agreements for other frequency ranges in the TR, it is most unlikely that FDD will be used for this frequency range. However, additional text should be added stating FDD is not precluded.

The additional information should be updated with the following text in the TR – “There is no defined 3GPP band for the 14800 - 15350 MHz frequency range. Similar to the 4400 – 4800 MHz and 7125 – 8400 MHz frequency ranges, RAN4 assumed TDD as a baseline duplexing for the 14800 – 15350 MHz frequency range even though FDD is not precluded. SBFD can be a candidate duplexing method for this frequency range. The core requirements for Rel-19 SBFD work item can be tracked through the list of impacted specs captured in [6].”

#### 2.3.2	Typical Channel Bandwidth

We suggest replying with a channel bandwidth which is already specified for NR, to avoid any conclusion on which maximum channel bandwidth should be considered. 100 MHz should be considered as the typical channel bandwidth for this frequency range, as wider channel bandwidths would need further study based on the establish regulations. However, wider channel bandwidths upto 400 MHz are not precluded and should be further studied when specifying the corresponding band.

But It is important to communicate to ITU-R some combinations of power/bandwidth which are feasible in terms of coverage and propose them as typical values in this band. Otherwise, if power and bandwidth remain untied, this leads to unclear Power Spectral Density (PSD), which plays a significant role in coexistence simulations, especially when there are coverage issues.

Power Spectral Density (PSD) has an impact on the co-existence simulations, depending on the combination of channel bandwidth and maximum transmission power.

RAN4 to consider 100 MHz as a typical channel bandwidth, with higher channel bandwidth not precluded for this range, and add a note on association of the typical channel bandwidth with maximum transmission power.

#### 2.3.3	Signal Bandwidth

To maintain consistency with the previous LS responses for 4400 – 4800 MHz and 7125 – 8400 MHz ranges, we propose to refer to TS 38.104 sub-clause 5.3.2 and to mention the dependency of signal bandwidth on the SCS and number of RBs.

Signal bandwidth = NRB x SCS x 12

RAN4 to not specify a fixed signal bandwidth but rather mention the dependency of signal bandwidth on the SCS and number of RBs.

## 2.4	BS parameters

### 2.4.1	Transmitter characteristics

#### 2.4.1.1	Spectral mask

In the previous IMT SI for 6.425 - 7.125 GHz and 10.0 - 10.5 GHz band, it was agreed to apply the same basic limits as for bands above 1 GHz (Clause 6.6.4.2 and 9.7.4.2 of TS 38.104) but the frequency offset step size was updated to 50 MHz.

Table 6.1.2-1: Wide Area BS operating band unwanted emission limits for 14800 to 15350 MHz for Category A

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Basic limits | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 50 MHz | 0.05 MHz f_offset < 50.05 MHz | -7dBm-750foffsetMHz-0.05 | 100 kHz |
| 50 MHz f < min(100 MHz, fmax) | 50.05 MHz f_offset < min(100.05 MHz, f_offsetmax) | -14 dBm | 100 kHz |
| 100 MHz f fmax | 100.5 MHz f_offset < f_offsetmax | -13 dBm | 1MHz |

Table 6.1.2-2: Wide Area BS operating band unwanted emission limits for 14800 to 15350 MHz for Category B

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Basic limits | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 50 MHz | 0.05 MHz f_offset < 50.05 MHz | -7dBm-750foffsetMHz-0.05 | 100 kHz |
| 50 MHz f < min(100 MHz, fmax) | 50.05 MHz f_offset < min(100.05 MHz, f_offsetmax) | -14 dBm | 100 kHz |
| 100 MHz f fmax | 100.5 MHz f_offset < f_offsetmax | -15 dBm | 1MHz |

RAN4 to consider the Table 6.1.2-1 and Table 6.1.2-2 from TR 38.921 as baseline for starting discussions on BS OBUE.

#### 2.4.1.2	ACLR

In TR 38.921, sub-clause 7.1.3, BS ACLR was agreed as 37 dB for 10-10.5 GHz range. The value for the 14800 to 15350 frequency range shall be derived once the simulation results are concluded.

#### 2.4.1.3	Spurious emissions

For spurious emissions use limits in TR 38.921 for 10 to 10.5 GHz as baseline. According to ERC 74-01 report, the upper frequency limit could be defined as 2nd harmonic.

Table 2.2.2.3-1: BS spurious emission limits for 14800 to 15350 MHz for Category A

| Frequency range | Limit | Measurement Bandwidth | Note |
| --- | --- | --- | --- |
| 30 MHz – 1 GHz | -13 dBm | 100 kHz | Note 1 |
| 1 GHz – 2nd harmonic of the upper frequency edge of the DL operating band |  | 1 MHz | Note 1, Note 2 |
| NOTE 1: Bandwidth as in ITU-R SM.329, s4.1 NOTE 2: Upper frequency as in ITU-R SM.329, s2.5 table 1. |  |  |  |

Table 2.2.2.3-2: BS spurious emission limits for 14800 to 15350 MHz for Category B

| Spurious frequency range | Limit | Measurement bandwidth |
| --- | --- | --- |
| 30 MHz – 1 GHz | -36 dBm | 100 kHz |
| 1 GHz – 18 GHz | -30 dBm | 1 MHz |
| 18 GHz – 2nd harmonic of the upper frequency edge of the DL operating band | -20 dBm | 10 MHz |

RAN4 to consider BS spurious emission limits in Table 2.2.3-1 and 2.2.3-2 as baseline for starting discussions.

#### 2.4.1.4	Maximum output power

There does not seem to be any difference between the two parameters, as Maximum output power is already an average power.

There is no upper limit specified for Wide Area Base Station, this shall be declared by the manufacturer. However, the BS maximum output power should be aligned with the antenna characteristics.

### 2.4.2	Receiver characteristics

#### 2.4.2.1	Sensitivity

It has been agreed for 4400 to 4800 and 7125 to 8400 MHz frequency ranges, that sensitivity is not a critical parameter for sharing and compatibility studies and “to be specified” in the LS responses sent to ITU-R. Similar agreements can be followed for 14800 to 15350 MHz range.

RAN4 to mention “to be specified” for sensitivity in the LS response to ITU-R for 14800 to 15350 MHz range.

#### 2.4.2.2	Blocking response

Co-existence with existing services would be more challenging if these parameters are further relaxed, which might end up with stringent requirements imposed by ITU-R WP5D. We also need to consider filter challenges at this frequency range, mounting to implementation cost and design challenges.

RAN4 to consider 100 MHz for both ΔfOBUE and ΔfOOB

#### 2.4.2.3	ACS

In TR 38.921, sub-clause 7.2.3, BS ACS was agreed as 40 dBc for 10-10.5 GHz range. The value for the 14800 to 15350 frequency range shall be derived once the simulation results are concluded.

## 2.5	UE parameters

### 2.5.1	Transmitter characteristics

#### 2.5.1.1	Power dynamic range

For the UE in sharing and compatibility studies, power dynamic range sets the range for Transmitter control output power. In TR 38.921, PC3 UEs were assumed for 6.425-7.025 GHz, 7.025-7.125 GHz, and 10.0-10.5 GHz study. From TS 38.101-1 [8], PC3 UEs have 23 dBm as Maximum output power and -33 dBm for 100 MHz Channel BW, therefore UE power dynamic range of 56 dB for 100 MHz can be considered as starting point.

RAN4 to consider power dynamic range of 56 dB for 100 MHz CBW as baseline for discussions.

#### 2.5.1.2	ACLR

In TR 38.921, sub-clause 7.1.3, UE ACLR was agreed as 24 dB for 10-10.5 GHz range. The value for the 14800 to 15350 frequency range shall be derived once the simulation results are concluded.

#### 2.5.1.3	Spurious emissions

From TS 38.101-1 [8], the general spurious can be defined as per clause 6.5.3.1 for this frequency range.

-	1 GHz ≤ f ≤ 26 GHz: - 30 dBm / 1 MHz

RAN4 to consider UE spurious emission limits from TS 38.101-1, - 30 dBm / 1 MHz as baseline for discussions.

#### 2.5.1.4	Maximum output power

The UE maximum output power can be considered as 23 dBm as a starting point. Several other studies have been carried out for adoption of High power UEs, where the problem of UL coverage can be improved for these frequency ranges.

High power UEs adoption can be further studied to solve the problem of UL coverage for these frequency ranges.

RAN4 to consider as 23 dBm UE maximum output power.

### 2.5.2	Receiver characteristics

#### 2.5.2.1	Noise figure

From TR 38.820, subclause 5.5.1.3.3 a system noise figure of 10 dB can be considered for the frequency range 14800 to 15350 MHz.

RAN4 to consider UE NF as 10 dB based on information in TR 38.820.

#### 2.5.2.2	Sensitivity

It has been agreed for 4400 to 4800 and 7125 to 8400 MHz frequency ranges, that sensitivity is not a critical parameter for sharing and compatibility studies and “to be specified” in the LS responses sent to ITU-R. Similar agreements can be followed for 14800 to 15350 MHz range.

RAN4 to mention “to be specified” for sensitivity in the LS response to ITU-R for 14800 to 15350 MHz range.

#### 2.5.2.3	ACS

In TR 38.921, sub-clause 7.2.3, UE ACS was agreed as 31 dBc for 10-10.5 GHz range. The value for the 14800 to 15350 frequency range shall be derived once the simulation results are concluded.

#### 2.5.2.4	Blocking response

The blocking characteristic specified in clause 7.6 of TS 38.101-1 [4] for frequency larger than 3300 MHz could be applied for this frequency range.

# Conclusion

In the previous sections we made the following observations:

Observation 1	Power Spectral Density (PSD) has an impact on the co-existence simulations, depending on the combination of channel bandwidth and maximum transmission power.

Observation 2	High power UEs adoption can be further studied to solve the problem of UL coverage for these frequency ranges.

Based on the discussion in the previous sections we propose the following:

Proposal 1	In Urban Macro scenario, RAN4 to consider sub-array size of 4x1 and drop 8x1 as a larger sub-array would limit the coverage angular range due to narrow sub-array radiation pattern in vertical axis.

Proposal 2	RAN4 to consider following BS array antenna architectures:

1.	Urban Macro: 4096 AEs with 16x32 array and 4x1 sub-array

2.	Indoor Hotspot: 4x4

3.	Dense Urban: Same HW as Urban Macro

Proposal 3	RAN4 to converge on the combination of the BS-UE antenna configuration that seems the most viable based on implementation feasibility aspects, to achieve a reasonable coverage.

Proposal 4	Considering the given timeline to agree on the parameters, RAN4 to consider the combination 4096 Antenna Elements (AE) (in total) (16x32 and 4x1 sub-array) and FR1-like UE - 1TX assumption with omnidirectional UE antenna as baseline.

Proposal 5	Most of the system parameters are dependent on other, so to make progress RAN4 should start by tentatively agreeing on parameters, and then confirm them only after verifying the overall system feasibility.

Proposal 6	The additional information should be updated with the following text in the TR – “There is no defined 3GPP band for the 14800 - 15350 MHz frequency range. Similar to the 4400 – 4800 MHz and 7125 – 8400 MHz frequency ranges, RAN4 assumed TDD as a baseline duplexing for the 14800 – 15350 MHz frequency range even though FDD is not precluded. SBFD can be a candidate duplexing method for this frequency range. The core requirements for Rel-19 SBFD work item can be tracked through the list of impacted specs captured in [6].”

Proposal 7	RAN4 to consider 100 MHz as a typical channel bandwidth, with higher channel bandwidth not precluded for this range, and add a note on association of the typical channel bandwidth with maximum transmission power.

Proposal 8	RAN4 to not specify a fixed signal bandwidth but rather mention the dependency of signal bandwidth on the SCS and number of RBs.

Proposal 9	RAN4 to consider the Table 6.1.2-1 and Table 6.1.2-2 from TR 38.921 as baseline for starting discussions on BS OBUE.

Proposal 10	RAN4 to consider BS spurious emission limits in Table 2.2.3-1 and 2.2.3-2 as baseline for starting discussions.

Proposal 11	RAN4 to mention “to be specified” for sensitivity in the LS response to ITU-R for 14800 to 15350 MHz range.

Proposal 12	RAN4 to consider 100 MHz for both ΔfOBUE and ΔfOOB

Proposal 13	RAN4 to consider power dynamic range of 56 dB for 100 MHz CBW as baseline for discussions.

Proposal 14	RAN4 to consider UE spurious emission limits from TS 38.101-1, - 30 dBm / 1 MHz as baseline for discussions.

Proposal 15	RAN4 to consider as 23 dBm UE maximum output power.

Proposal 16	RAN4 to consider UE NF as 10 dB based on information in TR 38.820.

Proposal 17	RAN4 to mention “to be specified” for sensitivity in the LS response to ITU-R for 14800 to 15350 MHz range.

# References

- R4-2400333, “ITU-R WP5D LS on Parameters of terrestrial component of IMT for sharing and compatibility studies in the frequency bands 4 400-4 800 MHz, 7 125-8 400 MHz, and 14.8-15.35 GHz”.

- R4-2405874, “Work Plan on IMT parameters relevant for sharing and compatibility for 4400 to 4800 MHz, 7125 to 8400 MHz, and 14800 to 1530 MHz frequency ranges”, 3GPP RAN4#110-bis, Ericsson

- R4-2414283, “WF on 15 GHz BS parameters”, 3GPP RAN4#112, ZTE Corporation

- R4-2414284, “WF on 15 GHz UE parameters”, 3GPP RAN4#112, Qualcomm

- R4-2412608, “3GPP TR 38.922 v0.2.0: Study on IMT parameters for 4400-4800 MHz, 7125-8400 MHz, and 14800-15350 MHz”, 3GPP RAN4#112

- 3GPP TR 38.921 v18.0.1: Study on IMT parameters for 6425 – 7025, 7025 – 7125 and 10000 - 10500 MHz”, 3GPP SA#103

- R4-2416171, “On 15 GHz co-existence simulation results”, 3GPP RAN4#112bis, Ericsson