3GPP TSG-RAN WG4 Meeting #112bis	R4-2416170

Hefei, China, October 14th-18th 2024

Agenda Item:	6.2.4.1

Source:	Ericsson

Title:	On 15 GHz co-existence simulation assumptions and results discussion

Document for:	Discussion

# 1	Introduction

ITU-R WP5D sent a LS [1] to 3GPP RAN seeking information on terrestrial component IMT-2030 parameters to be used for sharing and compatibility studies in preparation for WRC-27, for frequency ranges 4400 to 4800 MHz, 7125 to 8400 MHz, and 14800 to 15350 MHz.

In the previous 3GPP RAN4#110-bis meeting, the Work Plan [2] for delivery of information to ITU-R LS for those respective frequency ranges was agreed, following a phase approach to send information in 3 phases as below -

- Track 1: 4400 to 4800 MHz, the estimated date for completion is latest by May 2024 (RAN4#111).

- Track 2: 7125 to 8400 MHz, the estimated date for completion is latest by August 2024 (RAN4#112).

- Track 3: 14800 to 15350 MHz, the estimated date for completion is latest by November 2024 (RAN4#113).

WF [3] [4] and TR 38.922 v0.2.0 [5] captured initial agreements on co-existence simulation assumptions for this frequency range.

This contribution provides further discussion points on co-existence simulation assumptions and BS-UE antenna configuration relevant for 14800 to 15350 MHz frequency range.

# Discussion

In the previous 3GPP RAN4#112 meeting, a couple of agreements were made on the co-existence simulation assumptions for Urban scenarios which are captured in the TR 38.922 v0.2.0 [5]. However, some issues remain open and are discussed in the following sub-sections.

#### 2.1	Co-existence simulation scenarios

Considering the timeline to deliver the information to ITU-R latest by November 2024, there has been an agreement to prioritize co-existence evaluation with focus on Urban Macro, followed by Indoor Hotspot and then Dense Urban. RAN4 until the last meeting has only been able to discuss simulation parameters for Urban Macro scenarios, while the parameters for other scenarios remain yet to be agreed to carry out co-existence evaluations. If we look at the views on BS antenna size in RAN4 discussions, most are pointing towards a larger antenna array and sub-array which should be large enough to ensure sufficient vertical coverage to cover the vertical spread of users in Dense Urban scenarios.

Looking at the studies in TR 38.921 for 6 GHz and 10 GHz [6], Dense Urban scenario was down-prioritized as Urban Macro ACIR results would be more stringent between the two scenarios. On this basis, it is reasonable to agree to narrow down the simulation evaluation scope to Urban Macro and Indoor scenarios considering the estimated date of completing of this study item.

RAN 4 to narrow down the co-existence simulation evaluation scope to Urban Macro and Indoor scenarios.

#### 2.2	Channel Bandwidth

During RAN4#112 meeting as agreed in TR 38.922 [5], the possible channel bandwidths that are considered for study: 100/200/400 MHz. At the same time, the maximum BS and UE transmission powers (46 dBm and 23/26 dBm, respectively) have been defined independent to the channel bandwidth.

It is important to communicate to ITU-R some combinations of power/bandwidth which are feasible in terms of coverage and propose them as typical values in this band. Otherwise, if power and bandwidth remain untied, this leads to unclear Power Spectral Density (PSD), which plays a significant role in coexistence simulations, especially when there are coverage issues.

In our companion contribution [7], we have presented pre-liminary results on how UL coverage could be improved, by increasing UE maximum transmission power and reducing channel bandwidth or vice versa.

Power Spectral Density (PSD) has an impact on the co-existence simulations, and it depends on the combination of channel bandwidth and maximum transmission power.

RAN4 to clarify the association of typical channel bandwidth assumption with the maximum transmit power in both UL and DL.

#### 2.3	Inter-site distance for Urban Macro scenario

In our companion contribution [7], we have presented our preliminary results with different BS & UE antenna configurations/combinations for Urban Macro scenario at ISD of 450m. When considering 20% as Indoor UE ratio, the DL coverage is sufficient but the UL coverage still poses a challenge.

With the purpose to maintain the coverage of lower frequency bands and reuse the available grid, we propose to narrow down the ISD to 450m.

RAN4 to narrow down to ISD as 450m, to maintain the coverage of lower frequency bands and reuse the available macro grid

#### 2.4	Indoor UE ratio for Urban Macro scenario

Based on our pre-liminary results as presented in our companion contribution [7] with Indoor probability 20%, UL coverage still possess a challenge with different BS-UE antenna configurations Despite the challenges in UL coverage, we propose to prioritize scenarios with 20% indoor users, in order to fully exploit this band. However, 0% indoor users cannot be excluded, if UL coverage remains a challenge. As a second option, we could also consider 0% indoor users if UL coverage remains a challenge.

UL coverage is a challenge when considering Indoor users probability 20%.

To maximize the operation of this band, RAN4 to prioritize the scenario with 20% indoor user. Keep 0% Indoor user as second priority until other companies can verify UL coverage feasibility based on 20% probability.

#### 2.5	Coordinated and Uncoordinated operation based on UE type

In the previous meetings, it was agreed to focus on both uncoordinated and coordinated scenarios for UEs with FR1-link antennas and only on the coordinated case for UEs with FR2-like antennas.

From results presented in companion contribution [7], the uncoordinated case seems to be more restrictive in terms of requirements. Given the timeline to finalize the parameters, we propose to narrow down to only Uncoordinated scenarios for both FR1 and FR2 UE type.

The Uncoordinated scenario is the worst case and drives the requirements.

Given the timeline to finalize the parameters, we propose to narrow down to only Uncoordinated scenarios for both FR1 and FR2 UE type.

#### 2.6	UE max TX power – 23 dBm or 26 dBm

Based on the agreements in TR [5], it was agreed to consider two options for UE maximum transmission power, 23 dBm or 26 dBm. We consider that 23 dBm should be the preferred option if the UL coverage is feasible. Otherwise, the power may be increased to 26 dBm with a wider bandwidth to increase the PSD.

High power UEs adoption can be further studied to solve the problem of UL coverage for these frequency ranges, if considered important to have as baseline a channel bandwidth higher than reference 100 MHz.

RAN4 to consider as 23 dBm UE maximum output power when BW is 100 MHz.

#### 2.7	BS and UE antenna configuration

In the previous meeting many options for BS antenna configuration and a few for UE antenna configuration were proposed. To narrow down the options to be considered, we propose to combine a few BS and UE antenna configurations that would provide a reasonable coverage.

In the companion contribution [7], we presented results for the following combinations of BS and UE antenna configurations:

- Combination 1: 4096 Antenna Elements (AE) (in total) (16x32 and 4x1 sub-array) and 1 TX assumption with omnidirectional UE antenna.

- Combination 2: 2048 AEs (8x32 and 4x1 sub-array) and two discrete elements, with maximum-gain across elements selection criterion.

- Combination 3: 2048 AEs (8x32 and 4x1 sub-array) and FR2-like UE, two panels with 1x2 array at each panel.

- Combination 4: 1536 AEs (8x16 and 6x1 sub-array) and FR2-like UE, two panels with 1x2 array at each panel.

A possible approach could be to study combinations and go forward with the combination based on implementation feasibility aspects.

With the objective to narrow down the possible combinations of BS-UE antenna configurations, RAN4 to limit the combination that are feasible from UL coverage perspective and evaluate them to determine the more restrictive combination in terms of requirements.

It is also important to note the inter-dependency of some parameters – for example, Channel BW and UE Tx Power as we have highlighted in the companion contribution - when reaching an agreement in RAN4. These parameters should be tentatively agreed first, and then further validated later, after reaching out a conclusion on system feasibility.

As it can be seen in our companion contribution [7], a lower channel bandwidth of 100 MHz and 23 dBm transmission power increases the PSD, whereas a higher channel bandwidth of 200 MHz and 23 dBm transmission power decreases the PSD, and hence impacts the Uplink coverage.

It is important to share with ITU-R a typical combination of transmission power and bandwidth, and not leave the two parameters to be independent from each other.

For UE maximum transmission power narrow down to 23 dBm/100 MHz. Consider higher power of 26 dBm, only if the channel bandwidth is considered important to be higher than 100 MHz.

In general, we observe dependency of parameters on other, e.g. the antenna architectures at both BS and UEs, the power levels in UL and DL, the ISD, the channel bandwidth, etc. As a result, to make progress, RAN4 should start by tentatively agreeing on some system parameters and then confirm them only after validating the overall system feasibility.

Most of the system parameters are dependent on other, so to make progress RAN4 should start by tentatively agreeing on parameters, and then confirm them only after verifying the overall system feasibility.

# Conclusion

In the previous sections we made the following observations:

Observation 1	Power Spectral Density (PSD) has an impact on the co-existence simulations, and it depends on the combination of channel bandwidth and maximum transmission power.

Observation 2	UL coverage is a challenge when considering Indoor users probability 20%.

Observation 3	The Uncoordinated scenario is the worst case and drives the requirements.

Observation 4	High power UEs adoption can be further studied to solve the problem of UL coverage for these frequency ranges, if considered important to have as baseline a channel bandwidth higher than reference 100 MHz.

Observation 5	It is important to share with ITU-R a typical combination of transmission power and bandwidth, and not leave the two parameters to be independent from each other.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN 4 to narrow down the co-existence simulation evaluation scope to Urban Macro and Indoor scenarios.

Proposal 2	RAN4 to clarify the association of typical channel bandwidth assumption with the maximum transmit power in both UL and DL.

Proposal 3	RAN4 to narrow down to ISD as 450m, to maintain the coverage of lower frequency bands and reuse the available macro grid

Proposal 4	To maximize the operation of this band, RAN4 to prioritize the scenario with 20% indoor user. Keep 0% Indoor user as second priority until other companies can verify UL coverage feasibility based on 20% probability.

Proposal 5	Given the timeline to finalize the parameters, we propose to narrow down to only Uncoordinated scenarios for both FR1 and FR2 UE type.

Proposal 6	RAN4 to consider as 23 dBm UE maximum output power when BW is 100 MHz.

Proposal 7	With the objective to narrow down the possible combinations of BS-UE antenna configurations, RAN4 to limit the combination that are feasible from UL coverage perspective and evaluate them to determine the more restrictive combination in terms of requirements.

Proposal 8	For UE maximum transmission power narrow down to 23 dBm/100 MHz. Consider higher power of 26 dBm, only if the channel bandwidth is considered important to be higher than 100 MHz.

Proposal 9	Most of the system parameters are dependent on other, so to make progress RAN4 should start by tentatively agreeing on parameters, and then confirm them only after verifying the overall system feasibility.

# References

- R4-2400333, “ITU-R WP5D LS on Parameters of terrestrial component of IMT for sharing and compatibility studies in the frequency bands 4 400-4 800 MHz, 7 125-8 400 MHz, and 14.8-15.35 GHz”.

- R4-2405874, “Work Plan on IMT parameters relevant for sharing and compatibility for 4400 to 4800 MHz, 7125 to 8400 MHz, and 14800 to 1530 MHz frequency ranges”, 3GPP RAN4#110-bis, Ericsson

- R4-2414283, “WF on 15 GHz BS parameters”, 3GPP RAN4#112, ZTE Corporation

- R4-2414284, “WF on 15 GHz UE parameters”, 3GPP RAN4#112, Qualcomm

- R4-2412608, “3GPP TR 38.922 v0.2.0: Study on IMT parameters for 4400-4800 MHz, 7125-8400 MHz, and 14800-15350 MHz”, 3GPP RAN4#112

- 3GPP TR 38.921 v18.0.1: Study on IMT parameters for 6425 – 7025, 7025 – 7125 and 10000 - 10500 MHz”, 3GPP SA#103

- R4-2416171, “On 15 GHz co-existence simulation results”, 3GPP RAN4#112bis, Ericsson