3GPP TSG-RAN WG4 Meeting #112bis							R4-2416171

Hefei, China, October 14th-18th 2024

Agenda Item:	6.2.4.1

Source:	Ericsson

Title:	On 15 GHz co-existence simulation results

Document for:	Information

# 1	Introduction

ITU-R WP5D sent a LS [1] to 3GPP RAN seeking information on terrestrial component IMT-2030 parameters to be used for sharing and compatibility studies in preparation for WRC-27, for frequency ranges 4400 to 4800 MHz, 7125 to 8400 MHz, and 14800 to 1530 MHz.

In 3GPP RAN4#110-bis meeting, the Work Plan [2] for delivery of information to ITU-R LS for those respective frequency ranges was agreed, following a phase approach to send information in 3 phases as below -

- Track 1: 4400 to 4800 MHz, completed by May 2024 (RAN4#111).

- Track 2: 7125 to 8400 MHz, completed by August 2024 (RAN4#112).

- Track 3: 14800 to 15350 MHz, the estimated date for completion is latest by November 2024 (RAN4#113).

WF [1] [2] and TR 38.922 v0.2.0 [3] captured initial agreements on co-existence simulation assumptions for companies to present results in October meeting.

This contribution provides the simulation results for Urban Macro and Indoor scenario as per the latest agreements.

# 2	Discussion

In this section some preliminary simulation results for Urban macro and Indoor scenarios are presented, following the captured agreements, and the options listed in the sections below. The simulation assumptions for the Urban Macro scenario have been agreed in WF [1][2] and captured in TR [3].

## 2.1	Urban Macro

In the WF [1] for the Urban Macro scenario, many options are available for the Base Station (BS) and the User Equipment (UE) antenna configuration.

The following options are considered in the WF for BS parameters:

And the following options are considered in the WF [2] for the UE parameters:

We propose a reduced set of combinations from the proposed options, in order to narrow down the possible combinations of BS-UE configurations to study, with the aim to converge to a reasonable configuration to propose to ITU-R. For example, by selecting the one that provides the most restrictive requirements - we combine the bigger BS array, with the simpler option at the UE, and vice versa, in order to achieve a reasonable coverage.

The proposed BS/UE type combinations are the following:

- Combination 1: 4096 Antenna Elements (AE) (in total) (16x32 and 4x1 sub-array) and FR1-like UE – 1TX assumption with omnidirectional UE antenna.

- Combination 2: 2048 AEs (8x32 and 4x1 sub-array) and FR1-like UE - two discrete elements, with maximum gain across elements selection criterion.

- Combination 3: 2048 AEs (8x32 and 4x1 sub-array) and FR2-like UE - two panels with 1x2 array at each panel.

- Combination 4: 1536 AEs (8x16 and 6x1 sub-array) and FR2-like UE - two panels with 1x2 array at each panel.

Other relevant simulation assumptions are the following:

- ISD (Inter-Site Distance) = 450 m

- UE max Transmission power = 23 and 26 dBm

- Both coordinated and uncoordinated deployments are considered.

- Indoor probability: 20%

- Diversity gain: 0 dB

- Number of active UEs is 1

- Channel bandwidth: 100/200 MHz

In the rest of the contribution, we focus on analysis and discussion of simulation results, obtained for both uncoordinated and coordinated scenarios, where the grid-shift between BSs of different networks is 100% or 0%, respectively.

Results will be presented for 2 different combinations of channel bandwidth (100 MHz/200 MHz) and UE transmission power (23/26 dBm). The goal of these combinations is to illustrate the significant impact of both achievable Power Spectral Density (PSD) and maximum UE transmission power in a 15 GHz deployment, where the objective is to maintain the coverage of lower frequencies to reuse the available deployment grid, while also aiming to provide coverage to indoor users (the agreed assumption is 20% of indoor UEs in the simulations). We will start by analysing the 200 MHz bandwidth, with 46 dBm at the BS and 23 dBm at the UE, and then adjust to achieve coverage feasibility.

In general, we observe that there is high dependability in the definition of parameters, so all parameters should be agreed tentatively, in the first place, and only validated when the overall system is feasible.

### 2.1.1	Uncoordinated scenario

In this section we focus on analysis and discussion of simulation results for the uncoordinated scenario where the grid shift within Base Stations (BS) of different operators is 100%.

From the analysis of results, we observe that the requirements will be driven by the uncoordinated case, which represents the worst-case scenario.

Uncoordinated scenario represents the worst case and drives the ACIR requirements.

Figure 2.1.1-1 shows DL and UL SINR, for a case with high isolation between the two networks, that is, for an ACIR of 100 dB, as indicated in the figure legend. It can be observed that when we try to provide coverage also to 20% indoor users, with 200 MHz bandwidth and UE maximum power of 23 dBm, there is around 5% outage probability in UL. This results in challenging UL coverage and reduced statistics for UL 5%tile throughput, as it can be observed in Figure 2.1.1-2.

On the other hand, DL SINR in Figure 2.1.1-1 indicates a better coverage in DL. The high DL SINR saturates the link model defined in the TR [3]. Consequently, the mean user throughout is not significantly affected by the other operator interference, as it can be observed in Figure 2.2.1-3. As a result of that, the requirements are driven by the performance of 5%tile throughput that are shown in Figure 2.1.1-2. DL ACIR requirements are in the range of 15–17 dBs depending on the studied BS/UE combination. The most restrictive requirement is associated to combination 4, where the array at the BS is smaller (1536 AE) and consequently the transmissions are characterized by lower directionality compared to the other cases.

Considering the poor UL performance that are observed with 200 MHz bandwidth and 23 dBm UE transmission power, further efforts are needed at the UE side to improve the coverage. To this end, either the UE antenna should be enhanced, or the UE maximum transmission power increased. Considering the available options, we propose to increase the UE maximum transmission power to 26 dBm.

Figure 2.1.1-4 shows DL and UL SINR, when 200 MHz and 26 dBm as UE maximum transmission power are considered. It can be observed that the outage probability has decreased compared to Figure 2.1.1-1 where 200 MHz and 23 dBm are considered. This leads to more stable 5%tile UL throughput and consequently to more reliable statistics for UL throughput loss. The UL ACIR requirement in this case is between 5 and 7dBs, approximately, depending on the BS/UE combination.

For the DL and other plots shown in Figures 2.1.1-4 – 2.1.1-5 similar observations as already discussed for Figures 2.1.1-1 – 2.1.1-3 apply. We do not show again the mean user throughput loss, because it has the same trends as shown in Figure 2.1.1-3, that is, no significant throughput loss.

UL coverage is challenged by the presence of indoor users. UL 5%tile throughput statistics are not stable if UL coverage is poor. UL coverage can be improved by increasing UE maximum transmission power or the structure of the UE antenna array.

To achieve stable 5%tile throughput, and consequently requirements, 26 dBm can be considered as maximum UE transmission power, if the channel bandwidth is 200 MHz.

Another option is, not to increase the UL maximum transmission power, the channel bandwidth could be reduced in order to increase the Power Spectral Density (PSD). In general, even if in the specs power and bandwidth are not necessarily tied up, the resulting PSD has an impact on coexistence simulations, especially in cases where UL coverage is challenging. As a result, the information sent to ITU-R should include (especially for this new band) typical combinations of power and bandwidth values.

A lower channel bandwidth, for the same maximum transmission power, increases the PSD and consequently the coverage, so the channel bandwidth and the maximum transmission power to be proposed to ITU-R should be tied together in order to guarantee coverage feasibility.

It would be important to share with ITU-R a typical combination of transmission power and bandwidth, and not leave the two parameters to be independent from each other.

Figures 2.1.1-6 – 2.1.1-7 show SINR and 5%tile throughput loss for the case of 100 MHz channel bandwidth and 23 dBm UE maximum transmission power (23 dBm/100 MHz). It can be observed that the results for UL are very similar to those presented in Figures 2.1.1-4 – 2.1.1-5 for the case of 26 dBm and 200MHz.

It is possible to improve UL performance in the scenario, by increasing the UE maximum transmission power or by reducing the channel badiwdth, to increase the PSD.

In an Uncoordinated scenario, with ISD 450 mt and 20% indoor probability, ACIR requirements for the 4 different studied combinations are in the range of 15–17 dBs for the DL case, and 5–7 dBs for UL, for 26 dBm/200MHz or 23 dBm/100 MHz UE maximum transmission power/channel bandwidth.

Figure 2.1.1-1: Urban Macro DL and UL SINR for BS/UE combinations 1-4, indoor probability 20%, 200 MHz bandwidth and 23 dBm UE Tx power.

Figure 2.1.1-2: Urban Macro DL and UL 5%tile user throughput loss, for BS/UE combinations 1-4, indoor probability 20%, 200 MHz bandwidth and 23 dBm UE Tx power.

Figure 2.1.1-3: Urban Macro DL and UL mean user throughput loss, for BS/UE combinations 1-4, indoor probability 20%, 200 MHz bandwidth and 23 dBm UE Tx power.

Figure 2.1.1-4: Urban Macro DL and UL SINR for BS/UE combinations 1-4, indoor probability 20%, 200 MHz bandwidth and 26 dBm UE Tx power.

Figure 2.1.1-5: Urban Macro DL and UL 5%tile user throughput loss, for BS/UE combinations 1-4, indoor probability 20%, 200 MHz bandwidth and 26 dBm UE Tx power.

Figure 2.1.1-6: Urban Macro DL and UL SINR for BS/UE combinations 1-4, indoor probability 20%, 100 MHz bandwidth and 23 dBm UE Tx power.

Figure 2.1.1-7: Urban Macro DL and UL 5%tile user throughput loss, for BS/UE combinations 1-4, indoor probability 20%, 100 MHz bandwidth and 23 dBm UE Tx power.

### 2.1.2	Coordinated scenario

In the coordinated scenario similar trends to the uncoordinated scenario, discussed in the previous section, can be observed, except for the fact that the resulting requirements are less restrictive than for the uncoordinated case. As it can be observed from Figure 2.1.2-1 and Figure 2.1.2-2, UL coverage is challenging, and the UL 5%tile throughput performance is not stable: for BS/UE combinations 2 and 4, the 5%tile throughput is zero, and for BS/UE combination 1, it becomes zero when the isolation between the two networks is reduced to lower ACIR values.

As in the uncoordinated case, the UE maximum transmission power can be increased to 26 dBm to improve UL coverage. From Figure 2.1.2-4, it can be observed that the UL SINR is improved for the 5%tile of users, and 5%tile throughput loss derives requirements which are very low, less than 3 dBs, for the 4 considered combinations. As for DL, the requirements are again driven by the 5%tile throughput, and they fall in the range 7-10 dBs.

The UL coverage can be improved also reducing the power, to maintain the UE maximum transmission power, so with 23 dBm/100 MHz, which provides very similar results to 26 dBm/200 MHz.

The Coordinated scenario represents a best case scenario, with respect to the uncoordinated case. The requirements in UL and DL are less restrictive than the uncoordinated case, they fall in the range 7-10 dB for DL, and below 3 dB for UL.

Figure 2.1.2-1: Urban Macro DL and UL SINR for BS/UE combinations 1-4, indoor probability 20%, 200 MHz bandwidth and 23 dBm UE Tx power

Figure 2.1.2-2: Urban Macro DL and UL 5%tile user throughput loss, for BS/UE combinations 1-4, indoor probability 20%, 200 MHz bandwidth and 23 dBm UE Tx power

Figure 2.1.2-3: Urban Macro DL and UL mean user throughput loss, for BS/UE combinations 1-4, indoor probability 20%, 200 MHz bandwidth and 23 dBm UE Tx power

Figure 2.1.2-4: Urban Macro DL and UL SINR for BS/UE combinations 1-4, indoor probability 20%, 200 MHz bandwidth and 26 dBm UE Tx power

Figure 2.1.2-5: Urban Macro DL and UL 5%tile user throughput loss, for BS/UE combinations 1-4, indoor probability 20%, 200 MHz bandwidth and 26 dBm UE Tx power

Figure 2.1.2-6: Urban Macro DL and UL SINR for BS/UE combinations 1-4, indoor probability 20%, 100 MHz bandwidth and 23 dBm UE Tx power

## 2.2	Indoor

In the Indoor scenario, the following options have been assumed for simulation:

- UE max Transmission power = 23 dBm for FR1 like omnidirectional UE

- Antenna architectures: 4x4, aligned with the proposal included in our companion contributions [4,5]

- Diversity gain: 0 dB

- Number of active UEs is 1

- Channel bandwidth 100 MHz

Figure 2.2-1 shows the DL SINR for the indoor scenario and FR1 UE omnidirectional option. It can be observed that in both cases the coverage is good in this scenario.

Figure 2.2-1: Indoor DL SINR for 4x4 array, FR1 UE antenna

Figure 2.2-2: Indoor DL 5%tile and mean user throughput loss, for 4x4 array, FR1 UE antenna

Figure 2.2-2 shows the DL 5%tile and mean user throughout loss for FR1 UE omnidirectional antenna. The mean user throughout loss does not lead to any requirement, while for the DL 5%tile throughput loss we can observe that the requirements that we obtain are in the same magnitude.

Figure 2.2-3 shows the UL SINR for the indoor scenario and FR1 UE omnidirectional antenna. It can be observed that in both cases the coverage is good in this scenario.

Figure 2.2-3: Indoor UL SINR for 4x4 array, FR1 UE antenna

Figure 2.2-4: Indoor UL SINR for 4x4 array, FR1 UE antenna

Figure 2.2-4 shows the UL 5%tile and mean user throughout loss, for FR1 UE omnidirectional antenna. The mean user throughout loss does not lead to any requirement, while for the UL 5%tile throughput loss we can observe that also in this scenario the requirements are low.

With FR1 omni-directional UE in Indoor DL and UL, ACIR requirements are quite low in the proposed deployment.

The ACIR requirements in the indoor scenario are in the order of 8 and 10 dB for UL and DL, respectively.

# Conclusion

In the previous sections we made the following observations:

Observation 1	Uncoordinated scenario represents the worst case and drives the ACIR requirements.

Observation 2	UL coverage is challenged by the presence of indoor users. UL 5%tile throughput statistics are not stable if UL coverage is poor. UL coverage can be improved by increasing UE maximum transmission power or the structure of the UE antenna array.

Observation 3	To achieve stable 5%tile throughput, and consequently requirements, 26 dBm can be considered as maximum UE transmission power, if the channel bandwidth is 200 MHz.

Observation 4	A lower channel bandwidth, for the same maximum transmission power, increases the PSD and consequently the coverage, so the channel bandwidth and the maximum transmission power to be proposed to ITU-R should be tied together in order to guarantee coverage feasibility.

Observation 5	It would be important to share with ITU-R a typical combination of transmission power and bandwidth, and not leave the two parameters to be independent from each other.

Observation 6	It is possible to improve UL performance in the scenario, by increasing the UE maximum transmission power or by reducing the channel badiwdth, to increase the PSD.

Observation 7	In an Uncoordinated scenario, with ISD 450 mt and 20% indoor probability, ACIR requirements for the 4 different studied combinations are in the range of 15–17 dBs for the DL case, and 5–7 dBs for UL, for 26 dBm/200MHz or 23 dBm/100 MHz UE maximum transmission power/channel bandwidth.

Observation 8	The Coordinated scenario represents a best case scenario, with respect to the uncoordinated case. The requirements in UL and DL are less restrictive than the uncoordinated case, they fall in the range 7-10 dB for DL, and below 3 dB for UL.

Observation 9	With FR1 omni-directional UE in Indoor DL and UL, ACIR requirements are quite low in the proposed deployment.

Observation 10	The ACIR requirements in the indoor scenario are in the order of 8 and 10 dB for UL and DL, respectively.

# References

- R4-2414283, “WF on 15 GHz BS parameters”, 3GPP RAN4#112, ZTE Corporation

- R4-2414284, “WF on 15 GHz UE parameters”, 3GPP RAN4#112, Qualcomm

- R4-2412608, “3GPP TR 38.922 v0.2.0: Study on IMT parameters for 4400-4800 MHz, 7125-8400 MHz, and 14800-15350 MHz”, 3GPP RAN4#112

- R4-2416169, “On 15 GHz BS-UE radio and antenna parameters”, 3GPP RAN4#112bis, Ericsson

R4-2416170, “On 15 GHz co-existence simulation assumptions and results discussion”, 3GPP RAN4#112bis, Ericsson