3GPP TSG-RAN WG4 Meeting #112bis                                      R4-2416179

Hefei, Anhui, China, October 14 - 18, 2024

Source: 	Huawei, HiSilicon

Title: 	On MPR applicability for FR2 CA

Agenda Item:	6.1.1.2.3

Document for:	Approval

Introduction

During RAN4#112, MPR applicability for FR2 single carrier UL and intra-band UL CA with DL intra-band CA has been discussed. Remaining FFS points can be found in [1].

- Sub-topic 3-2: Applicable MPR - Issue 3-2-1: Applicable MPR for FR2 single carrier UL with DL intra band CA - Proposals - Proposal 1: For the case of DL intra band CA with single UL CC (activated or configured), the reference MPR (originally being Table 6.2A.2.4-1) is changed as the MPR requirement for single carrier. (Huawei) - WF - FFS in next meeting. - Issue 3-2-2: Applicable MPR for FR2 UL CA with DL intra band CA - WF - In general, MPR based on UL BWchannel_CA applies instead that based on cumulative aggregated channel BW (CABW) - If only 1 UL CC is activated, the MPR requirements of single carrier could be reused - No need to specify new column for <400MHz CABW in the MPR table. FFS specific spec change to accommodate 200MHz BW granularity - FFS whether to take the minimum values of MPR between the intra-band CA case and the single carrier for FR2-2 - Sub-topic 3-3: UE capability - Issue 3-3-1: Whether an optional UE capability is considered - Proposals - Proposal 1: no need to introduce a new UE capability for such MPR improvement as Network does not need to enable the MPR improvement originating from the application of MPR requirements. (CATT) - Proposal 2: The new capability shall be defined as optional and per UE, and only applicable to FR2. In addition, it could be release independent. (ZTE) - Proposal 3: RAN4 to consider as an optional capability. (Qualcomm) - Proposal 4: Subject to UE capability indication on the support of Rel-19 MPR applicability for FR2 CA. (Huawei) - WF - FFS in next meeting. - Issue 3-3-2: Whether UE capability, if needed, is agnostic to specific LO implementations - Proposals - Proposal 1: Consider all possible UE implementations, including UE with fast LO switching, or with dedicated LOs for Tx and Rx paths for the new UE capability. (ZTE) - Proposal 2: Capability should not be dependent on a specific implementation. (NTT DOCOMO) - WF - FFS in next meeting - Issue 3-3-3: Proposal for CC activation-based MPR improvement - Proposals - Proposal 1: FR2 UE CA MPR is based on the activated subset of the CCs in the CA configuration that are also enabled for UL, provided the activated CCs with UL comprise a contiguous block. (Qualcomm) - Recommended WF - FFS in next meeting

In this contribution, we would like to share our views regarding this topic.

Discussion

Regarding the applicable MPR for FR2 UL CA with DL intra band CA, the substitution of UL BWchannel_CA for cumulative aggregated channel BW (CABW) was agreed. One FFS point is how to accommodate <= 400MHz CBW case in the spec w/o introducing new column to the MPR table.

Looking into the TS 38.101-2, the FR2-1 intra band contiguous UL CA MPR requirements for different cumulative aggregated channel bandwidth are defined in Table 6.2A.2.4-1. Additionally, there is a special case targeting a contiguous RB allocation in a single CC but also subject to specific modulation order and waveform.

In case of a contiguous RB, DFT-s-BPSK or DFT-s-QPSK UL allocation in a single CC of a CA configuration with contiguous CCs, and whose cumulative aggregated BW 400 MHz, MPRC_CA shall be derived instead as MAX(MPR1, MPR2), where: MPR1 shall be determined from Table 6.2.2.3-1 if CABW 200 MHz, from Table 6.2.2.3-2 if CABW > 200 MHz. MPR2 shall be determined from Table 6.2.2.3-1 if UL BWchannel_CA 200 MHz, from Table 6.2.2.3-2 if UL BWchannel_CA > 200 MHz.

As now the Rel-19 enhancement is to determine the MPR based on UL BWchannel_CA, we think a similar replacement can be applied here, so we propose to change the MPRC_CA from MAX(MPR1, MPR2) to MPR2.

Proposal 1: Subject to UE capability indication on the support of Rel-19 MPR applicability for FR2 CA, following change can be introduced to the special case for contiguous RB allocation in a single CC:

In case of a contiguous RB, DFT-s-BPSK or DFT-s-QPSK UL allocation in a single CC of a CA configuration with contiguous CCs, and whose cumulative aggregated BW 400 MHz, MPRC_CA shall be derived instead as MPR2, where: MPR2 shall be determined from Table 6.2.2.3-1 if UL BWchannel_CA 200 MHz, from Table 6.2.2.3-2 if UL BWchannel_CA > 200 MHz.

About the UE capability, we think an optional UE capability can be introduced to distinguish the one supports Rel-19 FR2 UL CA MPR applicability feature from the one doesn’t, because from hardware implementation perspective the separate LO for Tx and Rx is assumed. The granularity can be per UE.

Proposal 2: Introduce an optional UE capability to indicate the support of Rel-19 MPR applicability for FR2 CA. The granularity is per UE.

Conclusion

In this contribution we discussed on the remaining issue for Rel-19 MPR applicability for FR2 CA. We have the following proposals:

Proposal 1: Subject to UE capability indication on the support of Rel-19 MPR applicability for FR2 CA, following change can be introduced to the special case for contiguous RB allocation in a single CC:

In case of a contiguous RB, DFT-s-BPSK or DFT-s-QPSK UL allocation in a single CC of a CA configuration with contiguous CCs, and whose cumulative aggregated BW 400 MHz, MPRC_CA shall be derived instead as MPR2, where: MPR2 shall be determined from Table 6.2.2.3-1 if UL BWchannel_CA 200 MHz, from Table 6.2.2.3-2 if UL BWchannel_CA > 200 MHz.

Proposal 2: Introduce an optional UE capability to indicate the support of Rel-19 MPR applicability for FR2 CA. The granularity is per UE.

- References

- R4-2414441, “WF on power domain enhancements”, Huawei, HiSilicon, 3GPP TSG-RAN4 Meeting #112, Maastricht, Netherlands, August 19-23.