3GPP TSG-RAN WG4 Meeting #112bis                                      R4-2416180

Hefei, Anhui, China, October 14 - 18, 2024

Source: 	Huawei, HiSilicon

Title: 	On 6Rx reference sensitivity requirements for FR1 UE

Agenda Item:	6.1.1.3.1

Document for:	Approval

Introduction

During RAN4#112, FR1 6Rx UE RF requirements have been discussed. Following conclusion regarding REFSENS can be found in [1].

- Topic 1: REFSENS (delta RIB,6R) - Sub-topic 1-1: General considerations for specifying ΔRIB,6R value Issue 1-1-1: Whether band n104 should be included in the high band (n77, n78 and n79) category for 6Rx case Agreement: Include n104 in the high band (n77, n78, n79) category. Issue 1-1-2: Whether to use same ΔRIB,6R value for handheld UE and FWA Agreement: Different value for handheld UE and FWA. Issue 1-1-3: Verification of 6Rx receiver requirements Way Forward: Companies encouraged to provide their views on the ZTE proposal in Proposal 5 in R4-2411883 and any further positions on verification of 6Rx receiver requirements at future meetings given that ΔRIB,6R values are now defined. - Sub-topic 1-2: ΔRIB,6R values for handheld UE and FWA Issue 1-2-1: Proposed ΔRIB,6R values for handheld UE and FWA Agreement: Based on the outcome of Issue 1-1-2, RAN4 to specify ΔRIB,6R values for handheld UE and FWA as follows. - Adopt the average value of ΔRIB,6R for handheld UE from company proposals (updated to include company proposals from RAN4#111) and specify it for handheld UE devices. Table: Averaged values for handheld UE based on proposals Operating Bands; ΔRIB,6R (dB). MediaTek; Band n41; -3.4. Band n77, n78, n79, n104; -3.1. Xiaomi; Band n41; -3.2. Band n77, n78, n79, n104; -3.0. LGE; Band n41; -3.5. Band n77, n78, n79, n104; -3.0. Spreadtrum; Band n41; -3.3. Band n77, n78, n79, n104; -3.0. Meta; Band n41; -3.5. Band n77, n78, n79, n104; -3.1. vivo #1; Band n41; -3.0. Band n77, n78, n79, n104; -3.0. vivo #2; Band n41; -3.2. Band n77, n78, n79, n104; -3.0. ZTE; Band n41; -3.5. Band n77, n78, n79, n104; -3.0. Ericsson; Band n41; -3.6. Band n77, n78, n79, n104; -3.2. Google; Band n41; -3.4. Band n77, n78, n79; -3.0. Huawei; Band n41; -3.3. Band n77, n78, n79, n104; -3.0. Qualcomm; Band n41; -3.4. Band n77, n78, n79, n104; -3.0. Apple (R4-2407071); Band n41; -3.3. Band n77, n78, n79, n104; -3.0. OPPO (R4-2408759); Band n41; -3.6. Band n77, n78, n79, n104; -3.2. Average; Band n41; -3.4. Band n77, n78, n79, n104; -3.0. - Adopt the average value of ΔRIB,6R for FWA UE from company proposals indicating different FWA UE values from handheld UE values and specify it for FWA UE devices. Table: Averaged values for FWA UE based on proposals Operating Bands; ΔRIB,6R (dB). LGE; Band n41; -3.7. Band n77, n78, n79, n104; -3.4. Spreadtrum; Band n41; -3.6. Band n77, n78, n79, n104; -3.2. vivo; Band n41; -3.4. Band n77, n78, n79, n104; -3.2. Google; Band n41; -3.7. Band n77, n78, n79; -3.3. Nokia; Band n41; -4.0. Band n77, n78, n79, n104; -3.4. Average; Band n41; -3.7. Band n77, n78, n79, n104; -3.3.

In this contribution, we would like to share our views regarding this topic.

Discussion

For the verification of 6Rx receiver requirements, the existing handling for 4Rx/8Rx, which is to traverse 2Rx, 4Rx and 6Rx for the single carrier REFSENS and bypass 2Rx and 4Rx for the other single carrier Rx requirements’ verification, would be a procedure that can serve the purpose on 6Rx hardware capability verification while keep the test burden acceptable. Unless other issues could be identified later, we suggest to apply the same handling to 6Rx verification.

Proposal 1: Check whether below methodology can be applied to 6Rx single carrier Rx requirements verification:

- For single carrier REFSEN requirements, additional requirement for four Rx antenna ports and six Rx antenna ports shall be verified in operating bands;

- For Rx requirements other than single carrier REFSEN, the UE shall be verified with six Rx antenna ports and skip both two and four Rx antenna ports requirements in operating bands.

Conclusion

In this contribution we discussed on the REFSENS for FR1 6Rx UE. We have the following observation and proposal:

Proposal 1: Check whether below methodology can be applied to 6Rx single carrier Rx requirements verification:

- For single carrier REFSEN requirements, additional requirement for four Rx antenna ports and six Rx antenna ports shall be verified in operating bands;

- For Rx requirements other than single carrier REFSEN, the UE shall be verified with six Rx antenna ports and skip both two and four Rx antenna ports requirements in operating bands.

- References

- R4-2414285, “WF on requirements for 6Rx UE requirements”, AT&T, 3GPP TSG-RAN4 Meeting #112, Maastricht, Netherlands, August 19-23, 2024.