3GPP TSG-RAN WG4 Meeting #112bis                                      R4-2416186

Hefei, Anhui, China, October 14 - 18, 2024

Source: 	Huawei, HiSilicon

Title: 	On potentially new requirements for SBFD

Agenda Item:	6.20.2.1

Document for:	Approval

Introduction

During RAN4#112, potentially new requirements for SBFD have been discussed. Following conclusions and FFS points are reproduced from [1].

- Requirement for transient period between SBFD and non-SBFD - Agreement: - RAN4 shall use existing transient period requirement as baseline for the following four cases identified for SBFD: Frequency regions for Transition; BS transmitter behavior during transition. Case-A (SBFD to DL); SBFD UL subband and guardband(s); BS transmitter OFF-to-ON. Case-B (SBFD to UL); SBFD DL subband; BS transmitter ON-to-OFF. Case-C (DL to SBFD); SBFD UL subband and guardband(s); BS transmitter ON-to-OFF. Case-D (UL to SBFD); SBFD DL subband; BS transmitter OFF-to-ON. - Transient period length between SBFD and non-SBFD - Agreement: - Transient period length between SBFD and non-SBFD: - The existing TDD BS transmitter transient period reused, i.e., 10us for FR1 and 3us for FR2-1. - Location of transient period between SBFD and non-SBFD - Agreement: - Location of transient period between SBFD and non-SBFD in RAN4 RF requirement: - FFS the location of transient period between SBFD and non-SBFD shall be located within the SBFD slot/symbol if it needs to be decided. - Requirement for transient period between different SBFD configurations - Agreement: - No need to specify transmitter transient period requirement between different SBFD configurations. - Necessity of in-channel adjacent subband leakage ratio requirements - Way forward: - FFS the necessity of in-channel adjacent subband leakage ratio requirements - Option 1: No necessity to introduce new requirement for in-channel adjacent subband leakage ratio. - Option 2: New requirement for in-channel adjacent subband leakage ratio should be defined. - Define a requirement on TX sub-band ACLR similar to the ACLR requirement and use existing ACLR requirement as baseline. - Necessity of in-channel adjacent subband selectivity requirements - Way forward: - FFS the necessity of in-channel adjacent subband selectivity requirements - Option 1: No necessity to introduce new requirement for in-channel adjacent subband selectivity. - Option 2: New requirement for in-channel adjacent subband selectivity should be defined. - Define a requirement on RX sub-band ACS similar to the ACS requirement and use existing ACS requirement as baseline. - Necessity of in-channel adjacent subband blocking requirement - Way forward: - FFS the necessity of in-channel adjacent subband blocking requirements - Option 1: No necessity to introduce new requirement for in-channel adjacent subband blocking. - Option 2: New requirement for in-channel adjacent subband blocking should be defined.

In this contribution, we would like to share our views regarding this topic.

Discussion

The requirement for transient period for SBFD

In previous meeting, conclusion on this issue was achieved. While there will be transient period between SBFD and non-SBFD slot, it is no need to specify transmitter transient period requirement between different SBFD configurations. The remaining issue is to determine where the transient period will be. Our preference is to select SBFD slot to accommodate the transient period for the necessary RF resource rearrangement, so the TDD slot can be intact.

Proposal 1: The location of transient period between SBFD and non-SBFD shall be located within the SBFD slot/symbol.

In-channel adjacent subband requirement for SBFD

On one hand, we do share the understanding that in-channel adjacent subband requirement could be needed in order to guarantee certain performance level under self-interference. On the other hand, the potential solutions to overcome that would highly depend on vendors but very hard to be reflected in this new requirement since it must be based on specific implementation assumptions. Somehow, the REFSENS requirement for SBFD will reflect the expected suppression on the self-interference from feasible solutions.

Observation: For in-channel adjacent subband leakage ratio derivation, it is difficult for RAN4 to agree on a reference scheme for self-interference suppression. While the REFSENS for SBFD will reflect such suppression to some extent.

We also have similar feelings for other intra-operator gNB-gNB interference e.g. inter-site or co-site inter-sector cases. It could be hard to align companies understanding regarding power level, interference handling solutions or other issues that will also be involved not only in requirements derivation but also in test setup.

Proposal 2: Prefer not to introduce new requirement for in-channel adjacent subband leakage ratio.

Proposal 3: Prefer not to introduce new requirement for in-channel adjacent subband selectivity or blocking.

Conclusion

In this contribution we discussed on the general aspects for SBFD. We have the following observation and proposals:

Observation: For in-channel adjacent subband leakage ratio derivation, it is difficult for RAN4 to agree on a reference scheme for self-interference suppression. While the REFSENS for SBFD will reflect such suppression to some extent.

Proposal 1: The location of transient period between SBFD and non-SBFD shall be located within the SBFD slot/symbol.

Proposal 2: Prefer not to introduce new requirement for in-channel adjacent subband leakage ratio.

Proposal 3: Prefer not to introduce new requirement for in-channel adjacent subband selectivity or blocking.

- References

- R4-2413514, “Way Forward for [112][307] NR_duplex_evo_General”, Samsung, 3GPP TSG-RAN4 Meeting #112, Maastricht, Netherlands, August 19-23, 2024.