3GPP TSG-RAN WG4 Meeting #112bis	R4-2416190

Hefei, China, Oct 14 – Oct 18, 2024

Source:	Nokia

Title:	Discussing general RRM requirements for Rel-19 WI and LS Reply to RAN2

Agenda item:	6.26.4.2

Document for:	Discussion

Introduction

In [1], RAN plenary has approved a new WI for NTN, which will progress the effort of specification for NTN systems in 3GPP technologies in Rel-19 (NR_NTN_PH3). The most recent version of the WID can be found in [2]. It includes a handful of objectives such as:

- Downlink Coverage Enhancements

- Uplink Capacity/Coverage Enhancements

- Signaling of the intended service area of a broadcast service (MBS broadcast)

- Support of regenerative payload for NTN systems

- Support of Rel-17 and Rel-18 RedCap and (e)RedCap UEs in NTN systems

The discussions of this WI in RAN4 were initiated in the previous meeting RAN4 #112. And as the outcome of the discussion a way-forward (WF) document was approved which contains a set of agreements and other way-forward (for further discussion) guidance for currently identified open issues [3].

In this document we provide our views, related to the RAN4 impact of the aforementioned objectives considering the scope of this WI and/or the open issues already identified in the WF.

Discussion

This document presents discussion and proposals for NR_NTN_PH3 work item. From the 5 objectives above mentioned in the introduction, one of them (number 5) is covered in a different document, which belongs to another sub-agenda item. So the focus herein will be on the first four objectives listed in the introduction.

It seems a consensus in RAN4 that – at current stage of work reached by other working groups – objectives 2 and 3 above do not have RRM impacts so far.

So far, no RRM impact related to the objectives related to MBS broadcast or UL capacity enhancements were identified in this WI.

As a consequence, our discussions in this paper will focus on objectives 1 and 4.

Support of Regenerative payload for NTN systems

Although in the discussions presented in the previous meeting, most companies did not show any concerns with impacts on the RRM requirements caused by the regenerative payload architecture.  But still, some companies raised a few issues that should be investigated in order to analyze whether any impact is expected. In total 6 open issues were observed for this objective in the previous WF, with five of them still open.

Timing Requirements and RAN2 LS

RAN2 has initiated their work on Regenerative Payload potential impacts and in what regards potential impact on UE transmit timing or transmit timing requirements, the following agreements were made:

RAN2 #126 - RAN2 assumes that the typical setting for common TA and kmac would be zero in case of regenerative mode with full gNB on board. FFS whether we capture in the spec that common TA and Kmac shall/should be set to zero. RAN2 #127 - RAN2 confirms the understanding that legacy UEs should be supported in a regenerative payload scenario. - Send an LS to RAN1 and RAN4 asking whether in a regenerative payload scenario it would be a problem to stick to 0 as the minimum possible value for ta-Common or whether we should e.g. introduce negative values for ta-Common. Also indicate that in any case legacy UEs would have to rely on existing signalling and then on a minimum value equal to 0.

The aforementioned LS was presented to RAN4 in R2-2407623 [4]. And the following question was posed to RAN4:

Question: Whether in a regenerative payload scenario it would be a problem to stick to 0 as the minimum possible value for TA-Common or whether we should e.g. introduce negative values for ta-Common. Additionally, RAN2 understands in any case legacy UEs would have to rely on existing signaling and then for legacy UEs, minimum value of Common TA is equal to 0.

Similar discussions are also captured in the RAN4 WF (issue 5-2-1).

In our view, the UE shall be capable to meet the transmission requirements, i.e., to be within  Te_NTN of the uplink transmit timing, which is NTA+NTA-offset+NTA,adjcommon+NTA,adjUE×Tc before the downlink reference timing. These 4 components must be such that all UE transmitting in the system are time aligned at the UL reference point (usually the gNB or the satellite).  This means that the values of these components must be meaningful from the system perspective.

All transmitting UEs must be time aligned at the system synchronization reference point (usually the satellite or the gNb)

In the case of transparent payload NTA,adjcommon was introduced to allow the system to compensate for the feeder link delay. In the case of regenerative architecture, there is no need to compensate for the feeder link delay, such that if NTN specs had been developed for regenerative-only, most likely this component would not be present.

Therefore, we think that there is no harm in considering NTA,adjcommon=0 for the regenerative architecture.

Besides, RAN2 adds a piece of valuable information in their reply: “Additionally,  RAN2 understands in any case legacy UEs would have to rely on existing signaling and then for legacy UEs, minimum value of Common TA is equal to 0.”

From the system point of view, it is not a good practice that the reference timing for legacy and new UEs are different, as we registered in Observation 2, all UEs must be time aligned from the system point of view. We shall indicate that to RAN2.

Reply to RAN2 that “in regenerative payload scenario, there is no problem to stick to 0 as the minimum possible value for TA-Common”.

Include on the LS reply to RAN2 that: “the UL reference timing – from the receiver point of view – shall be the same for all UEs, and legacy and Rel-19 UEs shall have a common understanding of the common delay.

Mobility Related Requirements

Other issues in the previous WF [3] were addressing whether the following RRM requirements are impacted:

- RRC_Inactive state mobility

- RRC  Re-establishment

- RACH-less HO

- Satellite switching with same PCI

Regarding those RRM aspects the following progress was achieved in RAN2:

RAN2 #126 - Regarding potential issues with RRC re-establishment, RAN2 will wait for RAN3 input, if any - Regarding potential issues with support of Inactive state, RAN2 will wait for RAN3 input, if any - RAN2 aims at supporting RACH-less handover for regenerative architecture. Can further discuss whether any optimization is needed - RAN2 assumes that network verified UE positioning is supported for regenerative architecture RAN2 #127 - RAN2 understanding is that for regenerative payload existing RACH-less HO could be applied directly, no further Uu enhancement is needed. - We do not optimize UE power saving for SDT procedure in regenerative payload architecture

Therefore, it is clear that so far, the RAN2 progress has not altered or modified RRC Re-establishment, inactive state mobility nor RACH-less HO requirements. From RAN4 point of view, it seems that none of those requirements are impacted either by assuming a regenerative architecture.

Unless indicated otherwise by further RAN2/3 progress, none of the following RRM requirements needs to be modified: RRC re-establishment, RACH-less HO and Inactive State mobility.

Regarding satellite switch with same PCI, we believe that, being the gNB on board of the satellite in the case of regenerative architecture, there is no benefit in switching the satellite while maintaining the logical cell ID, as this would require tight coordination between satellites.

Do not support satellite switching with same PCI for regenerative architecture in Rel-19.

Downlink Coverage Enhancements

RAN1 has already made some progress in their discussions regarding the DL coverage enhancements, and there is some indication that the SSB periodicity might be extended, which might require some analysis from RAN4 to whether this modification will impact the system and the mobility delays observed in the network (Issues 2-1-1, 2-2-1 in the WF).

It is our understanding that the most recent RAN1 progress is indicated below(highlights are our own):

Agreement As part of the NTN DL coverage enhancements at both system level and link level, RAN1 to consider: - Extending the periodicity of the half frames with SS/PBCH blocks assumed by UE during initial access. - Default value[s] with extended periodicity assumed by NTN UE for initial access can be: - One [or more] values from the list {40ms, 80 ms, 160 ms, 320ms, 640ms} - Potential enhancements for transmitting the DL common channels using a wider beam footprint, while DL/UL dedicated channels (incl. PRACH) may be transmitted using a narrower beam footprint - Link-level enhancements for the following channels: - PDCCH - PDSCH with Msg 4 - PDSCH with SIB1/SIB19. - Note: link-level enhancements for PDSCH with SIB1/SIB19 may be applicable to other SIBs, without additional specification impact. - Note: the above does not imply that all the channels above will be enhanced, but all of them should be considered based on this agreement

It is our understanding that there is the intention to increase the SSB periodicity, but there is still no agreement on the values to be considered and values from 40 to 640 ms are being considered.   This is also confirmed in RAN1 reply LS to RAN2 presented in [5].

According to the updated WID (RP-241667), SSB enhancement other than SSB periodicity extension is not considered. Thereby, RAN1 understanding is that enhancements to the existing SSB patterns, e.g. SSB position in burst, SSB index number, are not within the scope. The extension of the SSB periodicity is still under RAN1’s discussion.

Therefore, it is premature for RAN4 to start detailed analysis on the impact of the enlarged SSB periodicity considering there is no agreement in these values.  It is a good practice, on the other hand, to perform a general analysis on potential impacts.

From our point of view, initial cell search is beyond the scope of RAN4 RRM, as there are no requirements for this procedure. Enhancements associated to cell search are defined in order to aid the UE to find a cell as soon as possible.

Cell search optimizations are beyond the scope of RRM in RAN4.

For cell re-selection and other neighbor cell measurements, most of the requirements were designed using TN specs as baseline, and they assume that the SSB periodicity could be already larger than 20 ms, because, in fact, for PSS and SSS detection the periodicity of the SMTC was considered the baseline, not the periodicity of the SSB.

For neighbor cell measurements, the periodicity of the SMTC (and not the periodicity of the SSB) is the baseline.

The periodicity of the SMTC could already be above 20 ms in NTN, which is exemplified for example by NOTE1 in Table 4.2C.2.3-1 in TS 38.133

Note 1: M2 = 2 if SMTC periodicity of measured intra-frequency cell > 20 ms and 1<NSMTC 4 upon more than 1 SMTC configured at the UE; M2 = 1.5 if SMTC periodicity of measured intra-frequency cell > 20 ms and NSMTC=1 upon 1 SMTC configured at the UE; otherwise M2=1. Where, NSMTC is the number of SMTCs configured by SAN If different SMTC periodicities are configured for different cells, the SMTC periodicity in this note is the one used by the cell being identified. During PSS/SSS detection, the periodicity of the SMTC configured for the intra-frequency carrier is assumed, and if the actual SSB transmission periodicity is greater than the SMTC configured for the intra-frequency carrier, longer Tdetect, NR_intra is expected.

For that matter, we think that current values of SMTC and SSB periodicity allowed in specification are already covered in NTN specifications (up to 160 ms). Therefore we think that RAN1 progress might impact current RRM specifications only in the configurations that were unforeseen (SMTC and SSB periodicity above 160 ms) in Rel-17 NTN or that were not fully properly well defined (SMTC periodicity smaller than SSB periodicity).

Consider impact on neighbor cell measurements only if RAN1 agrees on SMTC/SSB periodicity above 160 ms, or if the larger SMTC period allowed in specification becomes smaller than the larger SSB periodicity.

Beam switching related issues

- In the previous WF, the following open issue was identified:

Issue 2-4-1: Beam switching related issues <Way Forward> : Further discuss the following proposal - Proposal 1 (Ericsson): RAN4 to check if beam steering latency other than 0 may impact RRM requirements. - Proposal 2 (Ericsson): RAN4 to check if timing offset/error (between beam switching timing and DL/UL framework) may impact RRM requirements.

From our point of view, the beam steering latency is still under discussion in RAN1 and is not clear whether other beam steering latency (othern than 0 ms) is to be considered. We prefer to wait for more progress in RAN1 before advancing in this discussion.

Wait for further progress on RAN1 on beam steering configurations before analyzing the impact in RRM requirements.

Conclusion

In this paper we analyzed RAN4 impact of the NR_NTN_PH3 objectives considering the scope of this WI and/or the open issues already identified in the WF.

The following Observations and Proposals were made:

Observation 1: So far, no RRM impact related to the objectives related to MBS broadcast or UL capacity enhancements were identified in this WI.

Observation 2: All transmitting UEs must be time aligned at the system synchronization reference point (usually the satellite or the gNb)

Proposal 1: Reply to RAN2 that “in regenerative payload scenario, there is no problem to stick to 0 as the minimum possible value for TA-Common”.

Proposal 2: Include on the LS reply to RAN2 that: “the UL reference timing – from the receiver point of view – shall be the same for all UEs, and legacy and Rel-19 UEs shall have a common understanding of the common delay.

Proposal 3: Unless indicated otherwise by further RAN2/3 progress, none of the following RRM requirements needs to be modified: RRC re-establishment, RACH-less HO and Inactive State mobility.

Proposal 4: Do not support satellite switching with same PCI for regenerative architecture in Rel-19.

Observation 3: Cell search optimizations are beyond the scope of RRM in RAN4.

Observation 4: For neighbor cell measurements, the periodicity of the SMTC (and not the periodicity of the SSB) is the baseline.

Proposal 5: Consider impact on neighbor cell measurements only if RAN1 agrees on SMTC/SSB periodicity above 160 ms, or if the larger SMTC period allowed in specification becomes smaller than the larger SSB periodicity.

Proposal 6: Wait for further progress on RAN1 on beam steering configurations before analyzing the impact in RRM requirements.

- References

- RP-234078, New WID: Non-Terrestrial Networks (NTN) for NR Phase 3, Huawei

- RP-241789 Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3, Thales, CATT

- R4-2414037, "The WF on RRM requirements for Rel-19 NR NTN phase3", Moderator (CATT), RAN4 #112, Maastricht, Netherlands, Aug. 19-23, 2024.

- R2-2407623, “LS on common TA in a regenerative payload scenario”, China Mobile, Maastricht, Netherlands, 19th – 23rd August 2024

- R1-2407538, “Draft Reply LS on DL coverage enhancements”, China Mobile, RAN1 #118, Maastricht, The Netherlands, 19th – 23rd August, 2024