3GPP TSG RAN WG4 Meeting #112-bis		R4-2416210

Hefei, China, Oct.14th – 18th, 2024

Source: 	Nokia

Title: 	Test methodology for FR1 non-RedCap headworn XR devices

Agenda item:       6.12.2.1

Document for:     Discussion

# 1	Introduction

Based on the WF from RAN4 #112 meeting [1], in this paper, we’d like to share our views of XR devices OTA test related issues as listed below:

- Sub-topic 2-2 Testing time reduction for XR OTA Issue 2-2-1: Coarse measurement grids Agreements: The following Options can be further discussed next meeting: Option 1: Stop the measurement in the middle and charge the battery, carefully re-place the XR device into the Chamber to continue the test. Option 2: Test TRP at [X] dB below the maximum transmit power and scale up [X] dB to obtain TRP at maximum transmit power. Options 3: - Measure TRP: Averaging the measurement results from 4 of interpolated 62-points measurement grids by TRPave=10log⁡(10TRP1/10+10TRP2/10+10TRP3/10+10TRP4/104) TRSave=10log4/(110TRS110+110TRS210+110TRS310+110TRS410 to achieve the same accuracy level of using 266-points measurement grids during OTA tests. (Define 4 of 62-points interpolated measurement grids based on 266-points measurement grids at the same time, the locations of each four of 62-points interpolated measurement grids is different from each other on the sphere, they can be considered orthogonal to each other.) - Measure Peak EIPR: Full sphere coarse girds measurement + partial fine grids measurement (For example, using 62-points measurement grids in OTA test to measure peak EIPR first, then based on the direction of peak EIPR, apply corresponding sectors (pre-defined 8 or 16 sectors) of 266-points measurement grids to measure the peak EIPR again to increase the accuracy of measured peak EIRP)

# 2	Discussion

2.1	Coarse measurement grids

The issue 2-2-1 was introduced due to that the XR device may not have big battery that can last for one complete OTA test. Therefore, three option are listed for discussion.

Option 1 is the straightforward choice if the battery is not enough for one complete OTA test. However, this is the issue itself, not a solution. Therefore, option2 and option3 are suggested.

Option 2 means the output power of XR device does not need to be maximum output power (MOP) during the whole OTA test. For the most of measurement grid, the XR output power is much lower than the MOP. After the whole OTA test, scale up the power to MOP accordingly by a scaler. The scaler for MOP power and real measurement power can be determined based on a pre-measurement.

However, the potential issue is that the PA performance will be impacted by temperature due to its non-linear nature if the temperature of PA will be different between working at MOP and not working at MOP. Therefore, using scaler to scale the real measurement power after OTA measurement to MOP will not be accurate. One may argue that to use UE’s calibration date of the PA to reduce the scaling inaccuracy after OTA test. However, the real practice of using UE’s calibration data for OTA test maybe difficult due to design security reason or other similar reasons since UE should not scale the power to MOP by itself without letting TE know what scaler is before the OTA test.

Regarding option3, it means to use 4 or 8 interpolated measurement grids to reduce the test time of each OTA test. Then, 4 or 8 of shorter-time OTA tests can be combined as one full OTA test. In the gap of each shorter-time OTA test, XR device can charge/change battery. However, this may introduce some errors caused by human, though this method can ignore the measurement grids mis-alignment issue compared with option 1 (stop the OTA test in the middle to change/charge battery).

Observation 1: There are both pros and cons of those options, it is difficult to judgement which one is better.

Proposal 1: Both the option 2 and option 3 can be applied depends on TE vendor.

# 3	Conclusions

In our contribution, related uplink 3Tx topics are discussed. Here is the list of our proposals:

Observation 1: There are both pros and cons of those two options, it is difficult to judgement which one is better.

Proposal 1: Both the option 2 and option 3 can be applied depends on TE vendor.

# References

- R4-2413535, WF for [112][333] TRP_TRS_MIMO_OTA.