3GPP TSG-RAN WG4 Meeting #112-bis                                                    R4-2416211

Hefei, China, 14th – 18th Otc., 2024

Source:		Nokia

Title:	Rel-19 FR1 OTA requirements

Agenda item:		6.12.2

Document for:	Discussion

# Introduction

Based on the WF from RAN4 #112 meeting [1], in this paper, we’d like to share our views of Rel-19 FR1 OTA requirements related issues as listed below:

Topic #5: Rel-19 FR1 OTA requirements - Sub-topic 5-1 FR1 TRP TRS requirements Issue 5-1-1: 1Tx requirements for TRP TRS Agreements: - No need to perform further lab alignment testing in Rel-19. Issue 5-1-2: 2Tx requirements for TRP TRS Agreements: - AC lab alignment for non-coherent UL MIMO at n41/n78 should be done in Rel-19, the framework can be similar to Rel-18, encourage test labs to join this activity. - Detailed framework will be provided next RAN4 meeting, also collect volunteer test labs feedback. Issue 5-1-3: Size 2 TRP TRS requirements (PDA hand) Agreements: - RAN4 further discuss how to specify size 2 UE TRP TRS requirements.

# Discussion

2.1	Size 2 TRP TRS requirements (PDA hand)

If the UE size is larger than 72mm, which is also called size 2 UE, then the performance should be better than those of the UE’s size less than 72mm in general.

There are two cases for the UE size larger than 72mm:

(1) Low-end UEs only equip one main antenna radiator for normal usage.

(2) high-end UEs equip multi-antenna radiators to receiving multiple data stream for high throughput.

In case (1): If the PCB size of the UE becomes larger, the antenna can be easier to design, and the surroundings that impact the radiation of the antenna patch inside UE can be moved further away. The radiation of the larger size of UE should be improved, so as the demodulation performance.

In case (2): If the PCB size of the UE becomes larger, there will be more space to put multiple antennas inside of the UE to achieve better radiation performance. Except of designing each antenna becoming easier, the positions of the multiple antennas can be optimized, therefore, the isolation between multiple antennas working at the same frequency can be improved. The benefit of using multiple antennas in terms of diversity can be better reflected.

There is one more impact during OTA test with phantom, the antenna radiation may be less impacted by the hand and head phantom, i.e., better radiation of the antenna in size 2 UEs, due to the antenna may be further away from the phantom than the antenna of a size 1 UE. Therefore, the performance of the single/multiple antenna(s) can be better, the overall performance during OTA test with phantom can be better.

Based on the above-mentioned reasons, it is possible to tighten the UE TRP/TIS specifications for size 2 UEs in the OTA tests. Regarding the quiet zone TE created for OTA test, if the current quiet zone is big enough to cover the size 2 UEs, then, TE vendor does not need to make changes immediately. However, TE vendor may need to verify if the current quiet zone can fully support OTA test of size 2 UEs without degradation.

Observation 1: If the UE size is larger than 72mm, which is also called size 2 UE, then, the performance should be better than those of the UE’s size less than 72mm in general both with and without phantom during the OTA test.

Proposal 2: Consider tightening the UE TRP/TIS specifications for size 2 UEs in the OTA tests.

# Conclusions

In this contribution, we make the following observations and conclusions:

Observation 1: If the UE size is larger than 72mm, which is also called size 2 UE, then, the performance should be better than those of the UE’s size less than 72mm in general both with and without phantom during the OTA test.

Proposal 2: Consider tightening the UE RF/Demod specifications for size 2 UEs in the OTA tests.

# References

[1]	R4-2413535, WF for [112][333] TRP_TRS_MIMO_OTA.