3GPP TSG RAN WG4 Meeting #112-bis		R4-2416213

Hefei, China, Oct.14th – 18th, 2024

Source: 	Nokia

Title: 	UE RF requirements (MIMO Phase5)

Agenda item:       6.19.2

Document for:     Discussion

# 1	Introduction

NR MIMO Phase 5 in Rel-19 has been revised in the last RAN plenary meeting [1]. Some of the topics from the WF in last RAN4 meeting may need corresponding changes. In this paper, we’d like to share our views for 3Tx:

Objectives based on revised WID: - Specify non-coherent UL codebook to facilitate 3-antenna-port codebook-based transmissions, enhancement(s) to enable 3T6R SRS antenna switching, as well as UE capability signaling for 3T3R antenna switching and 3-antenna-port non-codebook-based transmissions, without enhancement on UL full power transmission and without enhancement on SRS resource Note: UL full power transmission mode 1 and 2 are not supported. Note: Other than UE capability signaling, no other enhancement is specified for 3T3R SRS antenna switching. - Specify enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios, assuming intra-band intra-DU non-co-located mTRP scenarios, without changing existing cell definition or defining a new cell (e.g. UL-only cell), assuming the Rel-17/18 unified TCI framework and fully reusing the legacy QCL/UL spatial relation rules, targeting FR1 and FR2 - Two closed-loop PC adjustment states for SRS, both separate from PUSCH; and pathloss offset configurations for pathloss calculation to UL TRP(s), when the pathloss RS is from DL sTRP. - Two TAs through reusing Rel-18 specification of two TAs for multi-DCI-based multi-TRP and removing the restriction that coresetPoolIndex needs to be configured, assuming legacy PRACH resources

RAN4 WF from #112 meeting: <Topic 1> UE RF impact Issue 1-1: UE RF work plan approval Chair: encourage companies to follow the work plan in R4-2412134. Issue 1-2: UE RF impact of 3Tx - Proposals - Option 1: Identified. Start with 3Tx - Option 2: Not yet. Wait for further progress in RAN1 - Agreement - RAN4 will analyse the impact of 3Tx on UE RF requirements. Issue 1-3: UE RF impact of asymmetric DL sTRP/UL mTRP - Proposals - Option 1: Identified (e.g., spherical coverage with a different TCI state for UL compared to DL) - Option 2: Not yet. Wait for further progress in RAN1 - Way forward - Companies are encouraged to provide clear view based on WID and RAN1 status <Topic 2> 3Tx Issue 2-1: Applicable UE types - Proposals - Option 1: Handheld UE included - Option 2: Non-handheld UE only - Option 3: Depend on condition (e.g., ULFPTx) - Agreement - Both handheld and non-handheld UEs will be considered for UE RF requirements - FFS on whether a single set or two sets of UE RF requirements, e.g., MPR, will be specified. Issue 2-2: Applicable Frequency Ranges - Proposals - Option 1: FR1 only - Option 2: Others - Agreement - Both FR1 and FR2 will be considered for potential UL mTRP requirements - Only FR1 is considered for 3Tx objective Issue 2-3: Power classes and PA configuration for 3Tx UL MIMO - Proposals - Option 1: PC3 and PC2 - Option 2: PC1.5 only - Option 3: Others - Way forward - Companies are encouraged to provide clear view on 3Tx power class and its PA assumptions Issue 2-5: Non ULFPTx mode - Proposals - Option 1: 3-layer with precoding matrix of W=13100010001. - Option 2: Others - Agreement - 3-layer with precoding matrix of W=13100010001. Issue 2-6: ULFPTx mode 0 - Proposals - Option 1: Add both 1-layer and 2-layer for 3Tx with - Option 2: Add 1-layer for 3Tx with W=13100,13010,13001 - Tentative agreement - Take 1-layer as baseline - Further discuss whether 2-layer is needed <Topic 3> Asymmetric DL sTRP/UL mTRP Issue 3-1: FR2 UL enhancement scenario - FFS the following proposal (companies to check): - Option 1: UL TCI-state differs from DL TCI-state - Other options not precluded

# 2	Discussion

2.1	Discussion for 3Tx

Based on the revised WID for NR MIMO Phase 5, the 3Tx feature in Rel-19 is clarified. In the notes of objective 4, it clarified that UL full power transmission mode 1 and 2 are not supported. Therefore, the Issue 2-5 is clear out of scope.

Based on the current RF Architecture Review for ULFPTx mode 0 in 38.837 Ch 5.3.3, for the feature of 3Tx, if mode 0 should be supported, then, all the 3Tx chain should be able to achieve full power. The corresponding codebook to support 3Tx mode 0 can be developed, and the 3 layers can be supported according to the previous agreements from RAN1. In order to let network to configure UE to use 3Tx, the new UE capability of 3Tx should also be introduced.

Observation 1: In the revised WID, ULFPTx Mode 1 and mode 2 are not supported.

Proposal 1: ULFPTx  mode 1 and mode 2 are out of scope and the focus should be ULFPTx mode 0 only.

Proposal 2: All the Tx chain for 3Tx feature should achieve full power based on the revised WID that only ULFPTx mode 0 is supported.

Proposal 3: New UE capability to indicate UE support 3Tx should be introduced.

The feature of 3Tx mode 0 means that all three Tx chains should be able to achieve full power. For PC1.5/2 UE, the power supply is not a big issue compared with PC3 handheld UE devices. Therefore, the 3Tx ULFPTx mode 0 should be supported by PC1.5 and PC2 devices. However, this does not preclude that PC3 devices can support 3Tx.

Observation 2: The power supply to support ULFPTx mode 0 for PC3 devices maybe an issue compared with PC1.5 and PC2 devices. However, PC3 devices should not be precluded.

Proposal 4: The 3Tx ULFPTx mode 0 should be supported by PC1.5 and PC2 devices, but PC3 devices does not precluded.

Based on R4-2412135 from Samsung [3], the existing MPR gap between 2 Tx and 4Tx for PC1.5 is up to 1.5 dB gap for FWA devices as an example, it is possible to define new MPR requirement for 3Tx if necessary. However, it is reasonable for 3Tx to reuse existing MPR tables for 2Tx/4Tx as much as possible.

Observation 3: The MPR gap can be observed based on the current MPR tables defined for 2Tx and 4Tx for different power class UEs.

Proposal 5: It is possible to define new MPR requirement for 3Tx if necessary. However, it is reasonable to check if the current 2Tx/4Tx MPR tables can be reused for 3Tx.

2.2	Discussion for UL mTRP

The QCL information is conveyed by TCI state, UE will use the QCL type indicated by the network to configure the UL antenna ports and DL antenna ports of the UE. For transmitting the UL signals and receiving DL signals, UE does not necessarily need to use the same antenna ports in FR1 for adjusting itself in various channel scenarios. For example, 2 antennas for DL and one antenna for UL. Therefore, the UL and DL TCI states in FR1 can be different.

In FR2, QCL-D is mainly used for the purpose of beamforming, and UE antenna panel normally consists of a few antenna elements, for example, 8x1 antenna array. In DL, all the 8 antenna elements can be used at the same time to form a narrow beam to receive the signal from one TRP. In UL, the antenna elements can be split into 2 of 4x1 antenna arrays if UE Tx chain setup can support such configuration, then, UE can send UL signals to two different TRP, i.e., mTRP. Therefore, the UL TCI state and DL TCI state can be different as well.

Observation 4: The UL TCI state and DL TCI state can be different in terms of QCL type for configuring the antenna port of UE in both FR1 and FR2.

In the RAN1 #117 Fukuoka, it is agreed that for the asymmetric DL sTRP/UL mTRP deployment scenario, to reuse the rel-17 unified TCI/ICBM and rel-18 unified TCI framework:

- When rel-17 unified TCI/ICBM is configured:

  - For FR1: one joint TCI state or {one DL TCI state + one UL TCI state} can be applied.

  - For FR2: {one DL TCI state + one UL TCI state} can be applied.

- When rel-18 unified TCI is configured:

  - For FR1: up to two joint TCI states or {one DL TCI state + up to two UL TCI state} can be applied.

  - For FR2: {one DL TCI state + up to two UL TCI states} can be applied.

The unified TCI framework in Rel-17 and Rel-18 provide quite a few choices of how to flexibly configure the UL and DL TCI states in case they are different.

Observation 5: The choice of configuring UL and DL TCI-states based on Rel-17/18 unified TCI framework is quite flexible.

Proposal 6: The UL TCI state and DL TCI state can be different.

# 3	Conclusions

In our contribution, related uplink 3Tx topics are discussed. Here is the list of our proposals:

Observation 1: In the revised WID, ULFPTx Mode 1 and mode 2 are not supported.

Proposal 1: ULFPTx  mode 1 and mode 2 are out of scope and the focus should be ULFPTx mode 0 only.

Proposal 2: All the Tx chain for 3Tx feature should achieve full power based on the revised WID that only ULFPTx mode 0 is supported.

Proposal 3: New UE capability to indicate UE support 3Tx should be introduced.

Observation 2: The power supply to support ULFPTx mode 0 for PC3 devices maybe an issue compared with PC1.5 and PC2 devices. However, PC3 devices should not be precluded.

Proposal 4: The 3Tx ULFPTx mode 0 should be supported by PC1.5 and PC2 devices, but PC3 devices does not precluded.

Observation 3: The MPR gap can be observed based on the current MPR tables defined for 2Tx and 4Tx for different power class UEs.

Proposal 5: It is possible to define new MPR requirement for 3Tx if necessary. However, it is reasonable to check if the current 2Tx/4Tx MPR tables can be reused for 3Tx.

Observation 4: The UL TCI state and DL TCI state can be different in terms of QCL type for configuring the antenna port of UE in both FR1 and FR2.

Observation 5: The choice of configuring UL and DL TCI-states based on Rel-17/18 unified TCI framework is quite flexible.

Proposal 6: The UL TCI state and DL TCI state can be different.

# References

- RP-242394, “WID Revision: NR MIMO Phase 5,” RAN#105, Melbourne, Australia, September 9-12, 2024

- R4-2414310, WF on UE RF requirements for Rel-19 MIMO enhancement.

- R4-2412135, Views on UE RF impact of NR MIMO Phase 5, Samsung.