# 3GPP TSG-RAN WG4 Meeting #112bis 　　　　　　　　　　　　　　　　　	R4-2416226

Hefei, Anhui, China, 14th  – 18th October, 2024

Source: 	NTT DOCOMO INC.

Title:	Discussion on BS indication for MPR reduction

Agenda item:	6.1.1.2.1

Document for:	Discussion

- Introduction

In Rel-19, in the WI “UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4”[1], MPR reduction is included as one of the objectives. RAN4 has discussed approaches to enable MPR reduction, but the detail of the BS indication have not yet been agreed in the WF[2]. This paper provides our views on the signaling aspects.

- Discussion

Background:

According to the WID for this Rel-19 WI, ACLR/SEM/SE can be relaxed under particular conditions with BS indication as follows[1]:

Objective in [1] Power boosting and/or MPR reduction - Specify power domain enhancement, e.g., MPR reduction for NR single carrier and NR intra-band UL CA - Study the scenarios, and if feasible, specify the power domain enhancement, e.g., MPR reduction, for PC2 and PC3 with applicable ACLR/SEM/spurious emission modification with BS indication for NR FR1 on a single UL carrier - Include the following scenarios: - when there is no adjacent in-band/out-of-band co-existence issue - when a UE uses a narrower channel bandwidth within a wider BS bandwidth - Include both (e)RedCap UE (only PC3) and non-RedCap UE - Limited to QSPK and 16QAM

However, the details of the BS indication have not yet been agreed upon[2].

#### Issue 1-2-11: Signaling aspects - Proposals - Proposal 1: no changes of signalling are specified for the feature ‘narrower UE channel BW within wider BS bandwidth’ except possibly specification of a capability bit to indicate support of the feature. (Ericsson) - Proposal 2: Need to consider capability for extended UE CBW procedure. (LGE) - Proposal 3: If any reduction of MPR would be specified in the end, it should be an optional feature for UE with per band capability. (Sony) - Proposal 4: The frequency intervals for the UE CBW and the BS CBW should be judged by the NW and the indication of MPR reduction could be directly sent to the UE when the corresponding conditions are met. (vivo) - Proposal 5: Establish a start point and continue to details for BS indication with the deepen in MPR reduction mechanism. (CTC) - WF - To discuss the signalling aspects after sufficient evaluation of power boosting and/or MPR reduction in terms of relaxed requirements.

Discussion:

While the approved WF mentioned that the discussion on signalling aspects should be after sufficient evaluation of the work, there seemed some confusion among companies in last meeting regarding how to control this feature, especially for region where relaxation on emission requirement is not allowed, and thus let us provide our understanding for clarification.

According to the WID[2], the applicability of the ACLR/SEM/spurious emission modification is determined by BS indication. It means that this new feature can be under NW control. In other words, UE can enable this feature after UE receives related BS indication from NW. Therefore, even if the regulation in some regions does not allow relevant emission requirement relaxation, there is no problem as long as the NW does not give the indication. This aspect should be clarified to avoid any misunderstanding.

Observation 1: BS indication is needed before UE enable the MPR reduction by relaxing emission requirements according to the description in objective in the WID.

In addition to the principle mentioned above, if the BS indication signals a uniform modification for ACLR/SEM/spurious emission, in our view, the conditions under which this feature can be enabled are limited.

In light of the regulations, since the interference requirement influences the coexistence considerations with surrounding systems, its legislation varies by region. Therefore, uniform modifications based on BS indication can only be enabled in regions where all modifications are accepted. Furthermore, future legal revisions may prevent this function from being activated.

Additionally, the enabled interference requirement modifications depend on the specific scenario. For example, an ACLR modification might be allowed under one BS but not under another.

Observation 2: BS indication that determines a uniform modification of ACLR/SEM/spurious emission may limit the situations in which this feature can be enabled due to variations in regulations and scenarios.

Proposal: BS indication should be designed to allow selection of which ACLR/SEM/spurious emission can be relaxed to fully enable this feature.

- Conclusion

Here we summarize our proposals:

Observation 1: BS indication is needed before UE enable the MPR reduction by relaxing emission requirements according to the description in objective in the WID.

Observation 2: BS indication that determines a uniform modification of ACLR/SEM/spurious emission may limit the situations in which this feature can be enabled due to variations in regulations and scenarios.

Proposal: BS indication should be designed to allow selection of which ACLR/SEM/spurious emission can be relaxed to fully enable this feature.

4	References

[1] RP-241656, “Revised WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4”, RAN4 chair (Huawei)

[2] R4-2414441, “WF on power domain enhancements”, Huawei, HiSilicon